EDITORS NOTE:  Table of contents tables will be removed from individual clauses.  The TOC below is for draft review purposes only.

8	Spatial reference frames	117

8.1	Introduction	117

8.2	Spatial coordinate systems	117

8.3	Spatial reference frame	118

8.3.1	Specification	118

8.3.2	SRF specification elements	118

8.4	SRF induced surface reference frame	120

8.5	SRF templates	121

8.5.1	Introduction	121

8.5.2	Celestiocentric SRF	122

8.5.3	Local space rectangular 3D SRF	123

8.5.4	Local space rectangular 2D SRF	124

8.5.5	Celestiodetic SRF	125

8.5.6	Local tangent plane 3D SRF	126

8.5.7	Local azimuthal spherical tangent plane SRF	127

8.5.8	Surface local azimuthal tangent plane SRF	128

8.5.9	Azimuthal 2D SRF	129

8.5.10	Local cylindrical tangent plane SRF	130

8.5.11	Polar 2D SRF	131

8.5.12	Celestiomagnetic SRF	132

8.5.13	Equatorial inertial SRF	132

8.5.14	Solar ecliptic SRF	133

8.5.15	Solar equatorial SRF	133

8.5.16	Solar magnetospheric SRF	134

8.5.17	Solar magnetic SRF	134

8.5.18	Heliospheric Aries ecliptic SRF	135

8.5.19	Heliospheric Earth ecliptic SRF	135

8.5.20	Heliospheric Earth equatorial SRF	136

8.5.21	Mercator SRF	136

8.5.22	Oblique Mercator SRF	137

8.5.23	Transverse Mercator SRF	138

8.5.24	Lambert conformal conic SRF	139

8.5.25	Polar stereographic SRF	140

8.5.26	Equidistant cylindrical SRF	141

8.6	SRFs	141

8.7	SRF sets	146

Figure 8.1 — A spatial embedding of a surface CS	117

Table 8.1 — Compatible CS types	118

Table 8.2 — SRFT specification elements	121

Table 8.3 — SRFT directory	122

Table 8.4 — Celestiocentric SRFT	123

Table 8.5 — Local space rectangular 3D SRFT	123

Table 8.6 — Local space rectangular 2D SRF template	124

Table 8.7 — Celestiodetic SRFT	125

Table 8.8 — Local tangent plane 3D SRFT	126

Table 8.9 — Local azimuthal spherical tangent plane SRFT	127

Table 8.10 — Surface local azimuthal tangent plane SRFT	128

Table 8.11 — Azimuthal 2D SRFT	129

Table 8.12 — Local cylindrical tangent plane SRFT	130

Table 8.13 — Polar 2D SRFT	131

Table 8.14 — Celestiomagnetic SRFT	132

Table 8.15 — Equatorial inertial SRFT	132

Table 8.16 — Solar ecliptic SRFT	133

Table 8.17 — Solar equatorial SRFT	133

Table 8.18 — Solar magnetospheric SRFT	134

Table 8.19 — Solar magnetic SRFT	135

Table 8.20 — Heliospheric Aries ecliptic SRFT	135

Table 8.21 — Heliospheric Earth ecliptic SRFT	136

Table 8.22 — Heliospheric Earth equatorial SRFT	136

Table 8.23 — Mercator SRFT	137

Table 8.24 — Oblique Mercator SRFT	137

Table 8.25 — Transverse Mercator SRFT	138

Table 8.26 — Lambert conformal conic SRFT	139

Table 8.27 — Polar stereographic SRFT	140

Table 8.28 — Equidistant cylindrical SRFT	141

Table 8.29 — SRF specification elements	141

Table 8.30 — SRFs	142

Table 8.31 — SRF set specification elements	146

Table 8.32 — SRF sets	146

Table 8.33 — SRF set member specification elements	149

Table 8.34 — Explicit specifications of SRF set members	149

Table 8.35 — Implicit specifications of SRF set members	151

Table 8.36 — GTRS indexing scheme and set members directory	153

Table 8.37 — GTRS indexing scheme and set members: 88º-90º north	153

Table 8.38 — GTRS indexing scheme and set members: 86º-88º north	153

Table 8.39 — GTRS indexing scheme and set members: 84º-86º north	154

Table 8.40 — GTRS indexing scheme and set members: 80º-84º north	154

Table 8.41 — GTRS indexing scheme and set members: 78º-80º north	154

Table 8.42 — GTRS indexing scheme and set members: 71º-78º north	154

Table 8.43 — GTRS indexing scheme and set members: 60º-71º north	155

Table 8.44 — GTRS indexing scheme and set members: 60º south - 60º north	155

Table 8.45 — GTRS indexing scheme and set members: 71º-60º south	158

Table 8.46 — GTRS indexing scheme and set members: 78º-71º south	159

Table 8.47 — GTRS indexing scheme and set members: 80º-78º south	159

Table 8.48 — GTRS indexing scheme and set members: 84º-80º south	159

Table 8.49 — GTRS indexing scheme and set members: 86º-84º south	159

Table 8.50 — GTRS indexing scheme and set members: 88º-86º south	160

Table 8.51 — GTRS indexing scheme and set members: 90º-88º south	160

Table 8.52 — MTM (body) indexing scheme and set members: northern hemisphere	160

Table 8.53 — MTM (body) indexing scheme and set members: southern hemisphere	160

Table 8.54 — MTM (pole) indexing scheme and set members: northern hemisphere	160

Table 8.55 — MTM (pole) indexing scheme and set members: southern hemisphere	161

Table 8.56 — UPS indexing scheme and set members: northern hemisphere	161

Table 8.57 — UPS indexing scheme and set members: southern hemisphere	161

Table 8.58 — UTM indexing scheme and set members: northern hemisphere	161

Table 8.59 — UTM indexing scheme and set members: southern hemisphere	162

# Spatial reference frames

## Introduction

A spatial reference frame is a spatial coordinate system for a region of the space of an object formed by the binding of an abstract coordinate system to the linear embedding specified by an ORM. A full specification includes values for the CS parameters and a specification of the region of the space of the object. In some cases, some the CS parameters are bound by ORM parameters. In particular, a CS based on an oblate spheroid (or sphere) must match the parameters of the oblate spheroid (or sphere) RD of the ORM.

A spatial reference frame template is the basis for realizing spatial reference frames that share the same CS, similar ORMs, and the same structure in the binding of CS parameter values. Spatial reference frames may be organized into specified sets so as to form an atlas for a large region of space. This International Standard specifies a collection of spatial reference frame templates, realizations of those templates, and sets of those realizations.

## Spatial coordinate systems

Once an embedding of Rm is established, any N-dimensional CS for Rm (m = 1, 2, or 3) specifies an N-dimensional spatial CS that associates coordinates in coordinate space to points in object-space. This association is a binding of a CS via a spatial embedding. This is illustrated in Figure 8.1 that illustrates a spatial surface CS bound with an embedding of R3 to the 3D space of an object.  In this illustration, a surface coordinate (u, v) in coordinate-space is associated to a position (x, y, z) in the abstract position-space R3.  That position is then identified with a position in the space of an object via the embedding of R3 determined by the selection of an origin and three unit points.

Figure 8.1 — A spatial embedding of a surface CS

## Spatial reference frame

### Specification

A spatial reference frame (SRF) is a specification of an ORM together with a compatible spatial CS, where coordinates uniquely specify a position with respect to the spatial object for which that ORM was specified. A specification of an SRF includes:

- an ORM,

- a spatial CS compatible with the ORM,

- a binding of all parameters of the spatial CS,

- (optionally) kth–coordinate names, and

- (optionally) additional restrictions on the domain of valid coordinates in that spatial CS.

Spatial CS compatibility and the other elements of the specification of an SRF are defined in the following clauses.

### SRF specification elements

#### ORM and CS compatibility

The compatible CS type of the CS element of an SRF depends on the dimension of the ORM. The dimension of an ORM is defined as the dimension of the RD specification of the ORM. The compatible CS types by ORM dimension are specified in Table 8.1.

Table 8.1 — Compatible CS types

| ORM dimension | Compatible CS types |
| --- | --- |
| 1D | 1D CS |
| 2D | Curve CS 2D CS |
| 3D | Curve CS Surface CS 3D CS |

The use of surface CSs or 3D CSs that are based on an oblate spheroid (or sphere) are further restricted to ORMs based on an oblate spheroid (or respectively, sphere) RD.

The surface CSs that are based on an oblate spheroid (or sphere) are:

- surface geodetic, and

- all map projections.

The 3D CSs that are based on an oblate spheroid (or sphere) are:

- geodetic 3D, and

- all augmented map projections.

As a further restriction, some map projections are based on spheres only. These include:

- CS_MP_OBLIQUE_MERCATOR, and

- CS_MP_EQUIDISTANT_CYLINDRICAL.

#### CS parameter binding

All CS parameter values must be specified. In the case of a combination of a CS and an ORM based on an oblate spheroid (or sphere), the major semi-axis and minor semi-axis (or equivalently, the inverse flattening) (or respectively, sphere radius) of the ORM and CS shall match.

#### Coordinate names

A CS specification (see 5.4) includes the coordinate symbols with common names (if any). A specification of an SRF may optionally assign SRF-specific names to the kth-coordinates. The name assignment shall reflect the common use in the intended application domain.

EXAMPLE 1	For a spherical CS, the assignment of SRF-specific names to the kth-coordinates of “right ascension”  for , “declination” for  , and “radius” for .

#### Coordinate valid region

A CS specification (see 5.4) includes the specification of the domain and range where the generating function (or mapping equations) and its inverse(s) are defined. An SRF specification may further restrict the domain. A valid region is a restriction of the domain of the generating function (or mapping equations) for a CS as used in a SRF. An extended valid region is a second valid region that contains the first valid region as a subset. The specification of these restrictions is important for several (SRF specific) reasons:

- If the ORM is local, the restrictions are used to model, in coordinate-space, the local region of the space of the object.

- If the CS is a map projection or an augmented map projection, the restrictions are used to bound or otherwise limit distortions (see 5.3.3.1).

- The SRF may be used in conjunction with other SRFs to form an atlas for a large region. In this case, the restrictions are used to control the pair-wise overlap of the spatial coverage of members of the SRF collection.

- If the CS generating function (or map projection mapping equations) or the inverse function(s) have been implemented with a numerical approximation, the restrictions are used to control error bounds.

- Any combination of the reasons above.

The extended valid region is used primarily for overlapping regions in forming an atlas as in (c) above. Not all properties of the SRF that are true in the valid region will necessarily be true in the extended valid region. For example, a distortion error bound that holds in the valid region may not hold in the extended valid region.

A valid region may be described and/or specified. A valid region description is a descriptive statement of the region such as the spatial boundary of a named political entity.

EXAMPLE 2	“The German state of Baden-Wurttemberg.” and “The Baltic Sea.” are valid region descriptions.

In this International Standard, a valid region specification is a finite (or empty) list of coordinate constraints of the form:

kth-coordinate belongs to a non-empty interval of real numbers .

An extended valid region specification is a finite (or empty) list of coordinate constraints of the form:

kth-coordinate belongs to an interval of real numbers , where  has been specified and .

In the case of an SRF with an oblate spheroid (or sphere) based ORM, celestiodetic coordinates may be similarly constrained. In particular, a valid region specification for a map projection based SRF may specify coordinate constraints for easting, northing, latitude, and/or longitude.

EXAMPLE 3	The SRF is based on a Mercator map projection (see SRFT_ MERCATOR).

Valid region specification:			0 ≤ easting ≤ 10 000 000,  	0 ≤ northing ≤ 500 000
Extended valid region specification:	 -100 < easting,  			-100 < northing

In this example and are closed bounded intervals, and and are open semi-bounded intervals.

EXAMPLE 4	The SRF is based on a transverse Mercator map projection (see SRFT_TRANSVERSE_MERCATOR).

Valid region specification:			-78º ≤ longitude < -72º,		0º ≤ latitude < 84º
Extended valid region specification:	-78,5º ≤ longitude < -71,5º

In this example and are left-closed, right-open bounded intervals, as is . is not specified. This indicates that there are no constraints for latitude (except for the CS domain definition) in the extended valid region.

## SRF induced surface reference frame

In the case of an SRF specified with the combination of a 3D ORM and a 3D CS, the 3D CS induces a surface CS on each coordinate surface (see 5.2.4.2). If the 3D CS has a coordinate that can be identified as a vertical coordinate for the SRF, then the coordinate surface for the zero value of the vertical coordinate (if defined) is of special interest and the surface CS induced on that coordinate surface specifies an SRF surface CS.

The CS_3D_GEODETIC 3rd-coordinate (h: ellipsoidal height), and the 3rd-coordinate of any augmented map projection CS (h: ellipsoidal height) are identified in this International Standard as a vertical coordinate. When an SRF is specified with any of these 3D CSs, the h = 0 coordinate surface coincides with the surface of the oblate spheroid (or sphere) RD of the ORM. Any SRF based on these CSs intrinsically specifies the corresponding surface CS on the oblate spheroid (or sphere) RD surface.

In the case of other 3D CSs, a vertical coordinate may be specified in the coordinate name specification element of the SRF. In particular, the SRFT_3D_LOCAL_TANGENT_PLANE and in the SRFT_3D_LOCAL_CYLINDRICAL_TANGENT_PLANE the 3rd-coordinate is specified as the vertical coordinate.

SRF templates that are based on surface CSs that can be induced by a zero value vertical coordinate surface of an SRF based on a 3D CS are not separately specified. The induced surface CS is noted in the corresponding 3D CS based SRF template specification.

The zero value 3rd-coordinate surface of the 3D CS of the SRFT_LOCAL_AZIMUTHAL_SPHERICAL_TANGENT_PLANE induces the surface CS of the SRFT_SURFACE_LOCAL_AZIMUTHAL_TANGENT_PLANE. However, since that 3rd-coordinate is not specified as a vertical coordinate, the SRFT_SURFACE_LOCAL_AZIMUTHAL_TANGENT_PLANE has a separate specification.

## SRF templates

### Introduction

An SRF template (SRFT) is an abstraction of a set of specifications of SRFs that share the same CS and coordinate names, whose ORM components are similar, that model the same type of spatial object, and whose CS share the same binding rules. An SRF template allows for a consistent derivation of SRFs. It is not necessary that an appropriate SRFT be defined in order to define a new SRF; however in this International Standard all SRFs are derived from SRFTs.

The specification elements for SRFTs are defined in Table 8.2.

Table 8.2 — SRFT specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Description | A description including name as published or as commonly known. |
| Object type | One of: planet, satellite, star, non-celestial, abstract, Earth, and Sun. The relationships among these object types are specified in Figure 4.1. |
| ORM constraint | Criteria for allowable ORMs. |
| CS label | The label of a CS of compatible type. |
| CS coordinate names and/or symbols | SRF-specific names and/or symbols for the kth-coordinate names and/or symbols, and a designation of the vertical coordinate if applicable. |
| Template parameters | CS and RD parameters, if any, and/or SRF parameters that are not specified by a CS parameter binding rule. |
| CS parameter binding rules | A set of rules for binding for CS parameters and RD class parameters, if any, and/or SRF parameters. |
| Coordinate valid region | Optional restriction of the domain of the CS to a valid region. If a valid region is specified, optionally an extended valid region. If both are unspecified, then there are no additional constraints on coordinate validity. |
| Notes | Optional, additional, non-normative information such as a description of the SRF structure, modelled region, intended use, and/or application domain. |
| Reference type | The reference type (see 12.2.5). |
| References | The references (see 12.2.5). |

This International Standard specifies a collection of SRFTs as identified in Table 8.3. Additional SRFTs may be registered in accordance with Clause 12. Registered SRFs shall be derived only from standardized or registered SRFTs.

Table 8.3 — SRFT directory

| CS type | Description | SRFT label |
| --- | --- | --- |
| 3D | Celestiocentric | SRFT_CELESTIOCENTRIC |
|  | Local space rectangular | SRFT_3D_LOCAL_SPACE_RECTANGULAR |
|  | Celestiodetic | SRFT_CELESTIODETIC |
|  | Local tangent plane | SRFT_3D_LOCAL_TANGENT_PLANE |
|  | Local azimuthal spherical tangent plane | SRFT_LOCAL_AZIMUTHAL_SPHERICAL_-TANGENT_PLANE |
|  | Local cylindrical tangent plane | SRFT_LOCAL_CYLINDRICAL_TANGENT_PLANE |
|  | Celestiomagnetic | SRFT_CELESTIOMAGNETIC |
|  | Equatorial inertial | SRFT_EQUATORIAL_INERTIAL |
|  | Solar ecliptic | SRFT_SOLAR_ECLIPTIC |
|  | Solar equatorial | SRFT_SOLAR_EQUATORIAL |
|  | Solar magnetospheric | SRFT_SOLAR_MAGNETOSPHERIC |
|  | Solar magnetic | SRFT_SOLAR_MAGNETIC |
|  | Heliospheric Aries ecliptic | SRFT_HELIOSPHERIC_ARIES_ECLIPTIC |
|  | Heliospheric Earth ecliptic | SRFT_HELIOSPHERIC_EARTH_ECLIPTIC |
|  | Heliospheric Earth equatorial | SRFT_HELIOSPHERIC_EARTH_EQUATORIAL |
| Surface | Surface celestiodetic (induced) | SRFT_CELESTIODETIC |
|  | Surface local tangent plane (induced) | SRFT_3D_LOCAL_TANGENT_PLANE |
|  | Surface local azimuthal tangent plane | SRFT_SURFACE_LOCAL_AZIMUTHAL_-TANGENT_PLANE |
|  | Surface local polar tangent plane (induced) | SRFT_LOCAL_CYLINDRICAL_TANGENT_PLANE |
| 2D | Local space rectangular | SRFT_2D_LOCAL_SPACE_RECTANGULAR |
|  | Azimuthal | SRFT_2D_AZIMUTHAL |
|  | Polar | SRFT_2D_POLAR |
| Map projection and augmented map projection | Mercator | SRFT_MERCATOR |
|  | Oblique Mercator | SRFT_OBLIQUE_MERCATOR |
|  | Transverse Mercator | SRFT_TRANSVERSE_MERCATOR |
|  | Lambert conformal conic | SRFT_LAMBERT_CONFORMAL_CONIC |
|  | Polar stereographic | SRFT_POLAR_STEREOGRAPHIC |
|  | Equidistant cylindrical | SRFT_EQUIDISTANT_CYLINDRICAL |

### Celestiocentric SRF

Celestiocentric SRFs shall be derived from the SRFT specified in Table 8.4.

Table 8.4 — Celestiocentric SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_CELESTIOCENTRIC |
| Code | 1 |
| Description | Celestiocentric SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_3D_EUCLIDEAN |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None (no CS parameters). |
| Coordinate valid region | No additional restrictions. |
| Notes | When the object is Earth, this SRF is referred to as a geocentric SRF. |
| Reference type | IR |
| References | TBD |

### Local space rectangular 3D SRF

Local space rectangular SRFs shall be derived from the SRFT specified in Table 8.5.

Table 8.5 — Local space rectangular 3D SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_3D_LOCAL_SPACE_RECTANGULAR |
| Code | 2 |
| Description | Local space rectangular 3D SRF. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 3D abstract object. |
| CS label | CS_3D_LOCOCENTRIC_EUCLIDEAN |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | Primary axis (1, 2, or 3). Direction of up (+1, -1). Direction of forward (+1, -1). |
| CS parameter binding rules |  |
| Coordinate valid region | No additional restrictions. |
| Notes | CAD/CAM and other engineering applications. |
| Reference type | IR |
| References | TBD |

### Local space rectangular 2D SRF

Local space rectangular 2D SRFs shall be derived from the SRFT specified in Table 8.6.

Table 8.6 — Local space rectangular 2D SRF template

| Specification element | Value |
| --- | --- |
| Label | SRFT_2D_LOCAL_SPACE_RECTANGULAR |
| Code | 3 |
| Description | Local space rectangular 2D SRF. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | CS_3D_LOCOCENTRIC_EUCLIDEAN |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | Primary axis (1, 2). Direction of up (+1, -1). |
| CS parameter binding rules |  |
| Coordinate valid region | No additional restrictions. |
| Notes | CAD/CAM and 2D graphic applications. |
| Reference type | IR |
| References | TBD |

### Celestiodetic SRF

Celestiodetic SRFs shall be derived from the SRFT specified in Table 8.7.

Table 8.7 — Celestiodetic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_CELESTIODETIC |
| Code | 4 |
| Description | Celestiodetic SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_3D_GEODETIC |
| CS coordinate names and/or symbols | The same as the CS. Ellipsoidal height is the vertical coordinate. |
| Template parameters | None. |
| CS parameter binding rules | CS parameters equal RD values: Oblate spheroid RD case - Major semi-axis a, inverse flattening f -1 Sphere RD case - Radius a |
| Coordinate valid region | No additional restrictions. |
| Notes | - The CS_S3_SURFACE_GEODETIC CS is induced on the oblate spheroid (or sphere) RD surface. - When the object is Earth, this SRF is referred to as a geodetic SRF. |
| Reference type | IR |
| References | TBD |

### Local tangent plane 3D SRF

Local tangent plane 3D SRFs shall be derived from the SRFT specified in Table 8.8.

Table 8.8 — Local tangent plane 3D SRFT

| Specification element |  | Value |
| --- | --- | --- |
| Label |  | SRFT_3D_LOCAL_TANGENT_PLANE |
| Code |  | 5 |
| Description |  | Local Tangent Plane 3D SRF. |
| Object type |  | Celestial object. |
| ORM constraint |  | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label |  | CS_3D_LOCOCENTRIC_EUCLIDEAN |
| CS coordinate names and/or symbols |  | u: x (x) v: y (y) w: height (h) is the vertical coordinate. |
| Template parameters |  |  |
| CS parameter binding rules |  |  |
| Coordinate valid region |  | No additional restrictions. |
| Notes |  | - The CS_S3_TOPOCENTRIC_SURFACE_EUCLIDEAN CS is induced on the tangent plane surface. - Plane tangent to the oblate spheroid RD at surface celestiodetic coordinates (,).  is the azimuth of the x-axis. |
| Reference type | IR |  |
| References | TBD |  |

[Editor’s Note: Add diagram that illustrates the relationship between the tangent plane surface and the RD.]

### Local azimuthal spherical tangent plane SRF

Local azimuthal spherical tangent plane SRFs shall be derived from the SRFT specified in Table 8.9.

Table 8.9 — Local azimuthal spherical tangent plane SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_LOCAL_AZIMUTHAL_SPHERICAL_TANGENT_PLANE |
| Code | 6 |
| Description | Local azimuthal spherical tangent plane SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_3D_LOCOCENTRIC_AZIMUTHAL_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid region | No additional restrictions. |
| Notes | Used in radar localization. |
| Reference type | IR |
| References | TBD |

### Surface local azimuthal tangent plane SRF

Surface local azimuthal tangent plane SRFs shall be derived from the SRFT specified in Table 8.10.

Table 8.10 — Surface local azimuthal tangent plane SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_SURFACE_LOCAL_AZIMUTHAL_TANGENT_PLANE |
| Code | 7 |
| Description | Surface local azimuthal tangent plane SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_S3_LOCOCENTRIC_SURFACE_AZIMUTHAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid region | No additional restrictions. |
| Notes | Used in radar localization. |
| Reference type | IR |
| References | TBD |

### Azimuthal 2D SRF

Azimuthal 2D SRFs shall be derived from the SRFT specified in Table 8.11.

Table 8.11 — Azimuthal 2D SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_2D_AZIMUTHAL |
| Code | 8 |
| Description | Azimuthal 2D SRF. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | CS_2D_AZIMUTHAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | None. |
| Reference type | IR |
| References | TBD |

### Local cylindrical tangent plane SRF

Local cylindrical tangent plane SRFs shall be derived from the SRFT specified in Table 8.12.

Table 8.12 — Local cylindrical tangent plane SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_LOCAL_CYLINDRICAL_TANGENT_PLANE |
| Code | 9 |
| Description | Local cylindrical tangent plane SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_3D_LOCOCENTRIC_CYLINDRICAL |
| CS coordinate names and/or symbols |  |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid region | No additional restrictions. |
| Notes | - The CS_S3_LOCOCENTRIC_SURFACE_POLAR CS is induced on the tangent plane surface. - Plane tangent to the oblate spheroid (or sphere) RD at surface celestiodetic coordinates (,).  is the azimuth of the x-axis. |
| Reference type | IR |
| References | TBD |

### Polar 2D SRF

Polar 2D SRFs shall be derived from the SRFT specified in Table 8.13.

Table 8.13 — Polar 2D SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_2D_POLAR |
| Code | 10 |
| Description | Polar 2D SRF. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | CS_2D_POLAR |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | None. |
| Reference type | IR |
| References | TBD |

### Celestiomagnetic SRF

Celestiomagnetic SRFs shall be derived from the SRFT specified in Table 8.14.

Table 8.14 — Celestiomagnetic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_CELESTIOMAGNETIC |
| Code | 11 |
| Description | Magnetic SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be an ORM from Table 7.17. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | - See clause 7.5.7. - When the object is Earth, this SRF is referred to as a geomagnetic SRF. |
| Reference type | IR |
| References | TBD |

### Equatorial inertial SRF

Equatorial inertial SRFs shall be derived from the SRFT specified in Table 8.15.

Table 8.15 — Equatorial inertial SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_EQUATORIAL_INERTIAL |
| Code | 12 |
| Description | Equatorial Inertial SRF. |
| Object type | Planet. |
| ORM constraint | Shall be an ORM from Table 7.20. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols |  : right ascension (ra)  : declination (dec)  : radius (r) |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.2. |
| Reference type | IR |
| References | TBD |

### Solar ecliptic SRF

Solar ecliptic SRFs shall be derived from the SRFT specified in Table 8.16.

Table 8.16 — Solar ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_SOLAR_ECLIPTIC |
| Code | 13 |
| Description | Solar ecliptic SRF. |
| Object type | Planet. |
| ORM constraint | Shall be an ORM from Table 7.14. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.3. |
| Reference type | IR |
| References | TBD |

### Solar equatorial SRF

Solar equatorial SRFs shall be derived from the SRFT specified in Table 8.17.

Table 8.17 — Solar equatorial SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_SOLAR_EQUATORIAL |
| Code | 14 |
| Description | Solar equatorial SRF. |
| Object type | Planet. |
| ORM constraint | Shall be an ORM from Table 7.16. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.4. |
| Reference type | IR |
| References | TBD |

### Solar magnetospheric SRF

Solar magnetospheric SRFs shall be derived from the SRFT specified in Table 8.18.

Table 8.18 — Solar magnetospheric SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_SOLAR_MAGNETOSPHERIC |
| Code | 15 |
| Description | Solar magnetospheric SRF. |
| Object type | Planet. |
| ORM constraint | Shall be an ORM from Table 7.22. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.8. |
| Reference type | IR |
| References | TBD |

### Solar magnetic SRF

Solar magnetic SRFs shall be derived from the SRFT specified in Table 8.19.

Table 8.19 — Solar magnetic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_SOLAR_MAGNETIC |
| Code | 16 |
| Description | Solar magnetic SRF. |
| Object type | Planet. |
| ORM constraint | Shall be an ORM from Table 7.24. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.9. |
| Reference type | IR |
| References | TBD |

### Heliospheric Aries ecliptic SRF

Heliospheric Aries ecliptic SRFs shall be derived from the SRFT specified in Table 8.20.

Table 8.20 — Heliospheric Aries ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_HELIOSPHERIC_ARIES_ECLIPTIC |
| Code | 17 |
| Description | Heliospheric Aries ecliptic SRF. |
| Object type | Sun. |
| ORM constraint | Embedding origin is at solar mass centre. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See Clause TBD. |
| Reference type | IR |
| References | TBD |

### Heliospheric Earth ecliptic SRF

Heliospheric Earth ecliptic SRFs shall be derived from the SRFT specified in Table 8.21.

Table 8.21 — Heliospheric Earth ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_HELIOSPHERIC_EARTH_ECLIPTIC |
| Code | 18 |
| Description | Heliospheric Earth ecliptic SRF. |
| Object type | Sun. |
| ORM constraint | Embedding origin is at solar mass centre. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.5. |
| Reference type | IR |
| References | TBD |

### Heliospheric Earth equatorial SRF

Heliospheric Earth equatorial SRFs shall be derived from the SRFT specified in Table 8.22.

Table 8.22 — Heliospheric Earth equatorial SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_HELIOSPHERIC_EARTH_EQUATORIAL |
| Code | 19 |
| Description | Heliospheric Earth equatorial SRF. |
| Object type | Sun. |
| ORM constraint | Embedding origin is at solar mass centre. |
| CS label | CS_3D_SPHERICAL |
| CS coordinate names and/or symbols | The same as the CS. |
| Template parameters | None. |
| CS parameter binding rules | None. |
| Coordinate valid region | No additional restrictions. |
| Notes | See clause 7.5.6. |
| Reference type | IR |
| References | TBD |

### Mercator SRF

Mercator SRFs shall be derived from the SRFT specified in Table 8.23.

Table 8.23 — Mercator SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_MERCATOR |
| Code | 20 |
| Description | Mercator SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_MP_MERCATOR |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate spheroid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented Mercator CS induces the Mercator CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

### Oblique Mercator SRF

Oblique Mercator SRFs shall be derived from the SRFT specified in Table 8.24.

Table 8.24 — Oblique Mercator SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_OBLIQUE_MERCATOR |
| Code | 21 |
| Description | Oblique Mercator SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from ORMT_3D_SPHERE. |
| CS label | CS_MP_OBLIQUE_MERCATOR |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Radius a |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented oblique Mercator CS induces the oblique Mercator CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

### Transverse Mercator SRF

Transverse Mercator SRFs shall be derived from the SRFT specified in Table 8.25.

Table 8.25 — Transverse Mercator SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_TRANSVERSE_MERCATOR |
| Code | 22 |
| Description | Transverse Mercator SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_MP_TRANSVERSE_MERCATOR |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate spheroid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented transverse Mercator CS induces the transverse Mercator CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

### Lambert conformal conic SRF

Lambert conformal conic SRFs shall be derived from the SRFT specified in Table 8.26.

Table 8.26 — Lambert conformal conic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_LAMBERT_CONFORMAL_CONIC |
| Code | 23 |
| Description | Lambert conformal conic SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_MP_LAMBERT_CONFORMAL_CONIC |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate spheroid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented Lambert conformal conic CS induces the Lambert conformal conic CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

### Polar stereographic SRF

Polar stereographic SRFs shall be derived from the SRFT specified in Table 8.27.

Table 8.27 — Polar stereographic SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_POLAR_STEREOGRAPHIC |
| Code | 24 |
| Description | Polar stereographic SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from: ORMT_3D_OBLATE_SPHEROID or ORMT_3D_SPHERE |
| CS label | CS_MP_POLAR_STEREOGRAPHIC |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate spheroid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented polar stereographic CS induces the polar stereographic CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

### Equidistant cylindrical SRF

Equidistant cylindrical SRFs shall be derived from the SRFT specified in Table 8.28.

Table 8.28 — Equidistant cylindrical SRFT

| Specification element | Value |
| --- | --- |
| Label | SRFT_EQUIDISTANT_CYLINDRICAL |
| Code | 25 |
| Description | Equidistant cylindrical SRF. |
| Object type | Celestial object. |
| ORM constraint | Shall be derived from ORMT_3D_SPHERE. |
| CS label | CS_MP_EQUIDISTANT_CYLINDRICAL |
| CS coordinate names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Radius a, |
| Coordinate valid region | No additional restrictions. |
| Notes | The augmented equidistant cylindrical CS induces the equidistant cylindrical CS on the zero value vertical coordinate surface (which coincides with the RD surface). |
| Reference type | IR |
| References | TBD |

## SRFs

This International Standard specifies a collection of SRFs. These specifications appear in Table 8.30. These SRFs are each derived from a SRFT. Additional SRFs derived from SRFTs may be registered in accordance with Clause 12.

The specification elements for SRFTs are defined in Table 8.29.

Table 8.29 — SRF specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Description | A description including name as published or as commonly known. |
| SRFT | The label of the applicable SRF template. |
| ORM | The label of the applicable ORM. |
| Region | Optional restriction of the domain of the CS to a valid region. If a valid region is specified, optionally an extended valid region. If both are unspecified, then there are no additional constraints on coordinate validity. |
| Parameter values | The SRF template parameter values |
| Notes | Optional, additional, non-normative information concerning the SRF, such as description of its structure, modelled region, intended use, and/or application domain. |
| Reference type | The reference type (see 12.2.5). |
| References | The references (see 12.2.5). |

Table 8.30 — SRFs

| Label | SRF_BRITISH_NATIONAL_GRID | Code | 1 |
| --- | --- | --- | --- |
| Description | British National Grid |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM | ORM_OSGB_1936_MEAN_SOLUTION |
| Region | Valid region description: Great Britain. |  |  |
| Parameter values | Central meridian: λ0 = -2º Scale on central meridian: k0 = 0,999 601 271 7 True origin: (49ºN, 2ºW). False easting: uF = -400 000 m. False northing: vF = 100 000 m. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [OSTM] |
| Label | SRF_DELAWARE_SPCS | Code | 2 |
| Description | Delaware (US) State Plane Coordinate System |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM | ORM_N_AM_1983_CONUS |
| Region | Valid region description: State of Delaware (US). |  |  |
| Parameter values | Central meridian: λ0 = -75º 25′ Scale on central meridian: k0 = (1 - 1/200 000) True origin: (38ºN, 75º 25′W) False easting: uF = 200 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| Reference type | IR | References | [SNYD, Appendix C] |
| Label | SRF_GEOCENTRIC_EARTH_1984 | Code | 3 |
| Description | Geocentric WGS 1984 |  |  |
| SRF template | SRFT_CELESTIOCENTRIC | ORM | ORM_WGS_1984 |
| Region | Valid region description: Global Earth. |  |  |
| Parameter values | None. |  |  |
| Notes | Mass centred. |  |  |
| Reference type | IR | References | [83502T, Chapter 2.1] |
| Label | SRF_GEODETIC_AUSTRALIA- _1984 | Code | 4 |
| Description | Geodetic Australia 1984 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_AUSTRALIAN_GEOD_1984_7 |
| Region | Valid region description: Australia and Tasmania. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [CECT] |
| Label | SRF_GEODETIC_AUSTRALIA- _1990 | Code | 5 |
| Description | Geodetic Australia 1990 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_GEOCENTRIC_DATUM_AUSTRALIA |
| Region | Valid region description: Australia. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [CECT] |
| Label | SRF_GEODETIC_EARTH_1984 | Code | 6 |
| Description | Geodetic WGS 1984 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_WGS_1984 |
| Region | Valid region description: Global Earth. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [83502T, Chapter 3] |
| Label | SRF_GEODETIC_EUROPE_1950 | Code | 7 |
| Description | Geodetic Europe 1950 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_EUR_1950_MEAN_SOLUTION |
| Region | Valid region description: Europe. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [83502T, App. B.5, "EUR-M"], [HELM, "EUR-M"] |
| Label | SRF_GEODETIC_JAPAN_1991 | Code | 8 |
| Description | Geodetic Japan 1991 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_TOKYO_MEAN_SOLUTION |
| Region | Valid region description: Japan, Korea, and Okinawa. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [83502T, App. B.3, "TOY-M"] |
| Label | SRF_GEODETIC_N_AMERICAN-_1950 | Code | 9 |
| Description | Geodetic North American 1983 |  |  |
| SRF template | SRFT_CELESTIODETIC | ORM | ORM_N_AM_1983_CONUS |
| Region | Valid region description: Continental United States. |  |  |
| Parameter values | None. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [SNYD] |
| Label | SRF_IRISH_GRID | Code | 10 |
| Description | Irish Grid |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM | ORM_IRELAND_1965 |
| Region | Valid region description: Ireland. |  |  |
| Parameter values | Central meridian: λ0 = -8º Scale on central meridian: k0 = 1,000 035 True origin: (53º 30′N, 8ºW) False easting: uF = 200 000 m. False northing: vF = 250 000 m. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [IGRID] |
| Label | SRF_IRISH_TRANSVERSE- _MERCATOR | Code | 11 |
| Description | Irish Transverse Mercator |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM | ORM_ETRS_1989 |
| Region | Valid region description: Ireland. |  |  |
| Parameter values | Central meridian: λ0 = -8º Scale on central meridian: k0 = 0,999 820 True origin: (53º 30′N, 8ºW) False easting: uF = 600 000 m. False northing: vF = 750 000 m. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [NMPI] |
| Label | SRF_LAMBERT_93 | Code | 12 |
| Description | Lambert-93 |  |  |
| SRF template | SRFT_LAMBERT_CONFORMAL_CONIC | ORM | ORM_RGF_1993 |
| Region | Valid region description: France. |  |  |
| Parameter values | First parallel: φ1 = 44º Second parallel: φ2 = 49º True origin: (46º 30′N, 3ºE) False easting: uF = 700 000 m. False northing: vF = 6 600 000 m. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [PASG] |
| Label | SRF_LAMBERT_II_WIDE | Code | 13 |
| Description | Lambert II étendu (Lambert II wide) |  |  |
| SRF template | SRFT_LAMBERT_CONFORMAL_CONIC | ORM | ORM_NEW_FRENCH_ZERO |
| Region | Valid region description: France. |  |  |
| Parameter values | First parallel: φ1 = 45º 53′ 56,108″ Second parallel: φ2 = 47º 41′ 45,652″ True origin: (46º 48′N, 0ºE) False easting: uF = 600 000 m. False northing: vF = 2 200 000 m. |  |  |
| Notes | None. |  |  |
| Reference type | IR | References | [LIIE] |
| Label | SRF_MARYLAND_SPCS | Code | 14 |
| Description | Maryland (US) State Plane Coordinate System |  |  |
| SRF template | SRFT_LAMBERT_CONFORMAL_CONIC | ORM | ORM_N_AM_1983_CONUS |
| Region | Valid region description: State of Maryland (US). |  |  |
| Parameter values | First parallel: φ1 = 38º 18′ Second parallel: φ2 = 39º 27′ True origin: (37º 50′N, 77ºW) False easting: uF = 400 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| Reference type | IR | References | [SNYD, Appendix C] |

## SRF sets

Given an ORM, an SRF set is a finite parameterized set of two or more SRFs that:

- are derived from the same SRF template using the given ORM, and

- the valid regions of the set members are non-overlapping.

An SRF set specification may further restrict the ORM constraints of the SRFT. The specification elements for SRF sets are defined in Table 8.31.

Table 8.31 — SRF set specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Description | A description including name as published or as commonly known. |
| SRFT | The label of the applicable SRFT. |
| ORM constraints | Criteria for allowable ORMs. Specifying a single ORM indicates that only that ORM shall be used. |
| Region | Optional restriction of the domain of the SRF set to a valid region. If a valid region is specified, optionally an extended valid region. If both are unspecified, then there are no additional constraints on coordinate validity. |
| SRF set membership | A specification of the parameterization of the set members, by listing or parameter algorithm. |
| Notes | A description of the structure, modelled region, intended use, and/or application domain of the SRF set. |
| Reference type | The reference type (see 12.2.5). |
| References | The reference type (see 12.2.5). |

This International Standard specifies a collection of SRF sets. These specifications appear in Table 8.32. Additional SRF sets may be registered in accordance with Clause 12.

Table 8.32 — SRF sets

| Label | SRFS_ALABAMA_SPCS | Code | 1 |
| --- | --- | --- | --- |
| Description | Alabama (US) State Plane Coordinate System. |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM constraints | ORM_N_AM_1983_CONUS |
| Region | Valid region description: State of Alabama (US). |  |  |
| SRF set membership | Table 8.34, subheading SRFS_ALABAMA_SPCS. |  |  |
| Notes | A set of two localized adjacent SRFs where only one SRF is used for each county in the state and no overlap is allowed. |  |  |
| Reference type | IR | References | [SNYD, Appendix C] |
| Label | SRFS_GTRS_GLOBAL-_COORDINATE_SYSTEM | Code | 2 |
| Description | GTRS global coordinate system (Earth). |  |  |
| SRF template | SRFT_3D_LOCAL_TANGENT_PLANE | ORM constraints | A global model ERM such as ORM_WGS_1984. |
| Region | Valid region description: Earth (complete). |  |  |
| SRF set membership | Table 8.35, subheading SRFS_GTRS_GLOBAL_COORDINATE_SYSTEM. |  |  |
| Notes | A set of 49 896 localized (but overlapping) SRFs, each approximately 100 kilometres square, that are identified according to the geotile reference system indexing scheme. The commonly known GCS SRF set specifies the ORM_WGS_1984 as the ERM. The members of this SRF set are commonly known as cells. For much of the RD surface, each cell valid region covers one arc degree of geodetic latitude by one arc degree of geodetic longitude. However, near the poles, many arc degrees of longitude are grouped together into a single GCS cell since an arc degree of geodetic longitude becomes arbitrarily small near the poles. GCS cells are always one arc degree of geodetic latitude in extent. Within each GCS cell, a false origin offset is provided. The point of tangency is at the centre of the rectangular GCS cell, even if more than one arc degree of geodetic longitude falls within a GCS SRF cell. The SRFT_3D_LOCAL_TANGENT_PLANE azimuth parameter (α) is zero. |  |  |
| Reference type | IR | References | TBS |
| Label | SRFS_LAMBERT_NTF | Code | 3 |
| Description | Lambert projection-based mapping system for France associated with the NTF. |  |  |
| SRF template | SRFT_LAMBERT_CONFORMAL_CONIC | ORM constraints | ORM_NEW_FRENCH_ZERO |
| Region | Valid region description: France. |  |  |
| SRF set membership | Table 8.34, subheading SRFS_LAMBERT_NTF. |  |  |
| Notes | A set of four localized adjacent SRFs where only one SRF is used for each portion of France and no overlap is allowed. |  |  |
| Reference type | IR | References | [LIIE] |
| Label | SRFS_MARS_TRANSVERSE_MERCATOR-_BODY | Code | 4 |
| Description | Mars Transverse Mercator (MTM) map series system, body (non-polar) regions. |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM constraints | ORM_MARS |
| Region | Valid region specification: -82,5º ≤ latitude ≤ 82,5º |  |  |
| SRF set membership | Table 8.35, subheading SRFS_MARS_TRANSVERSE_MERCATOR_BODY. |  |  |
| Notes | A set of 1 944 localized (but overlapping) SRFs that are used to index 1:500 000 scale geologic and photomosaic maps of Mars non-polar regions. Uses planetocentric, rather than planetographic, referencing and shares a common boundary with SRFS_MARS_TRANSVERSE_MERCATOR_POLE. |  |  |
| Reference type | IR | References | [MTMM] |
| Label | SRFS_MARS_TRANSVERSE_MERCATOR_POLE | Code | 5 |
| Description | Mars Transverse Mercator (MTM) map series system, polar regions. |  |  |
| SRF template | SRFT_POLAR_STEREOGRAPHIC | ORM constraints | ORM_MARS |
| Region | Valid region specification: latitude < -82,5º or 82,5º ≤ latitude |  |  |
| SRF set membership | Table 8.35, subheading SRFS_MARS_TRANSVERSE_MERCATOR_POLE. |  |  |
| Notes | A set of 20 localized (but overlapping) SRFs that are used to index 1:500 000 scale geologic and photomosaic maps of Mars polar regions. Uses planetocentric, rather than planetographic, referencing and shares a common boundary with SRFS_MARS_TRANSVERSE_MERCATOR_BODY. |  |  |
| Reference type | IR | References | [MTMM] |
| Label | SRFS_UNIVERSAL_POLAR_STEREOGRAPHIC | Code | 6 |
| Description | Universal Polar Stereographic (Earth). |  |  |
| SRF template | SRFT_POLAR_STEREOGRAPHIC | ORM constraints | A global model ERM such as ORM_WGS_1984. |
| Region | Valid region specification: latitude < -80º or 84º ≤ latitude Extended valid region specification: latitude < -79,5º or 83,5º ≤ latitude |  |  |
| SRF set membership | Table 8.35, subheading SRFS_UNIVERSAL_POLAR_STEREOGRAPHIC. |  |  |
| Notes | A set of two localized adjacent SRFs, where limited overlap is modelled by extended validity of coordinates in the member SRFs. Shares a common boundary with SRFS_UNIVERSAL_TRANSVERSE_MERCATOR. |  |  |
| Reference type | IR | References | [83582] |
| Label | SRFS_UNIVERSAL_TRANSVERSE_MERCATOR | Code | 7 |
| Description | Universal Transverse Mercator (Earth). |  |  |
| SRF template | SRFT_TRANSVERSE_MERCATOR | ORM constraints | A global model ERM such as ORM_WGS_1984. |
| Region | Valid region specification: -80º ≤ latitude ≤ 84º Extended valid region specification: -80,5º < latitude ≤ 84,5º |  |  |
| SRF set membership | Table 8.35, subheading SRFS_UNIVERSAL_TRANSVERSE_MERCATOR. |  |  |
| Notes | A set of 120 localized SRFs, where limited overlap is modelled by extended validity of coordinates in the member SRFs. Shares a common boundary with SRFS_UNIVERSAL_POLAR_STEREOGRAPHIC. |  |  |
| Reference type | IR | References | [83582] |
| Label | SRFS_WISCONSIN_SPCS | Code | 8 |
| Description | Wisconsin (US) State Plane Coordinate System. |  |  |
| SRF template | SRFT_LAMBERT_CONFORMAL_CONIC | ORM constraints | ORM_N_AM_1983_CONUS |
| Region | Valid region description: State of Wisconsin (US). |  |  |
| SRF set membership | Table 8.34, subheading SRFS_WISCONSIN_SPCS. |  |  |
| Notes | A set of three localized adjacent SRFs where only one SRF is used for each county in the state and no overlap is allowed. |  |  |
| Reference type | IR | References | [WSCO] |

The specification elements for SRF set members are defined in Table 8.33.

Table 8.33 — SRF set member specification elements

| Specification element | Definition | Location |
| --- | --- | --- |
| SRF set label | The label of the SRF set (see 12.2.2). | Table 8.34 or Table 8.35 |
| Label | The label (see 12.2.2). | Table 8.34 or Table 8.35 |
| Code | The code (see 12.2.3); the set member parameter. | Table 8.34 or Table 8.35 |
| Description | A description including name as published or as commonly known. | Table 8.34 or Table 8.35 |
| Region | A valid region description. Optionally an extended valid region description. | Table 8.34 or Table 8.35 |
| Parameter values | The SRFT parameter values. | Table 8.34 or Table 8.35 |
| Notes | Optional, additional, non-normative information concerning the SRF set member. | Table 8.34 or Table 8.35 |

This International Standard specifies a collection of SRF sets. These specifications appear in Table 8.34 and Table 8.35. Additional SRF set members may be registered in accordance with Clause 12.

Table 8.34 — Explicit specifications of SRF set members

| SRFS_ALABAMA_SPCS |  |  |  |
| --- | --- | --- | --- |
| Label | WEST_ZONE | Code | 1 |
| Description | West zone. |  |  |
| Region | Valid region description: Alabama (US) counties of TBD. |  |  |
| Parameter values | Central meridian: λ0 = -87º 30′ Scale on central meridian: k0 = (1 - 1/15 000) True origin: (30ºN, 87º 30′W) False easting: uF = 600 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| Label | EAST_ZONE | Code | 2 |
| Description | East zone. |  |  |
| Region | Valid region description: Alabama (US) counties of TBD. |  |  |
| Parameter values | Central meridian: λ0 = -85º 50′ Scale on central meridian: k0 = (1 - 1/25 000) True origin: (30º&nbsp30′N, 85º 50′W) False easting: uF = 200 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| SRFS_LAMBERT_NTF |  |  |  |
| Label | ZONE_I | Code | 1 |
| Description | Zone I. |  |  |
| Region | Valid region specification: -5º ≤ λ ≤ 10º -53,5º ≤ φ < 57º |  |  |
| Parameter values | First parallel: φ1 = 48º 35′ 54,682″ Second parallel: φ2 = 50º 23′ 45,282″ True origin: (49,5ºN, 0ºE) False easting: uF = 600 000 m. False northing: vF = 200 000 m. |  |  |
| Notes | None. |  |  |
| Label | ZONE_II | Code | 2 |
| Description | Zone II. |  |  |
| Region | Valid region specification: -5º ≤ λ ≤ 10º -50,5º ≤ φ < 53,5º |  |  |
| Parameter values | First parallel: φ1 = 45º 53′ 56,108″ Second parallel: φ2 = 47º 41′ 45,652″ True origin: (46,8ºN, 0ºE) False easting: uF = 600 000 m. False northing: vF = 200 000 m. |  |  |
| Notes | None. |  |  |
| Label | ZONE_III | Code | 3 |
| Description | Zone III. |  |  |
| Region | Valid region specification: -5º ≤ λ ≤ 10º -47º ≤ φ < 50,5º |  |  |
| Parameter values | First parallel: φ1 = 43º 11′ 57,449″ Second parallel: φ2 = 44º 59′ 45,938″ True origin: (44,1ºN, 0ºE) False easting: uF = 600 000 m. False northing: vF = 200 000 m. |  |  |
| Notes | None. |  |  |
| Label | ZONE_IV | Code | 4 |
| Description | Zone IV. |  |  |
| Region | Valid region specification: The island of Corsica. |  |  |
| Parameter values | First parallel: φ1 = 41º 33′ 37,396″ Second parallel: φ2 = 42º 46′ 3,588″ True origin: (42º 9′ 54″N, 0ºE) False easting: uF = 234 358 m. False northing: vF = 185 861,369 m. |  |  |
| Notes | None. |  |  |
| SRFS_WISCONSIN_SPCS |  |  |  |
| Label | SOUTH_ZONE | Code | 1 |
| Description | South zone. |  |  |
| Region | Valid region description: Wisconsin (US) counties of TBD. |  |  |
| Parameter values | First parallel: φ1 = 42º 44′ Second parallel: φ2 = 44º 04′ True origin: (42ºN, 90ºW) False easting: uF = 600 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| Label | CENTRAL_ZONE | Code | 2 |
| Description | Central zone. |  |  |
| Region | Valid region description: Wisconsin (US) counties of TBD. |  |  |
| Parameter values | First parallel: φ1 = 44º 15′ Second parallel: φ2 = 45º 30′ True origin: (43º 50′N, 90ºW) False easting: uF = 600 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |
| Label | NORTH_ZONE | Code | 3 |
| Description | North zone. |  |  |
| Region | Valid region description: Wisconsin (US) counties of TBD. |  |  |
| Parameter values | First parallel: φ1 = 45º 34′ Second parallel: φ2 = 46º 46′ True origin: (45º 10′N, 90ºW) False easting: uF = 600 000 m. False northing: vF = 0 m. |  |  |
| Notes | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |

Table 8.35 — Implicit specifications of SRF set members

| SRFS_GTRS_GLOBAL_COORDINATE_SYSTEM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Label | “TILE_” + <code> | Code | 1…49 896: As specified in Table 8.37 through Table 8.51. |  |  |  |  |
| Description | Tile <code>. |  |  |  |  |  |  |
| Region | Valid region specification: As specified in Table 8.36. Extended valid region specification: Unrestricted. |  |  |  |  |  |  |
| Parameter values | True origin: As specified in Table 8.37 through Table 8.51. Azimuth: 0 rad. False easting: uF = -50 000 m. False northing: vF = -50 000 m. |  |  |  |  |  |  |
| Notes | None. |  |  |  |  |  |  |
| SRFS_MARS_TRANSVERSE_MERCATOR_BODY |  |  |  |  |  |  |  |
| Label | As specified in Table 8.52 and Table 8.53. |  |  | Code | 1… 1 944 |  |  |
| Description | Region map designator. |  |  |  |  |  |  |
| Region | Valid region specification: As specified in Table 8.52 and Table 8.53. |  |  |  |  |  |  |
| Parameter values | As specified in Table 8.52 and Table 8.53. |  |  |  |  |  |  |
| Notes | None. |  |  |  |  |  |  |
| SRFS_MARS_TRANSVERSE_MERCATOR_POLE |  |  |  |  |  |  |  |
| Label | As specified in Table 8.54 and Table 8.55. |  |  | Code | 1…20 |  |  |
| Description | Region map designator. |  |  |  |  |  |  |
| Region | Valid region specification: As specified in Table 8.54 and Table 8.55. |  |  |  |  |  |  |
| Parameter values | As specified in Table 8.54 and Table 8.55. |  |  |  |  |  |  |
| Notes | None. |  |  |  |  |  |  |
| SRFS_UNIVERSAL_POLAR_STEREOGRAPHIC |  |  |  |  |  |  |  |
| Label | As specified in Table 8.56 and Table 8.57. |  |  |  |  | Code | 1…2 |
| Description | Northern or southern pole. |  |  |  |  |  |  |
| Region | Valid region specification: As specified in Table 8.56 and Table 8.57. Extended valid region specification: As specified in Table 8.56 and Table 8.57. |  |  |  |  |  |  |
| Parameter values | Scale on central meridian: k0 = 0,994 True origin: As specified in Table 8.56 and Table 8.57. False easting: uF = 2 000 000 m. False northing: vF = 2 000 000 m. |  |  |  |  |  |  |
| Notes | None. |  |  |  |  |  |  |
| SRFS_UNIVERSAL_TRANSVERSE_MERCATOR |  |  |  |  |  |  |  |
| Label | As specified in Table 8.58 and Table 8.59. |  |  |  |  | Code | 1…120 |
| Description | UTM zone in northern or southern hemisphere. |  |  |  |  |  |  |
| Region | Valid region specification: As specified in Table 8.58 and Table 8.59. Extended valid region specification: As specified in Table 8.58 and Table 8.59. |  |  |  |  |  |  |
| Parameter values | Central meridian: As specified in Table 8.58 and Table 8.59. Scale on central meridian: k0 = 0,999 6 True origin: As specified in Table 8.58 and Table 8.59. False easting: uF = 500 000 m. False northing: As specified in Table 8.58 and Table 8.59. |  |  |  |  |  |  |
| Notes | None. |  |  |  |  |  |  |

Table 8.36 — GTRS indexing scheme and set members directory

| Band | Tile size | Valid range from true origin | Table number |
| --- | --- | --- | --- |
| 88º-90ºN | 1º x 30º | ±0,5º latitude, ±15º longitude | Table 8.37 |
| 86º-88ºN | 1º x 15º | ±0,5º latitude, ±7,5º longitude | Table 8.38 |
| 84º-86ºN | 1º x 10º | ±0,5º latitude, ±5º longitude | Table 8.39 |
| 80º-84ºN | 1º x 6º | ±0,5º latitude, ±3º longitude | Table 8.40 |
| 78º-80ºN | 1º x 5º | ±0,5º latitude, ±2,5º longitude | Table 8.41 |
| 71º-78ºN | 1º x 3º | ±0,5º latitude, ±1,5º longitude | Table 8.42 |
| 60º-71ºN | 1º x 2º | ±0,5º latitude, ±1º longitude | Table 8.43 |
| 60ºS - 60ºN | 1º x 1º | ±0,5º latitude, ±0,5º longitude | Table 8.44 |
| 71º-60ºS | 1º x 2º | ±0,5º latitude, ±1º longitude | Table 8.45 |
| 78º-71ºS | 1º x 3º | ±0,5º latitude, ±1,5º longitude | Table 8.46 |
| 80º-78ºS | 1º x 5º | ±0,5º latitude, ±2,5º longitude | Table 8.47 |
| 84º-80ºS | 1º x 6º | ±0,5º latitude, ±3º longitude | Table 8.48 |
| 86º-84ºS | 1º x 10º | ±0,5º latitude, ±5º longitude | Table 8.49 |
| 88º-86ºS | 1º x 15º | ±0,5º latitude, ±7,5º longitude | Table 8.50 |
| 90º-88ºS | 1º x 30º | ±0,5º latitude, ±15º longitude | Table 8.51 |

[Editor’s Note: Each of the following GTRS tables should be accompanied by a (non-normative) NOTE expressing the relationship between code and true origin in terms of a formula.]

Table 8.37 — GTRS indexing scheme and set members: 88º-90º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49885 (89,5º, -165º) | 49886…49895 (89,5º, -165º +[1…10]•30º) | 49896 (89,5º, 165º) |
| 49873 (88,5º, -165º) | 49874…49883 (88,5º, -165º +[1…10]•30º) | 49884 (88,5º, 165º) |

Table 8.38 — GTRS indexing scheme and set members: 86º-88º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49849 (87,5º, -172,5º) | 49850…49871 (87,5º, -172,5º +[1…22]•15º) | 49872 (87,5º, 172,5º) |
| 49825 (86,5º, -172,5º) | 49826…49847 (86,5º, -172,5º +[1…22]•15º) | 49848 (86,5º, 172,5º) |

Table 8.39 — GTRS indexing scheme and set members: 84º-86º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49789 (85,5º, -175º) | 49790…49823 (85,5º, -175º +[1…34]•10º) | 49824 (85,5º, 175º) |
| 49753 (84,5º, -175º) | 49754…49787 (84,5º, -175º +[1…34]•10º) | 49788 (84,5º, 175º) |

Table 8.40 — GTRS indexing scheme and set members: 80º-84º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49693 (83,5º, -177º) | 49694…49751 (83,5º, -177º +[1…58]•6º) | 49752 (83,5º, 177º) |
| 49633 (82,5º, -177º) | 49634…49691 (82,5º, -177º +[1…58]•6º) | 49692 (82,5º, 177º) |
| 49573 (81,5º, -177º) | 49574…49631 (81,5º, -177º +[1…58]•6º) | 49632 (81,5º, 177º) |
| 49513 (80,5º, -177º) | 49514…49571 (80,5º, -177º +[1…58]•6º) | 49572 (80,5º, 177º) |

Table 8.41 — GTRS indexing scheme and set members: 78º-80º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49441 (79,5º, -177,5º) | 49442…49511 (79,5º, -177,5º +[1…70]•5º) | 49512 (79,5º, 177,5º) |
| 49369 (78,5º, -177,5º) | 49370…49439 (78,5º, -177,5º +[1…70]•5º) | 49440 (78,5º, 177,5º) |

Table 8.42 — GTRS indexing scheme and set members: 71º-78º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49249 (77,5º, -178,5º) | 49250…49367 (77,5º, -178,5º +[1…118]•3º) | 49368 (77,5º, 178,5º) |
| 49129 (76,5º, -178,5º) | 49130…49247 (76,5º, -178,5º +[1…118]•3º) | 49248 (76,5º, 178,5º) |
| 49009 (75,5º, -178,5º) | 49010…49127 (75,5º, -178,5º +[1…118]•3º) | 49128 (75,5º, 178,5º) |
| 48889 (74,5º, -178,5º) | 48890…49007 (74,5º, -178,5º +[1…118]•3º) | 49008 (74,5º, 178,5º) |
| 48769 (73,5º, -178,5º) | 48770…48887 (73,5º, -178,5º +[1…118]•3º) | 48888 (73,5º, 178,5º) |
| 48649 (72,5º, -178,5º) | 48650…48767 (72,5º, -178,5º +[1…118]•3º) | 48768 (72,5º, 178,5º) |
| 48529 (71,5º, -178,5º) | 48530…48647 (71,5º, -178,5º +[1…118]•3º) | 48648 (71,5º, 178,5º) |

Table 8.43 — GTRS indexing scheme and set members: 60º-71º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 48349 (70,5º, -179º) | 48350…48527 (70,5º, -179º +[1…178]•2º) | 48528 (70,5º, 179º) |
| 48169 (69,5º, -179º) | 48170…48347 (69,5º, -179º +[1…178]•2º) | 48348 (69,5º, 179º) |
| 47989 (68,5º, -179º) | 47990…48167 (68,5º, -179º +[1…178]•2º) | 48168 (68,5º, 179º) |
| 47809 (67,5º, -179º) | 47810…47987 (67,5º, -179º +[1…178]•2º) | 47988 (67,5º, 179º) |
| 47629 (66,5º, -179º) | 47630…47807 (66,5º, -179º +[1…178]•2º) | 47808 (66,5º, 179º) |
| 47449 (65,5º, -179º) | 47450…47627 (65,5º, -179º +[1…178]•2º) | 47628 (65,5º, 179º) |
| 47269 (64,5º, -179º) | 47270…47447 (64,5º, -179º +[1…178]•2º) | 47448 (64,5º, 179º) |
| 47089 (63,5º, -179º) | 47090…47267 (63,5º, -179º +[1…178]•2º) | 47268 (63,5º, 179º) |
| 46909 (62,5º, -179º) | 46910…47087 (62,5º, -179º +[1…178]•2º) | 47088 (62,5º, 179º) |
| 46729 (61,5º, -179º) | 46730…46907 (61,5º, -179º +[1…178]•2º) | 46908 (61,5º, 179º) |
| 46549 (60,5º, -179º) | 46550…46727 (60,5º, -179º +[1…178]•2º) | 46728 (60,5º, 179º) |

Table 8.44 — GTRS indexing scheme and set members: 60º south - 60º north

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 46189 (59,5º, -179,5º) | 46190…46547 (59,5º, -179,5º +[1…358]•1º) | 46548 (59,5º, 179,5º) |
| 45829 (58,5º, -179,5º) | 45830…46187 (58,5º, -179,5º +[1…358]•1º) | 46188 (58,5º, 179,5º) |
| 45469 (57,5º, -179,5º) | 45470…45827 (57,5º, -179,5º +[1…358]•1º) | 45828 (57,5º, 179,5º) |
| 45109 (56,5º, -179,5º) | 45110…45467 (56,5º, -179,5º +[1…358]•1º) | 45468 (56,5º, 179,5º) |
| 44749 (55,5º, -179,5º) | 44750…45107 (55,5º, -179,5º +[1…358]•1º) | 45108 (55,5º, 179,5º) |
| 44389 (54,5º, -179,5º) | 44390…44747 (54,5º, -179,5º +[1…358]•1º) | 44748 (54,5º, 179,5º) |
| 44029 (53,5º, -179,5º) | 44030…44387 (53,5º, -179,5º +[1…358]•1º) | 44388 (53,5º, 179,5º) |
| 43669 (52,5º, -179,5º) | 43670…44027 (52,5º, -179,5º +[1…358]•1º) | 44028 (52,5º, 179,5º) |
| 43309 (51,5º, -179,5º) | 43310…43667 (51,5º, -179,5º +[1…358]•1º) | 43668 (51,5º, 179,5º) |
| 42949 (50,5º, -179,5º) | 42950…43307 (50,5º, -179,5º +[1…358]•1º) | 43308 (50,5º, 179,5º) |
| 42589 (49,5º, -179,5º) | 42590…42947 (49,5º, -179,5º +[1…358]•1º) | 42948 (49,5º, 179,5º) |
| 42229 (48,5º, -179,5º) | 42230…42587 (48,5º, -179,5º +[1…358]•1º) | 42588 (48,5º, 179,5º) |
| 41869 (47,5º, -179,5º) | 41870…42227 (47,5º, -179,5º +[1…358]•1º) | 42228 (47,5º, 179,5º) |
| 41509 (46,5º, -179,5º) | 41510…41867 (46,5º, -179,5º +[1…358]•1º) | 41868 (46,5º, 179,5º) |
| 41149 (45,5º, -179,5º) | 41150…41507 (45,5º, -179,5º +[1…358]•1º) | 41508 (45,5º, 179,5º) |
| 40789 (44,5º, -179,5º) | 40790…41147 (44,5º, -179,5º +[1…358]•1º) | 41148 (44,5º, 179,5º) |
| 40429 (43,5º, -179,5º) | 40430…40787 (43,5º, -179,5º +[1…358]•1º) | 40788 (43,5º, 179,5º) |
| 40069 (42,5º, -179,5º) | 40070…40427 (42,5º, -179,5º +[1…358]•1º) | 40428 (42,5º, 179,5º) |
| 39709 (41,5º, -179,5º) | 39710…40067 (41,5º, -179,5º +[1…358]•1º) | 40068 (41,5º, 179,5º) |
| 39349 (40,5º, -179,5º) | 39350…39707 (40,5º, -179,5º +[1…358]•1º) | 39708 (40,5º, 179,5º) |
| 38989 (39,5º, -179,5º) | 38990…39347 (39,5º, -179,5º +[1…358]•1º) | 39348 (39,5º, 179,5º) |
| 38629 (38,5º, -179,5º) | 38630…38987 (38,5º, -179,5º +[1…358]•1º) | 38988 (38,5º, 179,5º) |
| 38269 (37,5º, -179,5º) | 38270…38627 (37,5º, -179,5º +[1…358]•1º) | 38628 (37,5º, 179,5º) |
| 37909 (36,5º, -179,5º) | 37910…38267 (36,5º, -179,5º +[1…358]•1º) | 38268 (36,5º, 179,5º) |
| 37549 (35,5º, -179,5º) | 37550…37907 (35,5º, -179,5º +[1…358]•1º) | 37908 (35,5º, 179,5º) |
| 37189 (34,5º, -179,5º) | 37190…37547 (34,5º, -179,5º +[1…358]•1º) | 37548 (34,5º, 179,5º) |
| 36829 (33,5º, -179,5º) | 36830…37187 (33,5º, -179,5º +[1…358]•1º) | 37188 (33,5º, 179,5º) |
| 36469 (32,5º, -179,5º) | 36470…36827 (32,5º, -179,5º +[1…358]•1º) | 36828 (32,5º, 179,5º) |
| 36109 (31,5º, -179,5º) | 36110…36467 (31,5º, -179,5º +[1…358]•1º) | 36468 (31,5º, 179,5º) |
| 35749 (30,5º, -179,5º) | 35750…36107 (30,5º, -179,5º +[1…358]•1º) | 36108 (30,5º, 179,5º) |
| 35389 (29,5º, -179,5º) | 35390…35747 (29,5º, -179,5º +[1…358]•1º) | 35748 (29,5º, 179,5º) |
| 35029 (28,5º, -179,5º) | 35030…35387 (28,5º, -179,5º +[1…358]•1º) | 35388 (28,5º, 179,5º) |
| 34669 (27,5º, -179,5º) | 34670…35027 (27,5º, -179,5º +[1…358]•1º) | 35028 (27,5º, 179,5º) |
| 34309 (26,5º, -179,5º) | 34310…34667 (26,5º, -179,5º +[1…358]•1º) | 34668 (26,5º, 179,5º) |
| 33949 (25,5º, -179,5º) | 33950…34307 (25,5º, -179,5º +[1…358]•1º) | 34308 (25,5º, 179,5º) |
| 33589 (24,5º, -179,5º) | 33590…33947 (24,5º, -179,5º +[1…358]•1º) | 33948 (24,5º, 179,5º) |
| 33229 (23,5º, -179,5º) | 33230…33587 (23,5º, -179,5º +[1…358]•1º) | 33588 (23,5º, 179,5º) |
| 32869 (22,5º, -179,5º) | 32870…33227 (22,5º, -179,5º +[1…358]•1º) | 33228 (22,5º, 179,5º) |
| 32509 (21,5º, -179,5º) | 32510…32867 (21,5º, -179,5º +[1…358]•1º) | 32868 (21,5º, 179,5º) |
| 32149 (20,5º, -179,5º) | 32150…32507 (20,5º, -179,5º +[1…358]•1º) | 32508 (20,5º, 179,5º) |
| 31789 (19,5º, -179,5º) | 31790…32147 (19,5º, -179,5º +[1…358]•1º) | 32148 (19,5º, 179,5º) |
| 31429 (18,5º, -179,5º) | 31430…31787 (18,5º, -179,5º +[1…358]•1º) | 31788 (18,5º, 179,5º) |
| 31069 (17,5º, -179,5º) | 31070…31427 (17,5º, -179,5º +[1…358]•1º) | 31428 (17,5º, 179,5º) |
| 30709 (16,5º, -179,5º) | 30710…31067 (16,5º, -179,5º +[1…358]•1º) | 31068 (16,5º, 179,5º) |
| 30349 (15,5º, -179,5º) | 30350…30707 (15,5º, -179,5º +[1…358]•1º) | 30708 (15,5º, 179,5º) |
| 29989 (14,5º, -179,5º) | 29990…30347 (14,5º, -179,5º +[1…358]•1º) | 30348 (14,5º, 179,5º) |
| 29629 (13,5º, -179,5º) | 29630…29987 (13,5º, -179,5º +[1…358]•1º) | 29988 (13,5º, 179,5º) |
| 29269 (12,5º, -179,5º) | 29270…29627 (12,5º, -179,5º +[1…358]•1º) | 29628 (12,5º, 179,5º) |
| 28909 (11,5º, -179,5º) | 28910…29267 (11,5º, -179,5º +[1…358]•1º) | 29268 (11,5º, 179,5º) |
| 28549 (10,5º, -179,5º) | 28550…28907 (10,5º, -179,5º +[1…358]•1º) | 28908 (10,5º, 179,5º) |
| 28189 (9,5º, -179,5º) | 28190…28547 (9,5º, -179,5º +[1…358]•1º) | 28548 (9,5º, 179,5º) |
| 27829 (8,5º, -179,5º) | 27830…28187 (8,5º, -179,5º +[1…358]•1º) | 28188 (8,5º, 179,5º) |
| 27469 (7,5º, -179,5º) | 27470…27827 (7,5º, -179,5º +[1…358]•1º) | 27828 (7,5º, 179,5º) |
| 27109 (6,5º, -179,5º) | 27110…27467 (6,5º, -179,5º +[1…358]•1º) | 27468 (6,5º, 179,5º) |
| 26749 (5,5º, -179,5º) | 26750…27107 (5,5º, -179,5º +[1…358]•1º) | 27108 (5,5º, 179,5º) |
| 26389 (4,5º, -179,5º) | 26390…26747 (4,5º, -179,5º +[1…358]•1º) | 26748 (4,5º, 179,5º) |
| 26029 (3,5º, -179,5º) | 26030…26387 (3,5º, -179,5º +[1…358]•1º) | 26388 (3,5º, 179,5º) |
| 25669 (2,5º, -179,5º) | 25670…26027 (2,5º, -179,5º +[1…358]•1º) | 26028 (2,5º, 179,5º) |
| 25309 (1,5º, -179,5º) | 25310…25667 (1,5º, -179,5º +[1…358]•1º) | 25668 (1,5º, 179,5º) |
| 24949 (0,5º, -179,5º) | 24950…25307 (0,5º, -179,5º +[1…358]•1º) | 25308 (0,5º, 179,5º) |
| 24589 (-0,5º, -179,5º) | 24590…24947 (-0,5º, -179,5º +[1…358]•1º) | 24948 (-0,5º, 179,5º) |
| 24229 (-1,5º, -179,5º) | 24230…24587 (-1,5º, -179,5º +[1…358]•1º) | 24588 (-1,5º, 179,5º) |
| 23869 (-2,5º, -179,5º) | 23870…24227 (-2,5º, -179,5º +[1…358]•1º) | 24228 (-2,5º, 179,5º) |
| 23509 (-3,5º, -179,5º) | 23510…23867 (-3,5º, -179,5º +[1…358]•1º) | 23868 (-3,5º, 179,5º) |
| 23149 (-4,5º, -179,5º) | 23150…23507 (-4,5º, -179,5º +[1…358]•1º) | 23508 (-4,5º, 179,5º) |
| 22789 (-5,5º, -179,5º) | 22790…23147 (-5,5º, -179,5º +[1…358]•1º) | 23148 (-5,5º, 179,5º) |
| 22429 (-6,5º, -179,5º) | 22430…22787 (-6,5º, -179,5º +[1…358]•1º) | 22788 (-6,5º, 179,5º) |
| 22069 (-7,5º, -179,5º) | 22070…22427 (-7,5º, -179,5º +[1…358]•1º) | 22428 (-7,5º, 179,5º) |
| 21709 (-8,5º, -179,5º) | 21710…22067 (-8,5º, -179,5º +[1…358]•1º) | 22068 (-8,5º, 179,5º) |
| 21349 (-9,5º, -179,5º) | 21350…21707 (-9,5º, -179,5º +[1…358]•1º) | 21708 (-9,5º, 179,5º) |
| 20989 (-10,5º, -179,5º) | 20990…21347 (-10,5º, -179,5º +[1…358]•1º) | 21348 (-10,5º, 179,5º) |
| 20629 (-11,5º, -179,5º) | 20630…20987 (-11,5º, -179,5º +[1…358]•1º) | 20988 (-11,5º, 179,5º) |
| 20269 (-12,5º, -179,5º) | 20270…20627 (-12,5º, -179,5º +[1…358]•1º) | 20628 (-12,5º, 179,5º) |
| 19909 (-13,5º, -179,5º) | 19910…20267 (-13,5º, -179,5º +[1…358]•1º) | 20268 (-13,5º, 179,5º) |
| 19549 (-14,5º, -179,5º) | 19550…19907 (-14,5º, -179,5º +[1…358]•1º) | 19908 (-14,5º, 179,5º) |
| 19189 (-15,5º, -179,5º) | 19190…19547 (-15,5º, -179,5º +[1…358]•1º) | 19548 (-15,5º, 179,5º) |
| 18829 (-16,5º, -179,5º) | 18830…19187 (-16,5º, -179,5º +[1…358]•1º) | 19188 (-16,5º, 179,5º) |
| 18469 (-17,5º, -179,5º) | 18470…18827 (-17,5º, -179,5º +[1…358]•1º) | 18828 (-17,5º, 179,5º) |
| 18109 (-18,5º, -179,5º) | 18110…18467 (-18,5º, -179,5º +[1…358]•1º) | 18468 (-18,5º, 179,5º) |
| 17749 (-19,5º, -179,5º) | 17750…18107 (-19,5º, -179,5º +[1…358]•1º) | 18108 (-19,5º, 179,5º) |
| 17389 (-20,5º, -179,5º) | 17390…17747 (-20,5º, -179,5º +[1…358]•1º) | 17748 (-20,5º, 179,5º) |
| 17029 (-21,5º, -179,5º) | 17030…17387 (-21,5º, -179,5º +[1…358]•1º) | 17388 (-21,5º, 179,5º) |
| 16669 (-22,5º, -179,5º) | 16670…17027 (-22,5º, -179,5º +[1…358]•1º) | 17028 (-22,5º, 179,5º) |
| 16309 (-23,5º, -179,5º) | 16310…16667 (-23,5º, -179,5º +[1…358]•1º) | 16668 (-23,5º, 179,5º) |
| 15949 (-24,5º, -179,5º) | 15950…16307 (-24,5º, -179,5º +[1…358]•1º) | 16308 (-24,5º, 179,5º) |
| 15589 (-25,5º, -179,5º) | 15590…15947 (-25,5º, -179,5º +[1…358]•1º) | 15948 (-25,5º, 179,5º) |
| 15229 (-26,5º, -179,5º) | 15230…15587 (-26,5º, -179,5º +[1…358]•1º) | 15588 (-26,5º, 179,5º) |
| 14869 (-27,5º, -179,5º) | 14870…15227 (-27,5º, -179,5º +[1…358]•1º) | 15228 (-27,5º, 179,5º) |
| 14509 (-28,5º, -179,5º) | 14510…14867 (-28,5º, -179,5º +[1…358]•1º) | 14868 (-28,5º, 179,5º) |
| 14149 (-29,5º, -179,5º) | 14150…14507 (-29,5º, -179,5º +[1…358]•1º) | 14508 (-29,5º, 179,5º) |
| 13789 (-30,5º, -179,5º) | 13790…14147 (-30,5º, -179,5º +[1…358]•1º) | 14148 (-30,5º, 179,5º) |
| 13429 (-31,5º, -179,5º) | 13430…13787 (-31,5º, -179,5º +[1…358]•1º) | 13788 (-31,5º, 179,5º) |
| 13069 (-32,5º, -179,5º) | 13070…13427 (-32,5º, -179,5º +[1…358]•1º) | 13428 (-32,5º, 179,5º) |
| 12709 (-33,5º, -179,5º) | 12710…13067 (-33,5º, -179,5º +[1…358]•1º) | 13068 (-33,5º, 179,5º) |
| 12349 (-34,5º, -179,5º) | 12350…12707 (-34,5º, -179,5º +[1…358]•1º) | 12708 (-34,5º, 179,5º) |
| 11989 (-35,5º, -179,5º) | 11990…12347 (-35,5º, -179,5º +[1…358]•1º) | 12348 (-35,5º, 179,5º) |
| 11629 (-36,5º, -179,5º) | 11630…11987 (-36,5º, -179,5º +[1…358]•1º) | 11988 (-36,5º, 179,5º) |
| 11269 (-37,5º, -179,5º) | 11270…11627 (-37,5º, -179,5º +[1…358]•1º) | 11628 (-37,5º, 179,5º) |
| 10909 (-38,5º, -179,5º) | 10910…11267 (-38,5º, -179,5º +[1…358]•1º) | 11268 (-38,5º, 179,5º) |
| 10549 (-39,5º, -179,5º) | 10550…10907 (-39,5º, -179,5º +[1…358]•1º) | 10908 (-39,5º, 179,5º) |
| 10189 (-40,5º, -179,5º) | 10190…10547 (-40,5º, -179,5º +[1…358]•1º) | 10548 (-40,5º, 179,5º) |
| 9829 (-41,5º, -179,5º) | 9830…10187 (-41,5º, -179,5º +[1…358]•1º) | 10188 (-41,5º, 179,5º) |
| 9469 (-42,5º, -179,5º) | 9470…9827 (-42,5º, -179,5º +[1…358]•1º) | 9828 (-42,5º, 179,5º) |
| 9109 (-43,5º, -179,5º) | 9110…9467 (-43,5º, -179,5º +[1…358]•1º) | 9468 (-43,5º, 179,5º) |
| 8749 (-44,5º, -179,5º) | 8750…9107 (-44,5º, -179,5º +[1…358]•1º) | 9108 (-44,5º, 179,5º) |
| 8389 (-45,5º, -179,5º) | 8390…8747 (-45,5º, -179,5º +[1…358]•1º) | 8748 (-45,5º, 179,5º) |
| 8029 (-46,5º, -179,5º) | 8030…8387 (-46,5º, -179,5º +[1…358]•1º) | 8388 (-46,5º, 179,5º) |
| 7669 (-47,5º, -179,5º) | 7670…8027 (-47,5º, -179,5º +[1…358]•1º) | 8028 (-47,5º, 179,5º) |
| 7309 (-48,5º, -179,5º) | 7310…7667 (-48,5º, -179,5º +[1…358]•1º) | 7668 (-48,5º, 179,5º) |
| 6949 (-49,5º, -179,5º) | 6950…7307 (-49,5º, -179,5º +[1…358]•1º) | 7308 (-49,5º, 179,5º) |
| 6589 (-50,5º, -179,5º) | 6590…6947 (-50,5º, -179,5º +[1…358]•1º) | 6948 (-50,5º, 179,5º) |
| 6229 (-51,5º, -179,5º) | 6230…6587 (-51,5º, -179,5º +[1…358]•1º) | 6588 (-51,5º, 179,5º) |
| 5869 (-52,5º, -179,5º) | 5870…6227 (-52,5º, -179,5º +[1…358]•1º) | 6228 (-52,5º, 179,5º) |
| 5509 (-53,5º, -179,5º) | 5510…5867 (-53,5º, -179,5º +[1…358]•1º) | 5868 (-53,5º, 179,5º) |
| 5149 (-54,5º, -179,5º) | 5150…5507 (-54,5º, -179,5º +[1…358]•1º) | 5508 (-54,5º, 179,5º) |
| 4789 (-55,5º, -179,5º) | 4790…5147 (-55,5º, -179,5º +[1…358]•1º) | 5148 (-55,5º, 179,5º) |
| 4429 (-56,5º, -179,5º) | 4430…4787 (-56,5º, -179,5º +[1…358]•1º) | 4788 (-56,5º, 179,5º) |
| 4069 (-57,5º, -179,5º) | 4070…4427 (-57,5º, -179,5º +[1…358]•1º) | 4428 (-57,5º, 179,5º) |
| 3709 (-58,5º, -179,5º) | 3710…4067 (-58,5º, -179,5º +[1…358]•1º) | 4068 (-58,5º, 179,5º) |
| 3349 (-59,5º, -179,5º) | 3350…3707 (-59,5º, -179,5º +[1…358]•1º) | 3708 (-59,5º, 179,5º) |

Table 8.45 — GTRS indexing scheme and set members: 71º-60º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 3169 (-60,5º, -179º) | 3170…3347 (-60,5º, -179º +[1…178]•2º) | 3348 (-60,5º, 179º) |
| 2989 (-61,5º, -179º) | 2990…3167 (-61,5º, -179º +[1…178]•2º) | 3168 (-61,5º, 179º) |
| 2809 (-62,5º, -179º) | 2810…2987 (-62,5º, -179º +[1…178]•2º) | 2988 (-62,5º, 179º) |
| 2629 (-63,5º, -179º) | 2630…2807 (-63,5º, -179º +[1…178]•2º) | 2808 (-63,5º, 179º) |
| 2449 (-64,5º, -179º) | 2450…2627 (-64,5º, -179º +[1…178]•2º) | 2628 (-64,5º, 179º) |
| 2269 (-65,5º, -179º) | 2270…2447 (-65,5º, -179º +[1…178]•2º) | 2448 (-65,5º, 179º) |
| 2089 (-66,5º, -179º) | 2090…2267 (-66,5º, -179º +[1…178]•2º) | 2268 (-66,5º, 179º) |
| 1909 (-67,5º, -179º) | 1910…2087 (-67,5º, -179º +[1…178]•2º) | 2088 (-67,5º, 179º) |
| 1729 (-68,5º, -179º) | 1730…1907 (-68,5º, -179º +[1…178]•2º) | 1908 (-68,5º, 179º) |
| 1549 (-69,5º, -179º) | 1550…1727 (-69,5º, -179º +[1…178]•2º) | 1728 (-69,5º, 179º) |
| 1369 (-70,5º, -179º) | 1370…1547 (-70,5º, -179º +[1…178]•2º) | 1548 (-70,5º, 179º) |

Table 8.46 — GTRS indexing scheme and set members: 78º-71º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 1249 (-71,5º, -178,5º) | 1250…1367 (-71,5º, -178,5º +[1…118]•3º) | 1368 (-71,5º, 178,5º) |
| 1129 (-72,5º, -178,5º) | 1130…1247 (-72,5º, -178,5º +[1…118]•3º) | 1248 (-72,5º, 178,5º) |
| 1009 (-73,5º, -178,5º) | 1010…1127 (-73,5º, -178,5º +[1…118]•3º) | 1128 (-73,5º, 178,5º) |
| 889 (-74,5º, -178,5º) | 890…1007 (-74,5º, -178,5º +[1…118]•3º) | 1008 (-74,5º, 178,5º) |
| 769 (-75,5º, -178,5º) | 770…887 (-75,5º, -178,5º +[1…118]•3º) | 888 (-75,5º, 178,5º) |
| 649 ( -76,5º, -178,5º) | 650…767 (-76,5º, -178,5º +[1…118]•3º) | 768 (-76,5º, 178,5º) |
| 529 (-77,5º, -178,5º) | 530…647 (-77,5º, -178,5º +[1…118]•3º) | 648 (-77,5º, 178,5º) |

Table 8.47 — GTRS indexing scheme and set members: 80º-78º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 457 (-78,5º, -177,5º) | 458…527 (78,5º, 177,5º +[1…70]•5º) | 528 (-78,5º, 177,5º) |
| 385 (-79,5º, -177,5º) | 386…455 (79,5º, 177,5º +[1…70]•5º) | 456 (-79,5º, 177,5º) |

Table 8.48 — GTRS indexing scheme and set members: 84º-80º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 325 (-80,5º, -177º) | 326…383 (80,5º, 177º +[1…58]•6º) | 384 (-80,5º, 177º) |
| 265 (-81,5º, -177º) | 266…323 (81,5º, 177º +[1…58]•6º) | 324 (-81,5º, 177º) |
| 205 (-82,5º, -177º) | 206…263 (82,5º, 177º +[1…58]•6º) | 264 (-82,5º, 177º) |
| 145 (-83,5º, -177º) | 146…203 (83,5º, 177º +[1…58]•6º) | 204 (-83,5º, 177º) |

Table 8.49 — GTRS indexing scheme and set members: 86º-84º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 109 (-84,5º, -175º) | 110…143 (84,5º, 175º +[1…34]•10º) | 144 (-84,5º, 175º) |
| 73 (-85,5º, -175º) | 74…107 (85,5º, 175º +[1…34]•10º) | 108 (-85,5º, 175º) |

Table 8.50 — GTRS indexing scheme and set members: 88º-86º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 49 (-86,5º, -172,5º) | 50…71 (86,5º, 172,5º +[1…22]•15º) | 72 (-86,5º, 172,5º) |
| 25 (-87,5º, -172,5º) | 26…47 (87,5º, 172,5º +[1…22]•15º) | 48 (-87,5º, 172,5º) |

Table 8.51 — GTRS indexing scheme and set members: 90º-88º south

| First cell in row | Other cells in row | Last cell in row |
| --- | --- | --- |
| Code True origin | Code True origin | Code True origin |
| 13 (-88,5º, -165º) | 14…23 (88,5º, 165º +[1…10]•30º) | 24 (-88,5º, 165º) |
| 1 (-89,5º, -165º) | 2…11 (89,5º, 165º +[1…10]•30º) | 12 (-89,5º, 165º) |

Table 8.52 — MTM (body) indexing scheme and set members: northern hemisphere

| Code | TDB |
| --- | --- |
| Label | TDB |
| Description | TDB |
| Region | Valid region specification: TDB |
| Parameter values | TDB |

Table 8.53 — MTM (body) indexing scheme and set members: southern hemisphere

| Code | TDB |
| --- | --- |
| Label | TDB |
| Description | TDB |
| Region | Valid region specification: TDB |
| Parameter values | TDB |

Table 8.54 — MTM (pole) indexing scheme and set members: northern hemisphere

| Code | TDB |
| --- | --- |
| Label | TDB |
| Description | TDB |
| Region | Valid region specification: TDB |
| Parameter values | TDB |

Table 8.55 — MTM (pole) indexing scheme and set members: southern hemisphere

| Code | TDB |
| --- | --- |
| Label | TDB |
| Description | TDB |
| Region | Valid region specification: TDB |
| Parameter values | TDB |

Table 8.56 — UPS indexing scheme and set members: northern hemisphere

| Code | 1 |
| --- | --- |
| Label | NORTHERN_POLE |
| Description | UPS, northern pole. |
| Region | Valid region specification: φ ≥ 84º Extended valid region specification: φ ≥ 83,5º |
| Parameter values | Central meridian: λ0 = 180º True origin: (90ºN, 0ºE) |

Table 8.57 — UPS indexing scheme and set members: southern hemisphere

| Code | 1 |
| --- | --- |
| Label | SOUTHERN_POLE |
| Description | UPS, southern pole. |
| Region | Valid region specification: φ ≤ -80º Extended valid region specification: φ ≤ -79,5º |
| Parameter values | Central meridian: λ0 = 0º True origin: (90ºS, 180ºE) |

Table 8.58 — UTM indexing scheme and set members: northern hemisphere

| Code | 1…60 |
| --- | --- |
| Label | “ZONE_” + <code> + “_NORTHERN_HEMISPHERE” |
| Description | UTM Zone <code>, Northern hemisphere. |
| Region | Valid region specification: (-186º + (<code>)•6º) ≤ λ < (-180º + (<code>)•6º) 0º ≤ φ < 84º Extended valid region specification: (-186,5º + (<code>)•6º) ≤ λ < (-179,5º + (<code>)•6º) -0,5º ≤ φ < 84,5º |
| Parameter values | Central meridian: λ0 = (-183º + (<code>)•6º) True origin: (0ºN, λ0) False northing: vF = 0 m. |

Table 8.59 — UTM indexing scheme and set members: southern hemisphere

| Code | 61…120 |
| --- | --- |
| Label | “ZONE_” + <code> + “_SOUTHERN_HEMISPHERE” |
| Description | UTM Zone <code>, Southern hemisphere. |
| Region | Valid region specification: (-186º + (<code>)•6º) ≤ λ < (-180º + (<code>)•6º) -80º ≤ φ < 0º Extended valid region specification: (-186,5º + (<code>)•6º) ≤ λ < (-179,5º + (<code>)•6º) -80,5º ≤ φ < 0,5º |
| Parameter values | Central meridian: λ0 = (-183º + (<code>)•6º) True origin: (0ºN, λ0) False northing: vF = 10 000 000 m. |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

azimuthal 2D SRF	129

binding of a CS via a spatial embedding	117

celestiocentric SRF	122

celestiodetic SRF	125

celestiomagnetic SRF	132

dimension of an ORM	118

equatorial inertial SRF	132

equidistant cylindrical SRF	141

extended valid region	119

extended valid region specification	120

geocentric SRF	123

geodetic SRF	125

geomagnetic SRF	132

heliospheric Aries ecliptic SRF	135

heliospheric Earth ecliptic SRF	135

heliospheric Earth equatorial SRF	136

lambert conformal conic SRF	139

local azimuthal spherical tangent plane SRF	127

local cylindrical tangent plane SRF	130

local space rectangular 2D SRF	124

local space rectangular SRF	123

local tangent plane 3D SRF	126

Mercator SRF	136

oblique Mercator SRF	137

polar 2D SRF	131

polar stereographic SRF	140

solar ecliptic SRF	133

solar equatorial SRF	133

solar magnetic SRF	134

solar magnetospheric SRF	134

spatial reference frame	118

SRF set	146

SRF set member	149

SRF template	121

surface local azimuthal tangent plane SRF	128

transverse Mercator SRF	138

valid region	119

valid region description	119

valid region specification	119