EDITORS NOTE:  Table of contents tables will be removed from individual clauses.  The TOC below is for draft review purposes only.

3 Terms, definitions, symbols, and abbreviated terms 5 3.1 Notation 5 3.2 Terms and definitions 5 3.3 Symbols and abbreviated terms 6

Table 3.1 — Mathematical notation 5 Table 3.2 — Symbols 6 Table 3.3 — Abbreviated terms 7

# Terms, definitions, symbols, and abbreviated terms

## Notation

Table 3.1 lists the mathematical notation conventions used in this document.

Table 3.1 — Mathematical notation

| Style | Use | Examples |
| --- | --- | --- |
| lower case, bold, italic | points, vectors | x, p |
| lower case, italic | variables, scalars, real-valued functions, axes of a linear coordinate system | a, b, f, x (x-axis) |
| upper case, bold, italic | vector-valued functions, matrices | F, G, M |
| upper case, italic | sets | S, T |

Terms defined in the body of this document are presented in italics at the point where they are defined.  The purpose of this notation is to aid readers in determining those sentences that contain the definitions of terms.

## Terms and definitions

For the purposes of this document, the following terms and definitions apply. Symbols or abbreviations used as representations for the term are listed immediately following the term. After the definition, where necessary, an additional clarifying note may be provided.

3.3.1

barycentre

centre of mass of a system of bodies [SEID]

3.3.2

celestial sphere

imaginary sphere of arbitrary radius upon which spatial objects may be considered to be located [SEID]

NOTE: as circumstances require, the celestial sphere may be centred at the observer, at the centre of the Earth, or at any other location.

3.3.3

celestiocentric

located with respect to a centre of a spatial object

3.3.4

celestiodetic

approximating the surface of a spatial object as an oblate spheroid (or sphere) reference surface

3.3.5

coordinate

one of an ordered set of n numbers designating a position within a n-dimensional space [EDM]

3.3.6

geodetic azimuth

at a point P, the angle between the geodesic to point Q and the surface curve defined by the meridian through P

3.3.7

ecliptic plane

plane of the orbit of the planet

3.3.8

spatial object

real or virtual object to which spatial information applies

3.3.9

spatial operation

mathematical function that re-expresses coordinates, directions, and/or distances expressed in one spatial reference frame in terms of a different spatial reference frame

## Symbols and abbreviated terms

Table 3.2 lists the symbols used in this document.

Table 3.2 — Symbols

| Symbol | Definition |
| --- | --- |
| a | major semi axis length of an oblate spheroid |
| b | minor semi axis length of an oblate spheroid |
| h | celestiodetic height |
| k0 | map scale factor |
| Ν | geoidal separation at a point |
|  | convergence of the meridian |
|  | celestiodetic latitude |
|  | celestiodetic longitude |
|  | spherical latitude |

Table 3.3 lists the abbreviated terms used in this document.

Table 3.3 — Abbreviated terms

| Abbreviation | Full form |
| --- | --- |
| 1D | one Dimensional |
| 2D | two Dimensional |
| 3D | three Dimensional |
| AFWA | Air Force Weather Agency (US) |
| API | Application Program Interface |
| BCE | Before the Common Era |
| CLTP | Canonical Local Tangent Plane |
| COAMPS | Coupled Ocean/Atmospheric Mesoscale Prediction System |
| CS | Coordinate System |
| DGIWG | Digital Geographic Information Working Group |
| DIGEST | DIgital Geographic information Exchange STandard |
| DR | Dictionary Reference |
| E | East |
| EGM | Earth Gravity Model |
| ERM | Earth Reference Model |
| ETRS | European Terrestrial Reference System |
| FRG | Federal Republic of Germany |
| GCS | Global Coordinate System |
| GDA | Geodetic Datum of Australia |
| GGRS | Greek Geodetic Reference System |
| GRS | Geodetic Reference System |
| GTRS | Geo Tile Reference System |
| IAU | International Astronomical Union |
| IEC | International Electrotechnical Commission |
| IEEE | Institute of Electrical and Electronics Engineers |
| IAG | International Association of Geodesy |
| IGAC | Instituto Geografico Augustin Cadazzi |
| IGN | Institut Geographique National |
| IR | Informative Reference |
| ISO | International Organization for Standardization |
| ISTS | International Satellite Triangulation Station |
| ITU-R | International Telecommunication Union – Radio-communication |
| JTC | Joint Technical Committee |
| KKJ | KartastoKoordinaattiJarjestelma (Finland) |
| LTP | Local Tangent Plane |
| MM5 | Mesoscale (weather) Model 5 |
| MODTRAN | MODerate resolution TRANsmittance (atmospheric radiation transfer) |
| MSL | Mean Sea Level |
| MTM | Mars Transverse Mercator |
| N | North |
| NAVD | North American Vertical Datum |
| NGVD | National Geodetic Vertical Datum (United States/Canada) |
| NIMA | National Imagery and Mapping Agency (United States) |
| NOGAPS | Navy Operational Global Atmospheric Prediction System (United States) |
| NTF | Nouvelle Triangulation Français (France) |
| ORM | Object Reference Model |
| OSGM | Ordnance Survey Geoid Model |
| PR | Prescriptive Reference |
| RD | Reference Datum |
| RDC | Reference Datum Class |
| RDI | Reference Datum Class Instance |
| RGF | Reseau Géodésique Français (France) |
| S | South |
| SI | Système International d'unités (International System of Units) |
| SPCS | State Plane Coordinate System (United States) |
| SRF | Spatial Reference Frame |
| SRFS | Spatial Reference Frame Set |
| SRFT | Spatial Reference Frame Template |
| SRM | Spatial Reference Model |
| TAI | Temps Atomique International (International Atomic Time) |
| TDS | Test Data Set |
| TRDS | Test Reference Data Set |
| UK | United Kingdom |
| UPS | Universal (earth) Polar Stereographic |
| US | United States |
| UT | Universal Time |
| UT0 | Universal Time observed |
| UT1 | Universal Time polar motion corrected |
| UT2 | Universal Time Earth rotation corrected |
| UTC | Coordinated Universal Time |
| UTM | Universal (earth) Transverse Mercator |
| W | West |
| WGS | World Geodetic System |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

barycentre	6

celestial sphere	6

celestiocentric	6

celestiodetic	6

coordinate	6

ecliptic plane	6

geodetic azimuth	6

spatial object	6

spatial operation	6