EDITORS NOTE:  Table of contents tables will be removed from individual clauses. The TOC below is for draft review purposes only.

5 Coordinate systems 25 5.1.1 Introduction 25 5.2 Abstract coordinate systems 25 5.2.1 Preliminaries 25 5.2.2 Abstract CS 25 5.2.3 CS types 27 5.2.4 Coordinate surfaces, induced surface CSs, and coordinate curves 28 5.2.4.1 Introduction 28 5.2.4.2 Coordinate surfaces and induced surface CSs 28 5.2.4.2.1 Coordinate curves 29 5.2.5 CS properties 30 5.2.5.1 Linearity 30 5.2.5.2 Orthogonality 31 5.2.5.3 Linear CS properties: rectangular, Cartesian, and orthonormal 31 5.2.6 CS localization 31 5.3 Map projection coordinate systems 33 5.3.1 Map projections 33 5.3.2 Map projection as a surface CS 33 5.3.3 Map projection geometry 34 5.3.3.1 Introduction 34 5.3.3.2 Conformal map projections 34 5.3.3.3 Scale factor and point scale 35 5.3.3.4 Map scale and map distance 35 5.3.3.5 Map azimuth 35 5.3.3.6 Convergence of the meridian 35 5.3.4 Relationship to projection functions 35 5.3.5 Map projection CS common parameters 37 5.3.5.1 False origin 37 5.3.5.2 Standard latitude, latitude of origin and central scale 37 5.3.6 Augmented map projections 38 5.3.6.1 Augmentation with ellipsoidal height 38 5.3.6.2 Distortion in augmented map projections 38 5.4 CS specifications 39

Table 5.1 — CS types 28 Table 5.2 — Localization operators 31 Table 5.3 — Localization inverse operators 32 Table 5.4 — Localized CS type relationships 32 Table 5.5 — Coordinate system specification fields 39 Table 5.6 — Common parameters and functions of an oblate spheroid 40 Table 5.7 — CS specification tables 41 Table 5.8 — Euclidean 3D 41 Table 5.9 — Lococentric Euclidean 3D 43 Table 5.10 — Spherical 44 Table 5.11 — Lococentric spherical 45 Table 5.12 — Azimuthal spherical 47 Table 5.13 — Lococentric azimuthal spherical 48 Table 5.14 — Geodetic 3D 49 Table 5.15 — Cylindrical 52 Table 5.16 — Lococentric cylindrical 53 Table 5.17 — Surface geodetic 55 Table 5.18 — Lococentric surface Euclidean 56 Table 5.19 — Lococentric surface azimuthal 58 Table 5.20 — Lococentric surface polar 59 Table 5.21 — Euclidean 2D 61 Table 5.22 — Lococentric Euclidean 2D 62 Table 5.23 — Azimuthal 63 Table 5.24 — Lococentric azimuthal 64 Table 5.25 — Polar 66 Table 5.26 — Lococentric polar 67 Table 5.27 — Euclidean 1D 68 Table 5.28 — Mercator 69 Table 5.29 — Oblique Mercator 71 Table 5.30 — Transverse Mercator 73 Table 5.31 — Lambert conformal conic 75 Table 5.32 — Polar stereographic 78 Table 5.33 — Equidistant cylindrical 81

Figure 5.1 — Polar CS geometry 27 Figure 5.2 — The polar CS generating function 27 Figure 5.3 — Geodetic CS geometry, coordinate surface, and coordinate curves 30 Figure 5.4 — The generating function of a map projection 34 Figure 5.5 — Polar stereographic map projection 36 Figure 5.6 — Tangent and secant cylindrical map projections 37 Figure 5.7 — Vertical distortion 39

# Coordinate systems

### Introduction

This International Standard specifies coordinate systems used to identify positions. Each coordinate system has a type and other properties. Abstract coordinate systems are defined for positions in abstract Euclidean space Rm. Embeddings of an abstract Euclidean space Rm into object space combined with abstract coordinate systems of Rm produce spatial coordinate systems as specified in Clause 8. Abstract coordinate systems based on map projections are treated in 5.3.

## Abstract coordinate systems

### Preliminaries

Annex A provides a concise summary of mathematical concepts and specifies the notational conventions used in this International Standard. In particular, the concept of Rn as a vector space, the point-set topology of Rn, and the theory of real-valued functions on Rn are all assumed. Algebraic and analytic geometry, including the concepts of point, line, and plane, are also assumed. This International Standard takes a functional approach to the construction of coordinate systems.

### Abstract CS

An abstract CS is a means of identifying a set positions in an abstract Euclidean space that shall be comprised of:

- a CS domain,

- a generating function, and

- a CS range,

where:

- The CS domain shall be a connected replete domain in the Euclidean space of n-tuples (1  n  m), called the coordinate-space.

- The generating function shall be a one-to-one, smooth, orientation preserving function from  the CS domain onto the CS range.

- The CS range shall be the set of positions in a Euclidean space of dimension m (n  m  3), called the position-space. When n = 2 and m = 3, the CS range shall be a subset a smooth surface. When n = 1 and m = 2 or 3, the CS range shall be a subset an implicitly specified smooth curve.

Note 1     A replete set is a generalization of an open set that allows the inclusion of boundary points. Boundary points are important in the definitions of certain CSs. (See Annex A for the definitions of the terms replete, one-to-one, smooth, smooth surface, smooth curve orientation preserving, and connected.)

An element of the CS domain shall be called a coordinate. In particular, if the domain is a subset of 3D Euclidean space (R3), each coordinate is called a 3D coordinate. If the domain is a subset of 2D Euclidean space (R2), each coordinate is called a 2D coordinate.

The kth-component of a coordinate n-tuple (1  k  n) may be called the kth-coordinate component. Coordinate component is the collective term for any kth-coordinate component.

An element of the CS range shall be called a position.

The coordinate of a position p shall be the unique coordinate whose generating function value is p.

The generating function may be parameterized. The generating function parameters (if any) shall be called the CS parameters .

The inverse of the generating function shall be called the inverse generating function. Since a smooth orientation preserving one-to-one function is invertible and its inverse is also a smooth orientation preserving one-to-one function, a CS may equivalently be defined by specifying the inverse generating function.

Note 3     The generating function of a CS is often specified by an algebraic and/or trigonometric description of a geometric relationship (see 5.2.2 EXAMPLE 1). There are CSs that do not have geometric derivations. The Mercator map projection (see Table 5-26) is specified to satisfy a functional requirement of conformality (see 5.3.3.2) rather than by geometric construction.)

EXAMPLE 1    Polar CS. Considering the polar geometry depicted in Figure 5.1, define a generating function F as:

where:

The CS domain of F in coordinate-space is

|  | . |  |
| --- | --- | --- |

The CS range of F in position-space is

Figure 5.1 — Polar CS geometry

This generating function is illustrated in Figure 5.2. The grey boxes with lighter grey edges in this figure represent the fact that the range in position-space extends indefinitely, and that the domain in coordinate-space extends indefinitely along the R-axis. The dotted grey edges indicate an open boundary. This CS range, CS domain and generating function defines an abstract CS representing polar coordinates defined in mathematics [EDM, “Coordinates”]. The normative definition of the polar CS may be found in Table 5.24.

Figure 5.2 — The polar CS generating function

NOTE 4     In the special case where 1) the CS domain and CS range are both Rn and 2) the function is the identity function, this approach to defining coordinate systems reduces to the usual definition of coordinates on Rn where each point is identified by an n-tuple of real numbers [EDM, “coordinates”] (see Table 5.8 and Table 5.21).

### CS types

The coordinate-space and position-space dimensions characterize an abstract CS by type as defined in Table 5.1.

Table 5.1 — CS types

| CS type | Dimension of coordinate-space | Dimension of position-space |
| --- | --- | --- |
| 3D | 3 | 3 |
| surface | 2 | 3 |
| curve | 1 | 3 |
| 2D | 2 | 2 |
| plane curve | 1 | 2 |
| 1D | 1 | 1 |

### Coordinate surfaces, induced surface CSs, and coordinate curves

#### Introduction

The generating function of a 3D CS is a function of the 3 coordinate components of a coordinate 3-tuple. If one of the coordinate components is held fixed to a constant value, the generating function thus restricted to two variables can be viewed as a surface CS generating function (with a surface CS range). If two of the three coordinate components are held fixed, the generating function restricted to one variable can be viewed as a curve CS generating function (with curve CS range). These observations motivate the definitions of coordinate surfaces and curves.

#### Coordinate surfaces and induced surface CSs

If F is the generating function of a 3D CS, and u = (u0, v0, w0) is in the interior of the CS domain D, then three surface CS generating functions are defined by:

|  | , |  |
| --- | --- | --- |
|  | , and |  |
|  | . |  |

The CS domain for S1 is the connected component of which contains (v0, w0). 
The CS domain for S2 is the connected component of which contains (u0, w0).
The CS domain for S3 is the connected component of which contains (u0, v0).

Each of these surface CSs shall be called, respectively, the 1st, 2nd, and 3rd surface CS induced by F at u.

The CS ranges of these surface CSs are, respectively, the 1st, 2nd, and 3rd coordinate surface at u.

EXAMPLE 1    Coordinate surface: The geodetic 3D CS with generating function is specified in Table 5.13 with CS parameters a and b. The 3rd coordinate surface at is the surface the oblate spheroid with major semi-axis a and minor semi-axis b.

EXAMPLE 2    Induced surface CS: The surface geodetic CS is specified in Table 5.16. Its CS domain, CS range and generating function is identical to the 3rd surface CS induced by the geodetic 3D generating function at . In effect, if h is replaced by 0 in the formulae for the generating and inverse generating functions of the geodetic 3D CS, they reduce to the surface geodetic formulae.

##### Coordinate curves

Coordinate curves are defined for CSs of type 3D, surface, and 2D.

The 3D CS case:

If F is the generating function of a 3D CS defined on CS domain D in coordinate-space and u = (u0, v0, w0) is in the interior of the CS domain D, then the 1st, 2nd, and 3rd coordinate curve at u is parametrically specified, respectively, by the following smooth functions:

The domain for C1 is the connected component of which contains u0. 
The domain for C2 is the connected component of which contains v0.
The domain for C3 is the connected component of which contains w0.

NOTE    The intersection of two coordinate surfaces at u is (the locus of) a coordinate curve:  C1 = S2S3, C2 = S1S3, and C3 = S2S1.

The surface and 2D case:

If F is the generating function of a surface or 2D CS defined on CS domain D in coordinate-space and u = (u0, v0) is in the interior of the CS domain D, then the 1st and 2nd coordinate curve at u is parametrically specified, respectively, by the following smooth functions:

The domain for C1 is the connected component of which contains u0. 
The domain for C2 is the connected component of which contains v0.

EXAMPLE    If u = (r0, 0) is in the interior of the CS domain of the polar CS generating function F of 5.2.2 Example 1, then the 1st-coordinate curve is C1(  ) = F(r0, ) = (r0 cos( ), r0 sin( ) ), and the 2nd-coordinate curve is C2( r ) = F(r, 0) = (r cos(0), r sin(0)).

If F is the generating function for the geodetic 3D CS or the surface geodetic CS, and in the 2D case or in the surface case, then (see Figure 5.3):

- the 1st-coordinate curve at u shall be called the meridian at u, and

- the 2nd-coordinate curve at u shall be called the parallel at u.

The meridian at shall be called the prime meridian.

The parallel at shall be called the equator.

Figure 5.3 — Geodetic CS geometry, coordinate surface, and coordinate curves

### CS properties

#### Linearity

A CS with generating function F is a linear CS if F is an affine function. The CS domain of a linear coordinate system is all of the coordinate-space .

A curvilinear CS is a non-linear CS.

EXAMPLE 2    The polar CS of 5.2.2 Example 1 is a 2D curvilinear CS.

#### Orthogonality

A CS of type 3D, surface, or 2D is orthogonal if given any coordinate u in the interior of the CS domain of the generating function, the angle between any two coordinate curves at u is a right angle.

#### Linear CS properties: rectangular, Cartesian, and orthonormal

In a linear CS, the kth-coordinate curve is a (straight) line. The kth-coordinate curve at the origin 0 of a linear CS is the kth-axis.

In a linear CS, if the angles between coordinate curves at the origin, 0, are (pair-wise) right angles, then that is the case at all points. In particular, a linear CS is orthogonal if the axes are orthogonal.

In some publications, a Cartesian CS is defined the same way as an orthogonal linear CS. This International Standard, however, defines this concept differently. A linear CS with generating function F is a Cartesian CS if  (i.e., the points in position-space corresponding to the canonical basis of coordinate-space are all one unit distant from F(0)).

An orthonormal CS is a linear CS that is both orthogonal and Cartesian.

A 3D CS with generating function F is orientation preserving if the Jacobian determinant of F is positive.

EXAMPLE 3    The Lococentric Euclidean 3D CS specified in Table 5.9 is an orientation preserving orthonormal CS.

### CS localization

In some applications of a CS in the context of a spatial reference frame (see Clause 8) it is necessary to consider a displaced and/or rotated version of a CS. To consider such a CS as the CS of a spatial reference frame a generating function for the displaced and rotated version of the CS must be specified. These modified versions of CS generating functions can be specified in a uniform way through the process of localization define below.

Three parameterized operators, called localization operators, that operate on or between position-spaces are defined in Table 5.2 and their inverse operators are specified in Table 5.3.

Table 5.2 — Localization operators

| Localization operator | Domain | Range | Localization parameters | Operator definition |
| --- | --- | --- | --- | --- |
|  |  |  | q, r, s, in r and s are orthonormal |  |
|  |  |  | q, r, s, in r and s are orthonormal |  |
|  |  |  | q, r, s, in r and s are orthonormal |  |

Table 5.3 — Localization inverse operators

| Localization operator | Inverse operator definition |
| --- | --- |

There are several forms of CS localization depending on CS type and localization operator. A 3D or surface CS with generating function FC is localized by composing FC with the localization operator. The localized CS is of the same CS type (3D or surface, respectively). Its generating function is  and has the same CS domain as FC.

There are two localization operations for a 2D CS. One uses localization parameters in and produces a surface CS. The other uses localization parameters in and produces a 2D CS.

- A 2D CS with generating function FC is localized by composing F with the localization operator. The localized CS is a surface CS. Its generating function is  and has the same CS domain as FC.

- A 2D CS with generating function FC is localized by composing F with the localization operator. The localized CS is a surface CS. Its generating function is  and has the same CS domain as FC.

The localization operator parameter, q, shall be called the lococentre. A localized CS may be called a lococentric CS.

NOTE     CS localization preserves the following CS properties: linear/curvilinear, orthogonal, Cartesian, orthonormal.

The relationship between a CS type and its localized version(s) is summarized in Table 5.4.

Table 5.4 — Localized CS type relationships

| CS type | Localization operator | Lococentric CS type |
| --- | --- | --- |
| 3D |  | 3D |
| Surface |  | Surface |
| 2D |  |  |
|  |  | 2D |

## Map projection coordinate systems

### Map projections

Map projections are 2D models of a 3D curved surface. A map projection (MP) is comprised of

- a connected region of the surface of an oblate spheroid,

- a generating projection, and

- an MP range in 2D coordinate space,

where:

- the MP range is a connected replete set, and

- the generating projection is one-to-one from the region of the oblate spheroid onto its MP range and its inverse function is smooth and orientation preserving.

NOTE 1:     This definition may be generalized to any ellipsoid including tri-axial ellipsoids, but this International Standard only addresses map projections for oblate spheroids.

NOTE 2:     The domain of a map projection is always a proper sub-set of the oblate spheroid surface.

The generating projection, P, is specified in terms of surface geodetic CS coordinates. The component functions, P1 and P2, of the generating projection, P, are called the mapping equations:

where:

The MP range coordinate components, u and v, are called easting and northing respectively. The positive direction of the u-axis (the easting axis) is called map-east. The positive direction of the v-axis (the northing axis) is called map-north.

The inverse mapping equations are the component functions Q1 and Q2, of the inverse generating projection :

### Map projection as a surface CS

If the inverse generating projection of a map projection, Q, is composed with the surface geodetic CS generating function, , the resulting function, , is the generating function of a surface CS (see Figure 5.4). The CS domain is the MP range. In this International Standard, a map projection CS shall be surface CS for which the generating function is (implicitly) specified in terms of map projection mapping equations.

Figure 5.4 — The generating function of a map projection

### Map projection geometry

#### Introduction

In general, the Euclidean geometry that a surface CS 2D coordinate-space inherits from has no direct significance with the respect to the geometry of position-space (for example, the Euclidean distance between two surface geodetic coordinate tuples has no obvious meaning in position space). This is not the case for a map projection CS. The map projection CS generating projections specified in this International Standard are designed so that one or more geometric aspects of the MP region of the oblate spheroid are approximated or modelled by the corresponding aspect in coordinate-space. The length of the line segment between two map coordinates is related to the length of the corresponding surface curve. Similarly, directions, areas, the angles between two intersecting curves, and shapes are related approximately or exactly to the corresponding geometric aspect on the oblate spheroid surface. The extent to which these aspects are or are not closely related is an indication of distortion. Some map projection CSs are designed to eliminate distortion for one geometric aspect (for example, angles or area). Others are designed to reduce distortion for several geometric aspects. In general, distortion tends to increase with the size of the oblate spheroid region relative to the total oblate spheroid surface area. If a sufficiently small part of the oblate spheroid is involved, relevant surface geometric properties can be represented approximately so that distortions are small. Map projections specified in this International Standard in the context of an SRF may have areas of definition beyond which the projection should not be used for some application domains due to unacceptable distortion. No map projection CS can eliminate all distortion.

#### Conformal map projections

A conformal map projection preserves angles. If two surface curves lying on an oblate spheroid meet at an angle of , then in a conformal projection the image of those curves in the map coordinate-space meet at the same angle, . [THOM]. In addition, [THOM] contains a derivation based on the theory of complex variables to obtain conditions that specify when a projection is conformal. Most map projections specified in this International Standard are conformal.

NOTE:     The conformal property is local. A conformal map projection preserves angles at a point, but does not necessarily preserve shapes. For example, a projected triangle may appear distorted under a conformal map projection.

#### Scale factor and point scale

The scale factor is used in precise direction and distance calculations when using a map projection CS. The scale factor of a map projection CS at a coordinate is the ratio of coordinate-space arc length along a differentially small line in the coordinate-space with the coordinate as an end-point of the arc to the corresponding position-space arc length. Scale factor depends on both the coordinate (or equivalently, the location of the corresponding point) and on the direction of the line along which arc length is being measured.

If the scale factor of a map projection is independent of the direction of the line and depends only on the location of the point, it is a point scale. In a conformal map projection CS, the scale factor is a point scale.

#### Map scale and map distance

Map scale is a global approximation for converting map distance in coordinate-space (that is "on the map sheet") to geodesic distance on the oblate spheroid, and the value is usually published on the map sheet. Conceptually, map scale is the ratio of the Euclidean distance between two points in coordinate-space to the geodesic distance between the two corresponding positions on the oblate spheroid. Although map scale varies according to the choice of the two points, it is conventional to assign a single (nominal or principal) value to denote the scale of the entire map. The map scale represents a single nominal value for range of scale factor values that occur. This value is customarily selected to be the exact map scale value along at least one line connecting two points on the map sheet.

Usually, map scale is expressed by a ratio 1:n, so that the larger the map scale, the smaller the value of n. For maps of scale greater than 1:100 000 (that is, n < 100 000), map scale changes only slightly over a map sheet, and map scale approximates the ratio of distance on the map to distance in the domain(s) of the generating projection for all locations on the map sheet.

Map distance is defined the Euclidean distance (in coordinate-space) between two coordinates divided by the map scale. Map distance approximates the geodesic distance between the corresponding points on the oblate spheroid. Some MP generating projections are designed so that map distance is exact along lines connected to a designated point or in a designated direction.

EXAMPLE     Map distance is exact for lines running in the north/south direction in the case of the equidistant cylindrical MP specified in Table 5.33.

#### Map azimuth

In a map projection CS, the map azimuth from a coordinate point p1 to a coordinate point p2 is defined as the angle from the v-axis (map-north) to the (straight) line connecting p1 to p2. In general, the map azimuth will differ in value from the geodetic azimuth of the corresponding points on the oblate spheroid.

EXAMPLE     If p2 is directly map-north of p1 (it has a larger v-coordinate component), then the map azimuth is zero, but the geodetic azimuth may not be zero.

#### Convergence of the meridian

Given a point in the interior of the domain of a generating projection, the meridian through that point is projected to a curve in coordinate-space that passes through the corresponding coordinate. The angle at the coordinate between the curve and the northing axis direction is called the convergence of the meridian.

### Relationship to projection functions

Projection functions are defined in A.8. In some cases, the generating projection of a map projection CS is derived from a projection function. The derivation involves two steps. The first step is to restrict the projection function to a specified region of a given oblate spheroid so that the restricted function is one-to-one. The range of a projection function is a surface in 3D position-space. The second step is associate the surface of the range to 2D coordinate-space without introducing additional distortions.

In the case planar projection functions, including the orthographic, perspective, and stereographic projection functions, the range is in a plane that can be identified with 2D coordinate-space by selecting an origin and unit axis points.

In the case of the cylindrical and conic projection functions, the range surface is a cylinder or a cone, respectively. These surfaces are developable surfaces and, except for a line of discontinuity, are homeomorphic to a subset of 2D coordinate-space with a homeomophism that has a Jacobian determinant equal to one. Conceptually, these surfaces can be unwrapped to a flat plane without stretching the surface.

In the case of a spherical surface, the polar stereographic map projection (Table 5.32) is derived from the polar stereographic projection function and the map projection is conformal. The same derivation may be applied to a non-spherical oblate spheroid. However, the resulting map projection will not have the conformal property. For this reason, the generalization of the polar stereographic map projection mapping equations from the spherical case to the non-spherical oblate spheroid case is not derived from the spatial projection function. Instead it is derived analytically to preserve the conformal property. Similarly, the Mercator map projection is not derived from the cylindrical projection function even in the case of a sphere. The Mercator mapping equations are designed to have the conformal property.

EXAMPLE     Polar Stereographic: Given a sphere with a polar point p, the tangent plane to the spheroid at p and the opposite polar point v specify a stereographic planar projection function F (see A.8.2.3). The restriction of F to a subsurface of the spheroid that excludes v, is the generating projection for the spherical case of a polar stereographic map projection. In Figure 5.5 the position s on the sphere is projected to point t on a plane.

Figure 5.5 — Polar stereographic map projection

The use of spatial projections to derive map projections with desirable properties is limited, but does motivate some classifications of map projections. These classifications include tangent and secant map projections as well as conic and cylindrical map projections  [SNYD, p. 5].

A map projection is cylindrical if the generating projection image of:

- all meridians of the oblate spheroid are parallel straight lines and are equally spaced with respect to the longitude of the meridians, and

- all parallels of the oblate spheroid are parallel straight lines and perpendicular to the meridian images.

EXAMPLE 1      The Mercator map projection (Table 5.28) and the equidistant cylindrical map projection (Table 5.33) are both cylindrical map projections.

A cylindrical map projection is tangent if along the equator the point scale is equal to the map scale. It is secant if the point scale is equal to the map scale along two parallels equally spaced from the equator in latitude. In that case the positive latitude is called the latitude of origin. (see 5.3.5.2).

Figure 5.6 — Tangent and secant cylindrical map projections

A map projection is conic if the generating projection image of:

- all meridians of the oblate spheroid are radial straight lines and are equally spaced with respect to the longitude of the meridians, and

- all parallels of the oblate spheroid are concentric arcs and perpendicular to the meridian images.

EXAMPLE 2     Lambert conformal conic (see Table 5.29) is a conic projection.

A conic map projection is tangent if along one parallel the point scale is equal to the map scale. It is secant if the point scale is equal to the map scale along two parallels in the same hemisphere. In that case the latitudes are called standard latitudes. (see Figure to be supplied).

### Map projection CS common parameters

#### False origin

To avoid negative numbers in a region of interest in the coordinate-space of a map projection, it is common practice to add positive offsets to the values. The value added to the easting coordinate n is the false easting. The value added to the northing coordinate v is the false northing. The point with false coordinates is the false origin.

#### Standard latitude, latitude of origin and central scale

The map scale (see 5.3.3.4) is selected to be true on at least one line. In many map projections that line is the projection of a meridian or parallel. In a secant map projection, that line is a standard latitude. Specifying the central scale at the latitude of origin is equivalent to specifying the standard latitude for secant projections. This method generalizes to oblate spheroids and non-projection based map projections. Central scale appears as a CS parameter for several map projections.

The non-negative latitude for which the scale factor is equal to the map scale is called the latitude of origin.

### Augmented map projections

#### Augmentation with ellipsoidal height

A 3D CS can be specified from a map projection. The canonical embedding of a point (u, v) in R2 to the point (u, v, 0) in the uv-plane of R3 allows map points in 2D coordinate-space to be augmented with a third coordinate axis, the w-axis of R3. To be considered as a 3D CS, an augmented 3-tuple (u, v, w) of coordinates in the augmented map projection coordinate-space shall be associated to a unique position in position-space. The association is to ellipsoidal height h = sv w, where sv > 0 is a specified vertical scale factor. Given an augmented coordinate-tuple (u, v, w) for which (u, v) belongs to the coordinate range of the underlying generating projection, the associated position is given in 3D geodetic coordinates (, , h) where (, ) is projected to (u, v) by the map projection mapping equations. The third coordinate-space coordinate w is the vertical coordinate and the 3D geodetic coordinate constraints on negative values of h impose corresponding constraints on allowed values for w.

Augmented map projections inherit the geometry of R3.

#### Distortion in augmented map projections

In addition to the map projection distortions cited in 5.3.3.1, augmentation causes additional distortion. Consider the two straight-line segments between the pairs of coordinate-space points {(u1, v1, 0), (u2, v2, 0)} and {(u1, v1, w), (u2, v2, w)} with w > 0. In augmented map projection geometry, the two line segments have the same length. The corresponding curve in position-space of the first line segment is a surface curve of the oblate spheroid (or sphere). The corresponding second curve is outside of the oblate spheroid (or sphere) and has longer arc length than the first, and the length difference increases with w. (In the case of augmentation with elevation, the curves would additionally parallel geoidal surface undulations).

If the vertical map scale does not equal the (horizontal) map scale, vertical angles at the surface will be skewed. Even a Cartesian augmentation of a conformal map projection will not be (vertically) conformal.

These and other distortions have profound implications for dynamic equations that are beyond the scope of this International Standard.

Figure 5.7 — Vertical distortion

## CS specifications

The CSs specified in this International Standard are presented in Table 5.8 through Table 5.33.  Each CS specification specifies the values of all fields presented in Table 5.5.

Table 5.5 — Coordinate system specification fields

| Field | Specification |
| --- | --- |
| Description | A description of the CS including any common name. |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Function type | Either “generating function” or “map projection”. |
| CS type | One of: 3D linear, 3D curvilinear, surface linear, surface curvilinear, map projection, 2D linear, 2D curvilinear, 1D linear, or 1D curvilinear. |
| Properties | Either “None” or a list of one or more properties of the CS chosen from the following: orthogonal, not orthogonal, orthonormal, not orthonormal, conformal, or not conformal. Conformal and not conformal only apply to map projections. |
| CS parameters and constraints | The parameters (if any) that control aspects of the CS and constraints on how those parameters interrelate. |
| Coordinates | Coordinate symbols and common names. |
| Domain of the generating function or mapping equations | The domain of the CS generating function or mapping equations. |
| Generating function or mapping equations | The CS generating function or mapping equations. |
| Domain of the inverse of the generating function or mapping equations | The domain of the inverse of the CS generating function or mapping equations. |
| Inverse of the generating function or mapping equations | The inverse of the CS generating function or mapping equations. |
| Figure(s) | Zero or more figure(s) that explain and illustrate the CS. |
| Notes | Additional, non-normative information concerning the CS. |
| Reference type | The reference type (see 12.2.5). |
| References | The references (see 12.2.5). |

Several specified CS generating functions and mapping equations and/or their inverses use some common intermediate functions or parameters associated with oblate spheroids.  For clarity and concise presentation, these functions and parameters are defined in Table 5.6.

Table 5.6 — Common parameters and functions of an oblate spheroid

| Function or parameter | Symbol and defining expression |
| --- | --- |
| major semi-axis |  |
| minor semi-axis |  |
| flattening |  |
| (first) eccentricity | alternative equivalent expression: |
| second eccentricity |  |
| radius of curvature in the prime vertical |  |
| radius of curvature in the meridian | alternative equivalent expression: |
| meridional distance to equator |  |

Table 5.7 presents the CS specifications in this International Standard.  Each CS is specified in a separate table that can be accessed through the hyperlink in the Table number column. Additional CS can be added by registration.

Table 5.7 — CS specification tables

| Function type | CS type | Description | Table number |
| --- | --- | --- | --- |
| Generating function | 3D | Euclidean 3D | Table 5.8 |
|  |  | Lococentric Euclidean 3D | Table 5.9 |
|  |  | Spherical | Table 5.10 |
|  |  | Lococentric spherical | Table 5.11 |
|  |  | Azimuthal spherical | Table 5.12 |
|  |  | Lococentric azimuthal spherical | Table 5.13 |
|  |  | Geodetic 3D | Table 5.14 |
|  |  | Cylindrical | Table 5.15 |
|  |  | Lococentric cylindrical | Table 5.16 |
|  | Surface | Surface geodetic | Table 5.17 |
|  |  | Lococentric surface Euclidean | Table 5.18 |
|  |  | Lococentric surface azimuthal | Table 5.19 |
|  |  | Lococentric surface polar | Table 5.20 |
|  | 2D | Euclidean 2D | Table 5.21 |
|  |  | Lococentric Euclidean 2D | Table 5.22 |
|  |  | Azimuthal | Table 5.23 |
|  |  | Lococentric azimuthal | Table 5.24 |
|  |  | Polar | Table 5.25 |
|  |  | Lococentric polar | Table 5.26 |
|  | 1D | Euclidean 1D | Table 5.27 |
| Map projection | Surface and augmented-3D | Mercator | Table 5.28 |
|  |  | Oblique Mercator | Table 5.29 |
|  |  | Transverse Mercator | Table 5.30 |
|  |  | Lambert conformal conic | Table 5.31 |
|  |  | Polar stereographic | Table 5.32 |
|  |  | Equidistant cylindrical | Table 5.33 |

Table 5.8 — Euclidean 3D

| Field | Specification |
| --- | --- |
| Description | Euclidean 3D. |
| Label | CS_3D_EUCLIDEAN |
| Code | 1 |
| Function type | Generating function. |
| CS type | 3D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | None. |
| Coordinates | u, v, w |
| Domain of the generating function or mapping equations | R3 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R3 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Coordinate-space 3-tuples are identified with position-space 3-tuples. |
| Reference type | IR |
| References | [EDM] |

Table 5.9 — Lococentric Euclidean 3D

| Field | Specification |
| --- | --- |
| Description | Localized Euclidean 3D |
| Label | CS_3D_LOCOCENTRIC_EUCLIDEAN |
| Code | 2 |
| Function type | Generating function. |
| CS type | 3D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates | u, v, w |
| Domain of the generating function or mapping equations | R3 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R3 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - Euclidean 3D CS is a special case with . - The generating function is the composition of the generating function for Euclidean 3D CS (see Table 5.8) with the 3D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.10 — Spherical

| Field | Specification |
| --- | --- |
| Description | Spherical. |
| Label | CS_3D_SPHERICAL |
| Code | 3 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | None |
| Coordinates |  : longitude in radians,  : spherical latitude in radians,  : radius |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | A spherical CS may not be associated with a specific sphere. |
| Reference type | IR |
| References | [HCP] |

Table 5.11 — Lococentric spherical

| Field | Specification |
| --- | --- |
| Description | Localized spherical. |
| Label | CS_3D_LOCOCENTRIC_SPHERICAL |
| Code | 4 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates |  : longitude in radians,  : spherical latitude in radians,  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for spherical CS (see Table 5.10) with the 3D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.12 — Azimuthal spherical

| Field | Specification |
| --- | --- |
| Description | Azimuthal spherical. |
| Label | CS_3D_AZIMUTHAL_SPHERICAL |
| Code | 5 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | None. |
| Coordinates |  : azimuth in radians,  : depression/elevation angle in radians,  : radius or range |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | None. |
| Reference type | IR |
| References | [EDM] |

Table 5.13 — Lococentric azimuthal spherical

| Field | Specification |
| --- | --- |
| Description | Localized azimuthal spherical. |
| Label | CS_3D_LOCOCENTRIC_AZIMUTHAL_SPHERICAL |
| Code | 6 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates |  : azimuth in radians,  : depression/elevation angle in radians,  : radius or range |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for azimuthal spherical (see Table 5.11) with the 3D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.14 — Geodetic 3D

| Field | Specification |
| --- | --- |
| Description | Geodetic 3D. |
| Label | CS_3D_GEODETIC |
| Code | 7 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | a: major semi-axis length b: minor semi-axis length Constraints: a > b: (oblate spheroid) a = b: (spherical) |
| Coordinates |  : longitude in radians,  : latitude in radians ( in the spherical case), h : ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The surface geodetic CS (Table 5.17) is induced on the 3rd coordinate surface at any point for which h = 0. |
| Reference type | IR |
| References | [HEIK] |

Table 5.15 — Cylindrical

| Field | Specification |
| --- | --- |
| Description | Cylindrical. |
| Label | CS_3D_CYLINDRICAL |
| Code | 8 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | None. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | None. |
| Reference type | IR |
| References | [EDM] |

Table 5.16 — Lococentric cylindrical

| Field | Specification |
| --- | --- |
| Description | Localized cylindrical. |
| Label | CS_3D_LOCOCENTRIC_CYLINDRICAL |
| Code | 9 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | See Table 5.14. |
| Notes | 1. The generating function is the composition of the cylindrical generating function (see Table 5.14) with 3D localization operator (see 5.2.6). 2. The domain of the inverse of the generating function is all points not on the axis of the cylinder. |
| Reference type | IR |
| References | [EDM] |

Table 5.17 — Surface geodetic

| Field | Specification |
| --- | --- |
| Description | Surface geodetic. |
| Label | CS_S3_SURFACE_GEODETIC |
| Code | 10 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Oblate spheroid parameters: a: major semi-axis length b: minor semi-axis length a > b: (Oblate case) a = b: (Spherical case) |
| Coordinates |  : longitude in radians,  : geodetic latitude in radians |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the oblate spheroid (or sphere) surface excluding the pole points. - The geodetic 3D CS (Table 5.14) induces this CS on the 3rd coordinate surface at any point for which h = 0. - If a = b, the geodetic latitude, , coincides with the geocentric latitude . |
| Reference type | IR |
| References | [HEIK] |

Table 5.18 — Lococentric surface Euclidean

| Field | Specification |
| --- | --- |
| Description | Localization of Euclidean 2D CS into 3D position space. |
| Label | CS_S3_LOCOCENTRIC_SURFACE_EUCLIDEAN |
| Code | 11 |
| Function type | Generating function. |
| CS type | Surface linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates | U, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for Euclidean 2D (see Table 5.21) with the surface localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.19 — Lococentric surface azimuthal

| Field | Specification |
| --- | --- |
| Description | Localization of azimuthal CS into 3D position space |
| Label | CS_S3_LOCOCENTRIC_SURFACE_AZIMUTHAL |
| Code | 12 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates |  : azimuth in radians,  : radius or range |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for azimuthal CS (see Table 5.23) with the 3D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.20 — Lococentric surface polar

| Field | Specification |
| --- | --- |
| Description | Localization of polar CS into 3D position space. |
| Label | CS_S3_LOCOCENTRIC_SURFACE_POLAR |
| Code | 13 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for polar CS (see Table 5.25) with the 3D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.21 — Euclidean 2D

| Field | Specification |
| --- | --- |
| Description | Euclidean 2D. |
| Label | CS_2D_EUCLIDEAN |
| Code | 14 |
| Function type | Generating function. |
| CS type | 2D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | None. |
| Coordinates | u, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R2 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Coordinate-space 2-tuples are identified with position-space 2-tuples. |
| Reference type | IR |
| References | [EDM] |

Table 5.22 — Lococentric Euclidean 2D

| Field | Specification |
| --- | --- |
| Description | Localized Euclidean 2D |
| Label | CS_2D_LOCOCENTRIC_EUCLIDEAN |
| Code | 15 |
| Function type | Generating function. |
| CS type | 2D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinates | u, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R2 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - Euclidean 2D CS is a special case with . - The generating function is the composition of the generating function for Euclidean 2D CS (see Table 5.21) with the surface localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.23 — Azimuthal

| Field | Specification |
| --- | --- |
| Description | Azimuthal. |
| Label | CS_2D_AZIMUTHAL |
| Code | 16 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | None. |
| Coordinates |  : azimuth in radians,  : radius or range |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | None. |
| Reference type | IR |
| References | [EDM] |

Table 5.24 — Lococentric azimuthal

| Field | Specification |
| --- | --- |
| Description | Localization of azimuthal CS. |
| Label | CS_2D_LOCOCENTRIC_AZIMUTHAL |
| Code | 17 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for azimuthal CS (see Table 5.23) with the 2D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.25 — Polar

| Field | Specification |
| --- | --- |
| Description | Polar. |
| Label | CS_2D_POLAR |
| Code | 18 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | None. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | None. |
| Reference type | IR |
| References | [EDM] |

Table 5.26 — Lococentric polar

| Field | Specification |
| --- | --- |
| Description | Localized polar. |
| Label | CS_2D_LOCOCENTRIC_POLAR |
| Code | 19 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinates |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for polar (see Table 5.24) with the 2D localization operator (see 5.2.6). |
| Reference type | IR |
| References | [EDM] |

Table 5.27 — Euclidean 1D

| Field | Specification |
| --- | --- |
| Description | Euclidean 1D. |
| Label | CS_1D_EUCLIDEAN |
| Code | 20 |
| Function type | Generating function. |
| CS type | 1D linear |
| Properties | None. |
| CS parameters and constraints | None. |
| Coordinates | u |
| Domain of the generating function or mapping equations | R1 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R1 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | None. |
| Notes | Coordinate-space 1-tuples are identified with position-space 1-tuples. |
| Reference type | IR |
| References | [EDM] |

Table 5.28 — Mercator

| Field | Specification |
| --- | --- |
| Description | Mercator and augmented Mercator. |
| Label | CS_MP_MERCATOR |
| Code | 21 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2 <  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations | For  functional iteration is used for the functional representation of the inverse map projection[SNYD]. Superscripts involving i indicate elements in the iteration sequence. |
| Figure(s) |  |
| Notes | None. |
| Reference type | IR |
| References | [SNYD] |

Table 5.29 — Oblique Mercator

| Field | Specification |
| --- | --- |
| Description | Oblique Mercator. |
| Label | CS_MP_OBLIQUE_MERCATOR |
| Code | 22 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Conformal. |
| CS parameters and constraints | Applicable only when the oblate spheroid is a sphere (a = b). |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2 <  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | TDB |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | There are several ways of defining a central meridian for the oblique Mercator map projection. The approach taken here lets the user define a central meridian by selecting two points on the sphere that are connected by an arc of a great circle. Using spherical trigonometry this permits the determination of two new poles and an origin located at . |
| Reference type | IR |
| References | [SNYD] |

Table 5.30 — Transverse Mercator

| Field | Specification |
| --- | --- |
| Description | Transverse Mercator and augmented transverse Mercator. |
| Label | CS_MP_TRANSVERSE_MERCATOR |
| Code | 23 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2 <  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | Not geometrically derived. |
| Notes | As noted in [SNYD] and [LLEE] the closed solution for the transverse Mercator forward and inverse conversions were developed by Prof. E. H. Thompson in 1945 and were formally published by L. P. Lee in 1962 [LLEE] with the permission of Professor Thompson. In 1980, J. Dozier published a report that adapted Lee’s results to a computerized solution, including complete program listings in the C language [DOZI]. The formulation used here is further adapted from the work of Dozier to include a central scale factor, a non-zero latitude origin and false easting and false northing offsets for both the easting and northing components. The functional iteration scheme used to determine latitude from the isometric latitude was adapted from [SNYD]. |
| Reference type | IR |
| References | [DOZI], [LLEE], and [SNYD] |

Table 5.31 — Lambert conformal conic

| Field | Specification |
| --- | --- |
| Description | Lambert conformal conic and augmented Lambert conformal conic. |
| Label | CS_MP_LAMBERT_CONFORMAL_CONIC |
| Code | 24 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2 <  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | {(x, y)  R2 ¦ -a < x ≤ a} |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | None. |
| Notes | None. |
| Reference type | IR |
| References | [SNYD] |

Table 5.32 — Polar stereographic

| Field | Specification |
| --- | --- |
| Description | Polar stereographic and augmented polar stereographic. |
| Label | CS_MP_POLAR_STEREOGRAPHIC |
| Code | 25 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2 <  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations | For  functional iteration is used for the functional representation of the inverse map projection [SNYD]. Superscripts involving m indicate elements in the iteration sequence. |
| Figure(s) |  |
| Notes | In the case of a sphere, the mapping equations are derived from the stereographic projection (see the Figure.) The points x and x1 on the sphere are projected to points p and p1 on the plane. |
| Reference type |  |
| References |  |

Table 5.33 — Equidistant cylindrical

| Field | Specification |
| --- | --- |
| Description | Equidistant cylindrical and augmented equidistant cylindrical. |
| Label | CS_MP_EQUIDISTANT_CYLINDRICAL |
| Code | 26 |
| Function type | Mapping equations. |
| CS type | Map projection. |
| Properties | Non-conformal. |
| CS parameters and constraints |  |
| Coordinates | u: easting v: northing Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | {(, ) ¦ - < -origin ≤  and -/2<  < /2} |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | None. |
| Notes | None. |
| Reference type | IR |
| References | [SNYD] |

Editors note: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

2D coordinate	26

3D coordinate	26

abstract CS	25

augmented equidistant cylindrical CS	81

augmented Lambert conformal conic CS	76

augmented map projection	38

augmented Mercator CS	69

augmented polar stereographic CS	78

augmented transverse Mercator CS	73

axis	31

azimuthal CS	63

azimuthal spherical CS	47

Cartesian CS	31

central scale	37

conformal map projection	34

conic map projection	37

convergence of the meridian	35

coordinate	26

coordinate curve	29

coordinate- space	25

CS localization	32

CS parameters	26

CS type	27

curvilinear CS	30

cylindrical CS	52

cylindrical map projection	36

equator	30

equidistant cylindrical CS	81

Euclidean 1D CS	68

Euclidean 2D CS	61

Euclidean 3D CS	42

false easting	37

false northing	37

false origin	37

generating function	25

generating projection	33

geodetic 3D CS	49

inverse generating function	26

kth-coordinate component	26

Lambert conformal conic CS	76

latitude of origin	38

linear CS	30

localization operator	31

Lococentric Euclidean 3D CS	43

Map distance	35

map projection	32

map scale	35

mapping equations	33

Mercator CS	69

meridian	30

oblique Mercator CS	71

orientation preserving orthonormal CS	31

orthogonal CS	31

orthonormal CS	31

parallel (geodetic)	30

point scale	35

polar CS	66

polar stereographic CS	78

position	26

position-space	25

prime meridian	30

scale factor	35

secant conic map projection	37

secant cylindrical map projection	37

spherical CS	44

surface geodetic CS	55

tangent conic map projection	37

tangent cylindrical map projection	37

topocentric azimuthal CS	64

topocentric azimuthal spherical CS	45, 48

topocentric cylindrical CS	53

topocentric Euclidean 2D CS	62

topocentric polar CS	67

topocentric surface azimuthal CS	58

topocentric surface Euclidean CS	56

topocentric surface polar CS	59

transverse Mercator CS	73

The generating function properties and the implicit function theorem imply that for each point in the interior of the CS domain CS range, there is an open neighbourhood of the point whose image under the generating function lies in a smooth surface. This requirement specifies that there exists one smooth surface for all point in the CS domain. The requirement is needed to exclude mathematically pathological cases.

The generating function properties and the implicit function theorem imply that for each point in the interior of the CS domain CS range, there is an open neighbourhood of the point whose image under the generating function lies in a smooth curve. This requirement specifies that there exists one implicitly defined smooth curve for all point in the CS domain. The requirement is needed to exclude mathematically pathological cases.

Geographic information – Spatial referencing by coordinates (DIS 19111) defines this as a coordinate.

? The Geographic information -Positioning services (FDIS 19116) concept of linear reference system is a specialization of the curve CS and plane curve CS concepts.

Geographic information –Spatial referencing by coordinates (DIS 19111) defines meridian as intersection between an ellipsoid and a plane containing the semi-minor axis of the ellipsoid. That definition includes the antipodal meridian and the pole points.

Some publications use “rectangular” to denote an orthogonal linear CS, and oblique to denote a non-orthogonal linear CS.

Geographic information –Spatial referencing by coordinates (DIS 19111) defines Cartesian coordinate system as a coordinate system which gives the position of points relative to n mutually-perpendicular axes.

The proof of this assertion is beyond the scope of this document.