EDITORS NOTE:  Table of contents tables will be removed from individual clauses.  The TOC below is for draft review purposes only.

3 Terms, definitions, symbols, and abbreviated terms 5 3.1 Terms and definitions 5 3.2 Notation, Symbols and abbreviated terms 6

Table 3.1 — Mathematical notation 6 Table 3.2 — Symbols 6 Table 3.3 — Abbreviated terms 7

# Terms, definitions, symbols, and abbreviated terms

## Terms and definitions

For the purposes of this document, the following terms and definitions apply. Symbols or abbreviations used as representations for the term are listed immediately following the term. After the definition, where necessary, an additional clarifying note may be provided. Terms defined in the body of this document are presented in italics at the point where they are defined. The purpose of this notation is to aid readers in determining those sentences that contain the definitions of terms. The Index provides a directory of those terms defined in the body of this International Standard.

3.1.1

Earth Gravitational Model

spherical harmonic expansion of the disturbing potential of the gravitational field

NOTE: rotational effects are not included in this model, gravity includes rotational effects.

3.1.2

ecliptic plane

plane of the orbit of the planet

3.1.3

equatorial plane

plane through a designated centre of an object and normal to the rotational axis of the object

3.1.4

geodetic datum

datum describing the relationship of a coordinate system to the Earth

NOTE	In most cases, the geodetic datum includes an ellipsoid definition.

[ISO 19111]

3.1.5

north pole

The north pole is that pole of rotation that lies on the north side of the invariable plane of the solar system [RIIC].

NOTE 1	Some planets have retrograde rotation with respect to this definition.

NOTE 2	Map north (see Clause 5) may be unrelated to this direction.

3.1.5

spatial object

physical or virtual object to which spatial information applies

3.1.6

spatial operation

mathematical function that re-expresses coordinates, directions, and/or distances expressed in one spatial reference frame in terms of a different spatial reference frame

## Notation, Symbols and abbreviated terms

Table 3.1 lists the mathematical notation conventions used in this document.

Table 3.1 — Mathematical notation

| Style | Use | Examples |
| --- | --- | --- |
| lower case, bold, italic | points, vectors | x, p |
| lower case, italic | variables, scalars, real-valued functions, axes of a linear coordinate system | a, b, f, x (x-axis) |
| upper case, bold, italic | vector-valued functions, matrices | F, G, M |
| upper case, italic | sets | S, T |

Table 3.2 lists the symbols used in this document.

Table 3.2 — Symbols

| Symbol | Definition |
| --- | --- |
| a | major semi axis length of an oblate ellipsoid |
| b | minor semi axis length of an oblate ellipsoid |
| f | flattening (see Table 5.6) |
| h | ellipsoidal height |
| he | elevation |
| k0 | central scale |
| RM | radius of curvature in the meridian (see Table 5.6) |
| RN | radius of curvature in the prime vertical (see Table 5.6) |
| Rn | vector space of n-tuples |
|  | azimuth |
|  | (first) eccentricity (see Table 5.6) |
|  | second eccentricity (see Table 5.6) |
|  | geodetic latitude |
|  | convergence of the meridian |
|  | geodetic or planetodetic longitude |
|  | spherical latitude or depression/elevation angle or cylindrical angle or (polar) angle |
|  | radius or range |
|  | height |

Table 3.3 lists the abbreviated terms used in this document. In the specification of an abbreviation, the letters in the abbreviated term used to form the abbreviation are presented in upper-case and other letters are presented in lower-case with one exception. When the abbreviated term includes another abbreviation that is is used to form the abbreviation for that term, all the letters of the included abbreviation are presented in upper-case even if all of the letters are not used in the formed abbreviation.

Table 3.3 — Abbreviated terms

| Abbreviation | Abbreviated term |
| --- | --- |
| 1D | one Dimensional |
| 2D | two Dimensional |
| 3D | three Dimensional |
| AFWA | Air Force Weather Agency (US) |
| API | Application Program Interface |
| BCE | Before the Common Era |
| CAD | Computer Aided Design |
| CAM | Computer Aided Manufacturing |
| CLTSE | Canonical Local Tangent Space Euclidean |
| COAMPS | Coupled Ocean/Atmospheric Mesoscale Prediction System |
| CPU | Central Processing Unit |
| CS | Coordinate System |
| DBC | Dynamic Binding Category |
| DGIWG | Digital Geographic Information Working Group |
| DGRF | Definitive Geomagnetic Reference Field |
| DIGEST | DIgital Geographic information Exchange STandard |
| DRAM | Dynamic Random Access Memory |
| E | East |
| EC | Equidistant Cylindrical |
| EGM | Earth Gravitational Model |
| EI | Equatorial Inertial |
| ERM | Earth Reference Model |
| ETRS | European Terrestrial Reference System |
| ESA | European Space Agency |
| ESPG | European Petroleum Survey Group |
| FRG | Federal Republic of Germany |
| GCS | Global Coordinate System |
| GDA | Geocentric Datum of Australia |
| GGRS | Greek Geodetic Reference System |
| GIS | Geographic Information System |
| GPS | Global Positioning System |
| GRS | Geodetic Reference System |
| GTRS | Geo Tile Reference System |
| HAEC | Heliospheric Aries ECliptic |
| HEEC | Heliospheric Earth ECliptic |
| HEEQ | Heliospheric Earth EQuatorial |
| IAU | International Astronomical Union |
| IEC | International Electrotechnical Commission |
| IEEE | Institute of Electrical and Electronics Engineers |
| IAG | International Association of Geodesy |
| IGAC | Instituto Geografico Augustin Cadazzi |
| IGLD | International Great Lakes Datum |
| IGN | Institut Geographique National |
| IGRF | International Geomagnetic Reference Field |
| IHO | International Hydrographic Organization |
| ISO | International Organization for Standardization |
| ISTS | International Satellite Triangulation Station |
| ITRF | International Terrestrial Reference Frame |
| ITU-R | International Telecommunication Union – Radio-communication |
| IUGG | International Union of Geodesy and Geophysics |
| JGD | Japan Geodetic Datum |
| JTC | Joint Technical Committee |
| KKJ | KartastoKoordinaattiJarjestelma (Finland) |
| LCC | Lambert Conformal Conic |
| LSR | Local Space Rectangular |
| LTSAS | Local Tangent Space Azimuthal Spherical |
| LTSC | Local Tangent Space Cylindrical |
| LTSE | Local Tangent Space Euclidean SRF |
| M | Mercator |
| MM5 | Mesoscale (weather) Model 5 |
| MODTRAN | MODerate resolution TRANsmittance (atmospheric radiation transfer) |
| MSL | Mean Sea Level |
| MTM | Mars Transverse Mercator |
| N | North |
| NASA | National Aeronautics and Space Administration (United States) |
| NAVD | North American Vertical Datum |
| NGO | Norges Graendsers Oppmaling |
| NGVD | National Geodetic Vertical Datum (United States/Canada) |
| NIMA | National Imagery and Mapping Agency (United States) |
| NOGAPS | Navy Operational Global Atmospheric Prediction System (United States) |
| NTF | Nouvelle Triangulation Français (France) |
| OGC | Open Gis Consortium |
| OGIS | Open Geographic Information Systems |
| OMS | Oblique Mercator Spherical |
| ORM | Object Reference Model |
| ORMT | Object Reference Model Template |
| OSGM | Ordnance Survey Geoid Model |
| POSC | Petroleum Open Systems Consortium |
| PS | PS Polar Stereographic |
| RD | Reference Datum |
| RGF | Reseau Géodésique Français (France) |
| RT | Reference Transformation |
| S | South |
| SEC | Solar Ecliptic |
| SEQ | Solar Equatorial |
| SI | Système International d'unités (International System of Units) |
| SIRGAS | SIstema de Referencia Geocentrico para America del Sur |
| S-JTSK | System - Jednotne Trigonometricke Siti Katastralni |
| SMD | Solar Magnetic Dipole |
| SME | Solar Magnetic Ecliptic |
| SPCS | State Plane Coordinate System (United States) |
| SRF | Spatial Reference Frame |
| SRFS | Spatial Reference Frame Set |
| SRFT | Spatial Reference Frame Template |
| SRM | Spatial Reference Model |
| STOW | Synthetic Theater Of War |
| TAI | Temps Atomique International (International Atomic Time) |
| TDB | Barycentric Dynamical Time |
| TDS | Test Data Set |
| TDT | Terrestrial Dynamic Time |
| TM | Transverse Mercator |
| TRDS | Test Reference Data Set |
| UK | United Kingdom |
| UPS | Universal Polar Stereographic |
| US | United States |
| USGS | United States Geological Survey |
| UT | Universal Time |
| UT0 | Universal Time observed |
| UT1 | Universal Time polar motion corrected |
| UT2 | Universal Time Earth rotation corrected |
| UTC | Coordinated Universal Time |
| UTM | Universal Transverse Mercator |
| W | West |
| WGS | World Geodetic System |
| WMO | World Meteorological Organization |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

Earth Gravitational Model	5

ecliptic plane	5

equatorial plane	5

geodetic datum	5

north pole	5

spatial object	5

spatial operation	5