EDITORS NOTE: Table of contents tables will be removed from individual clauses. The TOC below is for draft review purposes only.

9	Object reference surfaces and vertical offset surfaces	179

9.1	Introduction	179

9.2	Surface models with respect to an ORM	179

9.2.1	Object reference surfaces	179

9.2.2	Equipotential surfaces and geoids	179

9.2.3	Vertical offset surface	180

9.2.4	Geoidal separation	181

9.2.5	Vertical offset height and elevation	181

9.3	Other vertical coordinate definitions	182

9.4	Geiodal and equipotential ORS specifications	182

Table 9.1 — Geoidal and equipotential VOS specification elements	183

Table 9.2 — Geoidal and equipotential VOS specifications	183

Figure 9.1 — Vertical offset surface for ellipsoidal height	180

Figure 9.2 — Vertical offset surface tangent plane	180

Figure 9.3 — Geoidal separation	181

Figure 9.4 — Vertical coordinate with respect to a vertical offset surface	181

Figure 9.5 — Orthometric height and elevation	182

# Object reference surfaces and vertical offset surfaces

## Introduction

Many spatial applications require reference surfaces or other surfaces of interest that are more complex than a surface RD. An RD surface generating function is restricted by definition to be a multi-variate polynomial of degree 2 or less. Other reference surfaces of are typically more complex than surfaces that can be modelled with a second degree RD surface. These surfaces are termed object reference surfaces if they can be modelled with a smooth surface generating function in position-space. These surfaces often model some aspect of physical object-space such as gravity equipotential surfaces.

Real-world measurements of position may include a 3rd-coordinate component value referenced to an object reference surface other than an RD component of an ORM specified in this International Standard. Such reference surfaces, termed vertical offset surfaces, may be specified in terms of an object reference surface. They are not, however, appropriate for direct use as RDs in an ORM. The vertical offset surface concept is an abstraction the concept of vertical datum as used in the field of geodesy. This International Standard specifies how the position of a point in object-space may be specified in part as a distance from a vertical offset surface.

Several SRFTs, including celestiodetic 3D and all the map projection based SRFTs, designate the 3rd coordinate component as the vertical coordinate component. This International Standard specifies how vertical coordinate component values may be adjusted to approximate the distance of a position in object-space to the vertical offset surface.

## Surface models with respect to an ORM

### Object reference surfaces

An object reference surface (ORS) is a smooth surface defined with respect to (the embedding of) an ORM. An ORS specification includes both the smooth surface specification and the ORM specification upon which it is based. An ORS may be used to model an application-specific aspect of the object-space.

Two important cases of ORSs are:

- equipotential surfaces including geoids, and

- models of mean sea level surfaces based on sounding and tidal data.

### Equipotential surfaces and geoids

An equipotential surface of a potential function P defined in (a portion of) position-space is an implicitly defined surface given by, where c is a value in the range of P. If P is a smooth function, the equipotential surface is a smooth surface. If the smooth surface is embedded into object-space with the normal embedding of an ORM it is an ORS.

An important special case of an equipotential surface is a mathematical model of the gravity potential of a celestial body. The geoid is a specific equipotential surface of the Earth’s gravity field that best fits the global mean sea surface in a minimum variance sense. Global, regional, and local approximations of the geoid are developed from empirical measurements in association with specific ERMs. Gravity equipotential surfaces have also been modelled for other planets.

NOTE	The geoid cannot be measured directly. Current models of the Earth’s gravity potential are usually realized as truncated power series in spherical harmonics.

### Vertical offset surface

Some 3D SRFTs specified in Clause 8 designate the 3rd-coordinate as a vertical offset coordinate (see 8.4). An ORS is a vertical offset surface (VOS) for a region of object space with respect to an SRF if each vertical component curve containing a point in the region of intersects the ORS exactly once.

The 3rd-coordinate zero surface is the set of points for which the vertical coordinate component value is zero (see 5.5.2). Given a point p on the 3rd-coordinate zero surface that is in the region of a VOS, the vertical offset at p is the value of the 3rd-coordinate component at the intersection of the VOS with the 3rd-coordinate curve that contains p. The vertical offset at p is denoted v(p).  If p is not in the region of a VOS or if a VOS has not been specified, the vertical offset at p shall be defined to be zero.

NOTE	All points on the same 3rd-coordinate curve have the same offset surface separation.

For a VOS with respect to an SRF based on an oblate ellipsoid, or sphere, ORM, the vertical offset at a point p on the oblate ellipsoid (or sphere) with surface geodetic coordinate  is denoted by .

EXAMPLE 1	If an SRF is derived from SRFT CELESTIODETIC or from a map projection SRFT, the ellipsoidal height coordinate component is the vertical component and  is the distance from the ellipsoid to the VOS along the ellipsoidal height curve at  (see Figure 9.1).

Figure 9.1 — Vertical offset surface for ellipsoidal height

EXAMPLE 2	If an SRF is derived from SRFT LOCAL_TANGENT_SPACE_EUCLIDEAN or SRFT LOCAL_TANGENT_SPACE_CYLINDRICAL, the vertical coordinate is height and the 3rd-coordinate zero surface is a plane. The vertical offset at point p in the plane is the distance from p to the VOS along a line normal to the plane (see Figure 9.2).

p

v(p)

VOS

plane of the vertical coordinate zero surface

Figure 9.2 — Vertical offset surface tangent plane

### Geoidal separation

If the VOS is a geoid, is called the geoidal separation at . (see Figure 9.3). The specification of the geoidal separation is equivalent to the specification of the geoid because the geoid ORS can be constructed from the set of geoidal separation values.

Figure 9.3 — Geoidal separation

NOTE	The geoidal separation with respect to an ERM is often published as a table of values of.

### Vertical offset height and elevation

Given a VOS with respect to an SRF with vertical coordinate component h, the vertical offset height he at point p is defined as he = h - v(p).

EXAMPLE	If the SRF is derived from SRFT CELESTIODETIC and If (, , h) is the coordinate of p, then he = h - v(, ).

If the VOS is a geoid, then he is called the elevation of p with respect to the geoid. Note that in the geoid case, (ellipsoidal height) - (elevation) = v(, ).

NOTE 1	he is an approximation to the distance from p to the VOS. In general, the 3rd-coordinate curve intersection with the VOS is not perpendicular to the VOS. When the intersection is not perpendicular, he does not equal the distance.

NOTE 2	VOS is similar in concept to vertical datum as defined in ISO 19111. ISO 19111 uses the term vertical datum to define a vertical coordinate reference system as part of a (3D) compound coordinate reference system.

Figure 9.4 — Vertical coordinate with respect to a vertical offset surface

## Other vertical coordinate definitions

In addition to vertical offset height (and elevation), different fields of application define other vertical coordinates. These include:

- Orthometric height h0 depends on a gravity model that specifies a potential for each position in position-space. The gradient operator on the geoidal equipotential surface specifies a vector field in position-space. A plumbline is defined to be a curve that follows the gradient vector field (i.e. the tangent vector at a point on the plumbline equals the potential gradient vector at that point). Let q be a position in position-space. The plumbline containing q intersects the geoid at a position p. The orthometric height of q is the plumbline arc length distance from p to q (see Figure 9.5). Note that the tangent to a plumbline at the point where it intersects the geoid is normal to the geoid.

- Other specific functional relationships of the form hm = F(u, v, h), where F is monotonic in h for fixed u and v (or in the geodetic case:  for fixed ).

EXAMPLE	F is standard pressure altitude.

ellipsoid surface

elevation (he)

q

geoid

ellipsoidal

height

(

h

)

(



,



)

plumbline

orthometric height (ho)

= arc length

p

Figure 9.5 — Orthometric height and elevation

NOTE	Terrain elevation is not a standardized term and can have several different definitions and, consequently, different values depending on the application domain. The phrase “height above mean sea level” is ambiguous for several reasons. In addition to the varying definitions/approximations/models of the mean sea level VOS, height can be measured along a straight line normal to the surface or a along a plumbline (plumbline curves may be empirical or depend on the gravity model used). These height measurements are called gravity-related heights in ISO 19111. The Global Positioning System uses ellipsoidal height referenced to the ORM_WGS84 ellipsoid.

## Geiodal and equipotential ORS specifications

The elements of an ORS specification are defined in Table 9.1. This International Standard specifies an ORS set ORS that including geoids and equipotential surfaces for other celestial bodies, in Table 9.2 and Table J.20. Additional geoids or equipotential surfaces may be registered in accordance with Clause 12.

All geoids specified in Table 9.2 are VOSs with respect to any SRF with a specified vertical coordinate component of ellipsoidal height for the intersection of the VOS region and the SRF valid region provided that the SRF is based on the same ORM as the geoid. In general, if the gravity equipotential surface of an ORS and the oblate ellipsoid RD of an ORM have both been selected to approximate the surface of a planet, then the ORS will be a VOS with respect to any SRF with a specified vertical coordinate component of ellipsoidal height based on that ORM.

Table 9.1 — Geoidal and equipotential VOS specification elements

| Element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Description | The published name of the ORS |
| ORM | The label of the ORM. |
| Global/local | Specifies whether the vertical offset surface is only a local vertical offset surface. |
| Notes | Additional, non-normative information. |
| References | The references (see 12.2.5). |

Table 9.2 — Geoidal and equipotential VOS specifications

| Object type: Earth |  |  |  |
| --- | --- | --- | --- |
| Label | EGM96_GEOID | Code | 1 |
| Description | WGS 84 EGM96 geoid |  |  |
| ORM | WGS_1984 | Global/Local | Global |
| Notes | The geopotential surface defined by the WGS 84 EGM96 Earth Gravitational Model that is closely associated with the mean ocean surface. |  |  |
| References | [83502T, Section 6] |  |  |
| Label | IGLD_1955 | Code | 2 |
| Description | International Great Lakes Datum (IGLD) 1955 |  |  |
| ORM | N_AM_1927 | Global/Local | Local |
| Notes | A system of geopotential elevations throughout the Great Lakes region that is based on mean water level at Pointe-au-Pere, Quebec, on the Gulf of St. Lawrence over the period 1941 through 1956. Implemented January 1, 1962. |  |  |
| References | [IGLD79] |  |  |
| Label | IGLD_1985 | Code | 3 |
| Description | International Great Lakes Datum (IGLD) 1985 |  |  |
| ORM | N_AM_1983 | Global/Local | Local |
| Notes | A revision of the International Great Lakes Datum 1955 to address the effects of crustal movement, the development of a common datum between Canada, the United States, and Mexico, new surveying methods, and the deterioration of the zero reference point gauge location. It is based on mean water level at Rimouski, Quebec, on the Gulf of St. Lawrence over the period 1970 through 1988. Implemented January 1, 1992. |  |  |
| References | [IGLD85] |  |  |
| Label | MSL | Code | 4 |
| Description | Mean sea level (MSL) |  |  |
| ORM | WGS_1984 | Global/Local | Global |
| Notes | A continuous surface over the oceans (or its hypothetical extension under the land masses) defined by the mean of sea level surfaces approximated and observed over 19 years. |  |  |
| References | [BOWD, Section 913] |  |  |
| Label | NAVD_1988 | Code | 5 |
| Description | North American Vertical Datum (NAVD) 1988 |  |  |
| ORM | NAD_1983 | Global/Local | Local |
| Notes | A fixed reference for elevations derived from a general adjustment of the first-order terrestrial levelling nets of the United States, Canada, and Mexico. In the adjustment, only the height of the primary tidal bench mark, referenced to the International Great Lakes Datum of 1985 local mean sea level height value, at Pointe-au-Pere, Quebec, on the Gulf of St. Lawrence was held fixed, thus providing minimum constraint. |  |  |
| References | [NAVD88] |  |  |
| Label | OSGM_2002 | Code | 7 |
| Description | Ordnance Survey Geoid Model (OSGM) 2002 |  |  |
| ORM | ETRS_1989 | Global/Local | Local |
| Notes | The geopotential surface defined by the OSGM of 2002, covering the region of Great Britain, 45,5ºN to 61,5ºN and 3,5ºW to 11,5ºE. |  |  |
| References | [OSGM02] |  |  |
| Label | WGS84_ELLIPSOID | Code | 8 |
| Description | WGS 84 ellipsoid |  |  |
| ORM | WGS_1984 | Global/Local | Global |
| Notes | The oblate ellipsoidal figure of the Earth defined by WGS 84. |  |  |
| References | [83502T, Section 3] |  |  |
| Label | WGS84_GEOID | Code | 9 |
| Description | WGS 84 geoid |  |  |
| ORM | WGS_1984 | Global/Local | Global |
| Notes | The geopotential surface defined by the WGS 84 Earth Gravity Model that is closely associated with the mean ocean surface. |  |  |
| References | [83502T, Section 6] |  |  |
| Object type: Planet (non-Earth) |  |  |  |
| Object type: Satellite |  |  |  |
| Object type: Star |  |  |  |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

elevation	181

equipotential surface	179

geoid	179

geoidal separation	181

object reference surface	179

orthometric height	182

plumbline	182

vertical offset	180

vertical offset height	181

vertical offset surface	180