EDITORS NOTE: Table of contents tables will be removed from individual clauses. The TOC below is for draft review purposes only.

5 Abstract coordinate systems 29 5.1 Introduction 29 5.2 Preliminaries 29 5.3 Abstract CS 29 5.4 CS types 31 5.5 Coordinate surfaces, induced surface CSs, and coordinate curves 32 5.5.1 Introduction 32 5.5.2 Coordinate component surfaces and induced surface CSs 32 5.5.3 Coordinate component curves 33 5.6 CS properties 34 5.6.1 Linearity 34 5.6.2 Orthogonality 34 5.6.3 Linear CS properties: Cartesian, and orthonormal 35 5.7 CS localization 35 5.8 Map projection coordinate systems 36 5.8.1 Map projections 36 5.8.2 Map projection as a surface CS 37 5.8.3 Map projection geometry 38 5.8.3.1 Introduction 38 5.8.3.2 Conformal map projections 38 5.8.3.3 Scale factor and point scale 39 5.8.3.4 Geodetic azimuth and map azimuth 39 5.8.3.5 Convergence of the meridian 40 5.8.4 Relationship to projection functions 41 5.8.5 Map projection CS common parameters 44 5.8.5.1 False and natural origins 44 5.8.5.2 Longitude and latitude of origin and central scale 44 5.8.6 Augmented map projections 44 5.8.6.1 Augmentation with ellipsoidal height 44 5.8.6.2 Distortion in augmented map projections 45 5.9 CS specifications 45

Table 5.1 — CS types 32 Table 5.2 — Localization operators 35 Table 5.3 — Localization inverse operators 36 Table 5.4 — Localized CS type relationships 36 Table 5.5 — Coordinate system specification fields 46 Table 5.6 — Common parameters and functions of an oblate ellipsoid 46 Table 5.7 — CS specification directory 47 Table 5.8 — Euclidean 3D CS 49 Table 5.9 — Lococentric Euclidean 3D CS 50 Table 5.10 — Spherical CS 51 Table 5.11 — Lococentric spherical CS 53 Table 5.12 — Azimuthal spherical CS 54 Table 5.13 — Lococentric azimuthal spherical CS 55 Table 5.14 — Geodetic CS 56 Table 5.15 — Planetodetic CS 59 Table 5.16 — Cylindrical CS 60 Table 5.17 — Lococentric cylindrical CS 62 Table 5.18 — Mercator CS 63 Table 5.19 — Oblique Mercator Spherical CS 65 Table 5.20 — Transverse Mercator CS 68 Table 5.21 — Lambert conformal conic CS 72 Table 5.22 — Polar stereographic CS 75 Table 5.23 — Equidistant cylindrical CS 79 Table 5.24 — Surface geodetic CS 81 Table 5.25 — Surface planetodetic CS 83 Table 5.26 — Lococentric surface Euclidean CS 84 Table 5.27 — Lococentric surface azimuthal CS 85 Table 5.28 — Lococentric surface polar CS 87 Table 5.29 — Euclidean 2D CS 88 Table 5.30 — Lococentric Euclidean 2D CS 89 Table 5.31 — Azimuthal CS 90 Table 5.32 — Lococentric azimuthal CS 91 Table 5.33 — Polar CS 93 Table 5.34 — Lococentric polar CS 94 Table 5.35 — Euclidean 1D CS 95

Figure 5.1 — Polar CS geometry 31 Figure 5.2 — The polar CS generating function 31 Figure 5.3 — Geodetic 3D CS geometry, and coordinate component surface and curves 34 Figure 5.4 — The generating function of a map projection 38 Figure 5.5 — Geodetic azimuths 12 from p1 to p2 and 34 from p3 to p4 40 Figure 5.6 — Convergence of the meridian 41 Figure 5.7 — Polar stereographic map projection 42 Figure 5.8 — Tangent and secant cylindrical map projections 43 Figure 5.9 — Tangent and secant conical map projections 44 Figure 5.10 — Vertical distortion 45

# Abstract coordinate systems

## Introduction

An abstract coordinate system is a means of identifying positions in position-space by coordinate n-tuples. An abstract coordinate system is completely defined in terms of the mathematical structure of position-space. In this International Standard the term “coordinate system”, if not otherwise qualified, shall mean “abstract coordinate system.” Each coordinate system has a coordinate system type (see 5.4). Other coordinate system related concepts defined in this clause include coordinate component surfaces and curves, linearity, and localization. Map projections and augmented map projections are defined and treated as special cases of the general abstract coordinate system concept.

In Clause 6 a temporal coordinate system is defined as a means of identifying events in the time continuum by coordinate 1-tuples using an abstract coordinate system of coordinate system type 1D. In Clause 8 a spatial coordinate system is defined as an abstract coordinate system suitably combined with a normal embedding (see Clause 7) as a means of identifying points in object-space by coordinate n-tuples.

## Preliminaries

Annex A provides a concise summary of mathematical concepts and specifies the notational conventions used in this International Standard. In particular, the concept of Rn as a vector space, the point-set topology of Rn, and the theory of real-valued functions on Rn are all assumed. Algebraic and analytic geometry, including the concepts of point, line, and plane, are also assumed. Together with such common concepts, a newly introduced concept “replete” will be used. A set D is replete if all points in D belong to the closure of the interior of D. A replete set is a generalization of an open set that allows the inclusion of boundary points. Boundary points are important in the definitions of certain coordinate systems. This International Standard takes a functional approach to the construction of coordinate systems.

## Abstract CS

An abstract Coordinate System (CS) is a means of identifying a set of positions in an abstract Euclidean space that shall be comprised of:

- a CS domain,

- a generating function, and

- a CS range,

where:

- The CS domain shall be a connected replete domain in the Euclidean space of n-tuples (1  n  m), called the coordinate-space.

- The generating function shall be a one-to-one, smooth, orientation preserving function from the CS domain onto the CS range.

- The CS range shall be the set of positions in a Euclidean space of dimension m (n  m  3), called the position-space. When n = 2 and m = 3, the CS range shall be a subset of a smooth surface. When n = 1 and m = 2 or 3, the CS range shall be a subset of an implicitly specified smooth curve.

NOTE 1   See Annex A for the definitions of the terms replete, one-to-one, smooth, smooth surface, smooth curve orientation preserving, and connected.

An element of the CS domain shall be called a coordinate. In particular, if the domain is a subset of 3D Euclidean space (R3), each coordinate is called a 3D coordinate. If the domain is a subset of 2D Euclidean space (R2), each coordinate is called a 2D coordinate.

The kth-component of a coordinate n-tuple (1  k  n) may be called the kth-coordinate component. Coordinate component is the collective term for any kth-coordinate component.

An element of the CS range shall be called a position.

The coordinate of a position p shall be the unique coordinate whose generating function value is p.

The generating function may be parameterized. The generating function parameters (if any) shall be called the CS parameters.

The inverse of the generating function shall be called the inverse generating function. The inverse generating function is one-to-one and is smooth and orientation preserving in the interior of its domain. A CS may equivalently be defined by specifying the inverse generating function when the domain is an open set.

NOTE 2	The generating function of a CS is often specified by an algebraic and/or trigonometric description of a geometric relationship (see 5.3 Example). There are CSs that do not have geometric derivations. The Mercator map projection (see Table 5.18) is specified to satisfy a functional requirement of conformality (see 5.8.3.2) rather than by geometric construction.

EXAMPLE	Polar CS. Considering the polar geometry depicted in Figure 5.1, define a generating function F as:

where:

The CS domain of F in coordinate-space is

|  | . |  |
| --- | --- | --- |

The CS range of F in position-space is

|  | . |  |
| --- | --- | --- |

y-axis

x-axis

 cos

 sin





Figure 5.1 — Polar CS geometry

This generating function is illustrated in Figure 5.2. The grey boxes with lighter grey edges in this figure represent the fact that the range in position-space extends indefinitely, and that the domain in coordinate-space extends indefinitely along the -axis. The dotted grey edges indicate an open boundary. This CS range, CS domain and generating function defines an abstract CS representing polar coordinates as defined in mathematics [EDM, “Coordinates”]. The normative definition of the polar CS may be found in Table 5.33.

position

-

space

coordinate

-

space

(, )

y = 

sin(



)

x = 

cos(



)

0

2



 -axis

p

F

x

-

axis

y

-

axis



-

axis

Figure 5.2 — The polar CS generating function

NOTE 3   In the special case where 1) the CS domain and CS range are both Rn and 2) the function is the identity function, this approach to defining coordinate systems reduces to the usual definition of Euclidean coordinates on Rn where each point is identified by an n-tuple of real numbers [EDM, “coordinates”] (see Table 5.8,  Table 5.29, and Table 5.35).

## CS types

The coordinate-space and position-space dimensions characterize an abstract CS by CS type as defined in Table 5.1.

Table 5.1 — CS types

| CS type | Dimension of coordinate-space | Dimension of position-space |
| --- | --- | --- |
| 3D | 3 | 3 |
| surface | 2 | 3 |
| curve | 1 | 3 |
| 2D | 2 | 2 |
| plane curve | 1 | 2 |
| 1D | 1 | 1 |

## Coordinate surfaces, induced surface CSs, and coordinate curves

### Introduction

The generating function of a 3D CS is a function of the 3 coordinate components of a coordinate 3-tuple. If one of the coordinate components is held fixed to a constant value, the generating function thus restricted to two variables may be viewed as a surface CS generating function (with a surface CS range). If two of the three coordinate components are held fixed, the generating function restricted to one variable may be viewed as a curve CS generating function (with curve CS range). These observations motivate the definitions of coordinate surfaces and curves. The coordinate surface and coordinate curve concepts are required to specify induced CS relationships and the definition of special coordinate curves.

### Coordinate component surfaces and induced surface CSs

If F is the generating function of a 3D CS, and u = (u0, v0, w0) is in the interior of the CS domain D, then three surface CS generating functions are defined by:

|  | , |  |
| --- | --- | --- |
|  | , and |  |
|  | . |  |

The CS domain for S1 is the connected component of which contains (v0, w0). 
The CS domain for S2 is the connected component of which contains (u0, w0).
The CS domain for S3 is the connected component of which contains (u0, v0).

Each of these surface CSs shall be called, respectively, the 1st, 2nd, and 3rd surface CS induced by F at u.

The CS ranges of these surface CSs are, respectively, the 1st-, 2nd-, and 3rd-coordinate component surface at u.

EXAMPLE 1	Coordinate component surface: The geodetic 3D CS with generating function is specified in Table 5.14 with CS parameters a and b. The 3rd coordinate component surface at is the surface of the oblate ellipsoid with major semi-axis a and minor semi-axis b.

EXAMPLE 2	Induced surface CS: The surface geodetic CS is specified in Table 5.24. Its CS domain, CS range and generating function are identical to the 3rd surface CS induced by the geodetic 3D generating function at. If h is replaced by 0 in the formulae for the generating and inverse generating functions of the geodetic 3D CS, they reduce to the surface geodetic formulae.

### Coordinate component curves

Coordinate component curves are defined for CSs of CS type 3D, CS type surface, and CS type 2D.

The CS type 3D case:

If F is the generating function of a CS of CS type 3D, D is the CS domain, and u = (u0, v0, w0) is in the interior of D, then the 1st-, 2nd-, and 3rd-coordinate component curves at u are parametrically specified, respectively, by the following smooth functions:

|  | , |  |
| --- | --- | --- |
|  | , and |  |
|  | . |  |

The domain for C1 is the connected component of which contains u0. 
The domain for C2 is the connected component of which contains v0.
The domain for C3 is the connected component of which contains w0.

NOTE	The intersection of two coordinate surfaces at u is (the locus of) a coordinate component curve: C1 = S2S3, C2 = S1S3, and C3 = S2S1.

The CS type surface and CS type 2D cases:

If F is the generating function of a CS of CS type surface or CS type 2D, D is the CS domain, and u = (u0, v0) is in the interior of D, then the 1st- and 2nd-coordinate component curve at u is parametrically specified, respectively, by the following smooth functions:

|  | , and |  |
| --- | --- | --- |
|  | . |  |

The domain for C1 is the connected component of which contains u0. 
The domain for C2 is the connected component of which contains v0.

EXAMPLE	If u = (r0, 0) is in the interior of the CS domain of the polar CS generating function F of 5.2.2 Example, then the 1st-coordinate component curve is C1(  ) = F(r0, ) = (r0 cos( ), r0 sin( ) ), and the 2nd-coordinate component curve is C2( r ) = F(r, 0) = (r cos(0), r sin(0)).

If F is the generating function for the geodetic 3D CS or the surface geodetic CS, and in the 3D case or in the surface case, then (see Figure 5.3):

- the 1st-coordinate component curve at u shall be called the parallel at u, and

- the 2nd-coordinate component curve at u shall be called the meridian at u.

The meridian at shall be called the prime meridian.

The parallel at shall be called the equator.

Figure 5.3 — Geodetic 3D CS geometry, and coordinate component surface and curves

## CS properties

### Linearity

A CS with generating function F is a linear CS if F is an affine function. The CS domain of a linear coordinate system is all of the coordinate-space.

A curvilinear CS is a non-linear CS.

EXAMPLE	The polar CS of 5.2.2 EXAMPLE is a 2D curvilinear CS.

### Orthogonality

A CS of CS type 3D, CS type surface, or CS type 2D is orthogonal if given any coordinate u in the interior of the CS domain of the generating function, the angle between any two coordinate component curves at u is a right angle.

EXAMPLE	The polar CS of 5.2.2 EXAMPLE is a 2D orthogonal CS.

### Linear CS properties: Cartesian, and orthonormal

In a linear CS, the kth-coordinate component curve is a (straight) line. The kth-coordinate component curve at the origin 0 of a linear CS is the kth-axis.

In a linear CS, if the angles between coordinate component curves at the origin 0 are (pair-wise) right angles, then that is the case at all points. In particular, a linear CS is orthogonal if the axes are orthogonal.

In some publications a Cartesian CS is defined the same way as an orthogonal linear CS. This International Standard, however, defines this concept differently. A linear CS with generating function F is a Cartesian CS if  (i.e., the points in position-space corresponding to the canonical basis of coordinate-space are all one unit distant from F(0)).

An orthonormal CS is a linear CS that is both orthogonal and Cartesian.

A 3D CS with generating function F is orientation preserving if the Jacobian determinant of F is positive.

EXAMPLE	The Lococentric Euclidean 3D CS specified in Table 5.9 is an orientation preserving orthonormal CS.

## CS localization

In some applications of a CS in the context of a spatial reference frame (see Clause 8), it is necessary to consider a displaced and/or rotated version of a CS. To consider such a CS, a generating function for the displaced and rotated version of the CS must be specified. These modified versions of CS generating functions may be specified in a uniform way through the process of localization defined below.

Three parameterized operators, called localization operators, that operate on or between position-spaces are defined in Table 5.2 and the inverses of these operators are specified in Table 5.3.

Table 5.2 — Localization operators

| Localization operator | Domain | Range | Localization parameters | Operator definition |
| --- | --- | --- | --- | --- |
|  |  |  | q, r, s, in r and s are orthonormal |  |
|  |  |  | q, r, s, in r and s are orthonormal |  |
|  |  |  | q, r, s, in r and s are orthonormal |  |

Table 5.3 — Localization inverse operators

| Localization operator | Inverse operator definition |
| --- | --- |

There are several forms of CS localization depending on CS type and localization operator. A 3D or surface CS with generating function FC is localized by composing FC with the localization operator. The localized CS is of the same CS type (CS type 3D or CS type surface, respectively). Its generating function is  and has the same CS domain as FC.

There are two localization operations for a 2D CS. One uses localization parameters in  and produces a surface CS. The other uses localization parameters in  and produces a 2D CS.

- A 2D CS with generating function FC is localized by composing F with the localization operator. The localized CS is a surface CS. Its generating function is  and has the same CS domain as FC.

- A 2D CS with generating function FC is localized by composing F with the localization operator. The localized CS is a surface CS. Its generating function is  and has the same CS domain as FC.

The localization operator parameter q shall be called the lococentre. A localized CS may be called a lococentric CS.

NOTE	CS localization preserves the following CS properties: linear/curvilinear, orthogonal, Cartesian, and orthonormal.

The relationship between a CS type and its localized version(s) is summarized in Table 5.4.

Table 5.4 — Localized CS type relationships

| CS type | Localization operator | Lococentric CS type |
| --- | --- | --- |
| 3D |  | 3D |
| Surface |  | Surface |
| 2D |  |  |
|  |  | 2D |

## Map projection coordinate systems

### Map projections

Map projections are 2D models of a 3D component curved surface. A map projection (MP) is comprised of

- An MP region in the surface of an oblate ellipsoid,

- a generating projection, and

- an MP range in 2D coordinate-space,

where:

- the MP region is a connected subset of the surface of the oblate ellipsoid,

- the MP range is a connected replete set, and

- the generating projection is one-to-one from the region of the oblate ellipsoid onto its MP range and its inverse function is smooth and orientation preserving.

NOTE 1	This definition may be generalized to any ellipsoid including tri-axial ellipsoids, but this International Standard only addresses map projections for oblate ellipsoids.

NOTE 2	The domain of a map projection is always a proper sub-set of the oblate ellipsoid surface. For example the domain of the Mercator map projection (see Table 5.18) omits the pole points.

The generating projection P is specified in terms of surface geodetic CS coordinates. The component functions P1 and P2 of the generating projection P are called the mapping equations:

where:

The MP range coordinate components u and v are called easting and northing respectively. The positive direction of the u-axis (the easting axis) is called map-east. The positive direction of the v-axis (the northing axis) is called map-north.

The inverse mapping equations are the component functions Q1 and Q2 of the inverse generating projection:

### Map projection as a surface CS

If the inverse generating projection of a map projection Q is composed with the surface geodetic CS generating function, the resulting function  is the generating function of a surface CS (see Figure 5.4). The CS domain is the MP range. In this International Standard, a map projection CS shall be a surface CS for which the generating function is (implicitly) specified in terms of map projection mapping equations.

NOTE	The CS generating function is not to be confused with the generating projection P.

G

MP

=

G

GD

Q

map projection

geodetic

coordinate-space

position-space

(



,



)

(

u

,

v

)

generating function

u

-

axis

v

-

axis



-

axis



-

axis

z

-

axis

y

-

axis

x

-

axis

P

Q

G

GD

mapping equations

coordinate-space

Figure 5.4 — The generating function of a map projection

### Map projection geometry

#### Introduction

In general, the Euclidean geometry that a surface CS 2D coordinate-space inherits from  has no direct significance with respect to the geometry of position-space. For example, the Euclidean distance between a pair of surface geodetic coordinates has no obvious meaning in position-space. In contrast, map projections are specifically designed so that coordinate-space geometry will model one or more geometric aspects of the corresponding surface in position-space.

The map projection CSs specified in this International Standard are designed so that one or more geometric aspects of the MP region of the oblate ellipsoid are approximated or modelled by the corresponding aspect in coordinate-space. The length of the line segment between two map coordinates is related to the length of the corresponding surface curve. Similarly, one or more of directions, areas, the angles between two intersecting curves, and shapes may be related approximately or exactly to the corresponding geometric aspect on the oblate ellipsoid surface.

The extent to which these aspects are or are not closely related is an indication of distortion. Some map projection CSs are designed to eliminate distortion for one geometric aspect (for example, angles or area). Others are designed to reduce distortion for several geometric aspects. In general, distortion tends to increase with the size of the oblate ellipsoid MP region relative to the total oblate ellipsoid surface area. Conversely, distortion errors may be reduced by restricting the size of the MP region. Map projections specified in this International Standard in the context of an SRF may have areas of definition beyond which the projection should not be used for some application domains due to unacceptable distortion.

#### Conformal map projections

A conformal map projection preserves angles. If two surface curves lying on an oblate ellipsoid meet at the angle, then in a conformal map projection the image of those curves in the map coordinate-space meet at the same angle  [THOM]. In addition [THOM] contains a derivation based on the theory of complex variables to obtain conditions that specify when a projection is conformal. The map projections specified in Table 5.18 through Table 5.22 are conformal.

NOTE	The conformal property is local. A conformal map projection preserves angles at a point, but does not necessarily preserve shape or area. For example, a large projected triangle may appear distorted under a conformal map projection.

#### Scale factor and point scale

One indicator of map projection distortion is the ratio of lengths between a line segment in coordinate-space and the corresponding curve in position-space. Given a point p on a smooth curve on the surface the ellipsoid the scale factor at p is the ratio of the differential distance in coordinate-space to the differential arc length at p along the curve as determined by the mapping equations. The scale factor along the meridian at a point p is denoted by j(p). The scale factor along the parallel at a point p is denoted by k(p).

If a map projection is conformal, then the scale factors at a point are independent of direction (j(p) = k(p)). In this case the scale factor value at the point is called the point scale [HTDP]. Point scale functions for specific map projections are specified in Clause 10.

Scale factor varies over the area of a map projection. A nominal or representative value for the scale factor is sometimes used to estimate position-space distances based on coordinate-space distances.

Note	The factor used to compress an area of a map projection onto a printed map sheet is called a map scale or a plot scale. Map scale should not be confused with scale factor.

#### Geodetic azimuth and map azimuth

The geodetic azimuth from a non-polar point p1 on the surface of an ellipsoid to a second point p2 on the surface is the angle measured clockwise from the meridian curve segment connecting p1 to the North pole to the geodesic containing p1 and p2 (see Figure 5.5). The range of azimuth values shall be [0, 2). The definition and range constraints apply to points in both hemispheres.

p1

p2

p3

p4

meridians

North pole

equator

geodesic

p3p4

geodesic

p1p2

12

34

Figure 5.5 — Geodetic azimuths 12 from p1 to p2 and 34 from p3 to p4

In a map projection CS, the map azimuth from a coordinate c1 to a coordinate c2 is defined as the angle from the v-axis (map-north) clockwise to the (straight) line segment connecting c1 to c2. In general, the map azimuth for a pair of coordinates will differ in value from the geodetic azimuth of the corresponding points on the oblate ellipsoid.

#### Convergence of the meridian

Given a point in the interior of the domain of a generating projection, the meridian through that point is projected to a curve in coordinate-space that passes through the corresponding coordinate. The angle  at the coordinate in the clockwise direction from the northing axis (the v-axis) direction to the curve is called the convergence of the meridian (COM). A typical geometry illustrating the COM at a point p is shown for the transverse Mercator map projection in Figure 5.6.

u-axis

v-axis

map-north



p

Figure 5.6 — Convergence of the meridian

EXAMPLE	If p2 is directly map-north of p1 (it has a larger v-coordinate component), then the map azimuth is zero, but the geodetic azimuth may not be zero. The geodetic azimuth is approximately the sum of the map azimuth and the convergence of the meridian if the points are sufficiently close together.

### Relationship to projection functions

Projection functions are defined in A.8. In some cases, the generating projection of a map projection CS is derived from a projection function. The derivation involves two steps. The first step is to restrict the projection function to a specified region of a given oblate ellipsoid so that the restricted function is one-to-one. The range of a projection function is a surface in 3D position-space. The second step is to associate the surface of the range to 2D coordinate-space without introducing additional distortions.

In the case of planar projection functions, including the orthographic, perspective, and stereographic projection functions, the range is in a plane that can be identified with 2D coordinate-space by selecting an origin and unit axis points.

In the case of the cylindrical and conic projection functions, the range surface is a cylinder or a cone, respectively. These surfaces are developable surfaces and, except for a line of discontinuity, are homeomorphic to a subset of 2D coordinate-space with a homeomorphism that has a Jacobian determinant equal to one. Conceptually, these surfaces can be unwrapped to a flat plane without stretching the surface.

In the case of a sphere surface, the polar stereographic map projection (Table 5.22) is derived from the polar stereographic projection function and the map projection is conformal. The same derivation may be applied to an oblate ellipsoid. However, the resulting map projection will not have the conformal property. For this reason, the generalization of the polar stereographic map projection mapping equations from the sphere case to the non-spherical oblate ellipsoid case is not derived from the spatial projection function. Instead it is derived analytically to preserve the conformal property. Similarly, the Mercator map projection is not derived from the cylindrical projection function even in the case of a sphere. The Mercator mapping equations are designed to have the conformal property.

EXAMPLE 1	Polar Stereographic: Given a sphere with a polar point p, the tangent plane to the sphere at p and the opposite polar point v specify a stereographic planar projection function F (see A.8.2.3). The restriction of F to a subsurface of the sphere that excludes v, is the generating projection for the sphere case of a polar stereographic map projection. In Figure 5.7 the position s on the sphere is projected to point t on a plane.

s

t

v

p

t = F(s)

Figure 5.7 — Polar stereographic map projection

The use of spatial projections to derive map projections with desirable properties is limited, but does motivate some classifications of map projections. These classifications include tangent and secant map projections as well as conic and cylindrical map projections [SNYD, p. 5].

A map projection is classified as cylindrical if the generating projection image of:

- all meridians of the oblate ellipsoid are parallel straight lines and are equally spaced with respect to the longitude of the meridians, and

- all parallels of the oblate ellipsoid are parallel straight lines and perpendicular to the meridian images.

EXAMPLE 2	The Mercator map projection (Table 5.18) and the equidistant cylindrical map projection (Table 5.23) are both classified as cylindrical map projections.

A cylindrical map projection is tangent if along the equator the scale factor is equal to one. It is secant if the scale factor is equal to one along two parallels equally spaced from the equator in latitude. In that case the positive latitude is called the standard latitude.  Tangent and secant cylindrical map projections are illustrated in Figure 5.8.

Figure 5.8 — Tangent and secant cylindrical map projections

A map projection is classified as conic if the generating projection image of:

- all meridians of the oblate ellipsoid are radial straight lines and are equally spaced with respect to the longitude of the meridians, and

- all parallels of the oblate ellipsoid are concentric arcs and perpendicular to the meridian images.

EXAMPLE 3	 Lambert conformal conic (see Table 5.21) is classified as a conic projection.

A conic map projection is tangent if along one parallel the scale factor is equal to one. It is secant if the scale factor is equal to one along two parallels that are not symmetric about the equator. In that case the two latitudes are called the standard latitudes (see Figure 5.9).

Figure 5.9 — Tangent and secant conical map projections

### Map projection CS common parameters

#### False and natural origins

To avoid negative coordinate component values or to reduce the magnitude of the values in a region of interest in the coordinate-space of a map projection, an offset may be added to each of the coordinate component values. The value uF added to the easting coordinate component u is the false easting. The value vF added to the northing coordinate component v is the false northing. The position with coordinates  is the false origin. The position  is the natural origin. The false origin and natural origin coincide when

#### Longitude and latitude of origin and central scale

The longitude and latitude at the natural origin are called the longitude of origin and latitude of origin respectively. The scale factor in the direction of the parallel at the natural origin is the central scale. If the central scale of a map projection of a sphere is equal to one, the map projection is a tangent map projection. If the central scale of a map projection of a sphere is less than one, the map projection is a secant map projection. Specifying a central scale less than one is one generalization of secant map projections to the case of an oblate ellipsoid. Central scale appears as a CS parameter for several map projections.

NOTE	The central scale parameter, when included with the CS parameters, is intended to control the tangent/secant characteristics of the map projection CS and is therefore close to, but does not exceed, 1.0. Map scales or plot scales (see 5.8.3.3 Note) are typically much smaller in magnitude and are applied directly to the coordinate-space.  For example, if a transverse Mercator map projection with central scale value 0,996 is to be scaled 50 000:1 on a map sheet, then the mapping equations  are evaluated with k0 = 0,996 and the points are plotted on the map sheet with s = (1/50 000).

### Augmented map projections

#### Augmentation with ellipsoidal height

A 3D CS can be specified from a map projection. The canonical embedding of a point (u, v) in R2 to the point (u, v, 0) in the uv-plane of R3 allows map points in 2D coordinate-space to be augmented with a third coordinate axis, the w-axis of R3. To be considered as a 3D CS, an augmented 3-tuple (u, v, w) of coordinates in the augmented map projection coordinate-space shall be associated to a unique position in position-space. The association is to ellipsoidal height where sv > 0 is a specified vertical scale factor. Given an augmented coordinate-tuple (u, v, w) for which (u, v) belongs to the coordinate range of the underlying generating projection, the associated position is given in 3D geodetic coordinates (, , h) where (, ) is projected to (u, v) by the map projection mapping equations. The third coordinate-space coordinate w is the vertical coordinate and the 3D geodetic coordinate constraints on negative values of h impose corresponding constraints on allowed values for w.

Augmented map projections inherit the geometry of R3.

#### Distortion in augmented map projections

In addition to map projection distortion (see 5.8.3.1), augmentation causes additional distortion. Consider the two straight-line segments between the pairs of coordinate-space points {(u1, v1, 0), (u2, v2, 0)} and {(u1, v1, w), (u2, v2, w)} with w > 0. In augmented map projection geometry, the two line segments have the same length. The corresponding curve in position-space of the first line segment is a surface curve of the oblate ellipsoid (or sphere). The corresponding second curve is outside of the oblate ellipsoid (or sphere) and has longer arc length than the first, and the length difference increases with w. (In the case of augmentation with elevation, the curves would additionally parallel geoidal surface undulations.)

If the vertical map scale does not equal the (horizontal) map scale, vertical angles at the surface will be skewed. Even a Cartesian augmentation of a conformal map projection will not be (vertically) conformal.

These and other distortions have profound implications for dynamic equations that are beyond the scope of this International Standard.

(

u

1

,

v

1

,

w

)

(

u

1

,

v

1

, 0)

(

u

2

,

v

2

,

w

)

(

u

2

,

v

2

,

0)

coordinate-space

position-space

p

p’

q’

q

Figure 5.10 — Vertical distortion

## CS specifications

The CSs specified in this International Standard are presented in Table 5.8 through Table 5.35. Each CS specification specifies the values of all fields presented in Table 5.5.

Table 5.5 — Coordinate system specification fields

| Field | Specification |
| --- | --- |
| Description | A description of the CS including any common name. |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Function type | Either “generating function” or “map projection”. |
| CS type | One of: 3D linear, 3D curvilinear, surface linear, surface curvilinear, map projection, 2D linear, 2D curvilinear, 1D linear, 1D curvilinear, or surface (map projection) and 3D (augmented map projection). |
| Properties | Either “none” or a list of one or more properties of the CS chosen from the following: orthogonal, not orthogonal, orthonormal, not orthonormal, conformal, or not conformal. Conformal and not conformal only apply to map projections. |
| CS parameters and constraints | The parameters (if any) that control aspects of the CS and constraints on how those parameters interrelate. |
| Coordinate components | Coordinate component symbols and common names. |
| Domain of the generating function or mapping equations | The domain of the CS generating function or mapping equations. |
| Generating function or mapping equations | The CS generating function or mapping equations. |
| Domain of the inverse of the generating function or mapping equations | The domain of the inverse of the CS generating function or mapping equations. |
| Inverse of the generating function or mapping equations | The inverse of the CS generating function or mapping equations. |
| Figure(s) | Zero or more figure(s) that explain and illustrate the CS. |
| Notes | Additional, non-normative information concerning the CS. |
| References | The references (see 12.2.5). |

NOTE	A coordinate component is identified by its name and/or symbol. The ordering of coordinate components is not part of a CS specification. A consistent coordinate component order for each specified CS is maintained in this International Standard only for clarity of presentation and to avoid ambiguity in the specification of the API.

Several specified CS generating functions and mapping equations and/or their inverses use some common intermediate functions or parameters associated with oblate ellipsoids. For clarity and concise presentation, these functions and parameters are defined in Table 5.6.

Table 5.6 — Common parameters and functions of an oblate ellipsoid

| Function or parameter | Symbol and defining expression |
| --- | --- |
| major semi-axis |  |
| minor semi-axis |  |
| flattening |  |
| (first) eccentricity | alternative equivalent expression: |
| second eccentricity |  |
| radius of curvature in the prime vertical |  |
| radius of curvature in the meridian | alternative equivalent expression: |
| meridional distance to equator |  |

Table 5.7 presents a directory of the CS specifications in this International Standard. Each listed CS is specified in a separate table that is indicated by the “Table number” hyperlink in the corresponding cell. Additional CSs may be added by registration.

Table 5.7 — CS specification directory

| Function type | CS type | Label and Description | Table number |
| --- | --- | --- | --- |
| Generating function | 3D | EUCLIDEAN_3D Euclidean 3D | Table 5.8 |
|  |  | LOCOCENTRIC_EUCLIDEAN_3D Lococentric Euclidean 3D | Table 5.9 |
|  |  | SPHERICAL Spherical | Table 5.10 |
|  |  | LOCOCENTRIC_SPHERICAL Lococentric spherical | Table 5.11 |
|  |  | AZIMUTHAL_SPHERICAL Azimuthal spherical | Table 5.12 |
|  |  | LOCOCENTRIC_AZIMUTHAL_SPHERICAL Lococentric azimuthal spherical | Table 5.13 |
|  |  | GEODETIC Geodetic 3D | Table 5.14 |
|  |  | PLANETODETIC Planetodetic 3D | Table 5.15 |
|  |  | CYLINDRICAL Cylindrical | Table 5.16 |
|  |  | LOCOCENTRIC_CYLINDRICAL Lococentric cylindrical | Table 5.17 |
| Map projection | Surface and augmented 3D | MERCATOR Mercator | Table 5.18 |
|  |  | OBLIQUE_MERCATOR_SPHERICAL Oblique Mercator spherical | Table 5.19 |
|  |  | TRANSVERSE_MERCATOR Transverse Mercator | Table 5.20 |
|  |  | LAMBERT_CONFORMAL_CONIC Lambert conformal conic | Table 5.21 |
|  |  | POLAR_STEREOGRAPHIC Polar stereographic | Table 5.22 |
|  |  | EQUIDISTANT_CYLINDRICAL Equidistant cylindrical | Table 5.23 |
| Generating function | Surface | SURFACE_GEODETIC Surface geodetic | Table 5.24 |
|  |  | SURFACE_PLANETODETIC Surface planetodetic | Table 5.25 |
|  |  | LOCOCENTRIC_SURFACE_EUCLIDEAN Lococentric surface Euclidean | Table 5.26 |
|  |  | LOCOCENTRIC_SURFACE_AZIMUTHAL Lococentric surface azimuthal | Table 5.27 |
|  |  | LOCOCENTRIC_SURFACE_POLAR Lococentric surface polar | Table 5.28 |
|  | 2D | EUCLIDEAN_2D Euclidean 2D | Table 5.29 |
|  |  | LOCOCENTRIC_EUCLIDEAN_2D Lococentric Euclidean 2D | Table 5.30 |
|  |  | AZIMUTHAL Azimuthal | Table 5.31 |
|  |  | LOCOCENTRIC_AZIMUTHAL Lococentric azimuthal | Table 5.32 |
|  |  | POLAR Polar | Table 5.33 |
|  |  | LOCOCENTRIC_POLAR Lococentric polar | Table 5.34 |
|  | 1D | EUCLIDEAN_1D Euclidean 1D | Table 5.35 |

Table 5.8 — Euclidean 3D CS

| Field | Specification |
| --- | --- |
| Description | Euclidean 3D. |
| Label | EUCLIDEAN_3D |
| Code | 1 |
| Function type | Generating function. |
| CS type | 3D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | none |
| Coordinate components | u, v, w |
| Domain of the generating function or mapping equations | R3 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R3 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Coordinate-space 3-tuples are identified with position-space 3-tuples. |
| References | [EDM] |

Table 5.9 — Lococentric Euclidean 3D CS

| Field | Specification |
| --- | --- |
| Description | Localized Euclidean 3D |
| Label | LOCOCENTRIC_EUCLIDEAN_3D |
| Code | 2 |
| Function type | Generating function. |
| CS type | 3D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components | u, v, w |
| Domain of the generating function or mapping equations | R3 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R3 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - Euclidean 3D CS (see Table 5.8) is a special case with. - The generating function is the composition of the generating function for Euclidean 3D CS (see Table 5.8) with the 3D localization operator (see 5.7). |
| References | [EDM] |

Table 5.10 — Spherical CS

| Field | Specification |
| --- | --- |
| Description | Spherical. |
| Label | SPHERICAL |
| Code | 3 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | none |
| Coordinate components |  : longitude in radians,  : spherical latitude in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The spherical CS is not intrinsically associated with any specific sphere. |
| References | [HCP] |

Table 5.11 — Lococentric spherical CS

| Field | Specification |
| --- | --- |
| Description | Localized spherical |
| Label | LOCOCENTRIC_SPHERICAL |
| Code | 4 |
| Function type | Generating function |
| CS type | 3D curvilinear |
| Properties | Orthogonal |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components |  : longitude in radians,  : spherical latitude in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for spherical CS (see Table 5.10) with the 3D localization operator (see 5.7). |
| References | [EDM] |

Table 5.12 — Azimuthal spherical CS

| Field | Specification |
| --- | --- |
| Description | Azimuthal spherical. |
| Label | AZIMUTHAL_SPHERICAL |
| Code | 5 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | none |
| Coordinate components |  : azimuth in radians,  : depression/elevation angle in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | none |
| References | [EDM] |

Table 5.13 — Lococentric azimuthal spherical CS

| Field | Specification |
| --- | --- |
| Description | Localized azimuthal spherical. |
| Label | LOCOCENTRIC_AZIMUTHAL_SPHERICAL |
| Code | 6 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components |  : azimuth in radians,  : depression/elevation angle in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for azimuthal spherical (see Table 5.12) with the 3D localization operator (see 5.7). |
| References | [EDM] |

Table 5.14 — Geodetic CS

| Field | Specification |
| --- | --- |
| Description | Geodetic 3D. |
| Label | GEODETIC |
| Code | 7 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | a: major semi-axis length b: minor semi-axis length Constraints: a > b: (oblate ellipsoid) a = b: (sphere) |
| Coordinate components |  : longitude in radians,  : geodetic latitude in radians, and h : ellipsoidal height. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The surface geodetic CS (Table 5.24) is induced on the 3rd coordinate surface at any point for which h = 0. - If a = b, the geodetic latitude  coincides with the spherical latitude  (see Table 5.10). |
| References | [HEIK] |

Table 5.15 — Planetodetic CS

| Field | Specification |
| --- | --- |
| Description | Planetodetic 3D. Geodetic 3D with longitude in opposite direction. |
| Label | PLANETODETIC |
| Code | 8 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | a: major semi-axis length b: minor semi-axis length Constraints: a > b: (oblate ellipsoid) a = b: (sphere) |
| Coordinate components |  : planetodetic longitude in radians,  : geodetic latitude in radians, and h : ellipsoidal height. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Similar to the geodetic 3D CS (see Table 5.14) except that longitude increases in the opposite direction. In particular, points on a planet surface rotating (prograde) into view have larger planetodetic longitudes than those points rotating out of view. This CS is also called planetocentric when a = b and planetographic when a < b. |
| References | [RIIC] |

Table 5.16 — Cylindrical CS

| Field | Specification |
| --- | --- |
| Description | Cylindrical. |
| Label | CYLINDRICAL |
| Code | 9 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | none |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | none |
| References | [EDM] |

Table 5.17 — Lococentric cylindrical CS

| Field | Specification |
| --- | --- |
| Description | Localized cylindrical. |
| Label | LOCOCENTRIC_CYLINDRICAL |
| Code | 10 |
| Function type | Generating function. |
| CS type | 3D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | See Table 5.16. |
| Notes | - The generating function is the composition of the cylindrical generating function (see Table 5.16) with 3D localization operator (see 5.7). - The domain of the inverse of the generating function is the set of all points not on the axis of the cylinder. |
| References | [EDM] |

Table 5.18 — Mercator CS

| Field | Specification |
| --- | --- |
| Description | Mercator and augmented Mercator map projection coordinate systems. |
| Label | MERCATOR |
| Code | 11 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations | For , functional iteration is used for the functional representation of the inverse map projection [SNYD]. Superscripts involving i indicate elements in the iteration sequence. |
| Figure(s) | Example Mercator map projection |
| Notes | Meridians project as straight lines that satisfy equations of the form u = a constant. Equally spaced meridians project to evenly-spaced straight lines orthogonal to the u-axis. Parallels project to straight lines orthogonal to the projected meridians and satisfy equations of the form v = a constant. Evenly spaced parallels project to unevenly spaced parallel lines on the projection. The spacing of these lines increases with distance from the u-axis. |
| References | [SNYD] |

Table 5.19 — Oblique Mercator Spherical CS

| Field | Specification |
| --- | --- |
| Description | Oblique Mercator and augmented oblique Mercator map projections of a sphere. |
| Label | OBLIQUE_MERCATOR_SPHERICAL |
| Code | 12 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations | and |
| Generating function or mapping equations | In this formulation u shall be computed using the double argument arctangent or the equivalent. |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations | In this formulation f(u,v) shall be computed using the double argument arctangent or the equivalent. |
| Figure(s) | Example oblique Mercator spherical map projection |
| Notes |  |
| References | [SNYD] |

Table 5.20 — Transverse Mercator CS

| Field | Specification |
| --- | --- |
| Description | Transverse Mercator and augmented transverse Mercator map projections. |
| Label | TRANSVERSE_MERCATOR |
| Code | 13 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | Example transverse Mercator map projection |
| Notes | - As noted in [SNYD] and [LLEE] the closed solution for the transverse Mercator forward and inverse conversions were developed by Prof. E. H. Thompson in 1945 and were formally published by L. P. Lee in 1962 [LLEE] with the permission of Professor Thompson. In 1980, J. Dozier published a report that adapted Lee’s results to a computerized solution, including complete program listings in the C language [DOZI]. The formulation used here is further adapted by Craig Rollins of the National Geospatial-Intelligence Agency, Arnold, Mo. USA to include a central scale factor, a non-zero latitude origin and false easting and false northing offsets for both the easting and northing components. - The complex functions sn(w ¦2), cn(w ¦2) and dn(w ¦2) are called Jacobian elliptic functions in [ABST] and [DOZI] and are called Jacobi functions in [LLEE]. - The domain of the inverse mapping equations covers the main region of interest. This domain can be extended to a larger region whose exact specification is complicated to define. |
| References | [DOZI], [LLEE], and [SNYD]. |

Table 5.21 — Lambert conformal conic CS

| Field | Specification |
| --- | --- |
| Description | Lambert conformal conic and augmented Lambert conformal conic map projections. |
| Label | LAMBERT_CONFORMAL_CONIC |
| Code | 14 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | The double argument arctangent function or the equivalent shall be used. |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | Example Lambert conformal conic map projection |
| Notes | 1. The signum function is defined as: 2. The point scale is unity along the standard parallel(s) 1 and 2. |
| References | [SNYD] |

Table 5.22 — Polar stereographic CS

| Field | Specification |
| --- | --- |
| Description | Polar stereographic and augmented polar stereographic map projections. |
| Label | POLAR_STEREOGRAPHIC |
| Code | 15 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations | For  functional iteration is used for the functional representation of the inverse map projection. Superscripts involving m indicate elements in the iteration sequence. |
| Figure(s) | The polar stereographic projection function for a sphere Example polar stereographic map projection |
| Notes | In the case of a sphere, the mapping equations are derived from the stereographic projection (see the Figure field). The points x and x1 on the sphere are projected to points p and p1 on the plane. |
| References | [SNYD] |

Table 5.23 — Equidistant cylindrical CS

| Field | Specification |
| --- | --- |
| Description | Equidistant cylindrical and augmented equidistant cylindrical map projections. |
| Label | EQUIDISTANT_CYLINDRICAL |
| Code | 16 |
| Function type | Mapping equations. |
| CS type | Surface (map projection) and 3D (augmented map projection). |
| Properties | Non-conformal. |
| CS parameters and constraints |  |
| Coordinate components | u: easting, and v: northing. Augmented coordinate: h: ellipsoidal height |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | Example equidistant cylindrical map projection |
| Notes | none |
| References | [SNYD] |

Table 5.24 — Surface geodetic CS

| Field | Specification |
| --- | --- |
| Description | Surface geodetic. |
| Label | SURFACE_GEODETIC |
| Code | 17 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | a: major semi-axis length b: minor semi-axis length Constraints: a > b: (oblate ellipsoid) a = b: (sphere) |
| Coordinate components |  : longitude in radians, and  : geodetic latitude in radians. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the oblate ellipsoid (or sphere) surface excluding the pole points. - The geodetic 3D CS (Table 5.14) induces this CS on the 3rd coordinate surface at any point for which h = 0. - If a = b, the geodetic latitude  coincides with the spherical latitude  (see Table 5.10). |
| References | [HEIK] |

Table 5.25 — Surface planetodetic CS

| Field | Specification |
| --- | --- |
| Description | Surface planetodetic. Surface geodetic with longitude in opposite direction. |
| Label | SURFACE_PLANETODETIC |
| Code | 18 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | a: major semi-axis length b: minor semi-axis length Constraints: a > b: (oblate ellipsoid) a = b: (sphere) |
| Coordinate components |  : planetodetic longitude in radians, and  : geodetic latitude in radians. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Similar to surface geodetic (see Table 5.24) except that longitude is in the opposite direction. In particular, points on a planet surface rotating (prograde) into view have larger planetodetic longitudes than those points rotating out of view. |
| References | [RIIC] |

Table 5.26 — Lococentric surface Euclidean CS

| Field | Specification |
| --- | --- |
| Description | Localization of Euclidean 2D CS into plane surface in 3D position-space. |
| Label | LOCOCENTRIC_SURFACE_EUCLIDEAN |
| Code | 19 |
| Function type | Generating function. |
| CS type | Surface linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components | u, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for Euclidean 2D (see Table 5.29) with the surface localization operator (see 5.7). |
| References | [EDM] |

Table 5.27 — Lococentric surface azimuthal CS

| Field | Specification |
| --- | --- |
| Description | Localization of azimuthal CS into plane surface in 3D position-space. |
| Label | LOCOCENTRIC_SURFACE_AZIMUTHAL |
| Code | 20 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components |  : azimuth in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for azimuthal CS (see Table 5.31) with the 3D localization operator (see 5.7). |
| References | [EDM] |

Table 5.28 — Lococentric surface polar CS

| Field | Specification |
| --- | --- |
| Description | Localization of polar CS into plane surface in 3D position-space. |
| Label | LOCOCENTRIC_SURFACE_POLAR |
| Code | 21 |
| Function type | Generating function. |
| CS type | Surface curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R3, and r, s: axis directions in R3. Constraints: r and s are orthonormal. |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - The CS surface is the plane specified by:. - The generating function is the composition of the generating function for polar CS (see Table 5.33) with the surface localization operator (see 5.7). |
| References | [EDM] |

Table 5.29 — Euclidean 2D CS

| Field | Specification |
| --- | --- |
| Description | Euclidean 2D. |
| Label | EUCLIDEAN_2D |
| Code | 22 |
| Function type | Generating function. |
| CS type | 2D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | none |
| Coordinate components | u, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R2 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | Coordinate-space 2-tuples are identified with position-space 2-tuples. |
| References | [EDM] |

Table 5.30 — Lococentric Euclidean 2D CS

| Field | Specification |
| --- | --- |
| Description | Localized Euclidean 2D |
| Label | LOCOCENTRIC_EUCLIDEAN_2D |
| Code | 23 |
| Function type | Generating function. |
| CS type | 2D linear. |
| Properties | Orthonormal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, and r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinate components | u, v |
| Domain of the generating function or mapping equations | R2 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R2 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | - Euclidean 2D CS is a special case with. - The generating function is the composition of the generating function for Euclidean 2D CS (see Table 5.29) with the 2D localization operator (see 5.7). |
| References | [EDM] |

Table 5.31 — Azimuthal CS

| Field | Specification |
| --- | --- |
| Description | Azimuthal coordinate system. |
| Label | AZIMUTHAL |
| Code | 24 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | none |
| Coordinate components |  : azimuth in radians, and  : radius. |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | none |
| References | [EDM] |

Table 5.32 — Lococentric azimuthal CS

| Field | Specification |
| --- | --- |
| Description | Localization of azimuthal CS. |
| Label | LOCOCENTRIC_AZIMUTHAL |
| Code | 25 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, and r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) |  |
| Notes | The generating function is the composition of the generating function for azimuthal CS (see Table 5.31) with the 2D localization operator (see 5.7). |
| References | [EDM] |

Table 5.33 — Polar CS

| Field | Specification |
| --- | --- |
| Description | Polar coordinate system. |
| Label | POLAR |
| Code | 26 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | none |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | y-axis x-axis  cos  sin   |
| Notes | none |
| References | [EDM] |

Table 5.34 — Lococentric polar CS

| Field | Specification |
| --- | --- |
| Description | Localized polar. |
| Label | LOCOCENTRIC_POLAR |
| Code | 27 |
| Function type | Generating function. |
| CS type | 2D curvilinear. |
| Properties | Orthogonal. |
| CS parameters and constraints | Localization parameters: q: the lococentric origin in R2, and r, s: axis directions in R2. Constraints: r and s are orthonormal. |
| Coordinate components |  |
| Domain of the generating function or mapping equations |  |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations |  |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | s y x u v y - axis p q r  x - axis  |
| Notes | The generating function is the composition of the generating function for the polar CS (see Table 5.33) with the 2D localization operator (see 5.7). |
| References | [EDM] |

Table 5.35 — Euclidean 1D CS

| Field | Specification |
| --- | --- |
| Description | Euclidean 1D. |
| Label | EUCLIDEAN_1D |
| Code | 28 |
| Function type | Generating function. |
| CS type | 1D linear |
| Properties | none |
| CS parameters and constraints | none |
| Coordinate components | u |
| Domain of the generating function or mapping equations | R1 |
| Generating function or mapping equations |  |
| Domain of the inverse of the generating function or mapping equations | R1 |
| Inverse of the generating function or mapping equations |  |
| Figure(s) | none |
| Notes | Coordinate-space 1-tuples are identified with position-space 1-tuples. |
| References | [EDM] |

Editors note: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

2D coordinate	30

3D coordinate	30

abstract CS	29

augmented equidistant cylindrical CS	79

augmented Lambert conformal conic CS	72

augmented map projection	44

augmented Mercator CS	63

augmented oblique Mercator CS	65

augmented polar stereographic CS	75

augmented transverse Mercator CS	68

axis	35

azimuthal CS	90

azimuthal spherical CS	54

Cartesian CS	35

central scale	44

conformal map projection	38

conic classification	43

convergence of the meridian	40

coordinate	30

Coordinate component	30

coordinate component curve	33

coordinate component surface	32

coordinate of a position	30

coordinate- space	29

CS domain	29

CS localization	36

CS parameters	30

CS range	29

CS type	31

curvilinear CS	34

cylindrical classification	42

cylindrical CS	60

easting	37

equator	34

equidistant cylindrical CS	79

Euclidean 1D CS	95

Euclidean 2D CS	88

Euclidean 3D CS	49

false easting	44

false northing	44

false origin	44

generating function	29

generating projection	37

geodetic 3D CS	56

geodetic azimuth	39

inverse generating function	30

kth-coordinate component	30

Lambert conformal conic CS	72

latitude of origin	44

linear CS	34

localization operator	35

lococentre	36

lococentric	36

lococentric azimuthal CS	91

lococentric azimuthal spherical CS	55

lococentric cylindrical CS	62

lococentric Euclidean 2D CS	89

Lococentric Euclidean 3D CS	50

lococentric polar CS	94

lococentric spherical CS	53

lococentric surface azimuthal CS	85

lococentric surface Euclidean CS	84

lococentric surface polar CS	87

longitude of origin	44

map azimuth	40

map projection	36

map-east	37

map-north	37

mapping equations	37

Mercator CS	63

meridian	34

natural origin	44

northing	37

oblique Mercator CS	65

orientation preserving orthonormal CS	35

orthogonal CS	34

orthonormal CS	35

parallel (geodetic)	34

planetodetic 3D CS	59

point scale	39

polar CS	93

polar stereographic CS	75

position	30

position-space	29

prime meridian	34

scale factor	39

secant conic map projection	43

secant cylindrical map projection	42

spherical CS	51

standard latitude cylindrical projection	42

standard latitudes conic projection	43

surface CS induced by F	32

surface geodetic CS	81

surface planetodetic CS	83

tangent conic map projection	43

tangent cylindrical map projection	42

transverse Mercator CS	68

vertical scale factor	45

The generating function properties and the implicit function theorem together imply that for each point in the interior of the CS domain, there is an open neighbourhood of the point whose image under the generating function lies in a smooth surface. This requirement specifies that there exists one smooth surface for all of the points in the CS domain. The requirement is needed to exclude mathematically pathological cases.

The generating function properties and the implicit function theorem together imply that for each point in the interior of the CS domain, there is an open neighbourhood of the point whose image under the generating function lies in a smooth curve. This requirement specifies that there exists one implicitly defined smooth curve for all the points in the CS domain. The requirement is needed to exclude mathematically pathological cases.

The ISO 19111 term for this concept is “coordinates”.

The ISO 19111 term for this concept is “coordinate”.

The ISO 19116 concept of a linear reference system is a specialization of the curve CS and plane curve CS concepts.

See preceding footnote.

ISO 19111 defines the term “meridian” as the intersection between an ellipsoid and a plane containing the semi-minor axis of the ellipsoid. That definition includes the antipodal meridian and the pole points and is therefore not the same concept as defined in this International Standard.

Some publications use “rectangular” to denote an orthogonal linear CS, and “oblique” to denote a non-orthogonal linear CS.

ISO 19111 defines “Cartesian coordinate system” as a coordinate system that gives the position of points relative to n mutually-perpendicular axes.

It is a consequence of the Theorema Egregium of Gauss that no map projection CS can eliminate all distortion.

More general definitions that allow measurements of azimuth angle clockwise or counter-clockwise and from the north or south side of the meridian are in use. The generalization to the case for which one or more of the two points not on the surface is treated in [RAPP1] and [RAPP2]. The more general definitions are not required for subsequent SRM concepts.