EDITORS NOTE: Table of contents tables will be removed from individual clauses. The TOC below is for draft review purposes only.

8	Spatial reference frames	135

8.1	Introduction	135

8.2	Spatial coordinate systems	135

8.3	Spatial reference frame	136

8.3.1	Specification	136

8.3.2	SRF specification elements	136

8.4	SRF induced surface spatial reference frame	138

8.5	SRF templates	139

8.5.1	Introduction	139

8.5.2	Celestiocentric SRF	141

8.5.3	Local space rectangular 3D SRF	142

8.5.4	Celestiodetic SRF	142

8.5.5	Planetodetic SRF	143

8.5.6	Local tangent space Euclidean SRF	144

8.5.7	Local tangent space azimuthal spherical SRF	146

8.5.8	Local tangent space cylindrical SRF	147

8.5.9	Celestiomagnetic SRF	148

8.5.10	Equatorial inertial SRF	149

8.5.11	Solar ecliptic SRF	150

8.5.12	Solar equatorial SRF	150

8.5.13	Solar magnetic ecliptic SRF	151

8.5.14	Solar magnetic dipole SRF	151

8.5.15	Heliospheric Aries ecliptic SRF	152

8.5.16	Heliospheric Earth ecliptic SRF	153

8.5.17	Heliospheric Earth equatorial SRF	153

8.5.18	Mercator SRF	154

8.5.19	Oblique Mercator Spherical SRF	154

8.5.20	Transverse Mercator SRF	156

8.5.21	Lambert conformal conic SRF	156

8.5.22	Polar stereographic SRF	157

8.5.23	Equidistant cylindrical SRF	158

8.5.24	Local space rectangular 2D SRF	159

8.5.25	Azimuthal 2D SRF	160

8.5.26	Polar 2D SRF	160

8.6	SRFs	161

8.7	SRF sets	166

Figure 8.1 — A spatial embedding of a surface CS	136

Figure 8.2 — Local tangent space Euclidean SRF	146

Table 8.1 — Compatible CS types	137

Table 8.2 — SRFT specification elements	139

Table 8.3 — SRFT directory	140

Table 8.4 — Celestiocentric SRFT	141

Table 8.5 — Local space rectangular 3D SRFT	142

Table 8.6 — Celestiodetic SRFT	143

Table 8.7 — Planetodetic SRFT	143

Table 8.8 — Local tangent space Euclidean SRFT	144

Table 8.9 — Local tangent space azimuthal spherical SRFT	146

Table 8.10 — Local tangent space cylindrical SRFT	147

Table 8.11 — Celestiomagnetic SRFT	149

Table 8.12 — Equatorial inertial SRFT	149

Table 8.13 — Solar ecliptic SRFT	150

Table 8.14 — Solar equatorial SRFT	150

Table 8.15 — Solar magnetic ecliptic SRFT	151

Table 8.16 — Solar magnetic dipole SRFT	152

Table 8.17 — Heliospheric Aries ecliptic SRFT	152

Table 8.18 — Heliospheric Earth ecliptic SRFT	153

Table 8.19 — Heliospheric Earth equatorial SRFT	153

Table 8.20 — Mercator SRFT	154

Table 8.21 — Oblique Mercator Spherical SRFT	155

Table 8.22 — Transverse Mercator SRFT	156

Table 8.23 — Lambert conformal conic SRFT	156

Table 8.24 — Polar stereographic SRFT	157

Table 8.25 — Equidistant cylindrical SRFT	158

Table 8.26 — Local space rectangular 2D SRF template	159

Table 8.27 — Azimuthal 2D SRFT	160

Table 8.28 — Polar 2D SRFT	161

Table 8.29 — SRF specification elements	161

Table 8.30 — SRFs	162

Table 8.31 — SRF set specification elements	166

Table 8.32 — SRF sets	167

Table 8.33 — SRF set member specification elements	169

Table 8.34 — Explicit specifications of SRF set members	170

Table 8.35 — Implicit specifications of SRF set members	176

Table 8.36 — GTRS natural origin and valid-region by code index	177

Table 8.37 — UPS indexing scheme and set members: northern hemisphere	177

Table 8.38 — UPS indexing scheme and set members: southern hemisphere	178

Table 8.39 — UTM indexing scheme and set members: northern hemisphere	178

Table 8.40 — UTM indexing scheme and set members: southern hemisphere	178

# Spatial reference frames

## Introduction

A spatial coordinate system is a means of associating a unique coordinate with a point in object-space. It is defined by binding an abstract CS to a normal embedding. This binding specifies all CS parameter values and combines the abstract CS generating function with the normal embedding of the CS position-space into the object-space.

A spatial reference frame is a spatial coordinate system for a region of object-space. It is formed by the binding of an abstract coordinate system to the normal embedding specified by an ORM for that object. A full specification specifies the CS and the ORM and includes values for CS parameters, if any, and a specification of the region of object-space. Some or all CS parameters may be bound by ORM parameters. In particular, a CS based on an oblate ellipsoid (or sphere) must match the parameters of the oblate ellipsoid (or sphere) RD of the ORM.

A spatial reference frame template is the basis for realizing spatial reference frames that share the same CS, similar ORMs, and the same structure in the binding of CS parameter values. Spatial reference frames may be organized into specified sets so as to form an atlas for a large region of space. This International Standard specifies a collection of spatial reference frame templates, realizations of those templates, and sets of those realizations.

## Spatial coordinate systems

If a normal embedding of position-space into object-space is defined, any abstract CS for a region of that position-space may be used to specify a spatial CS that associates coordinates in coordinate-space to points in object-space. This association is a binding of a CS via a normal embedding. The association is defined as:

|  |  | (8.1) |
| --- | --- | --- |

Figure 8.1 illustrates a spatial surface CS bound with a normal embedding of 3D position-space to the 3D object-space. In this illustration, a surface coordinate (u, v) in coordinate-space is associated to a position (x, y, z) in the abstract position-space. That position is then identified with a position in the space of an object via the normal embedding of position-space determined, in this example, by the selection of an origin and three unit points.

c

= (

u

, v)

coordinate-space

position-space

normal

embedding

E

v-axis

z

x

E

(

G

(

c

)) =

p

y

object-space

z

x

G

(

c

) =

(

x

,

y

, z)

y

CS generating function

G

Spatial CS association

origin

u-axis

x-axis

y-axis

x-axis

y-axis

z-axis

z-axis

Figure 8.1 — A spatial embedding of a surface CS

## Spatial reference frame

### Specification

A spatial reference frame (SRF) is a specification of an ORM together with a compatible CS, where coordinates uniquely specify positions with respect to the spatial object for which that ORM was specified. A specification of an SRF includes:

- an ORM,

- a CS compatible with the ORM,

- a binding of all parameters of the spatial CS,

- (optionally) kth–coordinate component names,

- (optionally) additional restrictions on the domain of valid coordinates in that spatial CS, and

- (optionally) if the CS is of CS type 3D, a vertical coordinate component identification (see 8.4).

An SRF implicitly specifies a spatial CS defined by the binding of the CS via the normal embedding associated with the ORM.

Spatial CS compatibility and the other elements of the specification of an SRF are defined in the following clauses.

### SRF specification elements

#### ORM and CS compatibility

The compatible CS type of the CS element of an SRF depends on the dimension of the ORM. The dimension of an ORM is defined as the dimension of the RD components of the specification of the ORM. The compatible CS types by ORM dimension are specified in Table 8.1.

Table 8.1 — Compatible CS types

| ORM dimension | Compatible CS types |
| --- | --- |
| 1D | 1D CS |
| 2D | Curve CS 2D CS |
| 3D | Curve CS Surface CS 3D CS |

The use of surface CSs or 3D CSs that are based on an oblate ellipsoid (or sphere) are restricted to ORMs that are based on an oblate ellipsoid (or respectively, sphere) RD.

The surface CSs that are based on an oblate ellipsoid (or sphere) are:

- surface geodetic, and

- all map projections.

The 3D CSs that are based on an oblate ellipsoid (or sphere) are:

- geodetic 3D, and

- all augmented map projections.

As a further restriction, some CSs are based on spheres only. CS OBLIQUE_MERCATOR_SPHERICAL has this restriction.

#### CS parameter binding

All CS parameter values must be specified. In the case of a combination of a CS and an ORM based on an oblate ellipsoid (or sphere), the major semi-axis and minor semi-axis (or equivalently, the inverse flattening) (or respectively, sphere radius) of the ORM and CS shall match.

#### Coordinate component names

A CS specification (see 5.4) includes the coordinate component symbols with common names (if any). A specification of an SRF may optionally assign SRF-specific names to the kth-coordinate components. The name assignment shall reflect the common use in the intended application domain.

EXAMPLE	For a spherical CS, the assignment of SRF-specific names to the kth-coordinate components of “right ascension” for , “declination” for  , and “radius” for .

#### Coordinate valid-region

A CS specification (see 5.4) includes the specification of the CS domain and CS range where the generating function (or mapping equations) and its inverse(s) are defined. An SRF specification may further restrict the CS domain. A valid-region is a restriction of the CS domain of the generating function (or mapping equations) for a CS as used in an SRF. An extended valid-region is a second valid-region that contains the first valid-region as a subset. The specification of these restrictions is important for several (SRF specific) reasons:

- If the ORM is local, the restrictions are used to model, in coordinate-space, the local region of the space of the object.

- If the CS is a map projection or an augmented map projection, the restrictions are used to bound or otherwise limit distortions (see 5.8.3.1).

- The SRF may be used in conjunction with other SRFs to form an atlas for a large region (see 8.7 SRF sets). In this case, the restrictions are used to control the pair-wise overlap of the spatial coverage of members of the SRF collection.

- If the CS generating function (or map projection mapping equations) or the inverse function(s) have been implemented with a numerical approximation, the restrictions are used to control error bounds.

- Any combination of the reasons above.

The extended valid-region is used primarily for overlapping regions in forming an atlas as in (c) above. Not all properties of the SRF that are true in the valid-region will necessarily be true in the extended valid-region. For example, a distortion error bound that holds in the valid-region may not hold in the extended valid-region.

A valid-region may be described and/or specified. A valid-region description is a descriptive statement of the region such as the spatial boundary of a named political entity.

EXAMPLE 1	“The German state of Baden-Wurttemberg” and “The Baltic Sea” are valid-region descriptions.

In this International Standard, a valid-region specification is a finite (or empty) list of coordinate component constraints of the form:

kth-coordinate component belongs to a non-empty interval of real numbers.

An extended valid-region specification is a finite (or empty) list of coordinate component constraints of the form:

kth-coordinate component belongs to an interval of real numbers, where  has been specified and.

In the case of an SRF with an oblate ellipsoid (or sphere) based ORM, celestiodetic coordinates may be similarly constrained. In particular, valid-region specifications for a map projection based SRF may specify coordinate component constraints for easting, northing, latitude, and/or longitude.

EXAMPLE 2	The SRF is based on a transverse Mercator map projection (see SRFT TRANSVERSE_MERCATOR).

Valid-region specification:			0 ≤ u ≤ 10 000 000, 	0 ≤ v ≤ 500 000
Extended valid-region specification:	 -100 < u, 			-100 < v

In this example and are closed bounded intervals, and and are open semi-bounded intervals.

EXAMPLE 3	The SRF is based on a transverse Mercator map projection (see SRFT TRANSVERSE_MERCATOR).

Valid-region specification:			-78º ≤  < -72º,		0º ≤  < 84º
Extended valid-region specification:	-78,5º ≤  < -71,5º

In this example and are left-closed, right-open bounded intervals, as is. is not specified. This indicates that there are no constraints for latitude (except for the CS domain definition) in the extended valid-region specification.

## SRF induced surface spatial reference frame

In the case of an SRF specified with the combination of a 3D ORM and a 3D CS, the 3D CS induces a surface CS on each coordinate component surface (see 5.5.2). An SRF specification may optionally identify the 3rd-coordinate component as the vertical coordinate component for the SRF. In that case, the surface CS induced on the zero value vertical coordinate component surface is the induced surface SRF for the specification. The vertical coordinate component is optionally specified in the coordinate component name specification element of the SRF.

The CS GEODETIC and the CS PLANETODETIC 3rd-coordinate components (h: ellipsoidal height), and the 3rd-coordinate component of any augmented map projection CS (h: ellipsoidal height) are identified in this International Standard as the vertical coordinate component. When an SRF is specified with any of these 3D CSs, the h = 0 coordinate component surface coincides with the surface of the oblate ellipsoid (or sphere) RD of the ORM. Any SRF based on these CSs intrinsically specifies the corresponding surface CS on the oblate ellipsoid (or sphere) RD surface.

In the SRFT LOCAL_TANGENT_SPACE_EUCLIDEAN specification and in the SRFT LOCAL_TANGENT_SPACE_CYLINDRICAL specification, the 3rd-coordinate component is specified as the vertical coordinate component. In these cases, the zero value vertical coordinate component surface is a plane that is tangent to the oblate ellipsoid (or sphere) RD of the ORM.

The zero value 3rd-coordinate component surface of the 3D CS SRFT LOCAL_TANGENT_SPACE_AZIMUTHAL_SPHERICAL induces a lococentric surface azimuthal CS on the tangent plane of the SRF. For the purpose of specifying an induced surface reference frame, the 3rd-coordinate component , depression/elevation angle, is specified as a vertical coordinate. The zero value vertical coordinate component surface is a plane that is tangent to the oblate ellipsoid (or sphere) RD of the ORM.

SRF templates that are based on surface CSs that can be induced by a zero value vertical coordinate component surface of an SRF based on a 3D CS are not separately specified. The induced surface CS is noted in the corresponding 3D CS based SRF template specification.

NOTE	Starting with a 3D SRF, SRM identifies surface SRFs on coordinate component surfaces. The relationship between a surface CS and the 3D CS which induces it is functionally similar to, but conceptually different from, the ISO 19111 concept of compound coordinate reference frame. A compound coordinate reference frame synthesizes a 3D reference frame from a surface and a vertical system. (See also 5.8.6.1 and Clause 9.)

## SRF templates

### Introduction

An SRF template (SRFT) is an abstraction of a set of specifications of SRFs that share the same CS and coordinate component names, whose ORM components are similar, that model the same type of spatial object, and that share the same CS parameter binding rules. An SRF template allows for a consistent derivation of SRFs. It is not necessary that an appropriate SRFT be defined in order to define a new SRF; however in this International Standard all SRFs are derived from SRFTs.

The specification elements for SRFTs are defined in Table 8.2.

Table 8.2 — SRFT specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Short name and description | A short name as published or as commonly known and an optional description. |
| Object or object type | One or more of: abstract, physical, Earth, planet, satellite, and Sun; and, optionally, additional restrictions. |
| ORM constraint | Criteria for allowable ORMs. |
| CS label | The label of a CS of compatible type. |
| CS coordinate component names and/or symbols | SRF-specific names and/or symbols for the kth-coordinate component names and/or symbols. If all coordinate component names and symbols are the same as the CS, the phrase “Same as the CS.” shall be used. The vertical coordinate component shall be designated in this field if applicable. |
| Template parameters | CS and RD parameters, if any, and/or SRF parameters that are not specified by a CS parameter binding rule. |
| CS parameter binding rules | A set of rules for binding for CS parameters and RD class parameters, if any, and/or SRF parameters. |
| Coordinate valid-region | Optional restriction of the domain of the CS to a valid-region. If a valid-region is specified, optionally an extended valid-region. If both are unspecified, then there are no additional constraints on coordinate validity. |
| Notes | Optional, additional, non-normative information such as a description of the SRF structure, modelled region, intended use, and/or application domain. |
| References | The references (see 12.2.5). |

This International Standard specifies a collection of SRFTs as identified in Table 8.3. Additional SRFTs may be registered in accordance with Clause 12. Registered SRFs shall be derived only from standardized or registered SRFTs.

Table 8.3 — SRFT directory

| CS type | Short name | SRFT label |
| --- | --- | --- |
| 3D | Celestiocentric | CELESTIOCENTRIC |
|  | Local space rectangular 3D | LOCAL_SPACE_RECTANGULAR_3D |
|  | Celestiodetic | CELESTIODETIC |
|  | Planetodetic | PLANETODETIC |
|  | Local tangent space Euclidean | LOCAL_TANGENT_SPACE_EUCLIDEAN |
|  | Local tangent space azimuthal spherical | LOCAL_TANGENT_SPACE_AZIMUTHAL_SPHERICAL |
|  | Local tangent space cylindrical | LOCAL_TANGENT_SPACE_CYLINDRICAL |
|  | Celestiomagnetic | CELESTIOMAGNETIC |
|  | Equatorial inertial | EQUATORIAL_INERTIAL |
|  | Solar ecliptic | SOLAR_ECLIPTIC |
|  | Solar equatorial | SOLAR_EQUATORIAL |
|  | Solar magnetic ecliptic | SOLAR_MAGNETIC_ECLIPTIC |
|  | Solar magnetic | SOLAR_MAGNETIC_DIPOLE |
|  | Heliospheric Aries ecliptic | HELIOSPHERIC_ARIES_ECLIPTIC |
|  | Heliospheric Earth ecliptic | HELIOSPHERIC_EARTH_ECLIPTIC |
|  | Heliospheric Earth equatorial | HELIOSPHERIC_EARTH_EQUATORIAL |
| Surface (map projection) and 3D (augmented map projection) | Mercator | MERCATOR |
|  | Oblique Mercator Spherical | OBLIQUE_MERCATOR_SPHERICAL |
|  | Transverse Mercator | TRANSVERSE_MERCATOR |
|  | Lambert conformal conic | LAMBERT_CONFORMAL_CONIC |
|  | Polar stereographic | POLAR_STEREOGRAPHIC |
|  | Equidistant cylindrical | EQUIDISTANT_CYLINDRICAL |
| Surface | Surface celestiodetic (induced) | CELESTIODETIC |
|  | Surface planetodetic (induced) | PLANETODETIC |
|  | Local tangent plane Euclidean (induced) | LOCAL_TANGENT_SPACE_EUCLIDEAN |
|  | Local tangent plane azimuthal (induced) | LOCAL_TANGENT_SPACE_AZIMUTHAL_SPHERICAL |
|  | Local tangent plane polar (induced) | LOCAL_TANGENT_SPACE_CYLINDRICAL |
| 2D | Local space rectangular 2D | LOCAL_SPACE_RECTANGULAR_2D |
|  | Local space azimuthal | LOCAL_SPACE_AZIMUTHAL |
|  | Local space polar | LOCAL_SPACE_POLAR |

### Celestiocentric SRF

Celestiocentric SRFs shall be derived from the SRFT specified in Table 8.4.

Table 8.4 — Celestiocentric SRFT

| Specification element | Value |
| --- | --- |
| Label | CELESTIOCENTRIC |
| Code | 1 |
| Short name and description | celestiocentric SRF The generalization of geocentric spatial reference frames to include non-Earth objects. |
| Object type | physical |
| ORM constraint | Shall be derived from any 3D ORM. |
| CS label | EUCLIDEAN_3D |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | None (no CS parameters). |
| Coordinate valid-region | No additional restrictions. |
| Notes | When the object is Earth, this SRF is referred to as a geocentric SRF. |
| References | [EDM] |

### Local space rectangular 3D SRF

Local space rectangular SRFs shall be derived from the SRFT specified in Table 8.5.

Table 8.5 — Local space rectangular 3D SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_SPACE_RECTANGULAR_3D |
| Code | 2 |
| Short name and description | local space rectangular 3D SRF A 3D Euclidean spatial reference frame for an abstract 3D space. |
| Object type | 3D abstract object. |
| ORM constraint | Shall be an ORM for a 3D abstract object. |
| CS label | LOCOCENTRIC_EUCLIDEAN_3D |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | r = vector direction of forward (forward axis). s = vector direction of up (up axis). |
| CS parameter binding rules |  |
| Coordinate valid-region | No additional restrictions. |
| Notes | CAD/CAM and other engineering applications. |
| References | [EDM] |

### Celestiodetic SRF

Celestiodetic SRFs shall be derived from the SRFT specified in Table 8.6.

Table 8.6 — Celestiodetic SRFT

| Specification element | Value |
| --- | --- |
| Label | CELESTIODETIC |
| Code | 3 |
| Short name and description | celestiodetic SRF The generalization of geodetic SRFs to include other planets and ellipsoidal bodies. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | GEODETIC |
| CS coordinate component names and/or symbols | The same as the CS. The vertical coordinate component is ellipsoidal height. |
| Template parameters | none |
| CS parameter binding rules | CS parameters match RD values. Oblate ellipsoid RD case with major semi-axis a and inverse flattening f -1: Sphere RD case with radius r : |
| Coordinate valid-region | No additional restrictions. |
| Notes | - The SURFACE_GEODETIC CS is induced on the oblate ellipsoid (or sphere) RD surface. - When the object is Earth, this SRF is referred to as a geodetic SRF. |
| References | [HEIK] |

### Planetodetic SRF

Planetodetic SRFs shall be derived from the SRFT specified in Table 8.7.

Table 8.7 — Planetodetic SRFT

| Specification element | Value |
| --- | --- |
| Label | PLANETODETIC |
| Code | 4 |
| Short name and description | planetodetic SRF Similar to celestiodetic SRF with reversed direction for longitude. |
| Object type | planet. |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | PLANETODETIC |
| CS coordinate names and/or symbols | The same as the CS. The vertical coordinate component is ellipsoidal height. |
| Template parameters | none |
| CS parameter binding rules | CS parameters match RD values: Oblate ellipsoid RD case with major semi axis a and inverse flattening f -1: Sphere RD case with radius r : |
| Coordinate valid region | No additional restrictions |
| Notes | Planetary science applications |
| References | [RIIC] |

### Local tangent space Euclidean SRF

Local tangent space Euclidean SRFs shall be derived from the SRFT specified in Table 8.8. The case with template parameters  = 0 and h0 = 0 is illustrated in Figure 8.2.

Table 8.8 — Local tangent space Euclidean SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_TANGENT_SPACE_EUCLIDEAN |
| Code | 5 |
| Short name and description | local tangent space Euclidean SRF Euclidean 3D spatial CS with the zero 3rd-coordinate component surface that is tangent to the oblate ellipsoid RD and with CS natural origin at the tangent point. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | LOCOCENTRIC_EUCLIDEAN_3D |
| CS coordinate component names and/or symbols | u: x (x) v: y (y) w: height (h) is the vertical coordinate component. The vertical coordinate component is height h. |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid-region | No additional restrictions. |
| Notes | - The TOPOCENTRIC_SURFACE_EUCLIDEAN CS is induced on the tangent plane surface. - The xy-plane is tangent to the oblate ellipsoid RD at the point with surface celestiodetic coordinate (,). -  is the geodetic azimuth of the x-axis. |
| References | [EDM] |

Figure 8.2 — Local tangent space Euclidean SRF

### Local tangent space azimuthal spherical SRF

Local tangent space azimuthal spherical SRFs shall be derived from the SRFT specified in Table 8.9.

Table 8.9 — Local tangent space azimuthal spherical SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_TANGENT_SPACE_AZIMUTHAL_SPHERICAL |
| Code | 6 |
| Short name and description | local tangent space azimuthal spherical SRF Azimuthal spherical spatial CS with the zero 3rd-coordinate component surface that is tangent to the oblate ellipsoid RD and with CS natural origin at the tangent point. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | LOCOCENTRIC_AZIMUTHAL_SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. : depression/elevation angle, is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid-region | No additional restrictions. |
| Notes | Used in radar localization. |
| References | [EDM] |

### Local tangent space cylindrical SRF

Local tangent space cylindrical SRFs shall be derived from the SRFT specified in Table 8.10.

Table 8.10 — Local tangent space cylindrical SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_TANGENT_SPACE_CYLINDRICAL |
| Code | 7 |
| Short name and description | local tangent space cylindrical SRF cylindrical spatial CS with the zero 3rd-coordinate component surface that is tangent to the oblate ellipsoid RD and with CS natural origin at the tangent point. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | LOCOCENTRIC_CYLINDRICAL |
| CS coordinate component names and/or symbols |  |
| Template parameters |  |
| CS parameter binding rules |  |
| Coordinate valid-region | No additional restrictions. |
| Notes | - The LOCOCENTRIC_SURFACE_POLAR CS is induced on the tangent plane surface. - The tangent plane is tangent to the oblate ellipsoid (or sphere) RD at surface celestiodetic coordinate (,). -  is the azimuth of the x-axis. |
| References | [EDM] |

### Celestiomagnetic SRF

Celestiomagnetic SRFs shall be derived from the SRFT specified in Table 8.11.

Table 8.11 — Celestiomagnetic SRFT

| Specification element | Value |
| --- | --- |
| Label | CELESTIOMAGNETIC |
| Code | 8 |
| Short name and description | celestiomagnetic SRF A spherical CS based SRF aligned with the magnetic dipole of a celestial object. |
| Object type | A planet or rotating satellite in a solar system with a magnetic dipole axis distinct from its rotational axis. |
| ORM constraint | Shall be a celestiomagnetic dynamic binding category ORM. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | - See 7.5.8. - When the object is Earth, this SRF is referred to as a geomagnetic SRF. - These SRFs are typically used at radii where the magnetic field is approximated by a dipole. |
| References | [CRUS] |

### Equatorial inertial SRF

Equatorial inertial SRFs shall be derived from the SRFT specified in Table 8.12.

Table 8.12 — Equatorial inertial SRFT

| Specification element | Value |
| --- | --- |
| Label | EQUATORIAL_INERTIAL |
| Code | 9 |
| Short name and description | equatorial Inertial SRF A spherical CS based SRF aligned with the equator of a planet and the direction to the Sun at the vernal equinox (at a specified epoch). |
| Object type | A planet in the solar system for which the ecliptic plane is distinct from the equatorial plane. |
| ORM constraint | Shall be an equatorial_inertial dynamic binding category ORM. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols |  : right ascension (ra)  : declination (dec)  : radius or range(r) |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | - See 7.5.3. - Star catalogues use right ascension and declination to specify directions. |
| References | [SEID] |

### Solar ecliptic SRF

Solar ecliptic SRFs shall be derived from the SRFT specified in Table 8.13.

Table 8.13 — Solar ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | SOLAR_ECLIPTIC |
| Code | 10 |
| Short name and description | solar ecliptic SRF A spherical CS based SRF aligned with the ecliptic plane of a planet and the direction of the Sun. |
| Object type | A planet in the solar system. |
| ORM constraint | Shall a solar_ecliptic dynamic binding category ORM. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.4. |
| References | [HAPG] |

### Solar equatorial SRF

Solar equatorial SRFs shall be derived from the SRFT specified in Table 8.14.

Table 8.14 — Solar equatorial SRFT

| Specification element | Value |
| --- | --- |
| Label | SOLAR_EQUATORIAL |
| Code | 11 |
| Short name and description | solar equatorial SRF A spherical CS based planet centred SRF aligned with the ecliptic plane and the rotational axis of the Sun. |
| Object type | A planet in the solar system for which the ecliptic plane is distinct from the equatorial plane. |
| ORM constraint | Shall be a SOLAR_EQUATORIAL dynamic binding category ORM. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.5. |
| References | [CRUS] |

### Solar magnetic ecliptic SRF

Solar magnetic ecliptic SRFs shall be derived from the SRFT specified in Table 8.15.

Table 8.15 — Solar magnetic ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | SOLAR_MAGNETIC_ECLIPTIC |
| Code | 12 |
| Short name and description | solar magnetic ecliptic SRF A Euclidean 3D CS based planet centred SRF aligned with the direction to the Sun and the plane form with that direction and the magnetic dipole of the planet. |
| Object type | A planet in the solar system with a magnetic dipole. |
| ORM constraint | Shall be a SOLAR_MAGNETIC_ECLIPTIC dynamic binding category ORM. |
| CS label | EUCLIDEAN |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | - See 7.5.9. - In the case of planet Earth, this STF is also known as a geocentric solar magnetospheric SRF. |
| References | [CRUS] |

### Solar magnetic dipole SRF

Solar magnetic dipole SRFs shall be derived from the SRFT specified in Table 8.16.

Table 8.16 — Solar magnetic dipole SRFT

| Specification element | Value |
| --- | --- |
| Label | SOLAR_MAGNETIC_DIPOLE |
| Code | 13 |
| Short name and description | solar magnetic dipole SRF A Euclidean 3D CS based planet centred SRF with the z-axis aligned magnetic dipole and the xz-plane containing the Sun. |
| Object type | A planet in the solar system with a magnetic dipole axis distinct from its rotational axis. |
| ORM constraint | Shall be a SOLAR_MAGNETIC_DIPOLE dynamic binding category ORM. |
| CS label | EUCLIDEAN |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.10. |
| References | [CRUS] , [BHAV] |

### Heliospheric Aries ecliptic SRF

Heliospheric Aries ecliptic SRFs shall be derived from the SRFT specified in Table 8.17.

Table 8.17 — Heliospheric Aries ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | HELIOSPHERIC_ARIES_ECLIPTIC |
| Code | 14 |
| Short name and description | Heliospheric Aries ecliptic SRF A spherical CS based Sun centred SRF with zero spherical latitude aligned with the ecliptic plane and zero longitude aligned to the first point of Aries. |
| Object type | Sun. |
| ORM constraint | Shall be a heliocentric_ARIES_ecliptic dynamic binding category ORM. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.6. |
| References | [HAPG] |

### Heliospheric Earth ecliptic SRF

Heliospheric Earth ecliptic SRFs shall be derived from the SRFT specified in Table 8.18.

Table 8.18 — Heliospheric Earth ecliptic SRFT

| Specification element | Value |
| --- | --- |
| Label | HELIOSPHERIC_EARTH_ECLIPTIC |
| Code | 15 |
| Short name and description | heliospheric Earth ecliptic SRF A spherical CS based Sun centred SRF with zero spherical latitude aligned with the ecliptic plane and zero longitude aligned to the centre of the Earth. |
| Object type | Sun. |
| ORM constraint | Shall be a Heliocentric_Planet_Ecliptic dynamic binding category ORM with respect to Earth. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.6. |
| References | [HAPG] |

### Heliospheric Earth equatorial SRF

Heliospheric Earth equatorial SRFs shall be derived from the SRFT specified in Table 8.19.

Table 8.19 — Heliospheric Earth equatorial SRFT

| Specification element | Value |
| --- | --- |
| Label | HELIOSPHERIC_EARTH_EQUATORIAL |
| Code | 16 |
| Short name and description | heliospheric Earth equatorial SRF A spherical CS based Sun centred SRF with zero spherical latitude aligned with the equator of the Sun and zero longitude aligned to the centre of the Earth. |
| Object type | Sun. |
| ORM constraint | Shall be a Heliocentric_Planet_Equatorial dynamic binding category ORM with respect to Earth. |
| CS label | SPHERICAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | See 7.5.7. |
| References | [HAPG] |

### Mercator SRF

Mercator SRFs shall be derived from the SRFT specified in Table 8.20.

Table 8.20 — Mercator SRFT

| Specification element | Value |
| --- | --- |
| Label | MERCATOR |
| Code | 17 |
| Short name and description | Mercator SRF. A Mercator and augmented Mercator map projection of the oblate or sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | MERCATOR |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate ellipsoid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented Mercator CS induces the Mercator CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Oblique Mercator Spherical SRF

Oblique Mercator Spherical SRFs shall be derived from the SRFT specified in Table 8.21.

Table 8.21 — Oblique Mercator Spherical SRFT

| Specification element | Value |
| --- | --- |
| Label | OBLIQUE_MERCATOR_SPHERICAL |
| Code | 18 |
| Short name and description | Oblique Mercator SRF for a sphere ORM. An oblique Mercator and augmented oblique Mercator map projection of the sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from ORMT SPHERE. |
| CS label | OBLIQUE_MERCATOR_SPHERICAL |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Radius R = a |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented oblique Mercator CS induces the oblique Mercator CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Transverse Mercator SRF

Transverse Mercator SRFs shall be derived from the SRFT specified in Table 8.22.

Table 8.22 — Transverse Mercator SRFT

| Specification element | Value |
| --- | --- |
| Label | TRANSVERSE_MERCATOR |
| Code | 19 |
| Short name and description | Transverse Mercator SRF A transverse Mercator and augmented transverse Mercator map projection of the oblate or sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | TRANSVERSE_MERCATOR |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate ellipsoid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented transverse Mercator CS induces the transverse Mercator CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Lambert conformal conic SRF

Lambert conformal conic SRFs shall be derived from the SRFT specified in Table 8.23.

Table 8.23 — Lambert conformal conic SRFT

| Specification element | Value |
| --- | --- |
| Label | LAMBERT_CONFORMAL_CONIC |
| Code | 20 |
| Short name and description | Lambert conformal conic SRF A Lambert conformal conic and augmented Lambert conformal conic map projection of the oblate or sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | LAMBERT_CONFORMAL_CONIC |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate ellipsoid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented Lambert conformal conic CS induces the Lambert conformal conic CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Polar stereographic SRF

Polar stereographic SRFs shall be derived from the SRFT specified in Table 8.24.

Table 8.24 — Polar stereographic SRFT

| Specification element | Value |
| --- | --- |
| Label | POLAR_STEREOGRAPHIC |
| Code | 21 |
| Short name and description | Polar stereographic SRF A polar stereographic and augmented polar stereographic map projection of the oblate or sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from: ORMT OBLATE_ELLIPSOID or ORMT SPHERE |
| CS label | POLAR_STEREOGRAPHIC |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Oblate ellipsoid RD case - Major semi-axis a, Sphere RD case - Radius a, |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented polar stereographic CS induces the polar stereographic CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Equidistant cylindrical SRF

Equidistant cylindrical SRFs shall be derived from the SRFT specified in Table 8.25.

Table 8.25 — Equidistant cylindrical SRFT

| Specification element | Value |
| --- | --- |
| Label | EQUIDISTANT_CYLINDRICAL |
| Code | 22 |
| Short name and description | equidistant cylindrical SRF A equidistant cylindrical and augmented equidistant cylindrical map projection of the oblate or sphere RD component of the ORM. |
| Object type | physical |
| ORM constraint | Shall be derived from ORMT SPHERE. |
| CS label | EQUIDISTANT_CYLINDRICAL |
| CS coordinate component names and/or symbols | Same as the CS. h: ellipsoidal height is the vertical coordinate component. |
| Template parameters |  |
| CS parameter binding rules | CS parameters match RD values: Radius R =a |
| Coordinate valid-region | No additional restrictions. |
| Notes | The augmented equidistant cylindrical CS induces the equidistant cylindrical CS on the zero value vertical coordinate component surface (which coincides with the RD surface). |
| References | [SNYD] |

### Local space rectangular 2D SRF

Local space rectangular 2D SRFs shall be derived from the SRFT specified in Table 8.26.

Table 8.26 — Local space rectangular 2D SRF template

| Specification element | Value |
| --- | --- |
| Label | LOCAL_SPACE_RECTANGULAR_2D |
| Code | 23 |
| Short name and description | local space rectangular 2D SRF A 2D Euclidean spatial reference frame for an abstract 2D space. |
| Object type | 2D abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | LOCOCENTRIC_EUCLIDEAN_2D |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | r = vector direction of forward (forward axis). |
| CS parameter binding rules |  |
| Coordinate valid-region | No additional restrictions. |
| Notes | CAD/CAM and 2D graphic applications. |
| References | [EDM] |

### Azimuthal 2D SRF

Azimuthal 2D SRFs shall be derived from the SRFT specified in Table 8.27.

Table 8.27 — Azimuthal 2D SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_SPACE_AZIMUTHAL |
| Code | 24 |
| Short name and description | Local space azimuthal 2D SRF An azimuthal CS based SRF for 2D abstract space. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | AZIMUTHAL |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | none |
| References | [EDM] |

### Polar 2D SRF

Polar 2D SRFs shall be derived from the SRFT specified in Table 8.28.

Table 8.28 — Polar 2D SRFT

| Specification element | Value |
| --- | --- |
| Label | LOCAL_SPACE_POLAR |
| Code | 25 |
| Short name and description | Local space polar 2D SRF A polar CS based SRF for 2D abstract space. |
| Object type | Abstract object. |
| ORM constraint | Shall be an ORM for a 2D abstract object. |
| CS label | POLAR |
| CS coordinate component names and/or symbols | The same as the CS. |
| Template parameters | none |
| CS parameter binding rules | none |
| Coordinate valid-region | No additional restrictions. |
| Notes | none |
| References | [EDM] |

## SRFs

This International Standard specifies a collection of SRFs. These specifications appear in Table 8.30. These SRFs are each derived from a SRFT. Additional SRFs derived from SRFTs may be registered in accordance with Clause 12.

The specification elements for SRFs are defined in Table 8.29.

Table 8.29 — SRF specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Short name | A short name as published or as commonly known and an optional description. |
| SRF template | The label of the applicable SRF template. |
| ORM | The label of the applicable ORM. |
| Valid-region | Optional restriction of the domain of the CS to a valid-region description and/or a valid-region specification. If a valid-region is specified, optionally, an extended valid-region may be specified. Valid-region specifications and extended valid-region specifications are specified by value or by reference. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). |
| Parameter values | The SRF template parameter values specified by value or by reference. If by reference, this field shall contain a citation(s) for the SRF template parameters values. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). Any parameter value that is not specified in the citation(s) shall be specified by value. |
| Notes | Optional, additional, non-normative information concerning the SRF, such as a description of its structure, modelled region, intended use, and/or application domain. |
| References | The references (see 12.2.5). |

Table 8.30 — SRFs

| Label | BRITISH_NATIONAL_GRID | Code | 1 |
| --- | --- | --- | --- |
| Short name | British National Grid |  |  |
| SRF template | TRANSVERSE_MERCATOR | ORM | OSGB_1936 |
| Valid-region | Valid-region description: Great Britain. |  |  |
| Parameter values | longitude of origin: λorigin = {True origin, Longitude} latitude of origin: origin = {True origin, Latitude} central scale: k0 = {Scale on central meridian (F0)} false easting: uF = {Grid coordinates of true origin, E0} (taken as a negative value) false northing: vF = {Grid coordinates of true origin, N0} (taken as a positive value) |  |  |
| Notes | Also known as the UK National Projection. |  |  |
| References | [OSTM, Section 7, "National projection"] |  |  |
| Label | DELAWARE_SPCS_1983 | Code | 2 |
| Short name | Delaware (US) State Plane Coordinate System |  |  |
| SRF template | TRANSVERSE_MERCATOR | ORM | N_AM_1983 |
| Valid-region | Valid-region description: State of Delaware (US). |  |  |
| Parameter values | longitude of origin: λorigin = {Table 8: Central meridian} latitude of origin: origin = {Table 8: Origin, Latitude} central scale: k0 = (1 - 1/{Table 8: Scale reduction})) false easting: uF = {Appendix C: Coordinates of origin (meters), x} (taken as a positive value) false northing: vF = {Appendix C: Coordinates of origin (meters), y} (taken as a positive value) |  |  |
| Notes | To convert a coordinate in metres to a grid coordinate in US survey feet, use 1m = (39,37 / 12) US survey feet. |  |  |
| References | [SNYD, Table 8 and Appendix C, "Delaware"] |  |  |
| Label | GEOCENTRIC_DATUM- _AUSTRALIA_1994 | Code | 3 |
| Short name | The Geocentric Datum of Australia (GDA). |  |  |
| SRF template | CELESTIOCENTRIC | ORM | WGS_1984 |
| Valid-region | Valid region description: Australia |  |  |
| Parameter values | none |  |  |
| Notes | Adopted at a meeting of the Inter-governmental Committee on Surveying and Mapping held in Canberra on 28-29 November 1994. |  |  |
| References | [GDA] |  |  |
| Label | GEOCENTRIC_WGS_1984 | Code | 4 |
| Short name | Geocentric WGS 1984 |  |  |
| SRF template | CELESTIOCENTRIC | ORM | WGS_1984 |
| Valid-region | Valid-region description: Earth, global. |  |  |
| Parameter values | none |  |  |
| Notes | Mass centred. |  |  |
| References | [83502T, Chapter 2.1] |  |  |
| Label | GEODETIC_AUSTRALIA_1984 | Code | 5 |
| Short name | Geodetic Australia 1984 |  |  |
| SRF template | CELESTIODETIC | ORM | AUSTRALIAN_GEOD_1984 |
| Valid-region | Valid-region description: Australia and Tasmania. |  |  |
| Parameter values | none |  |  |
| Notes | none |  |  |
| References | [CECT] |  |  |
| Label | GEODETIC_WGS_1984 | Code | 6 |
| Short name | Geodetic WGS 1984 |  |  |
| SRF template | CELESTIODETIC | ORM | WGS_1984 |
| Valid-region | Valid-region description: Earth, global. |  |  |
| Parameter values | none |  |  |
| Notes | none |  |  |
| References | [83502T, Chapter 3] |  |  |
| Label | GEODETIC_N_AMERICAN_1983 | Code | 7 |
| Short name | Geodetic North American 1983 |  |  |
| SRF template | CELESTIODETIC | ORM | N_AM_1983 |
| Valid-region | Valid region description: Continental United States |  |  |
| Parameter values | none |  |  |
| Notes | none |  |  |
| References | [SNYD |  |  |
| Label | GEODETIC_WGS_1984 | Code | 8 |
| Short name | Geodetic WGS 1984 |  |  |
| SRF template | CELESTIODETIC | ORM | WGS_1984 |
| Valid-region | Valid region description: Earth, Global |  |  |
| Parameter values | none |  |  |
| Notes | none |  |  |
| References | [83502T, Chapter 3] |  |  |
| Label | IRISH_GRID_1965 | Code | 9 |
| Short name | Irish Grid |  |  |
| SRF template | TRANSVERSE_MERCATOR | ORM | IRELAND_1965 |
| Valid-region | Valid-region description: Ireland. |  |  |
| Parameter values | Longitude of origin: λorigin = {True Origin, Longitude} Latitude of origin: origin = {True Origin, Latitude} Central scale: k0 = {Scale Factor on Central Meridian} False easting: uF = {Plane Co-ordinates of True Origin, East} (taken as a positive value) False northing: vF = {Plane Co-ordinates of True Origin, North} (taken as a positive value) |  |  |
| Notes | The Irish Grid has developed over more than two hundred years and is the coordinate reference system used in Ireland. |  |  |
| References | [IGRID, "The Transverse Mercator Map Projection"] |  |  |
| Label | IRISH_TRANSVERSE_MERCATOR_1989 | Code | 10 |
| Short name | Irish Transverse Mercator |  |  |
| SRF template | TRANSVERSE_MERCATOR | ORM | ETRS_1989 |
| Valid-region | Valid-region description: Ireland. |  |  |
| Parameter values | Longitude of origin: λorigin = {True Origin, Longitude} Latitude of origin: origin = {True Origin, Latitude} Central scale: k0 = {Scale on CM} False easting: uF = {False Origin (metres), West} (taken as a positive value) False northing: vF = {False Origin (metres), South} (taken as a positive value) |  |  |
| Notes | A newly derived projection. The true origin and central meridian defined in the Irish Grid are maintained, thus distributing the distortions due to the projection evenly. |  |  |
| References | [NMPI, Table 1, "ITM"] |  |  |
| Label | LAMBERT_93 | Code | 11 |
| Short name | Lambert-93 |  |  |
| SRF template | LAMBERT_CONFORMAL_CONIC | ORM | RGF_1993 |
| Valid-region | Valid-region description: France. |  |  |
| Parameter values | First parallel: 1 = {Parallèles d'échelle conservée (parallèles "standards"), 1 } Second parallel: 2 = {Parallèles d'échelle conservée (parallèles "standards"), 2 } Longitude of origin: λorigin = {Origine: méridien central: λ0} Latitude of origin: origin = {Origine: latitude origine: 0} False easting: uF = {Coordonnées de l'origine: X0} (taken as a positive value) False northing: vF = {Coordonnées de l'origine: Y0} (taken as a positive value) |  |  |
| Notes | Originally specified in September 1996. |  |  |
| References | [PASG, "Caractéristiques de la projection conique conforme (projection dite de Lambert)"] |  |  |
| Label | SRF_LAMBERT_II_WIDE | Code | 12 |
| Short name | Lambert II étendu (Lambert II wide) |  |  |
| SRF template | LAMBERT_CONFORMAL_CONIC | ORM | NTF_1896_PM_PARIS |
| Valid-region | Valid-region description: France. |  |  |
| Parameter values | First parallel: 1 = {Parallèles automécoïques} (first) Second parallel: 2 = {Parallèles automécoïques} (second) Longitude of origin: λorigin = {Longitude origine} Latitude of origin: origin = {Latitude origine} False easting: uF = {X0 : False Easting} (taken as a positive value) False northing: vF = {Y0 : False Northing} (taken as a positive value) |  |  |
| Notes | An extension of Lambert Zone II to cover all of France. Note that the prime meridian of the ORM is Paris (not Greenwich). |  |  |
| References | [LIIE, "Valeurs pour le calcul des coordonnes en projection Lambert de l'ellipsoïde de Clarke 1880 IGN.", "Zone lambert: II étendu"] |  |  |
| Label | MARS_PLANETOCENTRIC_2000 | Code | 13 |
| Short name | Mars planetodetic SRF |  |  |
| SRF template | CELESTIODETIC | ORM | MARS_SPHERE_2000 |
| Valid-region | Valid-region description: Mars, global. |  |  |
| Parameter values | none |  |  |
| Notes | Also referred to as "east/'ocentric"; adopted as the basis for current map production by the USGS, NASA, and ESA. Spherical latitude coincides with geodetic latitude. |  |  |
| References | [DUXB] |  |  |
| Label | MARS_PLANETOGRAPHIC | Code | 14 |
| Short name | Mars planetodetic SRF |  |  |
| SRF template | PLANETODETIC | ORM | MARS_2000 |
| Valid-region | Valid-region description: Mars, global. |  |  |
| Parameter values | none |  |  |
| Notes | Also referred to as "west/'ographic"; used historically for map production. Planetodetic longitude is positive westwards. |  |  |
| References | [DUXB] |  |  |
| Label | MARYLAND_SPCS_1983 | Code | 15 |
| Short name | Maryland (US) State Plane Coordinate System |  |  |
| SRF template | LAMBERT_CONFORMAL_CONIC | ORM | N_AM_1983_CONUS |
| Valid-region | Valid-region description: State of Maryland (US). |  |  |
| Parameter values | First parallel: 1 = {Table 8: Standard parallels} (first) Second parallel: 2 = {Table 8: Standard parallels} (second) Longitude of origin: λorigin = {Table 8: Origin, Longitude} Latitude of origin: origin = {Appendix C: Latitude of origin} False easting: uF = {Appendix C: Coordinates of origin (meters), x} (taken as a positive value) False northing: vF = {Appendix C: Coordinates of origin (meters), y} (taken as a positive value) |  |  |
| Notes | To convert a coordinate in metres to a grid coordinate in US survey feet, use 1m = (39,37 / 12) US survey feet. |  |  |
| References | [SNYD, Table 8 and Appendix C, "Maryland"] |  |  |

## SRF sets

Given an ORM, an SRF set is a finite parameterized set of two or more SRFs that:

- are derived from the same SRF template using the given ORM, and

- the valid-regions of the set members have non-overlapping interiors.

An SRF set specification may further restrict the ORM constraints of the SRFT. The specification elements for SRF sets are defined in Table 8.31.

Table 8.31 — SRF set specification elements

| Specification element | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Short name | A short name as published or as commonly known, and an optional description. |
| SRFT | The label of the applicable SRFT. |
| ORM constraints | Criteria for allowable ORMs. Specifying a single ORM indicates that only that ORM shall be used. |
| Coverage description | Optional description of the region corresponding to the union of the valid regions of all of the set members. |
| SRF set membership | A specification of the parameterization of the set members by listing or parameter algorithm, and valid-region descriptions or valid-region specifications. If valid-region specifications are included, extended valid-region specifications may also be included. References to other specification tables may be used for this purpose (see Table 8.33). Valid-region specifications and extended valid-region specifications are specified by value or by reference. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). |
| Notes | An optional description of the structure, modelled region, intended use, and/or application domain of the SRF set. |
| References | Optional references (see 12.2.5). |

This International Standard specifies a collection of SRF sets. These specifications appear in Table 8.32. The specified collection is not intended to be exhaustive. It includes national and regional grid systems as exemplars of the SRF set concept. Additional SRF sets may be registered in accordance with Clause 12.

Table 8.32 — SRF sets

| Label | ALABAMA_SPCS |  | Code |  |  | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Short name | Alabama (US) State Plane Coordinate System. |  |  |  |  |  |
| SRF template | TRANSVERSE_MERCATOR |  | ORM constraints |  |  | ORM N_AM_1983 |
| Coverage description | Valid-region description: State of Alabama (US). |  |  |  |  |  |
| SRF set membership | Table 8.34, subheading SRFS ALABAMA_SPCS. |  |  |  |  |  |
| Notes | A set of two localized adjacent SRFs where only one SRF is used for each county in the state and no overlap is allowed. |  |  |  |  |  |
| References | [SNYD, Table 8 and Appendix C, "Alabama" (East and West)] |  |  |  |  |  |
| Label | GTRS_GLOBAL_COORDINATE_SYSTEM | Code |  |  |  | 2 |
| Short name | GTRS global coordinate system (Earth). |  |  |  |  |  |
| SRF template | LOCAL_TANGENT_SPACE_EUCLIDEAN |  | ORM constraints |  |  | A global model ERM such as ORM WGS_1984. |
| Coverage description | Valid-region description: Earth (complete). |  |  |  |  |  |
| SRF set membership | Table 8.35, subheading SRFS GTRS_GLOBAL_COORDINATE_SYSTEM. |  |  |  |  |  |
| Notes | A set of 49 896 localized (but overlapping) SRFs, each approximately 100 kilometres square, that are identified according to the geotile reference system indexing scheme. The GTRS GCS SRF set specifies the ORM WGS_1984. The members of this SRF set are known as cells. For much of the RD surface, each cell valid-region covers one arc degree of geodetic latitude by one arc degree of geodetic longitude. However, near the poles, many arc degrees of longitude are grouped together into a single GCS cell since an arc degree of geodetic longitude becomes arbitrarily small near the poles. GCS cells are always one arc degree of geodetic latitude in extent. Within each GCS cell, a false origin offset is provided. The point of tangency is at the centre of the rectangular GCS cell, even if more than one arc degree of geodetic longitude falls within the GCS SRF cell. The SRFT LOCAL_TANGENT_SPACE_EUCLIDEAN azimuth parameter (α) is zero. |  |  |  |  |  |
| References | [I18025, Table 6.11, GTRS_GEOTILE] |  |  |  |  |  |
| Label | JAPAN_RECTANGULAR_PLANE_CS | Code |  |  |  | 3 |
| Short name | Japan Plane Coordinate System |  |  |  |  |  |
| SRF template | TRANSVERSE_MERCATOR | ORM constraints |  |  | ORM GRS_1980 |  |
| Coverage description | Valid-region description: Japan excluding northern territories. |  |  |  |  |  |
| SRF set membership | Table 8.34, subheading SRFS JAPAN_RECTANGULAR_PLANE_CS. |  |  |  |  |  |
| Notes | - The coordinate component ordering convention is: northing first, easting second. - A set of nineteen localized SRFs, each limited to 130 km eastward and westward from the central meridian. Valid-regions are described by political regions (cities, prefectures, counties, and/or partitions thereof). |  |  |  |  |  |
| References | [JMLIT] |  |  |  |  |  |
| Label | LAMBERT_NTF | Code |  |  |  | 4 |
| Short name | Lambert NTS. The Lambert projection-based mapping system for France associated with the NTF. |  |  |  |  |  |
| SRF template | LAMBERT_CONFORMAL_CONIC |  | ORM constraints |  |  | ORM NTF_PM_PARIS |
| Coverage description | Valid-region description: France. |  |  |  |  |  |
| SRF set membership | Table 8.34, subheading SRFS LAMBERT_NTF. |  |  |  |  |  |
| Notes | A set of four localized adjacent SRFs where only one SRF is used for each portion of France and no overlap is allowed. The prime meridian for each is Paris, France. |  |  |  |  |  |
| References | [LIIE, "Valeurs pour le calcul des coordonnes en projection Lambert de l'ellipsoïde de Clarke 1880 IGN.", "Zone lambert" (I, II, III, and IV)] |  |  |  |  |  |
| Label | UNIVERSAL_POLAR_STEREOGRAPHIC |  |  | Code |  | 5 |
| Short name | Universal Polar Stereographic (Earth). |  |  |  |  |  |
| SRF template | POLAR_STEREOGRAPHIC |  | ORM constraints |  |  | A global model ERM such as ORM WGS_1984. |
| Coverage description | Valid-region specification:  < -80º or 84º ≤  Extended valid-region specification:  < -79,5º or 83,5º ≤  |  |  |  |  |  |
| SRF set membership | Table 8.35, subheading SRFS UNIVERSAL_POLAR_STEREOGRAPHIC. |  |  |  |  |  |
| Notes | A set of two localized adjacent SRFs, where limited overlap is modelled by extended validity regions in the member SRFs. Shares a common boundary with SRFS UNIVERSAL_TRANSVERSE_MERCATOR. |  |  |  |  |  |
| References | [83582, "3-2.4 Specifications of the UPS."] |  |  |  |  |  |
| Label | UNIVERSAL_TRANSVERSE_MERCATOR |  | Code |  |  | 6 |
| Short name | Universal Transverse Mercator (Earth). |  |  |  |  |  |
| SRF template | TRANSVERSE_MERCATOR |  | ORM constraints |  |  | A global model ERM such as ORM WGS_1984. |
| Coverage description | Valid-region specification: -80º ≤  ≤ 84º Extended valid-region specification: -80,5º <  ≤ 84,5º |  |  |  |  |  |
| SRF set membership | Table 8.35, subheading SRFS UNIVERSAL_TRANSVERSE_MERCATOR. |  |  |  |  |  |
| Notes | A set of 120 localized SRFs, where limited overlap is modelled by extended validity regions in the member SRFs. Shares a common boundary with SRFS UNIVERSAL_POLAR_STEREOGRAPHIC. |  |  |  |  |  |
| References | [83582, "2-3 Specifications of the UTM."] |  |  |  |  |  |
| Label | WISCONSIN_SPCS | Code |  |  |  | 7 |
| Short name | Wisconsin (US) State Plane Coordinate System. |  |  |  |  |  |
| SRF template | LAMBERT_CONFORMAL_CONIC | ORM constraints |  |  | ORM N_AM_1983 |  |
| Coverage description | Valid-region description: State of Wisconsin (US). |  |  |  |  |  |
| SRF set membership | Table 8.34, subheading SRFS WISCONSIN_SPCS. |  |  |  |  |  |
| Notes | A set of three localized adjacent SRFs where only one SRF is used for each county in the state and no overlap is allowed. |  |  |  |  |  |
| References | [WSCO, "SPC 83" (South, Central, and North)] |  |  |  |  |  |

The specification elements for SRF set members are defined in Table 8.33.

Table 8.33 — SRF set member specification elements

| Specification element | Definition | Location |
| --- | --- | --- |
| SRF set label | The label of the SRF set (see 12.2.2). | Table 8.34 or Table 8.35 |
| Label | The label (see 12.2.2). | Table 8.34 or Table 8.35 |
| Code | The code (see 12.2.3); the set member parameter. | Table 8.34 or Table 8.35 |
| Description | A short description including name as published or as commonly known. | Table 8.34 or Table 8.35 |
| Valid-region | A valid-region description or specification. Optionally an extended valid-region specification. Valid-region specifications and extended valid-region specifications are specified by value or by reference. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). | Table 8.34 or Table 8.35 |
| Parameter values | The SRF template parameter values specified by value or by reference. If by reference, this field shall contain a citation(s) for the SRF template parameters values. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). Any parameter value that is not specified in the citation(s) shall be specified by value case. | Table 8.34 or Table 8.35 |
| Notes | Optional, additional, non-normative information concerning the SRF set member. | Table 8.34 or Table 8.35 |

This International Standard specifies a collection of SRF sets. These specifications appear in Table 8.34 and Table 8.35. Additional SRF set members may be registered in accordance with Clause 12.

Table 8.34 — Explicit specifications of SRF set members

| SRFS ALABAMA_SPCS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Label | WEST_ZONE |  | Code |  |  | 1 |  |
| Description | West zone. |  |  |  |  |  |  |
| Valid-region | Valid-region description: [ALSP, "west zone"] |  |  |  |  |  |  |
| Parameter values | Longitude of origin: λorigin = {Table 8: Central meridian} Latitude of origin: origin = {Table 8: Origin} (latitude) Central scale: k0 = (1 - 1/{Table 8: Scale reduction}) False easting: uF = {Appendix C: Coordinates of origin (meters), x} (taken as a positive value) False northing: vF = {Appendix C: Coordinates of origin (meters), y} (taken as a positive value) |  |  |  |  |  |  |
| Notes | To convert a coordinate in metres to a grid coordinate in US survey feet, use 1m = (39,37 / 12) US survey feet. |  |  |  |  |  |  |
| Label | EAST_ZONE |  | Code |  |  | 2 |  |
| Description | East zone. |  |  |  |  |  |  |
| Valid-region | Valid-region description: [ALSP, "east zone"] |  |  |  |  |  |  |
| Parameter values | Longitude of origin: λorigin = {Table 8: Central meridian} Latitude of origin: origin = {Table 8: Origin} (latitude) Central scale: k0 = (1 - 1/{Table 8: Scale reduction}) False easting: uF = {Appendix C: Coordinates of origin (meters), x} (taken as a positive value) False northing: vF = {Appendix C: Coordinates of origin (meters), y} (taken as a positive value) |  |  |  |  |  |  |
| Notes | To convert a coordinate in metres to a grid coordinate in US survey feet, use 1m = (39,37 / 12) US survey feet. |  |  |  |  |  |  |
| SRFS JAPAN_RECTANGULAR_PLANE_CS |  |  |  |  |  |  |  |
| Label |  | SYSTEM_I |  | Code | 1 |  |  |
| Description |  | System number I |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_II |  | Code | 2 |  |  |
| Description |  | System number II |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_III |  | Code | 3 |  |  |
| Description |  | System number III |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_IV |  | Code | 4 |  |  |
| Description |  | System number IV |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_V |  | Code | 5 |  |  |
| Description |  | System number V |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_VI |  | Code | 6 |  |  |
| Description |  | System number VI |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_VII |  | Code | 7 |  |  |
| Description |  | System number VII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_VIII |  | Code | 8 |  |  |
| Description |  | System number VIII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_IX |  | Code | 9 |  |  |
| Description |  | System number IX |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_X |  | Code | 10 |  |  |
| Description |  | System number X |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XI |  | Code | 11 |  |  |
| Description |  | System number XI |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XII |  | Code | 12 |  |  |
| Description |  | System number XII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XIII |  | Code | 13 |  |  |
| Description |  | System number XIII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XIV |  | Code | 14 |  |  |
| Description |  | System number XIV |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XV |  | Code | 15 |  |  |
| Description |  | System number XV |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XVI |  | Code | 16 |  |  |
| Description |  | System number XVI |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XVII |  | Code | 17 |  |  |
| Description |  | System number XVII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XVIII |  | Code | 18 |  |  |
| Description |  | System number XVIII |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| Label |  | SYSTEM_XIX |  | Code | 19 |  |  |
| Description |  | System number XIX |  |  |  |  |  |
| Valid-region |  | Valid region description: {Applicable Range} |  |  |  |  |  |
| Parameter values |  | Longitude of origin: λorigin = {Longitude (Eastward)} Latitude of origin: forigin = {Latitude (Northward)} Central scale: k0 = {Note 2} False easting: uF = {Note 3, X} False northing: vF = {Note 3, Y} |  |  |  |  |  |
| Notes |  | none |  |  |  |  |  |
| SRFS LAMBERT_NTF |  |  |  |  |  |  |  |
| Label |  | ZONE_I |  | Code | 1 |  |  |
| Description |  | Zone I |  |  |  |  |  |
| Valid-region |  | Valid region specification: {Zone application} |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {Parallèles automécoïques} (first) Second parallel: f2 = {Parallèles automécoïques} (second) Longitude of origin: λorigin = {Longitude origine} Latitude of origin: forigin = {Latitude origine} False easting: uF = {X0 : False Easting} (taken as a positive value) False northing: vF = {Y0 : False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | The prime meridian is Paris, France. |  |  |  |  |  |
| Label |  | ZONE_II |  | Code | 2 |  |  |
| Description |  | Zone II |  |  |  |  |  |
| Valid-region |  | Valid region specification: {Zone application} |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {Parallèles automécoïques} (first) Second parallel: f2 = {Parallèles automécoïques} (second) Longitude of origin: λorigin = {Longitude origine} Latitude of origin: forigin = {Latitude origine} False easting: uF = {X0 : False Easting} (taken as a positive value) False northing: vF = {Y0 : False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | The prime meridian is Paris, France. |  |  |  |  |  |
| Label |  | ZONE_III |  | Code | 3 |  |  |
| Description |  | Zone III |  |  |  |  |  |
| Valid-region |  | Valid region specification: {Zone application} |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {Parallèles automécoïques} (first) Second parallel: f2 = {Parallèles automécoïques} (second) Longitude of origin: λorigin = {Longitude origine} Latitude of origin: forigin = {Latitude origine} False easting: uF = {X0 : False Easting} (taken as a positive value) False northing: vF = {Y0 : False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | The prime meridian is Paris, France. |  |  |  |  |  |
| Label |  | ZONE_IV |  | Code | 4 |  |  |
| Description |  | Zone IV |  |  |  |  |  |
| Valid-region |  | Valid region specification: {Zone application} |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {Parallèles automécoïques} (first) Second parallel: f2 = {Parallèles automécoïques} (second) Longitude of origin: λorigin = {Longitude origine} Latitude of origin: forigin = {Latitude origine} False easting: uF = {X0 : False Easting} (taken as a positive value) False northing: vF = {Y0 : False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | The prime meridian is Paris, France. |  |  |  |  |  |
| SRFS WISCONSIN_SPCS |  |  |  |  |  |  |  |
| Label |  | SOUTH_ZONE |  | Code | 1 |  |  |
| Description |  | South zone |  |  |  |  |  |
| Valid-region |  | Valid region description: [SNYD, Figure 7, "SOUTH ZONE"] |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {1st Standard Parallel} Second parallel: f2 = {2nd Standard Parallel} Longitude of origin: λorigin = {Longitude of Central Meridian} Latitude of origin: forigin = {Latitude of Origin} False easting: uF = {False Easting} (taken as a positive value) False northing: vF = {False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |  |  |  |
| Label |  | CENTRAL_ZONE |  | Code | 2 |  |  |
| Description |  | Central zone |  |  |  |  |  |
| Valid-region |  | Valid region description: [SNYD, Figure 7, "CENTRAL ZONE"] |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {1st Standard Parallel} Second parallel: f2 = {2nd Standard Parallel} Longitude of origin: λorigin = {Longitude of Central Meridian} Latitude of origin: forigin = {Latitude of Origin} False easting: uF = {False Easting} (taken as a positive value) False northing: vF = {False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |  |  |  |
| Label |  | NORTH_ZONE |  | Code | 3 |  |  |
| Description |  | North zone |  |  |  |  |  |
| Valid-region |  | Valid region description: [SNYD, Figure 7, "NORTH ZONE"] |  |  |  |  |  |
| Parameter values |  | First parallel: f1 = {1st Standard Parallel} Second parallel: f2 = {2nd Standard Parallel} Longitude of origin: λorigin = {Longitude of Central Meridian} Latitude of origin: forigin = {Latitude of Origin} False easting: uF = {False Easting} (taken as a positive value) False northing: vF = {False Northing} (taken as a positive value) |  |  |  |  |  |
| Notes |  | To convert coordinates in metres to grid coordinates in US survey feet, use 1 m. = (39,37 / 12) US survey feet. |  |  |  |  |  |

Table 8.35 — Implicit specifications of SRF set members

| SRFS GTRS_GLOBAL_COORDINATE_SYSTEM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label | “TILE_” + <code>, where the symbol “+” denotes concatenation. | Code | 1…49823: As specified in Table 8.36. |  |
| Description | Tile <code>. |  |  |  |
| Valid-region | Valid-region specification: As specified in Table 8.36. as range from the natural origin Extended valid-region specification: Unrestricted. |  |  |  |
| Parameter values | Natural origin: As specified in Table 8.36. Azimuth: α = 0 rad. Offset height: h0 = 0 m. false easting: uF = -50 000 m. false northing: vF = -50 000 m. |  |  |  |
| Notes | none |  |  |  |
| SRFS UNIVERSAL_POLAR_STEREOGRAPHIC |  |  |  |  |
| Label | As specified in Table 8.37 and Table 8.38. |  | Code | 1…2 |
| Description | Northern or southern pole. |  |  |  |
| Valid-region | Valid-region specification: As specified in Table 8.37 and Table 8.38. Extended valid-region specification: As specified in Table 8.37 and Table 8.38. |  |  |  |
| Parameter values | longitude of origin: λorigin as specified in Table 8.37 and Table 8.38. latitude of true scale: 1 as specified in Table 8.37 and Table 8.38. scale at 1: k1 = 0,994 false easting: uF = 2 000 000 m. false northing: vF = 2 000 000 m. |  |  |  |
| Notes | none |  |  |  |
| SRFS UNIVERSAL_TRANSVERSE_MERCATOR |  |  |  |  |
| Label | As specified in Table 8.39 and Table 8.40. In those tables the “+” symbol shall denote concatenation of character strings. |  | Code | 1…120 |
| Description | UTM zone in northern or southern hemisphere. |  |  |  |
| Valid-region | Valid-region specification: As specified in Table 8.39 and Table 8.40. Extended valid-region specification: As specified in Table 8.39 and Table 8.40. |  |  |  |
| Parameter values | longitude of origin: λorigin as specified in Table 8.39 and Table 8.40. latitude of origin: origin as specified in Table 8.39 and Table 8.40. central scale: k0 = 0,999 6 false easting: uF = 500 000 m. false northing: As specified in Table 8.39 and Table 8.40. |  |  |  |
| Notes | none |  |  |  |

Table 8.36 — GTRS natural origin and valid-region by code index

| Latitude band (Tile size) | Tile code | Natural origin (λorigin, origin) | Valid-region specification |
| --- | --- | --- | --- |
| 88º-90ºN (1º x 30º) | 1 + 12•m + n (m=0,1; n=0,..,11) | (-89,5º + m•1º, -165º + n•30º) | -15º ≤ λ - λorigin ≤+15º -0,5º ≤  - origin ≤+0,5º |
| 86º-88ºN (1º x 15º) | 25 + 24•m + n (m=0,1; n=0,..,23) | (-87,5º + m•1º, -172,5º + n•15º) | -7,5º ≤ λ - λorigin ≤+7,5º -0,5º ≤  - origin ≤+0,5º |
| 84º-86ºN (1º x 10º) | 73 + 36•m + n (m=0,1; n=0,..,35) | (-85,5º + m•1º, -175º + n•10º) | -5º ≤ λ - λorigin ≤+5º -0,5º ≤  - origin ≤+0,5º |
| 80º-84ºN (1º x 6º) | 145 + 60•m + n (m=0,..,3; n=0,..,59) | (-83,5º + m•1º, -177º + n•6º) | -3º ≤ λ - λorigin ≤+3º -0,5º ≤  - origin ≤+0,5º |
| 78º-80ºN (1º x 5º) | 385 + 72•m + n (m=0,1; n=0,..,71) | (-79,5º + m•1º, -177,5º + n•5º) | -2,5º ≤ λ - λorigin ≤+2,5º -0,5º ≤  - origin ≤+0,5º |
| 71º-78ºN (1º x 3º) | 529 + 120•m + n (m=0,..,6; n=0,..,119) | (-77,5º + m•1º, -178.5º + n•3º) | -1,5º ≤ λ - λorigin ≤+1,5º -0,5º ≤  - origin ≤+0,5º |
| 60º-71ºN (1º x 2º) | 1369 + 180•m + n (m=0,..,10; n=0,..,179) | (-70,5º + m•1º, -179º + n•2º) | -1º ≤ λ - λorigin ≤+1º -0,5º ≤  - origin ≤+0,5º |
| 60ºS - 60ºN (1º x 1º) | 3349 + 360•m + n (m=0,..,119; n=0,..,359) | (-59,5º + m•1º, -179,5º + n•1º) | -0,5º ≤ λ - λorigin ≤+0,5º -0,5º ≤  - origin ≤+0,5º |
| 71º-60ºS (1º x 2º) | 46549 + 180•m + n (m=0,..,10; n=0,..,179) | (60,5º + m•1º, -179º + n•2º) | -1º ≤ λ - λorigin ≤+1º -0,5º ≤  - origin ≤+0,5º |
| 78º-71ºS (1º x 3º) | 48529 + 120•m + n (m=0,..,6; n=0,..,119) | (71,5º + m•1º, -178.5º + n•3º) | -1,5º ≤ λ - λorigin ≤+1,5º -0,5º ≤  - origin ≤+0,5º |
| 80º-78ºS (1º x 5º) | 49369 + 72•m + n (m=0,1; n=0,..,71) | (78,5º + m•1º, -177,5º + n•5º) | -2,5º ≤ λ - λorigin ≤+2,5º -0,5º ≤  - origin ≤+0,5º |
| 84º-80ºS (1º x 6º) | 49513 + 60•m + n (m=0,..,3; n=0,..,59) | (80,5º + m•1º, -177º + n•6º) | -3º ≤ λ - λorigin ≤+3º -0,5º ≤  - origin ≤+0,5º |
| 86º-84ºS (1º x 10º) | 49752 + 36•m + n (m=0,1; n=0,..,35) | (84,5º + m•1º, -175º + n•10º) | -5º ≤ λ - λorigin ≤+5º -0,5º ≤  - origin ≤+0,5º |
| 88º-86ºS (1º x 15º) | 49825 + 24•m + n (m=0,1; n=0,..,23) | (86,5º + m•1º, -172,5º + n•15º) | -7,5º ≤ λ - λorigin ≤+7,5º -0,5º ≤  - origin ≤+0,5º |
| 90º-88ºS (1º x 30º) | 49873 + 12•m + n (m=0,1; n=0,..,11) | (88,5º + m•1º, -165º + n•30º) | -15º ≤ λ - λorigin ≤+15º -0,5º ≤  - origin ≤+0,5º |

Table 8.37 — UPS indexing scheme and set members: northern hemisphere

| Label | NORTHERN_POLE |
| --- | --- |
| Code | 1 |
| Description | UPS, northern pole. |
| Valid-region | Valid-region specification:  ≥ 84º Extended valid-region specification:  ≥ 83,5º |
| Parameter values | longitude of origin: λorigin = +0º latitude of true scale: 1 = +90º |

Table 8.38 — UPS indexing scheme and set members: southern hemisphere

| Label | SOUTHERN_POLE |
| --- | --- |
| Code | 1 |
| Description | UPS, southern pole. |
| Valid-region | Valid-region specification:  ≤ -80º Extended valid-region specification:  ≤ -79,5º |
| Parameter values | longitude of origin: λorigin = +180º latitude of true scale: 1 = -90º |

Table 8.39 — UTM indexing scheme and set members: northern hemisphere

| Code | 1…60 |
| --- | --- |
| Label | “ZONE_” + <code> + “_NORTHERN_HEMISPHERE” |
| Description | UTM Zone <code>, Northern hemisphere. |
| Valid-region | Valid-region specification: (-186º + (<code>)•6º) ≤ λ < (-180º + (<code>)•6º) 0º ≤  < 84º Extended valid-region specification: (-186,5º + (<code>)•6º) ≤ λ < (-179,5º + (<code>)•6º) -0,5º ≤  < 84,5º |
| Parameter values | longitude of origin: λorigin = (-183º + (<code>)•6º) latitude of origin: origin = 0º. false northing: vF = 0 m. |

Table 8.40 — UTM indexing scheme and set members: southern hemisphere

| Code | 61…120 |
| --- | --- |
| Label | “ZONE_” + (<code> - 60) + “_SOUTHERN_HEMISPHERE” |
| Description | UTM Zone <code>, Southern hemisphere. |
| Valid-region | Valid-region specification: (-186º + (<code> - 60)•6º) ≤ λ < (-180º + (<code> - 60)•6º) -80º ≤  < 0º Extended valid-region specification: (-186,5º + (<code> - 60)•6º) ≤ λ < (-179,5º + (<code> - 60)•6º) -80,5º ≤  < 0,5º |
| Parameter values | longitude of origin: λorigin = (-183º + (<code> - 60)•6º) latitude of origin: origin = 0º. false northing: vF = 10 000 000 m. |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

azimuthal 2D SRF	160

binding of a CS via a normal embedding	135

celestiocentric SRF	141

celestiodetic SRF	142, 143

celestiomagnetic SRF	148

dimension of an ORM	136

equatorial inertial SRF	149

equidistant cylindrical SRF	158

extended valid region	137

extended valid region specification	138

geocentric SRF	141

geodetic SRF	143

geomagnetic SRF	149

heliospheric Aries ecliptic SRF	152

heliospheric Earth ecliptic SRF	153

heliospheric Earth equatorial SRF	153

lambert conformal conic SRF	156

local space rectangular 2D SRF	159

local space rectangular SRF	142

local tangent space azimuthal spherical SRF	146

local tangent space cylindrical SRF	147

local tangent space Euclidean SRF	144

Mercator SRF	154

oblique Mercator SRF	154

polar 2D SRF	160

polar stereographic SRF	157

solar ecliptic SRF	150

solar equatorial SRF	150

solar magnetic dipole SRF	151

solar magnetic ecliptic SRF	151

spatial CS	135

spatial reference frame	136

SRF set	166

SRF set member	169

SRF template	139

transverse Mercator SRF	156

valid region	137

valid region description	138

valid region specification	138

vertical coordinate component	138

In ISO 19111 terminology, the tangent plane is an engineering datum.

In ISO 19111 terminology, the tangent plane is an engineering datum.