EDITORS NOTE: Table of contents tables will be removed from individual clauses. The TOC below is for draft review purposes only.

7 Reference datums, embeddings, and object reference models 101 7.1. Introduction 101 7.2. Reference datums 101 7.2.1 Introduction 101 7.2.2 Reference datums 101 7.2.3 RDs of category oriented surface based on ellipsoids 103 7.2.4 RDs associated with physical objects 105 7.2.5 RD binding 106 7.3. Embedding position-space into object-space 106 7.3.1 Introduction 106 7.3.2 Normal embeddings 107 7.3.3 Specification of 3D normal embeddings 107 7.3.4 Specification of 2D normal embeddings 109 7.4. Object reference model 109 7.4.1 Introduction 109 7.4.2 ORM 110 7.4.3 Binding constraint 110 7.4.4 ORM template 111 7.4.5 Standardized ORMs 118 7.5. Celestial body dynamic binding categories 121 7.5.1 Binding categories 121 7.5.2 Terminology 122 7.5.3 Equatorial inertial 122 7.5.4 Solar ecliptic 124 7.5.5 Solar equatorial 125 7.5.6 Heliocentric Aries ecliptic 126 7.5.7 Heliocentric planet ecliptic 127 7.5.8 Heliocentric planet equatorial 128 7.5.9 Celestiomagnetic 129 7.5.10 Solar magnetic ecliptic 131 7.5.11 Solar magnetic dipole 132

Table 7.1 — RD categories 101 Table 7.2 — RD specification fields 102 Table 7.3 — RD specification directory 102 Table 7.4 — 2D RDs of category point 102 Table 7.5 — 3D RDs of category point 103 Table 7.6 — 2D RDs of category directed curve 103 Table 7.7 — 3D RDs of category directed curve 103 Table 7.8 — 3D RDs of category oriented surface 103 Table 7.9 — Physical object RD specification fields 105 Table 7.10 — Physical RD specification table locations 106 Table 7.11 — ORMT specification fields 111 Table 7.12 — ORMT specification directory 111 Table 7.13 — 2D ORMT specifications 112 Table 7.14 — 3D ORMT specifications 113 Table 7.15 — ORM specification fields 119 Table 7.16 — Reference transformation specification fields 120 Table 7.17 — Binding category specification fields 121 Table 7.18 — Dynamic binding category specification directory 121 Table 7.19 — Equatorial inertial dynamic binding category 122 Table 7.20 — Equatorial inertial ORM directory 123 Table 7.21 — Solar ecliptic dynamic binding category 124 Table 7.22 — Solar ecliptic ORM directory 125 Table 7.23 — Solar equatorial dynamic binding category 125 Table 7.24 — Solar equatorial ORM directory 126 Table 7.25 — Heliocentric Aries ecliptic dynamic binding category 126 Table 7.26 — Heliocentric Aries ecliptic ORM directory 127 Table 7.27 — Heliocentric planet ecliptic dynamic binding category 127 Table 7.28 — Heliocentric planet ecliptic ORM directory 128 Table 7.29 — Heliocentric planet equatorial dynamic binding category 128 Table 7.30 — Heliocentric planet equatorial ORM directory 129 Table 7.31 — Celestiomagnetic dynamic binding category 129 Table 7.32 — Celestiomagnetic ORM directory 130 Table 7.33 — Solar magnetic ecliptic dynamic binding category 131 Table 7.34 — Solar magnetic ecliptic ORM directory 132 Table 7.35 — Solar magnetic dipole dynamic binding category 132 Table 7.36 — Solar magnetic dipole ORM directory 133

Figure 7.1 — Oblate and prolate ellipsoids 104 Figure 7.2 — An RD bound to an abstract object and to a real object 106 Figure 7.3 — A right-handed normal embedding 107 Figure 7.4 — 3D normal embedding relationships 108 Figure 7.5 — Rotation between E1 and E2 in two conventions 109 Figure 7.6 — Oblate ellipsoid ORMT binding 117 Figure 7.7 — Equatorial inertial dynamic binding category 123 Figure 7.8 — Solar ecliptic ORM binding 124 Figure 7.9 — Solar equatorial ORM binding 126 Figure 7.10 — Heliocentric Aries ecliptic ORM binding 127 Figure 7.11 — Heliocentric planet ecliptic dynamic binding category 128 Figure 7.12 — Heliocentric planet equatorial dynamic binding category 129 Figure 7.13 — Celestiomagnetic ORM binding 130 Figure 7.14 — Solar magnetic ecliptic ORM binding 132 Figure 7.15 — Solar magnetic dipole ORM binding 133

# Reference datums, embeddings, and object reference models

## Introduction

This International Standard specifies reference datums as geometric constructs that can be bound to define an aspect of a normal embedding of position-space into object-space. A reference datum binding identifies a reference datum in position-space with a corresponding constructed entity in object-space.

A normal embedding is a position-space model of object-space formed by a one-to-one distance preserving function of positions in position-space to points in object-space. A normal embedding establishes a basis for modelling that object-space with coordinate systems.

When reference datums are bound with properly constrained geometric constructs in object-space, a unique normal embedding is determined. Such a constrained set of bound reference datums is called an object reference model. The sets of reference datums and compatible binding constraints that can be used to realize such object reference models are abstracted in the notion of an object reference model template.

Object reference model realizations for specific celestial objects and the relationships between two such realizations for the same object are specified.

## Reference datums

### Introduction

A reference datum (RD) is a geometric construct in position-space that is used to specify an aspect of an embedding of position-space into object-space. In this International Standard, reference datums are defined for 1D, 2D, and 3D position-space. In the 2D and 3D cases, this International Standard specifies a small set of reference datums for use in its own specifications, however this set is not intended to be exhaustive. Users of this International Standard may specify additional reference datums by registration in accordance with Clause 12.

### Reference datums

In this International Standard, an RD is expressed analytically in position-space and corresponds to a geometric construct in object-space. These geometric constructs are limited to a point, a directed curve or an oriented surface. The analytic form of the position-space representation and the corresponding object-space geometric representation is described by category and position-space dimension in Table 7.1. An RD of a given category is specified by the parameters and/or the analytic expression of its position-space representation.

Table 7.1 — RD categories

| Category | Position-space representation |  |  | Object-space representation |
| --- | --- | --- | --- | --- |
|  | 1D | 2D | 3D |  |
| Point | (a) real a | (a, b) real a, b | (a, b, c) real a, b, c | a position in object-space |
| Directed curve |  |  |  | a curve in object-space with a designation of direction along the curve |
| Oriented surface |  |  |  | a surface in object-space with a designation of one side as positive |

This International Standard specifies 2D and 3D RDs by RD category in Table 7.4 through Table 7.8. The fields of those tables are defined in Table 7.2. 3D RDs based on ellipsoids are described in 7.2.3 and 7.2.4 and specified in Annex D with specification fields defined in Table 7.9.  Table 7.3 is a directory of RD specification tables or, in the case of 3D RDs based on ellipsoids, RD specification directories. Table J.1 is a directory of deprecated RDs.

Table 7.2 — RD specification fields

| Field | Definition |
| --- | --- |
| RD label | The label (see 12.2.2). |
| RD code | The code (see 12.2.3). |
| Description | A description of the RD including any common name for the concept. |
| Position-space representation | The analytic formulation of the RD in position-space |

Table 7.3 — RD specification directory

| Position-space dimension | RD category | Table number |
| --- | --- | --- |
| 2D | point | Table 7.4 |
| 3D | point | Table 7.5 |
| 2D | directed curve | Table 7.6 |
| 3D | directed curve | Table 7.7 |
| 3D | oriented surface | Table 7.8 and Table 7.10 |

Table 7.4 — 2D RDs of category point

| RD label | RD code | Description | Position-space representation |
| --- | --- | --- | --- |
| ORIGIN_2D | 1 | Origin in 2D | (0,0) |
| X_UNIT_POINT_2D | 2 | x-axis unit point in 2D | (1,0) |
| Y_UNIT_POINT_2D | 3 | y-axis unit point in 2D | (0,1) |

Table 7.5 — 3D RDs of category point

| RD label | RD code | Description | Position-space representation |
| --- | --- | --- | --- |
| ORIGIN_3D | 4 | Origin in 3D | (0,0,0) |
| X_UNIT_POINT_3D | 5 | x-axis unit point in 3D | (1,0,0) |
| Y_UNIT_POINT_3D | 6 | y-axis unit point in 3D | (0,1,0) |
| Z_UNIT_POINT_3D | 7 | z-axis unit point in 3D | (0,0,1) |

Table 7.6 — 2D RDs of category directed curve

| RD label | RD code | Description | Position-space representation |
| --- | --- | --- | --- |
| X_AXIS_2D | 8 | x-axis in 2D |  |
| Y_AXIS_2D | 9 | y-axis in 2D |  |

Table 7.7 — 3D RDs of category directed curve

| RD label | RD code | Description | Position-space representation |
| --- | --- | --- | --- |
| X_AXIS_3D | 10 | x-axis in 3D |  |
| Y_AXIS_3D | 11 | y-axis in 3D |  |
| Z_AXIS_3D | 12 | z-axis in 3D |  |

Table 7.8 — 3D RDs of category oriented surface

| RD label | RD code | Description | Position-space representation |
| --- | --- | --- | --- |
| XY_PLANE_3D | 13 | xy-plane |  |
| XZ_PLANE_3D | 14 | xz-plane |  |
| YZ_PLANE_3D | 15 | yz-plane |  |

### RDs of category oriented surface based on ellipsoids

The RDs specified in this International Standard include 3D RDs of category oriented surface for oblate ellipsoids, prolate ellipsoids, and tri-axial ellipsoids. These RDs are specified based upon certain geometrically-defined parameters. The position-space representations of oblate and prolate ellipsoid RDs are expressed in the form:

|  |  | () |
| --- | --- | --- |

When  an RD of this form is an oblate ellipsoid RD with major semi-axis a and minor semi-axis b as illustrated in Figure 7.1.

Spheres shall be considered as a special case of oblate ellipsoids. When  an oblate ellipsoid RD may be called a sphere RD. In this case, the value  is the radius of the sphere RD.

When  an RD of this form is a prolate ellipsoid RD with major semi-axis b and minor semi-axis a, as illustrated in Figure 7.1.

Figure 7.1 — Oblate and prolate ellipsoids

Instead of specifying the parameters of oblate ellipsoid RDs as major semi-axis a and minor semi-axis b it is both equivalent and sometimes convenient to use the major semi-axis a and inverse flattening ratio f -1 as defined in Equation (7.2). The minor semi-axis b may be expressed in terms of the major semi-axis a and flattening ratio f as in Equation (7.3).

|  |  | () |
| --- | --- | --- |
|  |  | () |

The position-space representation of a tri-axial ellipsoid RD is expressed in the form:

|  |  | () |
| --- | --- | --- |

The semi-axes a, b, and c shall be positive non-zero and .

### RDs associated with physical objects

In the case of ellipsoid RDs intended for modelling aspects of physical objects, published parameter values for these objects are used. The specification of these RDs includes the published ellipsoid parameters and the identification of the associated physical object. The specification fields are defined in Table 7.9.

Table 7.9 — Physical object RD specification fields

| Field | Specification |  |
| --- | --- | --- |
| RD label | The label (see 12.2.2). |  |
| RD code | The code (see 12.2.3). |  |
| Description | The description including the name as published or as commonly known. |  |
| Physical object | The name of the physical object. |  |
| Parameters | Oblate ellipsoid case | Major semi-axis, a Inverse flattening, f –1 |
|  | Sphere case | Radius, r |
|  | Prolate ellipsoid case | Minor semi-axis, a Major semi-axis, b |
|  | Tri-axial ellipsoid case | x-semi-axis, a y-semi-axis, b z-semi-axis, c |
|  | RD parameters shall be specified by value or by reference (see 12.2.5). If by value, the value(s) shall be specified. The value(s) shall be followed by a error estimate expressed in one of the following forms: - error estimate: unknown - error estimate: assumed precise - error estimate (1): <parameter name>:<error value> EXAMPLE error estimate (1): . If by reference, this field shall express the value(s) and error estimate(s) using the terminology found in the reference. These terms shall be enclosed in brackets ( {} ). Any parameter value that is not specified in the citation(s) shall be specified as in the “by value” case. |  |
| Date | The date the RD parameters were specified or published. |  |
| References | The references (see 12.2.5). |  |

The RDs associated with physical objects are specified in Annex D and Table J.1. Table 7.10 is a directory of these RDs organized by type of ellipsoid. The semi-axis and radius parameters are unitless in position-space, but are bound to metre lengths when the RD is identified as a corresponding physical object-space construct.

Table 7.10 — Physical RD specification table locations

| Type of ellipsoid | RD table |
| --- | --- |
| Oblate ellipsoid | Table D.2 and Table J.2 |
| Sphere | Table D.3 and Table J.3 |
| Prolate ellipsoid | Table D.4 and Table J.4 |
| Tri-axial ellipsoid | Table D.5 and Table J.5 |

Additional RDs associated with physical objects may be specified by registration in accordance with Clause 12.

### RD binding

An RD is bound when the RD in position-space is identified with a corresponding constructed entity in object-space. The term corresponding in this context means that each RD is bound to a constructed entity of the same geometric object type. That is, position-space points are bound to identified points in object-space, position-space directed lines to constructed lines in object-space, position-space directed curves to constructed curves in object-space, position-space oriented planes to constructed planes in object-space, and position-space oriented surfaces to constructed surfaces in object-space.

Figure 7.2 illustrates two distinct bindings of a point RD. On the left, it is bound to a specific point in the abstract object-space of a CAD model. On the right, it is bound to a point on an object in physical object-space that has been manufactured from that CAD model.

Figure 7.2 — An RD bound to an abstract object and to a real object

In this International Standard, in the case of physical objects, one unit in position-space corresponds to one metre in object-space. In the case of abstract objects, one unit in position-space corresponds to the designated length scale unit in object-space.

## Embedding position-space into object-space

### Introduction

An embedding is a position-space model of object-space formed by a one-to-one function of positions in position-space to points in object-space.

### Normal embeddings

A function E from position-space to object-space is distance preserving if for any n-tuples p and q in position-space, the measured distance in object-space from E(p) to E(q) in metres is equal to the Euclidean distance d(p, q). A normal embedding is a distance preserving function E from position-space into object-space.

In all dimensions of position-space, the point E(0) is the origin of the normal embedding E, and the point E(e1) is the x-axis unit point of the normal embedding E. If the dimension of position-space is 2D or 3D, the point E(e2) is the y-axis unit point of the normal embedding E. If the dimension of position-space is 3D, the point E(e3) is the z-axis unit point of the normal embedding E.

A normal embedding of a 3D position-space is right-handed if the directed triangle formed by the three points, x-axis unit point, y-axis unit point, and z-axis unit point, in that sequence, has a clockwise orientation when viewed from the origin of the embedding. Otherwise, the embedding is left-handed. A right-handed normal embedding is illustrated in Figure 7.3. All 3D normal embeddings in this International Standard shall be right-handed.

Figure 7.3 — A right-handed normal embedding

NOTE 1	As a consequence of the distance preserving property, a normal embedding is one to one, and the image in object-space of the lines associated with the position-space axes are orthogonal.

Note 2	Table 7.5 defines four reference datums of category point. They may be bound as follows. RD ORIGIN_3D: (0, 0, 0) is identified as the origin of the embedding. RD X_UNIT_POINT_3D: (1, 0, 0) is identified as the x-axis unit point. RD Y_UNIT_POINT_3D: (0, 1, 0) is identified as the y-axis unit point. RD Z_UNIT_POINT_3D: (0, 0, 1) is identified as the z-axis unit point.

### Specification of 3D normal embeddings

A 3D object-space may have many normal embeddings of 3D position-space. Given two 3D normal embeddings E1 and E2 into the same object-space, one embedding can be expressed in terms of the other normal embedding. Given a position  in position-space, the normal embedding E2 associates to it a unique point p in object-space. The normal embedding E1 uniquely associates some position  to the same point p. The association of  to  is the embedding transformation from E2 to E1.

In general, the origin of the E2 normal embedding may be displaced with respect to the origin of the E1 normal embedding and the axes of the E2 normal embedding may also be rotated and/or differently scaled with respect to the axes of the E1 normal embedding (see Figure 7.4). If E1 associates the position  to the origin of the E2 normal embedding, the embedding transformation from E2 to E1 may be specified in the form of the seven parameter transformation:

where:

and where:

|  | , |  |
| --- | --- | --- |
|  | , |  |
|  | , and |  |
|  | . |  |



z



y



x













y

-

axis

E1

x

-

axis

E1

z

-

axis

E1

x

-

axis

E2

z

-

axis

E2

y

-

axis

E2

p

Figure 7.4 — 3D normal embedding relationships

The scale adjustment is needed to account for differing length scales in abstract object-space. In the case of physical object-space, small non-zero values of  may be required. This is addressed in 7.4.5.

The convention of viewing the rotations with respect to E1 is the position vector rotation convention. The coordinate frame rotation convention views rotations with respect to E2 instead of E1. The rotations  in Equation (7.5) are position vector rotations. If coordinate frame rotations are used, the rotations reverse sign (see Figure 7.5).

Note 1	Sign reversal does not affect cosine terms in the equation. Only the sine terms reverse sign.

E1 axis

E2 axis



= coordinate frame rotation



= position vector rotation



CFR



PVR



=



PVR

CFR

PVR

CFR

Figure 7.5 — Rotation between E1 and E2 in two conventions

NOTE 2	A small rotation approximation of the seven parameter transformation is described in Annex B.

The seven parameter embedding specification of E2 with respect to E1 is defined by the seven parameter values  in the position vector rotation convention (as in Equation ).

NOTE 3	In the cases that  the formula for the transformation from  to  reduces to a translation operation .

### Specification of 2D normal embeddings

Given two 2D normal embeddings E1 and E2 into the same 2D object-space, one embedding can be expressed in terms of the other normal embedding. Given a position  in position-space, the normal embedding E2 associates to it a unique point p in object-space. The normal embedding E1 uniquely associates some position  to the same point p. The association of  to  is the embedding transformation from E2 to E1.

In general, the origin of the E2 embedding may be displaced with respect to the origin of the E1 embedding and the axes of the E2 embedding may also be rotated and/or differently scaled with respect to the axes of the E1 embedding. If E1 associates  to the origin of the E2 embedding,  is the scale factor, and  is the relative rotation, the 2D embedding transformation from  to  may be specified in the form of the four parameter transformation:

with parameters:

## Object reference model

### Introduction

A normal embedding of position-space establishes a basis for modelling that object-space with spatial coordinates. RDs bound with properly constrained geometric constructs in object-space can determine a unique normal embedding and consequently a model of the object-space, called an object reference model. Of particular interest are object reference models that include an oriented surface RD that models a surface significant to the object. The sets of RDs and compatible binding constraints that can be used to realize such object reference models are abstracted in this International Standard in the notion of object reference model templates.

### ORM

A normal embedding of position-space into object-space is compatible with an RD binding if the image of the locus of the RD in position-space is coincident with the points (and direction or orientation, as applicable) of the geometric construction of the binding.

An object reference model (ORM) for a spatial object is a set of RDs bound by identification with geometric constructions in object-space for which there exists exactly one normal embedding of position-space into object-space that is compatible with each RD binding in the set. In the 3D case, this unique embedding shall also be right-handed.

An ORM is object-fixed if the binding measurements (for example: positions, directions) are fixed with respect to the embedding of position-space from the time of the binding, otherwise it is called object-dynamic. The object-fixed definition assumes that the object itself is not changing in time by an amount significant for the accuracy and time scale of an application. The embedding of position-space determined by an ORM is, correspondingly, either an object-fixed embedding or an object-dynamic embedding.

An ORM is often selected to contain one or more RDs of category oriented surface to correspond to one or more (physical or conceptual) surfaces that are significant to the modelled spatial object. An RD is chosen and its position with respect to the object is bound so that that the RD instance is a “best fit” to the object in some application-specific sense. In particular, if the RD surface is “fitted” to a specific part of the object surface, the ORM is a local model. If the RD is selected to best fit the entire surface, the ORM is a global model.

An Earth reference model (ERM) is an ORM for which the spatial object is Earth.

EXAMPLE	If the object is a planet, then an ORM containing an ellipsoid RD is usually selected, to model all or part of the general shape of the planet.

NOTE 1	The constructed point in object-space to which RD ORIGIN_3D is bound is the origin of an embedding if, and only if, that embedding is compatible.

NOTE 2	The directed line constructed in object-space to which RD X_AXIS_3D is bound is the locus of the x-axis image under a compatible embedding, and similarly for other axis RDs.

### Binding constraint

In an ORM, the existence of a unique and compatible normal embedding depends on establishing certain geometric relationships, called binding constraints, when the RDs are bound.

A binding constraint is a relationship in object-space between two or more geometric constructions corresponding to RD bindings. Binding constraints used in this International Standard include:

- containment of a point in a curve or a surface;

- containment of a curve in a surface;

- coincidence of a line with an axis of symmetry of a surface;

- in the 3D case, the right-handedness of sets of directed lines or oriented planes; or

- distance measurement in object-space between points.

In this International Standard every abstract object-space has an associated length scale. The term “(scaled) metres” in a binding constraint shall mean “metres” in the case of physical objects and shall mean “length scaled to metres with respect to the length scale of object-space” in the case of abstract objects.

### ORM template

ORMs that have similar RDs and that have the same binding constraints are abstracted in the notion of an ORM template.

An ORM template (ORMT) is a set of RDs and binding constraints such that when each RD in the set is bound in conformance to the binding constraints, the bound set of RDs is an ORM.

A realization of an ORMT is an ORM that is specified by binding each RD in the RD set of the ORMT in conformance to its binding constraints.

An ORMT is minimal if removing any RD or binding constraint results in a set of RDs and binding constraints that is no longer an ORMT.

This International Standard specifies a set of minimal ORMTs for 2D and 3D position-space in Table 7.14. The specification fields are defined in Table 7.11. Additional ORMTs may be registered in accordance with Clause 12. Table 7.12 is a directory of ORMT specification tables.

Table 7.11 — ORMT specification fields

| Field |  | Definition |
| --- | --- | --- |
| ORMT label |  | The label (see 12.2.2). |
| ORMT code |  | The code (see 12.2.3). |
| ORMT specification | Description | A description of an ORM realization of the template. |
|  | RD set | A list of RDs in the set. |
|  | Binding constraints | Binding constraints. |
|  | Notes | Optional notes. |

Table 7.12 — ORMT specification directory

| Position-space dimension | Table number |
| --- | --- |
| 2D | Table 7.13 |
| 3D | Table 7.14 |

Table 7.13 — 2D ORMT specifications

| ORMT label | ORMT code | ORMT specification |
| --- | --- | --- |
| BI_AXIS_ORIGIN_2D | 1 | Description: x- and y-axes determined by directed perpendicular lines passing through the origin RD set: RD 1. RD ORIGIN_2D RD 2. RD X_AXIS_2D RD 3. RD Y_AXIS_2D Binding constraints: BC 1. The constructed point bound to RD 1 shall be contained in the constructed directed line bound to RD 2 and the constructed directed line bound to RD 3. BC 2. The constructed directed lines bound to RD 2 and RD 3 shall be perpendicular. Notes: - The constructed point bound to RD 1 determines the origin of the embedding. - The perpendicular directed lines passing through the origin uniquely determine the x-axis and y-axis of the embedding. |

Table 7.14 — 3D ORMT specifications

| ORMT label | ORMT code | ORMT specification |
| --- | --- | --- |
| SPHERE | 2 | Description: 3D sphere with designated directional axis and xz-plane RD set: RD 1. The sphere RD with radius r. RD 2. RD Z_AXIS_3D RD 3. RD XZ_PLANE_3D Binding constraints: BC 1. The constructed directed line bound to RD 2 shall contain the centre of the constructed sphere bound to RD 1. BC 2. The constructed plane bound to RD 3 shall contain the constructed directed line bound to RD 2. BC 3. The radius of the constructed sphere bound to RD 1 shall be r (scaled) metres. Notes: - The centre of the constructed sphere bound to RD 1 determines the origin of the embedding. - The constructed directed line bound to RD 2 passing through the origin of the embedding uniquely determines the z-axis of the embedding. - The plane through the origin of the embedding perpendicular to the z-axis of the embedding determines the xy-plane of the embedding. The constructed plane bound to RD 3 determines the xz-plane of the embedding. The intersection of the constructed xz-plane with the xy-plane is the locus of the x-axis of the embedding. The positive side of the xz-plane is designated in the binding and together with the direction of the z-axis of the embedding determines the direction of the x-axis of the embedding. - The line perpendicular to the xz-plane of the embedding through the origin of the embedding determines the locus of the y-axis of the embedding. Its direction is determined by the right-handedness of the embedding. - The distance binding constraint BC 3 is required for compatibility with a normal embedding. |
| OBLATE_ELLIPSOID | 3 | Description: 3D oblate ellipsoid with designated minor axis direction and xz-plane RD set: - The oblate ellipsoid RD with major semi-axis a and minor semi-axis b. - RD Z_AXIS_3D - RD XZ_PLANE_3D Binding constraints: - The spatial plane RD 3 shall contain the minor axis of the spatial oblate ellipsoid RD 1. - The spatial z-axis RD 2 shall coincide with the minor axis of the spatial oblate ellipsoid RD 1. - The RD 1 length of the spatial major semi-axis shall be a (scaled) metres and the length of the spatial minor semi-axis shall be b (scaled) metres. Notes: - The centre of the constructed oblate ellipsoid bound to RD 1 determines the origin of the embedding. - The constructed directed line bound to RD 2 passing through the origin (as required by BC 1) uniquely determines the z-axis of the embedding. - The z-axis of the embedding determines the xy-plane as the perpendicular plane through the origin of the embedding. BC 2 requires the bound xz-plane to contain the z-axis. The line formed by the intersection of the bound xz-plane with the xy-plane is the x-axis line. The positive side of the constructed plane bound to RD 3 determines the xz-plane and together with the direction of the z-axis determines the direction of the x-axis. - The y-axis is determined by the required right-handedness of the embedding. - The distance binding constraint BC 3 is required for compatibility with a normal linear embedding. |
| PROLATE_ELLIPSOID | 4 | Description: 3D prolate ellipsoid with designated major axis direction and xz-plane RD set: - The prolate ellipsoid RD with minor semi-axis a and major semi-axis b. - RD Z_AXIS_3D - RD XZ_PLANE_3D Binding constraints: - The spatial plane shall contain the major axis of the spatial prolate ellipsoid. - The spatial z-axis shall coincide with the major axis of the spatial prolate ellipsoid. - The length of the spatial major semi-axis shall be b (scaled) metres and the length of the spatial minor semi-axis shall be a (scaled) metres. |
| TRI_AXIAL_ELLIPSOID | 5 | Description: 3D tri-axial ellipsoid with designated z-axis direction and xz-plane RD set: - The tri-axial ellipsoid RD with x-semi-axis a, y-semi-axis b, and z-semi-axis c. - RD Z_AXIS_3D - RD XZ_PLANE_3D Binding constraints: - The spatial plane shall contain the z-axis of the spatial tri-axial ellipsoid. - The spatial z-axis shall coincide with the z-axis of the spatial tri-axial ellipsoid. - The length of the spatial x-semi-axis shall be a (scaled) metres, spatial y-semi-axis shall be b (scaled) metres, and the length of the spatial z-semi-axis shall be c (scaled) metres. |
| BI_AXIS_ORIGIN_3D | 6 | Description: x- and y-axes determined by directed perpendicular lines passing through the origin RD set: RD 1. RD ORIGIN_3D RD 2. RD Z_AXIS_3D RD 3. RD X_AXIS_3D Binding constraints: BC 1. The constructed point bound to RD 1 shall be contained in the constructed directed line bound to RD 2 and the constructed directed line bound to RD 3. BC 2. The constructed directed lines bound to RD 2 and RD 3 shall be perpendicular. Notes: - The constructed point bound to RD 1 determines the origin of the embedding. - The perpendicular directed lines passing through the origin uniquely determine the z-axis and x-axis of the embedding. - The y-axis is determined by the required right-handedness of the embedding. |
| TRI_PLANE | 7 | Description: Origin determined by the intersection of three planes RD set: - RD XZ_PLANE_3D - RD XY_PLANE_3D - RD YZ_PLANE_3D Binding constraints: - The spatial planes shall be pair-wise perpendicular. - Collective orientations shall be right-handed. Notes: - The intersection of all three planes RD 1, RD 2, and RD 3 determine the origin of the embedding. - The intersection of the xz-plane RD 1 and the xy-plane RD 2 determine the line of the x-axis. - The positive side designations of the two planes RD 1 and RD 2 together with the right-handedness requirement determines the positive x-axis direction. The directed line and the origin point determine the x-axis of the embedding. - The z- and y-axes of the embedding are similarly determined. - The collective orientation binding constraint BC 2 is required for compatibility with the right-handedness requirement. |

The specification of an ORMT does not determine an embedding. An embedding is determined when an ORMT is realized as an ORM.

EXAMPLE 1	An ORM realization of the ORMT OBLATE_ELLIPSOID (see Table 7.14) is constructed from an oblate ellipsoid RD (with specified parameter values a and b), an RD Z_AXIS_3D and an RD XZ_PLANE_3D. Each RD identifies a position-space construct with a corresponding geometric construct realized in the object-space of a specific spatial object. One method for the realization of these geometric constructs in compliance with the binding constraints of this ORMT is to specify by direct or indirect measurement in object-space the following:

- a point on the spatial oblate ellipsoid surface,

- the direction of the oblate ellipsoid outward surface normal at the point,

- the positive direction of the z-axis (that coincides with the oblate ellipsoid minor axis as a binding constraint), and

- the positive normal direction of the xz-plane (that is perpendicular to the z-axis direction since the z-axis line is contained in the plane as a binding constraint).

Specifications (a) – (c) determine a unique oblate ellipsoid surface in the object-space. The equatorial plane of the oblate ellipsoid is (by compatibility with the oblate ellipsoid surface generating function) the xy-plane, and its intersection with the oblate ellipsoid axis of rotation determines the origin point and the z-axis. Specification (d) together with the origin and the xy-plane determine the x-axis. Since there is one, and only one, oriented yz-plane that is both perpendicular to the xz-plane and right-handed compatible, the right-handed orthonormal embedding is uniquely determined (see Figure 7.6).

specified surface point

z

-

axis

x

-

axis

y

-

axis

specified xz-plane

specified surface normal

specified z-axis direction

Figure 7.6 — Oblate ellipsoid ORMT binding

EXAMPLE 2	The North American Datum 1927 may be specified as an oblate ellipsoid ORM with the Clarke 1866 ellipsoid ( RD CLARKE_1866 in Table D.2) oblate ellipsoid RD and bound to the Earth object-space as follows:

- the direction of the z-axis is specified as the north direction of the Earth’s rotational axis,

- a position (latitude 39° 13’ 26,686” N) on the ellipsoid is identified to a position in the Earth object-space (Meades Ranch, Kansas, US),

- the direction of the surface normal to the ellipsoid is specified ( = -1,32  = 1,93), and

- the direction of the positive xz-plane normal is indirectly determined by specifying the longitude of Meades Ranch (98° 32’ 30,506” W).

The preceding examples illustrate the concept that RDs can be bound to an appropriate spatial object in accordance to an ORMT and thereby effect an ORM realization that determines a normal embedding of position-space.

NOTE 1	7.4.4 Example 2 is based on a published specification of the North American Datum 1927, however the actual binding methodology used was a complex process involving a mathematical best fit of the Clarke 1866 ellipsoid to a network of geodetic survey control points spanning the continental United States.

NOTE 2	Realization and binding techniques draw from disciplines ranging from astronomy, surveying, and geophysics to satellite geodesy. In general, there are many ways to realize an ORMT for a spatial object. Binding techniques and methodologies are outside of the scope of this International Standard.

EXAMPLE 3	In Figure 7.6, if the point used to bind an ORMT OBLATE_ELLIPSOID realization is not a polar point and if the xy-plane is bound to the (unique) plane that contains that point (in addition to containing the oblate ellipsoid axis), the point and directions will be fixed with respect to the 3D embedding, and the ORM is object-fixed.

EXAMPLE 4	In contrast, if the celestial object in the example is the Earth, and instead of binding the xy-plane to contain the point used to bind the oblate ellipsoid RD, it is bound as the (unique) plane that contains the centre of the Sun (in addition to containing the oblate ellipsoid axis). In that binding, the point and its oblate ellipsoid surface move in a manner that depends on both the Earth’s rotation and its orbital position and therefore the ORM is object-dynamic.

### Standardized ORMs

The ORMs specified in this International Standard, are ORMT realizations. Standardized ORMs shall include:

a.	a specification of the spatial object and optionally a region in object-space,

- a specification of the ORM template,

- a specification of the ellipsoid RD (if any), and

- the binding year if the spatial object is a physical object.

A standardized ORM does not include a specification of the binding of its RD components. The binding specification is not directly needed for the scope of this International Standard. An ORM indirectly designates a unique normal embedding. A specification of an ORM normal embedding is important when two or more ORMs have been specified for the same spatial object. To inter-convert between spatial relationships with respect to two different normal embeddings, one embedding shall be expressed in terms of the other embedding by means of an embedding transformation, or both shall be expressed in terms of a third (reference) embedding by means of an embedding transformation for each ORM with respect to a third (reference) ORM.

If two or more object-fixed ORMs for the same object are specified (or registered), one of the ORMs shall be designated as the reference ORM for that object.

A standardized object-fixed ORM shall either be the reference ORM for the object or its specification shall additionally include the seven parameter specification of an embedding transformation of the ORM with respect to the normal embedding of the reference ORM for the object. The embedding transformation shall be called a reference transformation (RT) for the ORM. The reference transformation from the normal embedding of a source ORM to the reference ORM shall be denoted by HSR (see Table 10.1).

Some ERMs that are specified in Table E.4 are based on local geodetic datums. Historic local geodetic datums may exhibit distortions with respect to the reference ERM. In those cases, a reference embedding transformation specified by the seven transformation parameters of the ERM is an approximation based on empirical measurements. In some of these cases, embedding transformation parameters for approximations based on different sets of measurements and/or sub-regions appear as multiple reference transformation entries in Table E.4. In those cases, the reference transformation labels shall share a common prefix derived from the name of the related local geodetic datum.

NOTE 1	The scale adjustment component of the seven parameter specification is needed to account for differing length scales in abstract object-space. Theoretically,  for two normal embeddings of a physical object-space. In practice, when the embeddings are indirectly determined by the RD bindings of an ORM, the determination of the parameters is an approximation that is the result of a mathematical best fit and the seventh parameter () is used to provide an additional degree of freedom to achieve the desired accuracy in the region of interest. In the case of the object Earth, published values for  are typically one part per million in magnitude (10-6) or smaller.

A directory of reference ORMs is provided in Table E.1. The reference ORM for Earth is ORM WGS_1984. This ORM is an Earth-fixed global model. The actual binding definition of ORM WGS_1984 (see [83502] ) is a realization of ORMT BI_AXIS_ORIGIN_3D, however, the RD WGS_1984 is associated with this ORM so that its ORM specification for purposes of this International Standard is an ORMT OBLATE_ELLIPSOID realization.

The fields of an ORM specification are defined in Table 7.15. Standardized ORMs are specified in Annex E and Annex J. Additional ORMs may be registered in accordance with Clause 12.

Table 7.15 — ORM specification fields

| Field | Definition |
| --- | --- |
| ORM Label | The label (see 12.2.2). |
| ORM Code | The code (see 12.2.3). |
| Published name | The name(s) given to the concept embodied in this ORM in the reference(s). |
| Reference ORM | The label of the reference ORM for this object. If this ORM is the reference ORM for this object, then this field shall contain the object name preceded by the phrase “This is the reference ORM for “. |
| Binding information | If the ORM is dynamic, or a time fixed (object fixed) instance of a dynamic ORM, the label of the dynamic binding category (see 7.5). If the ORM is object fixed and the spatial object is a physical object, the date that the ORM component RDs were bound in object-space. This case includes time fixed instances of dynamic ORMs for a physical object. If the ORM is object fixed and the spatial object is Earth, if Greenwich, UK is not contained in the x-positive xz-half-plane of the embedding, then the significant location contained in the x-positive xz-half-plane of the embedding shall be specified. Specification of any required binding constraints on the component RDs. Optional binding notes. In the case of abstract ORMs, the string “none”. |
| Region | The approximate subset of object-space to which the model applies expressed as either a spatial extent or the description as specified in the reference. |
| ORMT | The label of the ORM template for this ORM. |
| RD parameterization | The label of the ellipsoidal RD, if any; otherwise “N/A”. |
| Reference transformation | If the ORM is object fixed, or a time fixed (object fixed) instance of a dynamic ORM, one or more reference transformation specifications (see Table 7.16). |
| References | The references (see 12.2.5), or “none” if defined in this International Standard. |

Table 7.16 — Reference transformation specification fields

| Field | Definition |
| --- | --- |
| RT Label | The label (see 12.2.2). |
| RT Code | The code (see 12.2.3). |
| RT Region | A description of the extent and/or the spatial bounds of the region for which this reference transformation is applicable. |
| RT Parameters | The values of the seven parameters shall be specified by value or by reference (see 12.2.5). If by value, the values of the seven parameters specifying the reference transformation HSR (see Table 10.1) shall be specified. These values shall be followed by a error estimate expressed in one of the following forms: - error estimate: unknown - error estimate: assumed precise - error estimate (1): <parameter name>:<error value> EXAMPLE error estimate (1): . If by reference, this field shall contain a citation(s) for the values of the seven parameters and error estimates. Terms appearing in the references that are cited for a value shall be enclosed in brackets ( { } ). Any parameter value that is not specified in the citation(s) shall be specified as in the “by value” case. To avoid loss of precision, are expressed in arc seconds(“) and, in cases of a large rotation, in arc degrees (°). NOTE The rotations must be converted to radians when used in Clause 10 and Clause 11 formulations and in the Bursa-Wolf equation (see Annex B). |
| Date published | The date that the RT was published. |
| References | The references (see 12.2.5), or “none” if defined in this International Standard. |

If an ORM is the reference ORM of a spatial object, then it shall have a reference transformation with the RT label “IDENTITY” and the RT parameters of that RT shall be  and there shall be no error estimate.

If an ORM is not the reference ORM of a spatial object and the RT parameters for it are   within the limits of precision in measurement and associated errors, then the RT label shall be “IDENTITY_BY_MEASUREMENT”.

If an ORM is not the reference ORM of a spatial object and the RT parameters for it are   by intent, then the RT label shall be “IDENTITY_BY_DEFAULT”.

NOTE 2	 In the case of Earth-fixed ERM bindings axis rotations are either zero, or are very small with the following exceptions:

- Cases for which the xz-plane does not contain Greenwich, UK have relatively large  rotations.

- The Earth-fixed approximations of ERMs in the celestiomagnetic dynamic binding category have large  rotations (see “GEOMAGNETIC” entries in Table 7.32).

NOTE 3 	A geodetic datum that is global or geocentric is used as a basis for establishing a geocentric SRF (see 8.5.2). If geocentric coordinates are associated to a 3D linear embedding, then such a datum is conceptually equivalent to an ERM. The WGS 84 global datum (see [83502T]) is conceptually equivalent to the ORM WGS_1984 in Table E.4.

## Celestial body dynamic binding categories

### Binding categories

A binding restriction for an ORMT is an object specific restriction for the binding of a single RD in the RD set of the ORMT.

A binding category is an ORMT with:

- a binding category name,

- a label and code

- a set of binding restrictions for the ORMT, and

- object restrictions that delineate the objects for which the binding restrictions apply.

The specification fields for a binding category are defined in Table 7.17.

Table 7.17 — Binding category specification fields

| Field | Definition |
| --- | --- |
| Label | The label (see 12.2.2). |
| Code | The code (see 12.2.3). |
| Name | A descriptive name. |
| Object restrictions | A specification of a set of objects for which the binding restrictions apply. |
| ORMT | The label of the ORMT. |
| Binding restrictions | A specification of the binding restrictions. |
| References | Optional references (see 12.2.5). |

A binding category is a dynamic binding category (DBC) if each realization of the binding category ORMT compliant with the object and binding restrictions is object-dynamic. This International Standard specifies a collection of dynamic binding categories as identified in Table 7.18. Additional binding categories may be registered in accordance with Clause 12.

Table 7.18 — Dynamic binding category specification directory

| Dynamic binding category | Table number |
| --- | --- |
| equatorial inertial | Table 7.19 |
| solar ecliptic | Table 7.21 |
| solar equatorial | Table 7.23 |
| heliocentric Aries ecliptic | Table 7.25 |
| heliocentric planet ecliptic | Table 7.27 |
| heliocentric planet equatorial | Table 7.29 |
| celestiomagnetic | Table 7.31 |
| solar magnetic ecliptic | Table 7.33 |
| solar magnetic dipole | Table 7.35 |

A binding category name may be used to describe an ORM that realizes the binding category.

### Terminology

Several DBCs specified below set the x-axis direction in an ORMT BI_AXIS_ORIGIN_3D realization for a planet (including Earth) to the direction of the Sun when the planet is at its vernal equinox, as a binding restriction. The ecliptic plane and equatorial plane of a planet intersect in a line. An equinox is one of the two points of intersection between this line and the orbit of the planet. The vernal equinox is the equinox at which the direction from the planet to the Sun begins to ascend to the northern side of the equatorial plane.

The term northwards as used in several DBC specifications below shall mean in a direction making an acute angle with respect to the direction of the rotational north pole of the object from its rotational south pole.

An inertial direction is a direction with respect to the universe that is time invariant. A quasi-inertial direction is a direction that changes relatively slowly with respect to the universe.

At any given epoch as defined in 6.2.1, the direction of the Sun at the vernal equinox is fixed with respect to the Milky Way and other (detectible) galaxies. In the case of Earth, the first point of Aries is the direction from the vernal equinox to the Sun.

NOTE	The effects of precession and nutation on the spin axis of a planet cause this direction to change over time. The change in direction of the first point of Aries is very slow (approximately one full rotation in 26 000 years).

The first point of Aries at a given epoch is an inertial direction, and the current first point of Aries (which is called Aries true of date) is a quasi-inertial direction.

### Equatorial inertial

The equatorial inertial dynamic binding category is specified in Table 7.19. See also Figure 7.7.

Table 7.19 — Equatorial inertial dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | equatorial_inertial |
| Code | 1 |
| Name | equatorial inertial |
| Object restrictions | A planet in the solar system for which the ecliptic plane is distinct from the equatorial plane. |
| ORMT | BI_AXIS_ORIGIN_3D. |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD X_AXIS_3D points in the direction to the Sun when the planet is at its vernal equinox. - The RD Z_AXIS_3D is parallel to the rotational axis and points northwards. |
| References | [SEID] |

The axis directions are quasi-inertial, but vary with respect to any object-fixed ORM for the planet.

In the case of ORM EARTH_INERTIAL_J2000r0, the International Earth Rotation Service (IERS) specifies a very precise transformation to the ORM WGS_1984 reference embedding with a matrix whose coefficients represent the effects of polar motion, Earth rotation, nutation and precession.

Under the assumption that the ORM z-axis and ORM WGS_1984 z-axis are parallel, the transformation simplifies to a single rotation matrix. This simplified form is described in Annex B.

E

q

u

a

t

o

r

z-axis

First point

of Aries

North rotational pole

y-axis

x-axis

Figure 7.7 — Equatorial inertial dynamic binding category

Three equatorial inertial ERMs labelled ORM EARTH_INERTIAL_ARIES_1950, ORM EARTH_INERTIAL_J2000r0 and ORM EARTH_INERTIAL_ARIES_TRUE_OF_DATE, are specified in Table E.5. They are based on three determinations of the first point of Aries and the rotational axis. The first two have epoch fixed inertial directions. ORM EARTH_INERTIAL_ARIES_TRUE_OF_DATE has a quasi-inertial x-axis direction that varies as a function of time.

Table 7.20 is a directory of these Earth and other celestial object equatorial inertial ORMs.

Table 7.20 — Equatorial inertial ORM directory

| ORM label | Published name |
| --- | --- |
| EARTH_INERTIAL_ARIES_1950 | Earth equatorial inertial, Aries mean of 1950 |
| EARTH_INERTIAL_ARIES_TRUE_OF_DATE | Earth equatorial inertial, Aries true of date |
| EARTH_INERTIAL_J2000r0 | Earth equatorial inertial, J2000.0 |
| JUPITER_INERTIAL | Jupiter equatorial inertial |
| MARS_INERTIAL | Mars equatorial inertial |
| MERCURY_INERTIAL | Mercury equatorial inertial |
| NEPTUNE_INERTIAL | Neptune equatorial inertial |
| PLUTO_INERTIAL | Pluto equatorial inertial |
| SATURN_INERTIAL | Saturn equatorial inertial |
| URANUS_INERTIAL | Uranus equatorial inertial |
| VENUS_INERTIAL | Venus equatorial inertial |

### Solar ecliptic

The solar ecliptic dynamic binding category is specified in Table 7.21. See [BHAV, 3.2.5] and Figure 7.8.

Table 7.21 — Solar ecliptic dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | solar_ecliptic |
| Code | 2 |
| Name | solar ecliptic |
| Object restrictions | A planet in the solar system. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD X_AXIS_3D is in the ecliptic plane and points in the direction of the Sun. - The RD Z_AXIS_3D is perpendicular to the ecliptic plane and points northward. |
| References | [HAPG] |

z-axis

ecliptic

plane

direction of the Sun

North rotational pole

y-axis

x-axis

Figure 7.8 — Solar ecliptic ORM binding

NOTE	The x-axis slowly rotates once per orbit of the planet around the Sun. The y-axis also lies in the ecliptic plane.

In the case of Earth, the transformation from a solar ecliptic ORM embedding to the ORM WGS_1984 reference embedding is described in Annex B.

Table 7.22 is a directory of these Earth and other planet object solar ecliptic ORMs.

Table 7.22 — Solar ecliptic ORM directory

| ORM label | Published name |
| --- | --- |
| EARTH_SOLAR_ECLIPTIC | Solar ecliptic |
| JUPITER_SOLAR_ECLIPTIC | Jupiter solar ecliptic |

### Solar equatorial

A solar equatorial dynamic binding category is specified in Table 7.23. See also Figure 7.9.

Table 7.23 — Solar equatorial dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | solar_eQUATORIAL |
| Code | 3 |
| Name | solar equatorial |
| Object restrictions | A planet in the solar system for which the ecliptic plane is distinct from the equatorial plane. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD X_AXIS_3D is in the ecliptic plane and points in the direction of the Sun. - The RD Z_AXIS_3D is perpendicular to the RD X_AXIS_3D in the plane determined by the RD X_AXIS_3D and the rotational axis of the Sun and points northward. |
| References | [CRUS] |

planet

Sun

rotational axis

of the Sun

x-axis

y-axis

z-axis

plane of

x-axis and Sun

Figure 7.9 — Solar equatorial ORM binding

NOTE	The xz-plane of a solar equatorial ORM contains the rotational axis of the Sun. The x-axis slowly rotates once per orbit of the object around the Sun. The y-axis is parallel to the solar equatorial plane and points towards dusk [BHAV, 3.2.6].

Table 7.24 is a directory of these Earth and other planet solar equatorial ORMs.

Table 7.24 — Solar equatorial ORM directory

| ORM label | Published name |
| --- | --- |
| EARTH_SOLAR_EQUATORIAL | Solar equatorial |
| JUPITER_SOLAR_EQUATORIAL | Jupiter solar equatorial |

### Heliocentric Aries ecliptic

The heliocentric Aries ecliptic dynamic binding category is specified for a planet in Table 7.25. See [HAPG] and Figure 7.10.

Table 7.25 — Heliocentric Aries ecliptic dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | heliocentric_ARIES_ecliptic |
| Code | 4 |
| Name | heliocentric Aries ecliptic |
| Object restrictions | The Sun. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the Sun. - The RD Z_AXIS_3D is perpendicular to the ecliptic plane of the Earth and points northwards. - The RD X_AXIS_3D is in the ecliptic plane of the Earth and points towards the first point of Aries. |
| References | [HAPG] |

z-axis

ecliptic

plane

of Earth

First point

of Aries

North rotational pole

y-axis

x-axis

Sun

Figure 7.10 — Heliocentric Aries ecliptic ORM binding

See Figure 7.10. Table 7.26 is a directory of these Earth and other planet solar equatorial ORMs. The heliocentric Aries ecliptic axis directions are inertial for ORM HELIOCENTRIC_ARIES_ECLIPTIC_J2000r0 and quasi-inertial ORM HELIOCENTRIC_ARIES_ECLIPTIC_TRUE_OF_DATE.

Table 7.26 — Heliocentric Aries ecliptic ORM directory

| ORM label | Published name |
| --- | --- |
| HELIOCENTRIC_ARIES_ECLIPTIC_J2000r0 | Heliocentric Aries ecliptic, J2000.0 |
| HELIOCENTRIC_ARIES_ECLIPTIC_TRUE_OF_DATE | Heliocentric Aries ecliptic, true of date |

### Heliocentric planet ecliptic

The heliocentric planet ecliptic dynamic binding category is specified for a planet in Table 7.27. See [HAPG] and Figure 7.11.

Table 7.27 — Heliocentric planet ecliptic dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | heliocentric_planet_ecliptic |
| Code | 5 |
| Name | heliocentric planet ecliptic |
| Object restrictions | The Sun. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the Sun. - The RD Z_AXIS_3D is perpendicular to the ecliptic plane for a specified planet and points northwards. - The RD X_AXIS_3D is in the ecliptic plane of the specified planet and points towards the planet. |
| References | [HAPG] |

planet

Sun

x-axis

y-axis

z-axis

Ecliptic plane

Figure 7.11 — Heliocentric planet ecliptic dynamic binding category

Table 7.28 is a directory of these Earth and other planet solar equatorial ORMs.

Table 7.28 — Heliocentric planet ecliptic ORM directory

| ORM label | Published name |
| --- | --- |
| HELIOCENTRIC_EARTH_ECLIPTIC | Heliocentric Earth ecliptic |

### Heliocentric planet equatorial

The heliocentric planet equatorial dynamic binding category is specified for a planet in Table 7.29. See also Figure 7.12.

Table 7.29 — Heliocentric planet equatorial dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | heliocentric_planet_equatorial |
| Code | 6 |
| Name | heliocentric planet equatorial |
| Object restrictions | The Sun. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the Sun. - The RD Z_AXIS_3D is perpendicular to the solar equatorial plane of the specified planet and points northwards. - The RD X_AXIS_3D is perpendicular to the RD Z_AXIS_3D in the plane determined by the RD Z_AXIS_3D and the planet centre and points towards the specified planet. |
| References | [HAPG] |

planet

rotational axis

of the Sun

Sun

z-axis

y-axis

x-axis

plane of

z-axis and

planet centre

Figure 7.12 — Heliocentric planet equatorial dynamic binding category

Table 7.30 is a directory of these Earth and other planet solar equatorial ORMs.

Table 7.30 — Heliocentric planet equatorial ORM directory

| ORM label | Published name |
| --- | --- |
| HELIOCENTRIC_EARTH_EQUATORIAL | Heliocentric Earth equatorial |

### Celestiomagnetic

The celestiomagnetic dynamic binding category is specified in Table 7.31. See [BHAV, 3.3.1] and Figure 7.13.

Table 7.31 — Celestiomagnetic dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | celestiomagnetic |
| Code | 7 |
| Name | celestiomagnetic |
| Object restrictions | A planet or rotating satellite in a solar system with a magnetic dipole axis distinct from its rotational axis. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD Z_AXIS_3D is parallel to the magnetic dipole axis and points towards magnetic north. - The RD X_AXIS_3D is contained in the plane through the origin that is parallel to the dipole and rotational axes, perpendicular to the RD Z_AXIS_3D and pointing away from the dipole northward direction. |
| References | [CRUS] |

NOTE	The rotational south pole is contained in the x-positive xz-half-plane unless the planet has retrograde rotation. This binding is not applicable to Saturn whose magnetic and rotational poles are not distinguished.

magnetic

dipole

axis

North rotational pole

z-axis

y-axis

x-axis

Figure 7.13 — Celestiomagnetic ORM binding

In the case of Earth, this dynamic ERM is approximated as an Earth-fixed ERM for a five year epoch. The other celestial objects that have observed magnetic dipoles have object-fixed ORM approximations for the corresponding dynamic ORMs.

Table 7.32 is a directory of these Earth and other object-fixed celestiomagnetic ORM approximations.

Table 7.32 — Celestiomagnetic ORM directory

| ORM label | Published name |
| --- | --- |
| GANYMEDE_MAGNETIC | Ganymede magnetic |
| GEOMAGNETIC_1945 | Geomagnetic 1945 |
| GEOMAGNETIC_1950 | Geomagnetic 1950 |
| GEOMAGNETIC_1955 | Geomagnetic 1955 |
| GEOMAGNETIC_1960 | Geomagnetic 1960 |
| GEOMAGNETIC_1965 | Geomagnetic 1965 |
| GEOMAGNETIC_1970 | Geomagnetic 1970 |
| GEOMAGNETIC_1975 | Geomagnetic 1975 |
| GEOMAGNETIC_1980 | Geomagnetic 1980 |
| GEOMAGNETIC_1985 | Geomagnetic 1985 |
| GEOMAGNETIC_1990 | Geomagnetic 1990 |
| GEOMAGNETIC_1995 | Geomagnetic 1995 |
| GEOMAGNETIC_2000 | Geomagnetic 2000 |
| JUPITER_MAGNETIC | Jupiter magnetic |
| NEPTUNE_MAGNETIC | Neptune magnetic |
| SATURN_MAGNETIC | Saturn magnetic |
| URANUS_MAGNETIC | Uranus magnetic |

### Solar magnetic ecliptic

The solar magnetic ecliptic dynamic binding category is specified in Table 7.33. See [BHAV, 3.3.4] and Figure 7.14.

Table 7.33 — Solar magnetic ecliptic dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | solar_magnetic_ecliptic |
| Code | 8 |
| Name | solar magnetic ecliptic |
| Object restrictions | A planet in the solar system with a magnetic dipole. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD X_AXIS_3D is in the ecliptic plane of the planet pointing in the direction of the Sun. - The RD Z_AXIS_3D is perpendicular to the RD X_AXIS_3D and points northwards in the plane determined by the x-axis and the planetary magnetic dipole axis. |
| References | [CRUS] |

x-axis

y-axis

direction

to the Sun

magnetic dipole axis

direction

z-axis

plane of dipole

and x-axis

Figure 7.14 — Solar magnetic ecliptic ORM binding

Table 7.34 is a directory of these Earth and other planet solar magnetic ecliptic ORMs.

Table 7.34 — Solar magnetic ecliptic ORM directory

| ORM label | Published name |
| --- | --- |
| EARTH_SOLAR_MAGNETOSPHERIC | Solar magnetospheric |
| JUPITER_SOLAR_MAG_ECLIPTIC | Jupiter solar magnetic ecliptic |

### Solar magnetic dipole

The solar magnetic dipole dynamic binding category is specified in Table 7.35. See [BHAV, 3.3.5] and Figure 7.15.

Table 7.35 — Solar magnetic dipole dynamic binding category

| Specification element | Value |
| --- | --- |
| Label | solar_magnetic_dipole |
| Code | 9 |
| Name | solar magnetic dipole |
| Object restrictions | A planet in the solar system with a magnetic dipole. |
| ORMT | BI_AXIS_ORIGIN_3D |
| Binding restrictions | - The RD ORIGIN_3D is the mass-centre of the planet. - The RD Z_AXIS_3D is parallel to the planetary magnetic dipole axis and points towards magnetic north. - The RD X_AXIS_3D is perpendicular to the RD Z_AXIS_3D and pointing towards the Sun in the plane determined by the Sun and the RD Z_AXIS_3D. |
| References | [CRUS], [BHAV] |

y-axis

x-axis

z-axis

plane of z-axis

and Sun center

magnetic

dipole

Figure 7.15 — Solar magnetic dipole ORM binding

Table 7.36 is a directory of these Earth and other celestial object solar magnetic dipole ORMs.

Table 7.36 — Solar magnetic dipole ORM directory

| ORM label | Published name |
| --- | --- |
| EARTH_SOLAR_MAG_DIPOLE | Solar magnetic dipole |
| JUPITER_SOLAR_MAG_DIPOLE | Jupiter solar magnetic dipole |

EDITORS NOTE: This is a temporary table of index entries that is used to create the master index. It will eventually be removed:

binding category	121

binding constraint	110

binding restriction	121

bound RD	106

celestiomagnetic dynamic binding category	129

compatible normal embedding	110

coordinate frame rotation	109

distance preserving embedding	107

dynamic binding category	121

Earth reference model	110

embedding transformation 2D case	109

embedding transformation 3D case	107

equatorial inertial dynamic binding category	122

equinox	122

first point of Aries	122

four parameter transformation	109

global model	110

heliocentric Aries ecliptic dynamic binding category	126

heliocentric planet ecliptic dynamic binding category	127

heliocentric planet equatorial dynamic binding category	128

inertial direction	122

inverse flattening	104

left-handed embedding	107

local model	110

minimal ORMT	111

normal embedding	107

northwards	122

object reference model	110

object-dynamic embedding	110

object-dynamic ORM	110

object-fixed embedding	110

object-fixed ORM	110

oblate ellipsoid RD	103

origin of the embedding	107

ORM template	111

ORMT realization	111

position vector rotation	109

prolate ellipsoid RD	104

quasi-inertial direction	122

reference datum	101

reference ORM	118

reference transformation	118

right-handed embedding	107

seven parameter embedding specification	109

seven parameter transformation	108

solar ecliptic dynamic binding category	124

solar equatorial dynamic binding category	125

solar magnetic dipole dynamic binding category	132

solar magnetic ecliptic dynamic binding category	131

sphere RD	104

vernal equinox	122

x-axis embedding unit point	107

y-axis embedding unit point	107

z-axis embedding unit point	107

In general usage spheres are a limiting case of oblate, prolate, and tri-axial ellipsoids. To remove ambiguity, in this International Standard spheres are a special case of oblate ellipsoids only.

The y-axis and z-axis are in the plane of the presentation, and the x-axis is directed generally towards the observer.

For near-real-time orbit determination applications, Earth orientation parameters (polar motion and Earth rotation variations) that are needed to build the matrix must be predicted values. Because the driving forces that influence polar motion and Earth rotation variations are difficult to characterize, these Earth orientation predictions are performed weekly [83502T].