SEDRIS LO

Comments on EDCS WD5

# General

The following comments apply to the entire document:

| SEDRIS_G001 | ##### Clauses 4 & 6 |
| --- | --- |
|  | Units should not be assigned to those attributes which are of Value Type INTEGER. All existing INTEGER attributes have EQL = PURE_NUMBER. This EQ has two members: PERCENT, and UNITLESS. In all cases, PERCENT is an inappropriate unit, even in those cases which use _DENSITY in the EAL label. Therefore, these EA seem to be intrinsically unitless and should not be assigned Eqs |
|  | Clause 4 should state that Units are used only with EAs of value type REAL. Cases of INTEGER type should be corrected to blank/gray in the EQL column. Affected entries in clause 6 are cites below as technical comments. |

Response:	Accepted.

SEDRIS_G002

The following section should be added to clause 4 between section 4.4 and 4.5 (meta-data action item):

<INSERT>

4.[?4bis?] Numeric attribute data characteristic values and meta-data

This International standard defines a dictionary (EM) of terms used to quantify or otherwise describe information about a collection, set, or other application specific data structure of numeric values of a given attribute of EDCS Value Type REAL, INTEGER, COUNT,  or INDEX.  This additional information falls into two categories: characteristic values, and meta-data.

Characteristic values (also known as sentinel values) are widely used in application data sets to flag special meaning to a given attribute value that overrides or otherwise supplants the numerical meaning as defined in the EA entry definition.  Such “flag” values are usually selected by the data provider to be outside of the range of “normal” values for the attribute data set.  Table 4.?.1 defines characteristic values.

To extend the sentinel mechanism to attributes of value type ENUMERANT or LOGICAL, it is necessary to add out-of-range values that can be assigned special meanings.  These special values are termed pre-specified EDCS attribute enumerants (see 4.5.2).  The UNBOUNDED_UPPER and UNBOUNDED_LOWER are not meaningful for an EA of value type ENUMERANT or LOGICAL and are consequently not pre-specified.

Numeric meta data specifies numeric information about the ensemble of data belonging to a specific collection, set, or other application specific data structure of numeric values of a given numeric attribute.  Table 4.?.2 define numeric meta-data values.

Table 4.?.1 EM dictionary of numeric attribute characteristic values

| EM Label | Definition | EM Code |
| --- | --- | --- |
| NOT_APPLICABLE | There is no value that is applicable for the given EA in the context of the given data set. | 1 |
| MISSING | The function cannot determine the value for the given EA in the context of the given data set. | 2 |
| VALUE_WITHHELD | The function can determine the value for the given EA in the context of the given data set, but due to policy considerations does not return it. | 3 |
| MULTIPLE | There is more than one value that is applicable for the given EA in the context of the given data set. | 4 |
| UNDESIGNATED | The function cannot return a value for the given EA in the context of the given data set, for some reason other than it is NOT_APPLICABLE, MISSING, VALUE_WITHHELD or MULTIPLE. | 5 |
| reserved for future standardization | reserved for future standardization | 6 |
| reserved for future standardization | reserved for future standardization | 7 |
| reserved for future standardization | reserved for future standardization | 8 |
| reserved for future standardization | reserved for future standardization | 9 |
| reserved for future standardization | reserved for future standardization | 10 |
| UNBOUNDED_UPPER | The function returns “plus infinity” for the value for the given EA in the context of the given data set. | 11 |
| UNBOUNDED_LOWER | The function returns “minus infinity” for the value for the given EA in the context of the given data set. | 12 |

Table 4.?.1 EM dictionary of numeric attribute meta-data

| EM Label | Definition | EM Code |
| --- | --- | --- |
| CONSTANT_VALUE | All values of data for the given EA in the context of the given data set, are equal to this value. | 13 |
| MAXIMUM_VALUE | All values of data for the given EA in the context of the given data set, must be less than or equal to this value, to be considered allowed or legal. | 14 |
| MINIMUM_VALUE | All values of data for the given EA in the context of the given data set, must be greater than or equal to this value, to be considered allowed or legal. | 15 |
| UPPER_BOUND | All values of data for the given EA in the context of the given data set instance, are observed to be less than or equal to this value. | 16 |
| LOWER_BOUND | All values of data for the given EA in the context of the given data set instance, are observed to be greater than or equal to this value. | 17 |
| TOLERANCE | All values of data for the given EA in the context of the given data set instance, are rounded to this value. | 18 |
| SIGNIFICANT_DIGITS | All values of data for the given EA in the context of the given data set instance, are reported to these many significant digits. | 19 |
| MEASUREMENT_ERROR | All values of data for the given EA in the context of the given data set instance, were measured or estimated with a standard error given by this value. | 20 |

</INSERT>

Add to the end of the first paragraph of 4.5.2 Pre-specified EDCS Attribute Enumerants:

These labels, definitions, and codes correspond to the first ten characteristic values of table 4.?.1 EM dictionary of numeric attribute characteristic values.  These ten enumerants serve a parallel purpose to characteristic values as “out of range values” flagging special a meaning over the normal meaning for the EA.

Add EM to the list of dictionaries.

Response:	After much discussion, the proposal is recognized as tackling a thorny problem that requires additional team-analysis, starting from the position that the “meatdata solution” should consistently apply to ALL attribute value types (including ENUMERATED, STRING, and KEY). AI 08-11 was assigned to Carson/Puk/Hembree to study this problem further.

SEDRIS_G003

The following section should be added to clause 4 (action item 06-05):

(Note: Code assignments (EVC and ETC) for table 4.2 are proposed technical comment to clause 4)

<INSERT>

4.u.v Conventions for attribute type and value specifications

Some of the concepts used in this clause may be used together to form various levels of  attribute specification.  The highest level specifies a pattern for values of a specific attribute.  The lowest level specifies a specific attribute value (an instance of an attribute).  The least constrained pattern specification is referred to as an attribute type specification.  An attribute type specification with additional constraints is referred to as a constrained attribute type specification .  The lowest level specification is referred to as an attribute value specification.

The components required for these specifications vary between attributes of REAL data value types and attributes of one of the other data value types.  This section will cite the use of codes, however corresponding labels may be substituted as logically equivalent for purposes of this concept.

4.u.v.1 Attribute value specification

An attribute value is specified by:

- an attribute code (EAC)

- an EDCS storage type  (ETC) (see Table 4.2)

- an attribute data value expressed in conformance with the EDCS storage type

- and, in the case of REAL attributes, a unit code (EUC) and a scale factor code (ESC).

4.u.v.2 Attribute type specification

An attribute type specification consists of:

- an attribute code EAC

- an assignment of an attribute data value type code (EVC) from Table 4.2.

- if the REAL data value type is assigned, a Unit Equivalence Code (EQC) is required.

- if the ENUMERATED data value type is assigned, an EA specific enumeration must be specified.

Attributes in this international standard use an attribute type specification in the definition of each EAC.

4.u.v.3 Constrained attribute type specification

A constrained attribute type specification is an attribute type specification with one or more additional constraints.  Constraints may apply to value ranges, storage types, and in the case of REAL types, units and scale factors.  Constrained attribute type specifications may be used in the specification of a Profile (see 4.13.10).

4.u.v.3.1 Value range constraints

Value range constraints depend on the attribute value type.

For an ENUMERATED value type, a range constraint consists of the specification of a subset of the enumerants of the specific attribute enumeration.

For LOGICAL and NULL value types, a range constraint may specify the exclusion of one or more pre-specified enumerant values (see 4.5.2).

For STRING and KEY value types, a range constraint may consist of a specification of a list of allowed strings or string patterns.

For INDEX, COUNT, and INTEGER value types, a range constraint consists of range bounds, or a fixed value list.

For REAL value types, a range constraint may consist of range bounds and/or a value tolerance specification.

4.u.v.3.2 Storage type constraints

ENUMERATED, LOGICAL, and NULL value types cannot be storage type constrained.

For STRING and KEY value types, a storage type constraint may consist of a string length limit.

For the INDEX value type, a storage type constraint consists of specifying either SHORT_INDEX or LONG_INDEX.

For the COUNT, and INTEGER value type, a storage type constraint consists of specifying either SHORT_COUNT or LONG_COUNT.

For the INTEGER value type, a storage type constraint consists of specifying either SHORT_INTEGER or LONG_INTEGER.

For the REAL value type, a storage type constraint consists of specifying either FIXED_POINT, DOUBLE_FIXED_POINT, FLOATING_POINT, or DOUBLE_FLOATING_POINT.

4.u.v.3.3 Unit and scale constraints

Unit and scale constraints only apply to REAL value types.  A unit constraint is the specification of unit code (EUC) for a unit with membership in the unit equivalence class of the attribute.  A scale constraint is the specification of a specific scale code (ESC).  A scale constraint is useful is conjunction with a storage type constraint of type FIXED_POINT, or DOUBLE_FIXED_POINT.

</INSERT>

Response:	Accepted in principle; the editors may revise for clarity (e.g., potentially remove material on storage types; place attribute type/value material in clause 4; add single statement to conformance addressing restrictions-on-ranges).

| SEDRIS_G004 | Where | Tables 6.32 to 6.37 |
| --- | --- | --- |
|  | Problem | Several enumerations are derived from other standards, however, the ordering of the enumerants has been changed, which unnecessarily complicates the mapping of the data sets using the existing standards to the SEDRIS enumerations. |
|  | Solution | Put the enumerants in their “natural order”, ensuring that the ordering of the enumerants is the same as the standard from which they were derived. Several of the technical comments (SEDRIS T0362-74) are directed at identifying those related to the atmosphere and specify the correct ordering. |

Response:	Accepted.

| SEDRIS_G005 | Where | EDCS handling of Sets |
| --- | --- | --- |
|  | Problem | Several entries in EDCS describe concepts that are, in fact, collections of other concepts. These originate from one or more perspectives of environmental data which are common within the intended user community. Initially, one or more of these entries were included in the EDCS as attributes with extended sets of enumerant values. Analysis of the EDCS standard in its entirety seems to indicate that a revision of approach needs to be considered in the case of these “collections of other concepts.” Such a revision, however, should not be attempted without a thorough analysis by the SEDRIS organization of these community perspectives and their reconciliation with the purpose and scope of the proposed EDCS standard. |
|  | Solution | The following action is recommended: --that WG8 assign the SEDRIS Organization to conduct this analysis, develop revised entries for these concepts which reconcile their original intent to the intent of the EDCS standard, and post the results to the WG8 reflector for any discussion prior to their inclusion with all other accepted comments in the next EDCS draft document. |

Response:	This comment generally addresses the issues identified by comments UK_T359 through UK_T434. AI 08-02 on the SEDRIS Organization was generated to propose an alternative to supporting (something like) these concepts (individually) rather than simple deletion (as proposed in comments UK_T359 through UK_T434).

| SEDRIS_G006 | ##### Where | ##### Clause 6 |
| --- | --- | --- |
|  | Problem | Many of the acronyms used in dictionary labels are not included in the appropriate table(s) in Annex H because they are already included in Table 3.2. However, the titles for the Annex H tables indicate that these acronym entries should be duplicated in the appropriate Annex H table(s). |
|  | Solution | Determine whether the acronyms: (1) will appear in both tables, and make the appropriate entries; or (2) change the titles and associated text descriptions of the appropriate table(s) in Annex H. |

Response:	Accepted alternative (1), with the proviso that where the acronym appears in Annex H it is hyperlinked to the acronym in Table 3.2 (and no definition is included in Annex H).

| SEDRIS_G007 | ##### Where | ##### Clause 6 |
| --- | --- | --- |
|  | Problem | In many labels and definitions, references are made to typical environmental shorthand terms such as “U component”, “V component”, “X direction”, “Y direction”, and the compass headings N, S, E, W; yet these are not defined anywhere. |
|  | Solution | Either add them to Table 3.2, to the appropriate table(s) in Annex H, or to both, in accordance with the disposition of comment SEDRIS-G006. |

Response:	Accepted in principle. however locating these elsewhere in clause 3 is preferred to Table 3.2 as these are not acronyms.

# Technical

## Forward

| SEDRIS_T0001 | Where | ##### Forward, last sentence: |
| --- | --- | --- |
|  | Problem | The sentence “Annexes B, C, D, E, F and G form a normative part of this International Standard. Annexes A, H and I are for information only. “, does not agree with the various Annex title blocks. |
|  | Solution | Change to: Annexes B, G and H form a normative part of this International Standard. Annexes A, C, D, E, F and I are for information only. |

Response:	Accepted.

## Clause 3

| SEDRIS_T0002 | Where | Clause 3 |
| --- | --- | --- |
|  | Problem | The term ‘wetlands’ is used in various EAs without being properly defined (e.g. WETLAND SURFACE FORM). |
|  | Solution | Quoting from FFENVSCI, (see comments about bibliographic additions) The regulatory definition used by the USEPA and the U.S. Army Corps of Engineers reads, “Wetlands are areas that are inundated or saturated by surface or ground water at a frequency and duration sufficient to support, and that under normal circumstances do support a prevalence of vegetation typically adapted for life in saturated soil conditions. Wetlands generally include swamps, marshes, bogs, and similar areas.” The three parts of the definition address soil hydrology, soil types, and vegetation. The Wetlands Delineation Manual published by the Army Corps of Engineers in 1987 is the official guide used for wetlands identification. In 1989, a new wetlands manual, the Federal Manual for Identifying and Delineating Jurisdictional Wetlands, was released, and a 1991 revision was also proposed, but Congress voted to continue with the 1987 guidance pending further study. A new manual incorporating the improvements is pending. |

Response:	Accepted; the term will be added to the glossary.

SEDRIS_T0003 Withdrawn

| SEDRIS_T0004 | Where | ##### Table 3.2 |
| --- | --- | --- |
|  | Problem | BC |
|  | Solution | Replace with BCE – Before the Common Era |

Response:		Accepted.

| SEDRIS_T0005 | Where | Table 3.2 |
| --- | --- | --- |
|  | Problem | “NOTAM” expansion is incorrect. |
|  | Solution | Change “Notice to Mariners” to “Notice to Airmen.” |

Response:		Accepted.

## Clause 4

| SEDRIS_T0006 | Where | ##### 4.1.2 Overview, d. |
| --- | --- | --- |
|  | Problem | ##### Each EDCS Dictionary Entry consists of the following fields: ##### … ##### d. references and reference types, and This is violated by EDCS Unit Scale Dictionary which has no reference fields, but does have a general reference. |
|  | Solution | Change to: d. either references and reference type fields, or a general dictionary reference, and |

Response:	Accepted in principle.

| SEDRIS_T0007 | Where | ##### 4.2.1 Definitions |
| --- | --- | --- |
|  | Problem | Section name conflicts with clause 3 |
|  | Solution | Rename to: EDCS Definitions |

Response:	Accepted.

| SEDRIS_T0008 | Where | ##### 4.2.2(g) |
| --- | --- | --- |
|  | Problem | “unique” is not scoped. |
|  | Solution | Change g. within a dictionary, each label uniquely denotes a concept. |

Response:	Accepted.

| SEDRIS_T0009 | Where | ##### 4.2.3(a) |
| --- | --- | --- |
|  | Problem | “unique” is not scoped. |
|  | Solution | Change a. within a dictionary, each code uniquely denotes a concept. |

Response:	Accepted.

| SEDRIS_T0010 | Where | ##### 4.2.4.3 Types of references |
| --- | --- | --- |
|  | Problem | The two reference types: (1) Non-referenced definition (ND), and (2) Un-referenced definition (UD) are too similar in name and will cause confusion |
|  | Solution | Change Non-referenced definition (ND) to: Implicit Definition (ID) |

Response:	Accepted.

| SEDRIS_T0011 | Where | ##### 4.2.5 Related concepts |
| --- | --- | --- |
|  | Problem | Not all relationships are covered by (a) and (b). E.g.: AGRICULTURAL_FACILITY and GRAIN_ELEVATOR |
|  | Solution | Add: c. because one is a generalization or a specific case of the other. |

Response:	Withdrawn.

| SEDRIS_T0012 | Where | ##### 4.3 last sentence. |
| --- | --- | --- |
|  | Problem | Misleading |
|  | Solution | Change last sentence: This International Standard does not associate any EC with any EA(s), but it does provide for such associations in registered profiles (see 4.13.10). |

Response:	Accepted.

| SEDRIS_T0013 | Where | ##### 4.4 last sentence. |
| --- | --- | --- |
|  | Problem | Misleading |
|  | Solution | Change last sentence: This International Standard does not associate any EA with any EC(s), but it does provide for such associations in registered profiles (see 4.13.10). |

Response:	Accepted.

| SEDRIS_T0014 | Where | ##### Table 4.2 -- EDCS Attribute Value Types and EDCS Storage Types |
| --- | --- | --- |
|  | Problem | KEY value type definition is not sufficiently scoped to define the representation |
|  | Solution | Change to: A opaque fixed length character string used for identification purposes. |

Response:		Moot.

| SEDRIS_T0015 | Where | ##### Table 4.2 -- EDCS Attribute Value Types and EDCS Storage Types |
| --- | --- | --- |
|  | Problem | KEY_STORAGE storage type definition is not sufficiently scoped to define the representation |
|  | Solution | Change to: An array of fixed size whose elements are characters conforming to an attribute specific structure rule. |

Response:		Accepted; the editors will consider this while working on responses to other comments.

| SEDRIS_T0016 | Where | ##### Table 4.2 -- EDCS Attribute Value Types and EDCS Storage Types |
| --- | --- | --- |
|  | Problem | FIXED_POINT storage definition is not sufficiently scoped to define the representation |
|  | Solution | Change to: A representation of a real number as an integer scaled by an ES scale factor. |

Response:		Accepted.

| SEDRIS_T0017 | Where | ##### Table 4.2 -- EDCS Attribute Value Types and EDCS Storage Types |
| --- | --- | --- |
|  | Problem | EDCS Attribute Value Types & EDCS Attribute Value Types need codes so that this type information can be represented in computing applications |
|  | Solution | Add code columns: EVC & ETC for EDCS Attribute Value Types and EDCS Storage Types, respectively, and add sentence to 4.4.2: EDCS Attribute Value Types are encoded with an EDCS Attribute Value Type Code (EVC), and EDCS Attribute Value Types are encoded with EDCS Attribute Value Type Codes (ETC) as shown in Table 4.2. |

Response:		Accepted in principle; the specific resolution will be addressed in the metadata AI.

| SEDRIS_T0018 | Where | ##### Section 4.9 First paragraph |
| --- | --- | --- |
|  | Problem | The use of the word set in two different ways is confusing |
|  | Solution | Reword: This International Standard uses EDCS Organizational Schema (EOs) to arrange ECs and EAs into groups sets based on similarity. The groups sets need not be disjoint. An EDCS Organizational Schema (EO) is the end result of a process of grouping classification and consists of a set of EDCS Groups (EGs). In general, no further organization shall be required among the EGs in an EO. In particular, the EGs may, or may not, form a hierarchy. |

Response:	Accepted.

| SEDRIS_T0019 | Where | ##### 4.13.2(m) Guidelines for definitions for registered items. |
| --- | --- | --- |
|  | Problem | There are two exceptions. |
|  | Solution | Change: References shall not be embedded in definitions with one two exceptions. Where a reference is used not as the source of a definition, but rather to define the format of an EA whose EDCS Attribute Value Type is STRING or KEY, that reference may be part of the definition. |

Response:	Accepted.

| SEDRIS_T0020 | Where | ##### 4.13.2(n) Guidelines for definitions for registered items. |
| --- | --- | --- |
|  | Problem | There are two exceptions. |
|  | Solution | Change: Definitions shall be atomic, self-contained and independent of all other EDCS Dictionary Entries in the same EDCS Dictionary with one two exceptions. - A definition may intentionally express a composite of other concepts, if those concepts are individually defined within the same EDCS Dictionary. - An EE definition may be dependent on the definition of its corresponding EA. |

Response:	Accepted.

| SEDRIS_T0021 | Where | ##### 4.13.3(d ii) Guidelines for constructing labels for registered items |
| --- | --- | --- |
|  | Problem | “.” and “,” are radix symbols, “r” is not. It is a symbol for the radix delimiter. |
|  | Solution | Change d. ii. to: the radix delimiter symbol "r". |

Response:	Accepted.

| SEDRIS_T0022 | Where | ##### 4.13.4 Rules for assigning codes for registered items |
| --- | --- | --- |
|  | Problem | Deprecated codes cannot be re-used. |
|  | Solution | Change first sentence: Codes not assigned in this International Standard (including the deprecated codes of Annex B) are reserved for future standardization or for registration. |

Response:	Accepted.

| SEDRIS_T0023 | Where | ##### 4.13.4 Rules for assigning codes for registered items |
| --- | --- | --- |
|  | Problem | As discussed at meeting 7, there is no need to reserve code ranges |
|  | Solution | Remove both the second sentence, and table 4.4, and change bullet d: d. The Registration Authority for Graphical Items shall assign codes in increasing order beginning at the first the first available code in the set of codes reserved for registration within each set of codes. |

Response:	Accepted (see also response to UK_T663).

| SEDRIS_T0024 | Where | ##### 4.13.5(b) Guidelines for related concepts for registered items |
| --- | --- | --- |
|  | Problem | Not all relationships are covered by (i) and (ii). E.g.: BUILDING and CABIN |
|  | Solution | Add: iii. because one is a generalization or a specific case of the other. |

Response:	Withdrawn; an AI was assigned to the editors should clarify the intent of “related concept” and its utility.

| SEDRIS_T0025 | Where | ##### 4.13.7 Guidelines for the registration of EEs, first sentence |
| --- | --- | --- |
|  | Problem | May contradict 4.13.2 |
|  | Solution | Change: The following additional guidelines apply to the registration of EEs: |

Response:	Accepted.

| SEDRIS_T0026 | Where | ##### 4.13.7(a) Guidelines for the registration of Ees |
| --- | --- | --- |
|  | Problem | Too restrictive |
|  | Solution | Change: Definitions of EEs shall not repeat any portion of the definition of the corresponding EA, unless such repetitions significantly enhance the readability of the definition. |

Response:	Accepted.

| SEDRIS_T0027 | Where | ##### 4.13.7(f) Guidelines for the registration of EEs |
| --- | --- | --- |
|  | Problem | Too general for only EEs |
|  | Solution | Move to 4.13.2 |

Response:	Accepted.

| SEDRIS_T0028 | Where | ##### 4.13.7(g) Guidelines for the registration of EEs |
| --- | --- | --- |
|  | Problem | Does not apply. |
|  | Solution | Move to new section: 4.13.x Guidelines for the registration of EAs |

Response:	Accepted.

| SEDRIS_T0029 | Where | ##### 4.13.7(h) Guidelines for the registration of EEs |
| --- | --- | --- |
|  | Problem | Does not apply. |
|  | Solution | Move to new section: 4.13.x Guidelines for the registration of EAs |

Response:	Accepted (and remove “INTEGER”; see response to SEDRIS_G001).

## Clause 5

| SEDRIS_T0030 | Where | 5.2 EDCS Classification Dictionary |
| --- | --- | --- |
|  | Problem | Relation between Tables 5.4-29 and annex C needs clarification. |
|  | Solution | Add to end of first paragraph: The first four fields are found in Tables 6.4 through 6.29. The Label field together with last three fields are found in Annex C, Tables C.2 through C.27. The Label field content in Tables 5.4 through 5.29 are hyperlinked to the corresponding entries in Tables C.2 through C.27. |

Response:	Accepted in principle.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Definition | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0031 | EC |  | A | ATM_OBS_TRSRF_CHARSET | An evaluation of the state of the atmosphere as observed from a point at the surface of the earth. | EC ATMOSPHERE, EC ATM_OBS_CHARSET | ATMOSPHERE | ND |  |

|  | #### Where | #### Table C.2 – Table 5.4 – EDCS Classifications whose label begins with A |
| --- | --- | --- |
|  | Problem | Need to add this classification to make the set of atmospheric observations classifications more complete. |

Response:	Accepted.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0032 | EC | BOUNDARY | AC |  |  | EC_ADMINISTRATIVE_BOUNDARY |  |  |  |
| SEDRIS_T0033 | EC | BOUNDARY | AC |  |  | EC_ARMISTICE_LINE |  |  |  |
| SEDRIS_T0034 | EC | BOUNDARY | AC |  |  | EC_CEASE_FIRE_LINE |  |  |  |
| SEDRIS_T0035 | EC | BOUNDARY | AC |  |  | EC_CHANGE_LINE |  |  |  |
| SEDRIS_T0036 | EC | BOUNDARY | AC |  |  | EC_COASTLINE |  |  |  |
| SEDRIS_T0037 | EC | BOUNDARY | AC |  |  | EC_COASTLINE_OR_SHORELINE |  |  |  |
| SEDRIS_T0038 | EC | BOUNDARY | AC |  |  | EC_COMPLEX_OUTLINE |  |  |  |
| SEDRIS_T0039 | EC | BOUNDARY | AC |  |  | EC_DATA_QUALITY_BOUNDARY |  |  |  |
| SEDRIS_T0040 | EC | BOUNDARY | AC |  |  | EC_DEFACTO_BOUNDARY |  |  |  |
| SEDRIS_T0041 | EC | BOUNDARY | AC |  |  | EC_FENCE |  |  |  |
| SEDRIS_T0042 | EC | BOUNDARY | AC |  |  | EC_INLAND_SHORELINE |  |  |  |
| SEDRIS_T0043 | EC | BOUNDARY | AC |  |  | EC_MARITIME_LIMIT_BOUNDARY |  |  |  |
| SEDRIS_T0044 | EC | BOUNDARY | AC |  |  | EC_REGULATED_LINE |  |  |  |
| SEDRIS_T0045 | EC | BOUNDARY | AC |  |  | EC_SAND_LINE |  |  |  |
| SEDRIS_T0046 | Withdrawn |  |  |  |  |  |  |  |  |
| SEDRIS_T0047 | Withdrawn |  |  |  |  |  |  |  |  |
| SEDRIS_T0048 | EC | BUILDING | AC |  |  | EC_ADMINISTRATION_BUILDING |  |  |  |
| SEDRIS_T0049 | EC | BUILDING | AC |  |  | EC_ARMOURY |  |  |  |
| SEDRIS_T0050 | EC | BUILDING | AC |  |  | EC_AUDITORIUM |  |  |  |
| SEDRIS_T0051 | EC | BUILDING | AC |  |  | EC_CABIN |  |  |  |
| SEDRIS_T0052 | EC | BUILDING | AC |  |  | EC_CARPENTRY_BUILDING |  |  |  |
| SEDRIS_T0053 | EC | BUILDING | AC |  |  | EC_CASTLE |  |  |  |
| SEDRIS_T0054 | EC | BUILDING | AC |  |  | EC_CEMETERY_BUILDING |  |  |  |
| SEDRIS_T0055 | EC | BUILDING | AC |  |  | EC_COMMERCIAL_BUILDING |  |  |  |
| SEDRIS_T0056 | EC | BUILDING | AC |  |  | EC_COMMUNICATION_BUILDING |  |  |  |
| SEDRIS_T0057 | EC | BUILDING | AC |  |  | EC_COMMUNITY_CENTRE |  |  |  |
| SEDRIS_T0058 | EC | BUILDING | AC |  |  | EC_CORRECTIONAL_BUILDING |  |  |  |
| SEDRIS_T0059 | EC | BUILDING | AC |  |  | EC_DIPLOMATIC_BUILDING |  |  |  |
| SEDRIS_T0060 | EC | BUILDING | AC |  |  | EC_DORMITORY |  |  |  |
| SEDRIS_T0061 | EC | BUILDING | AC |  |  | EC_FACTORY |  |  |  |
| SEDRIS_T0062 | EC | BUILDING | AC |  |  | EC_FARM_BUILDING |  |  |  |
| SEDRIS_T0063 | EC | BUILDING | AC |  |  | EC_FILTRATION_BUILDING |  |  |  |
| SEDRIS_T0064 | EC | BUILDING | AC |  |  | EC_FIRE_AND_POLICE_STATION |  |  |  |
| SEDRIS_T0065 | EC | BUILDING | AC |  |  | EC_FIRE_STATION |  |  |  |
| SEDRIS_T0066 | EC | BUILDING | AC |  |  | EC_GREENHOUSE |  |  |  |
| SEDRIS_T0067 | EC | BUILDING | AC |  |  | EC_GUARD_ROOM |  |  |  |
| SEDRIS_T0068 | EC | BUILDING | AC |  |  | EC_HEALTH_OFFICE |  |  |  |
| SEDRIS_T0069 | EC | BUILDING | AC |  |  | EC_HOSPITAL_BUILDING |  |  |  |
| SEDRIS_T0070 | EC | BUILDING | AC |  |  | EC_HOSTEL |  |  |  |
| SEDRIS_T0071 | EC | BUILDING | AC |  |  | EC_HOTEL |  |  |  |
| SEDRIS_T0072 | EC | BUILDING | AC |  |  | EC_HOUSE |  |  |  |
| SEDRIS_T0073 | EC | BUILDING | AC |  |  | EC_INDUSTRIAL_BUILDING |  |  |  |
| SEDRIS_T0074 | EC | BUILDING | AC |  |  | EC_JAIL_OR_PRISON |  |  |  |
| SEDRIS_T0075 | EC | BUILDING | AC |  |  | EC_KENNEL |  |  |  |
| SEDRIS_T0076 | EC | BUILDING | AC |  |  | EC_LIBRARY |  |  |  |
| SEDRIS_T0077 | EC | BUILDING | AC |  |  | EC_MISSION |  |  |  |
| SEDRIS_T0078 | EC | BUILDING | AC |  |  | EC_MOTEL |  |  |  |
| SEDRIS_T0079 | EC | BUILDING | AC |  |  | EC_MULTI_UNIT_DWELLING |  |  |  |
| SEDRIS_T0080 | EC | BUILDING | AC |  |  | EC_MUSEUM_BUILDING |  |  |  |
| SEDRIS_T0081 | EC | BUILDING | AC |  |  | EC_NEWSPAPER_PLANT |  |  |  |
| SEDRIS_T0082 | EC | BUILDING | AC |  |  | EC_OBSERVATORY |  |  |  |
| SEDRIS_T0083 | EC | BUILDING | AC |  |  | EC_OPERA_HOUSE |  |  |  |
| SEDRIS_T0084 | EC | BUILDING | AC |  |  | EC_OUTBUILDING |  |  |  |
| SEDRIS_T0085 | EC | BUILDING | AC |  |  | EC_PALACE |  |  |  |
| SEDRIS_T0086 | EC | BUILDING | AC |  |  | EC_PALACE_COMPLEX |  |  |  |
| SEDRIS_T0087 | EC | BUILDING | AC |  |  | EC_POLICE_STATION |  |  |  |
| SEDRIS_T0088 | EC | BUILDING | AC |  |  | EC_POST_OFFICE |  |  |  |
| SEDRIS_T0089 | EC | BUILDING | AC |  |  | EC_POWER_GENERATION_BUILDING |  |  |  |
| SEDRIS_T0090 | EC | BUILDING | AC |  |  | EC_PROCESSING_BUILDING |  |  |  |
| SEDRIS_T0091 | EC | BUILDING | AC |  |  | EC_PUBLIC_INN |  |  |  |
| SEDRIS_T0092 | EC | BUILDING | AC |  |  | EC_R_AND_D_BUILDING |  |  |  |
| SEDRIS_T0093 | EC | BUILDING | AC |  |  | EC_RANGER_STATION |  |  |  |
| SEDRIS_T0094 | EC | BUILDING | AC |  |  | EC_REFORMATORY_BUILDING |  |  |  |
| SEDRIS_T0095 | EC | BUILDING | AC |  |  | EC_REPAIR_BUILDING |  |  |  |
| SEDRIS_T0096 | EC | BUILDING | AC |  |  | EC_ROUNDHOUSE |  |  |  |
| SEDRIS_T0097 | EC | BUILDING | AC |  |  | EC_SCHOOL_BUILDING |  |  |  |
| SEDRIS_T0098 | EC | BUILDING | AC |  |  | EC_SENIOR_CITIZENS_HOME |  |  |  |
| SEDRIS_T0099 | EC | BUILDING | AC |  |  | EC_TRANSPORTATION_BUILDING |  |  |  |
| SEDRIS_T0100 | EC | CABLE | AC |  |  | EC_AERIAL_CABLE |  |  |  |
| SEDRIS_T0101 | EC | CONSULATE | AC |  |  | EC_AMBASSADORIAL_RESIDENCE |  |  |  |
| SEDRIS_T0102 | EC | CONSULATE | AC |  |  | EC_EMBASSY |  |  |  |
| SEDRIS_T0103 | EC | DIPLOMATIC_BUILDING | AC |  |  | EC_CONSULATE |  |  |  |
| SEDRIS_T0104 | EC | DIPLOMATIC_BUILDING | AC |  |  | EC_LEGATION_BUILDING |  |  |  |
| SEDRIS_T0105 | EC | GOVERNMENT_BUILDING | AC |  |  | EC_CAPITOL_BUILDING |  |  |  |
| SEDRIS_T0106 | EC | GOVERNMENT_BUILDING | AC |  |  | EC_CITY_HALL |  |  |  |
| SEDRIS_T0107 | EC | GOVERNMENT_BUILDING | AC |  |  | EC_COURTHOUSE |  |  |  |
| SEDRIS_T0108 | EC | GOVERNMENT_BUILDING | AC |  |  | EC_CUSTOMS_HOUSE |  |  |  |
| SEDRIS_T0109 | EC | NONCOMMUNICATION_TOWER | AC |  |  | EC_AERIAL_CABLE_PYLON |  |  |  |
| SEDRIS_T0110 | EC | NONCOMMUNICATION_TOWER | AC |  |  | EC_BRIDGE_TOWER |  |  |  |
| SEDRIS_T0111 | EC | NONCOMMUNICATION_TOWER | AC |  |  | EC_COOLING_TOWER |  |  |  |
| SEDRIS_T0112 | EC | NONCOMMUNICATION_TOWER | AC |  |  | EC_MARTELLO_TOWER |  |  |  |
| SEDRIS_T0113 | EC | TOWER | AC |  |  | EC_COMMUNICATION_TOWER |  |  |  |
| SEDRIS_T0114 | EC | TOWER | AC |  |  | EC_WATER_INTAKE_TOWER |  |  |  |
| SEDRIS_T0115 | EC | TOWER | AC |  |  | EC_WATER_TOWER |  |  |  |
| SEDRIS_T0116 | EC | WATER | AC |  |  | EC_OCEAN |  |  |  |
| SEDRIS_T0117 | EC | WATER | AC |  |  | EC_SEA |  |  |  |
| SEDRIS_T0118 | EC | WATER | AC |  |  | EC_WATER_EXCEPT_INLAND |  |  |  |
| SEDRIS_T0119 | EC | AIR_WARNING_LIGHT | AC |  |  | EC_NAVIGATIONAL_LIGHT |  |  |  |
| SEDRIS_T0120 | EC | AMUSEMENT_PARK | AC |  |  | EC_RECREATIONAL_COMPLEX |  |  |  |
| SEDRIS_T0121 | EC | BARN_OR_SHED | AC |  |  | EC_FARM_BUILDING |  |  |  |
| SEDRIS_T0122 | EC | BEACON | AC |  |  | EE_AERONAUTICAL_LIGHT_FUNCTION_IDENTIFICATION_BEACON |  |  |  |
| SEDRIS_T0123 | EC | BLOCKHOUSE | AC |  |  | EC_FORTIFICATION |  |  |  |
| SEDRIS_T0124 | EC | COMMUNITY_CENTRE | AC |  |  | EC_COMMUNITY_REC_FACILITY |  |  |  |
| SEDRIS_T0125 | EC | GRAIN_ELEVATOR | AC |  |  | EC_GRAIN_STORAGE_STRUCTURE |  |  |  |
| SEDRIS_T0126 | EC | LIVESTOCK_PEN | AC |  |  | EC_AGRICULTURAL_FACILITY |  |  |  |
| SEDRIS_T0127 | EC | ADMINISTRATION_BUILDING | AC |  |  | EC_MILITARY_OPERATIONS_BLDG |  |  |  |
| SEDRIS_T0128 | EC | INDUSTRIAL_BUILDING | AC |  |  | EC_PETROLEUM_BUILDING |  |  |  |
| SEDRIS_T0129 | EC | CORRECTIONAL_FACILITY | AC |  |  | EC_PRISON |  |  |  |
| SEDRIS_T0130 | EC | PROCESSING_BUILDING | AC |  |  | EC_SAWMILL |  |  |  |
| SEDRIS_T0131 | EC | STORAGE | AC |  |  | EC_WAREHOUSE |  |  |  |
| SEDRIS_T0132 | EC | SCHOOL_BUILDING | AC |  |  | EC_SEMINARY |  |  |  |
| SEDRIS_T0133 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_CONIFER_BOREAL_FOREST |  |  |  |
| SEDRIS_T0134 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_CONIFER_FOREST |  |  |  |
| SEDRIS_T0135 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_CONIFEROUS_FOREST |  |  |  |
| SEDRIS_T0136 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_COOL_BROADLEAF_FOREST |  |  |  |
| SEDRIS_T0137 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_COOL_CONIFER_FOREST |  |  |  |
| SEDRIS_T0138 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_COOL_MIXED_FOREST |  |  |  |
| SEDRIS_T0139 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_COOL_RAIN_FOREST |  |  |  |
| SEDRIS_T0140 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_COOL_S_HEMI_MIXED_FORESTS |  |  |  |
| SEDRIS_T0141 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_DECID_BROADLEAF_FOREST |  |  |  |
| SEDRIS_T0142 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_DECID_BROADLEAF_FOREST_2 |  |  |  |
| SEDRIS_T0143 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_DECID_CONIFER_FOREST |  |  |  |
| SEDRIS_T0144 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_DECID_MIXED_BOREAL_FOREST |  |  |  |
| SEDRIS_T0145 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_EVERGREEN_BROADLEAF |  |  |  |
| SEDRIS_T0146 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_MIXED_FOREST |  |  |  |
| SEDRIS_T0147 | EC | MOUNTAINOUS_REGION | AC |  |  | EE_ECOSYSTEM_TYPE_MONTAINE_TROPICAL_FORESTS |  |  |  |
| SEDRIS_T0148 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_MONTAINE_TROPICAL_FORESTS |  |  |  |
| SEDRIS_T0149 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_RAIN_GREEN_TROPICAL_FOREST |  |  |  |
| SEDRIS_T0150 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_S_HEMI_MIXED_FOREST |  |  |  |
| SEDRIS_T0151 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_SEASONAL_TROPICAL_FOREST |  |  |  |
| SEDRIS_T0152 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_TROPICAL_DEGRADED_FOREST |  |  |  |
| SEDRIS_T0153 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_TROPICAL_RAINFOREST |  |  |  |
| SEDRIS_T0154 | EC | FOREST | AC |  |  | EE_ECOSYSTEM_TYPE_WET_SCLEROPHYLIC_FOREST |  |  |  |

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | ECs that should be related to more general concepts, if SEDRIS G000 was accepted. (If it was rejected, remove any portions of these relationships that already exist.) |
|  | Solution | Add 2 way ‘related concepts’ relationships between these ECs. These are all instances of specialization/generalization relationships. |

Response:	Withdrawn.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0155 | EC | BIOLOGIC_HAZARD | RL | BIOLOGIC_HAZARD_REGION |  |  |  |  |  |
| SEDRIS_T0156 | EC | CHEMICAL_HAZARD | RL | CHEMICAL_HAZARD_REGION |  |  |  |  |  |
| SEDRIS_T0157 | EC | GEOGRAPHIC_INFORMATION | RL | GEOGRAPHIC_INFORMATION_REGION |  |  |  |  |  |
| SEDRIS_T0158 | EC | LAND_FLOODS_PERIODICALLY | RL | PERIODICALLY_FLOODED_REGION |  |  |  |  |  |
| SEDRIS_T0159 | EC | RADIOLOGIC_HAZARD | RL | RADIOLOGIC_HAZARD_REGION |  |  |  |  |  |

Response:		Accept the HAZARDS and LAND_FLOODS_PERIODICALLY; leave GEOGRAPHIC_INFORMATION unchanged.

| SEDRIS_T0160 | EC | BIOLOGIC_H AZARD | DC |  |  | EC_CHEMICAL_HAZARD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0161 | EC | BIOLOGIC_HAZARD | DC |  |  | EC_RADIOLOGIC_HAZARD |  |  |  |
| SEDRIS_T0162 | EC | CHEMICAL_HAZARD | DC |  |  | EC_BIOLOGIC_HAZARD |  |  |  |
| SEDRIS_T0163 | EC | CHEMICAL_HAZARD | DC |  |  | EC_RADIOLOGIC_HAZARD |  |  |  |
| SEDRIS_T0164 | EC | RADIOLOGIC_HAZARD | DC |  |  | EC_BIOLOGIC_HAZARD |  |  |  |
| SEDRIS_T0165 | EC | RADIOLOGIC_HAZARD | DC |  |  | EC_CHEMICAL_HAZARD |  |  |  |

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | The labels of some region ECs are misleading. Also, BIOLOGIC_HAZARD, CHEMICAL_HAZARD, and RADIOLOGIC_HAZARD do not participate any of the kinds of relationships that would allow them to be related concepts. |

Response:		Accepted (all).

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0166 | EC | FOREST | RD |  | The phrase “tract or region” is redundant, according to the definition of ‘tract’ in clause 3; change to “region”. |  |  | ND |  |

Response:	Accepted.

| SEDRIS_T0167 | EC | DIPLOMATIC_BUILDING | RD |  | A building that supports the transaction of diplomatic business, e.g. the activities of an ambassador, the official residence of a diplomat. |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle (delete the “e.g., …”).

| SEDRIS_T0168 | EC | AERATOR | RL | AERATION_PLANT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0169 | EC | FILTRATION_PLANT | RL | FILTRATION_OR_AERATION_PLANT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted with appropriate revisions of the definition to a “structured” form.

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | The current definition of forest contains a redundant phrase. The current definition of diplomatic building is too restrictive.The current labels of AERATOR and FILTRATION PLANT are misleading. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0170 | EC | NONCOMMUNICATION_TOWER | AC |  |  | COMMUNICATION_TOWER |  |  |  |

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | ECs that should be related to their opposite concepts. |
|  | Solution | Add the missing two way relationship. |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0171 | Withdrawn |  |  |  |  |  |  |  |  |
| SEDRIS_T0174 | EC | MARITIME_POLICE_STATION | AC |  |  | EE_AVAILABLE_FACILITIES_TYPE_WATER_POLICE |  |  |  |
| SEDRIS_T0175 | EC | ARMISTICE_LINE | AC |  |  | EE_BOUNDARY_TYPE_ARMISTICE_LINE |  |  |  |
| SEDRIS_T0176 | EC | ARMOURED_FIGHTING_VEHICLE | AC |  |  | EE_ASSET_TYPE_ARMOURED_FIGHTING_VEH |  |  |  |
| SEDRIS_T0177 | EC | ARMOURED_UTILITY_VEHICLE | AC |  |  | EE_ASSET_TYPE_ARMOURED_UTILITY_VEH |  |  |  |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0178 | EC | BOTTOM_TOPO_CONFIGURATION | RD |  | Add “such as (but not restricted to) a pinnacle, ridge, or trench.” |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:		Rejected proposed change to definition.

| SEDRIS_T0179 | EC | BREAKWATER | AC |  |  | EC_BREAKWATER_OR_GROIN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0180 | EC | CAISSON | AC |  |  | EE_NAUTICAL_GATE_TYPE_CAISSON |  |  |  |
| SEDRIS_T0181 | EC | FLOOD_BARRAGE | AC |  |  | EE_NAUTICAL_GATE_TYPE_TIDAL |  |  |  |
| SEDRIS_T0182 | EC | RECREATIONAL_PIER | AC |  |  | EE_BERTH_TYPE_PIER |  |  |  |
| SEDRIS_T0183 | EC | WHARF | AC |  |  | EE_BERTH_TYPE_WHARF |  |  |  |
| SEDRIS_T0184 | EC | HOSPITAL_BUILDING | AC |  |  | EC_MEDICAL_CENTRE |  |  |  |

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | ECs that should be related to similar concepts. |
|  | Solution | Add the missing 2 way relationships. |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0185 | EC | CHURCH | D |  |  |  |  |  |  |

Response:	Rejected.

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | Some ECs that have to do with buildings used for religious purposes have malformed related concepts. CHURCH is redundant with WORSHIP_PLACE, except that it unnecessarily singles out a specific religion as being most likely to be designated. |
|  | Solution | Delete CHURCH and use the more neutral WORSHIP_PLACE instead, which can be attributed if there is a need to specify a specific religion. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0186 | EC | PETROLEUM_BUILDING | RD |  | Drop the second instance of ‘production’. |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0187 | EC | BOUNDARY_MONUMENT | RL | BOUNDARY_MARKER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle, however revise the definition from “marker” to “monument” and leave Label unchanged.

| SEDRIS_T0188 | EC | EQUIPMENT_COMPONENT | RD |  | Replace “or a vehicle” with “of a vehicle”. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0189 | EC | ZOO | RD |  | Remove “etc.,” from definition |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0190 | EC | JULIAN_DAY | RD |  | The Julian day number associated with the solar day which is the number assigned to a day in a continuous count of days beginning with the Julian day number “0” assigned to the day starting at Greenwich mean noon on 1 January 4 713 BCE, Julian proleptic calendar –4 712. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | Malformed definitions, poorly designed labels. |
|  | Solution | Fix the offending words and phrases. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0191 | EC | RADAR_DEVICE | AG |  |  |  | INFRASTRUCTURE |  |  |
| SEDRIS_T0192 | EC | RADAR_DEVICE | AG |  |  |  | AIR_TRANSP |  |  |
| SEDRIS_T0193 | EC | RADAR_TRANSMITTER | D |  |  |  |  |  |  |

Response:	Accept the group additions; AI 08-15: revise definition for radar transmitter (do not delete) and add a separate radar receiver.

|  | Where | Table 5.21 |
| --- | --- | --- |
|  | Problem | RADAR_TRANSMITTER is redundant with RADAR_DEVICE. In addition, the label of RADAR_TRANSMITTER incorrectly implies that it is defined to transmit only, so of the two ECs, RADAR_DEVICE has the better label. |
|  | Solution | Map all current usage of RADAR_TRANSMITTER to RADAR_DEVICE and delete RADAR_TRANSMITTER. Add RADAR_DEVICE to the necessary groups. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Definition | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0194 | EC | BOTTOM_RETURN | RL | OC_BOTTOM_RETURN |  |  |  |  |  |
| SEDRIS_T0195 | EC | BOTTOM_RETURN | RD |  | A detectable active sonar return signal returned from the ocean bottom. |  |  |  |  |

Response:	Accepted.

|  | Table 5.5 – EDCS Classifications whose label begins with B |
| --- | --- |
|  | The signal is not identified as the ocean bottom. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Definition | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0196 | EC | OC_COLUMN_SCATSTRN_CHARSET | RD |  | A set of acoustic scattering strength characteristics of the ocean water column. |  |  | ND |  |
| SEDRIS_T0197 | EC | OC_LAYER_SCATSTRN_CHARSET | RD |  | A set of acoustic scattering strength characteristics of an ocean horizontal layer. |  |  | ND |  |

Response:	Accepted.

|  | Table 5.18 – EDCS Classifications whose label begins with O |
| --- | --- |
|  | Scattering strength is not “along” an object. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Definition | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0198 | EC | OCEAN_BIOLOGIC | RD |  | A sound source in the ocean of biological nature, e.g., a fish in a school or kelp bed, a whale pod, or snapping shrimp bed. |  |  |  |  |
| SEDRIS_T0199 | EC | OCEAN_BIOLOGIC | RL | OC_BIOLOGIC_SOUND_SOURCE |  |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0200 | EC | OCEAN_BIOLOGIC | AC |  |  | EC_SHRIMP_BED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0201 | EC | OCEAN_BIOLOGIC | AC |  |  | EC_WHALE_POD |  |  |  |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0202 | EC |  | A | KELP_BED | An area of the ocean floor covered with a significant density of kelp weed | EC_OCEAN_BIOLOGIC | PLANT, ACOUSTIC_PHENOMENON | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted, however remove “weed”.

|  | Where | Table 5.18 – EDCS Classifications whose label begins with O |
| --- | --- | --- |
|  | Problem | OCEAN_BIOLOGIC refers to sound produced from biological sources rather than the plant or animals themselves. Consequently, the definition should make this clear, and specific instances of OCEAN_BIOLOGIC should be marked as related concepts. In addition, KELP_BED does not currently have an EC. |
|  | Solution | Provide new definitions for OCEAN_BIOLOGIC, and add the missing 2 way relationships. |

Table 5.17  Action Item #15.

| Comment Tag | Dic | Current label | Chg Type | New Label | Definition | Related Concepts | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0203 | EC |  | A | AID_TO_NAVIGATION | Objects (e.g., lights, signage, or electronic beacons) designed to assist in the determination of a safe course or of a vehicle’s position, or to warn of dangers and obstructions. |  | MARITIME_OBJ, AERONAUTICS, AIR_TRANSPORTATION, HYDROGRAPHIC_TRANSPORTATION | UD | (taken from Table 6.34, EE_AIDS_TO_NAVIGATION) |
| SEDRIS_T0204 | EC |  | A | CULTURAL_LANDMARK | An object relating to civilization located in the landscape which, by its conspicuousness, serves as a guide in the direction of one’s course (esp. as a guide to sailors in navigation). |  | LAND_TRANSPORTATION, HYDROGRAPHIC_OBJ, NAUTICAL | ND |  |
| SEDRIS_T0205 | EC |  | A | MISC_HYDRO_SVY_CONTACT | A miscellaneous object detected during a hydrographic survey. |  | HYDROGRAPHY, BATHYMETRY | ND |  |
| SEDRIS_T0206 | EC |  | A | NON_SUB_HYDRO_SVY_CONTACT | An object detected during a hydrographic survey which was determined not to be a submarine. |  | HYDROGRAPHY, BATHYMETRY | ND |  |

Response:	Accepted.

|  | Table 5.17 |
| --- | --- |
|  | EA_COVERAGE and related enumerants actually describe groups of “things” or, in a limited number of cases, the “things” themselves. If EA_COVERAGE and related enumerants are eliminated, then new ECs are required. |
|  | Add new classification codes required by elimination of EA_COVERAGE and related enumerants |

## Clause 6

| SEDRIS_T0207 | ##### 6.2 EDCS Attribute Dictionary |
| --- | --- |
|  | Relation between Tables 6.4-29 and annex D needs clarification. |
|  | Add to end of first paragraph: The first six fields are found in Tables 6.4 through 6.29. The Label field together with last three fields are found in Annex D, Tables D.2 through D.27. The Label field content in Tables 6.4 through 6.29 are hyperlinked to the corresponding entries in Tables D.2 through D.27. |

Response:	Accepted.

| SEDRIS_T0208 | ##### Table 6.2 – EDCS Attribute Dictionary Entry fields, field= Unit Equivalence Class label |
| --- | --- |
|  | An attribute should not have more than one EQ |
|  | Change content cell: If the EDCS Attribute Value Type is INTEGER or REAL then this field contains a list of one or more an EQLs (see 4.8 Unit Equivalence Classes). The presentation of this field is grey gray for all other EDCS Attribute Value Types. |

Response:	Accepted, however EAs currently having more than one EQ should be split into separate EAs with appropriate changes in their definition/label, and “grey” is correct.

| SEDRIS_T0209 | ##### Table 6.2 – EDCS Attribute Dictionary Entry fields, field= Unit Equivalence Class label |
| --- | --- |
|  | Only REAL attributes should have an EQ |
|  | Change content cell: If the EDCS Attribute Value Type is INTEGER or REAL then this field contains a list of one or more an EQLs (see 4.8 Unit Equivalence Classes). The presentation of this field is grey gray for all other EDCS Attribute Value Types. |

Response:	Accepted (see SEDRIS_G001).

| SEDRIS_T0210 | ##### 6.3 EDCS Attribute Enumerant Dictionary, First Paragraph |
| --- | --- |
|  | Relation between Tables 6.32-57 and annex E needs clarification. |
|  | Add to end of first paragraph: The References and Reference type fields are do not appear in Tables 6.32 through 6.57. The these fields together with the Attribute label and Enumerant label fields are found in Annex E, Tables E.2 through E.27. |

Response:	Accepted.

| SEDRIS_T0211 | ##### 6.3 EDCS Attribute Enumerant Dictionary, First Paragraph |
| --- | --- |
|  | Relation between Tables 6.32-57 and annex E needs clarification. |
|  | Enumerant Labels found in Tables 6.32 through 6.57 should be hyperlinked to corresponding entries in Tables E.2 through E.27. |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0212 | EA | ACNOISE_SPECCRV_CORRECTION | RR |  |  |  |  |  |  | IR | OAML, Shipping Noise Data Base |
| SEDRIS_T0213 | EA | ACOUSTIC_PWRLVLSPEC_CRRCT | RD |  | The omni directional ambient noise spectrum level correction at a specified frequency referenced to a 50 hertz reference level; this quantity may be qualified with an acoustic frequency |  |  |  |  | IR | OAML, Shipping Noise Data Base |

Response:	Accepted.

|  | #### Table 6.4 – EDCS Attributes whose label begins with A |
| --- | --- |
|  | Type was UD |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0214 | EA | ACOUSTIC_FREQUENCY_CODED | RD |  | The acoustic frequency center of the one-third octave frequency bands as defined by ANSII. |  |  |  |  | IR | OAML, Wind And Residual Noise Data Base |

Response:	AI 08-27: See response to JAPAN_T013.

|  | Problem: current definition is incomplete “The acoustic frequency.” |
| --- | --- |
|  | Replace the definition of the acoustic frequency coded with the new definition. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0215 | EA | AVAILABLE_FACILITIES_TYPE | RD |  | Add at the end “of a harbour or marina.” |  |  |  |  |  |  |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | AVAILABLE_FACILITIES_TYPE seems intended to cover only facilities in a maritime context, but its definition is currently too general. (For instance, ‘ticket sales’ would apply to passenger ships and the like, but currently the definition makes it appear that it should also apply to ECs like OPERA HOUSE. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0216 | EE | AERONAUTICAL_LIGHT_FUNCTION | AERONAUTICAL | D |  |  |  |  |  |
| SEDRIS_T0217 | EE | AERONAUTICAL_LIGHT_FUNCTION | AUXILIARY | D |  |  |  |  |  |
| SEDRIS_T0218 | EE | AERONAUTICAL_LIGHT_FUNCTION | BEACON | RL | BEACON_UNSPECIFIED |  |  |  |  |
| SEDRIS_T0219 | EE | AERONAUTICAL_LIGHT_FUNCTION | CENTRE_ROW_2_HI_VISIBILITY | RD |  | Replace “1” with “2”. |  |  |  |
| SEDRIS_T0220 | EE | AERONAUTICAL_LIGHT_FUNCTION | FOG_DETECTOR | D |  |  |  |  |  |

Response:	The editors will investigate.

| SEDRIS_T0221 | EE | AERONAUTICAL_LIGHT_FUNCTION | HARBOUR | DC |  |  |  |  | EC_HARBOUR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0222 | EE | AERONAUTICAL_LIGHT_FUNCTION | RUNWAY_OR_RUNWAY_EDGE | RD |  | Change “RUNWAY_EDGE” to “EE_RUNWAY_EDGE” so that it can hyperlink properly. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | Missing related concepts, missing or poor definitions. - AERONAUTICAL is a redundant EE here; it should be in LIGHT_FUNCTION instead. If the intent of the EE was to convey ‘unspecified’ or some such thing, the label should be changed and the definition supplied accordingly. - AERONAUTICAL_LIGHT_FUNCTION_FOG_DETECTOR is redundant with LIGHT_FUNCTION_FOG_DETECTOR. - AERONAUTICAL_LIGHT_FUNCTION_HARBOUR isn’t a HARBOUR, it’s just a light used in a harbour. - AUXILIARY seems redundant with SECONDARY. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0223 | EE | AQUEDUCT_TYPE | SHAFT | RD |  | The 36eflo form of aqueduct constructed by identifying a high point on the water table, tapping it, and channeling the water via an underground tunnel to its eventual destination. Vertical shafts leading from the surface down into the channel are used for access during construction, and for later maintenance. | IR | http://members.aol.com/_ht_a/cbeekman/research/qanatlaventa.html Beekman, C.S. P.C. Weigand, and J.J. Pint. 1999. Old World Irrigation Technology in a New World Context: Qanats in Spanish Colonial Western Mexico. Antiquity 73(279): 440446. See also Scientific American, 1988. H.E. Wulff, The Qanats of Iran. |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0224 | EE | ASSET_TYPE | ARMOURED_FIGHTING_VEH | AC |  |  |  |  | EC_ARMOURED_FIGHTING_VEHICLE |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0225 | EE | ASSET_TYPE | ARMOURED_FIGHTING_VEH | RD |  | See EC_ARMOURED_FIGHTING_VEHICLE. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T0226 | EE | ASSET_TYPE | ARMOURED_UTILITY_VEH | AC |  |  |  |  | EC_ARMOURED_UTILITY_VEHICLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0227 | EE | ASSET_TYPE | ARMOURED_UTILITY_VEH | RD |  | See EC_ARMOURED_UTILITY_VEHICLE. | ND |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T0228 | EE | ASSET_TYPE | LARGE_TRACK_UTILITY_VEH | AC |  |  |  |  | EE_ASSET_TYPE_SMALL_TRACK_UTILITY_VEH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0229 | EE | ASSET_TYPE | LARGE_WHEEL_UTILITY_VEH | AC |  |  |  |  | EE_ASSET_TYPE_SMALL_WHEEL_UTILITY_VEH |
| SEDRIS_T0230 | EE | ASSET_TYPE | MORTARS | AC |  |  |  |  | EC_MORTAR |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0231 | EE | ASSET_TYPE | MORTARS | RD |  | A muzzleloading cannon for firing shells at low velocities, short ranges and high trajectories. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0232 | EE | ASSET_TYPE | MORTARS | RL | MORTAR |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0233 | EE | ASSET_TYPE | SELF_PROPELLED_ARTILLERY | AC |  |  |  |  | EC_SELF_PROPELLED_ARTILLERY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0234 | EE | ASSET_TYPE | SELF_PROPELLED_ARTILLERY | RD |  | A selfpropelled largecalibre firing weapon that is mounted or manned by a crew; e.g., howitzer, or cannon. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0235 | EE | ASSET_TYPE | SMALL_TRACK_UTILITY_VEH | AC |  |  |  |  | EE_ASSET_TYPE_LARGE_TRACK_UTILITY_VEH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0236 | EE | ASSET_TYPE | SMALL_WHEEL_UTILITY_VEH | AC |  |  |  |  | EE_ASSET_TYPE_LARGE_WHEEL_UTILITY_VEH |
| SEDRIS_T0237 | EE | ASSET_TYPE | TOWED_ARTILLERY | AC |  |  |  |  | EC_TOWED_ARTILLERY |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0238 | EE | ASSET_TYPE | TOWED_ARTILLERY | RD |  | A towed largecalibre firing weapon that is mounted or manned by a crew; e.g., an antitank gun, howitzer, or cannon. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Question: What EC could actually be attributed by ASSET TYPE? |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0239 | EA | BELOW_STATION_CLD_TOP_TYP | RL | BELOW_STATION_CLD_TOP_DESCR |  |  |  |  |  |  |  |

Response:	Accepted.

|  | Where | Table 6.5 – EDCS Attributes whose label begins with B |
| --- | --- | --- |
|  | Problem | The use of TYP in the label could lead to confusion. What is being enumerated is a description of the cloud top, and not the cloud type. In the reference, it is called cloud top description. TYP does not appear in Table H.3 |

| Comment Tag | Dic | Current EA Label |  | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0240 | EA |  | BOT_RETURN_ALLEGIANCE | RD |  | The military alignment of an object identified by a sidescan sonar pulse bottom return. |  |  |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0241 | EA | BOT_RETURN_ALLEGIANCE | AC |  |  | MINE_ALLEGIANCE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | #### Table 6.5 – EDCS Attributes whose label begins with B |
| --- | --- | --- |
|  | Problem | This amplification is indicated by the enumerant definitions. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0242 | EA |  | A | FOG_PRESENT | Flag indicating if fog is present. | EA FOG_COVERAGE, EA FOG_PROBABILITY | LOGICAL |  | ATMOSPHERE | UD |  |

Response:	Accepted.

|  | Where | Table 6.9 – EDCS Attributes whose label begins with F and Table D.7 – Eas whose label begins with F |
| --- | --- | --- |
|  | Problem | The new attribute is needed to indicate if fog is present or not. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0243 | EA |  | A | SNOW_ACCUMULATION | The depth of snow that has accumulated over a specified time interval. | EA SNOW_DEPTH, EA SNOW_DEPTH_EQUIVALENT | REAL | LENGTH | ATMOSPHERE | UD |  |

Response:	Accepted.

|  | Where | Table 6.22 – EDCS Attributes whose label begins with S and Table D.20 – Eas whose label begins with S |
| --- | --- | --- |
|  | Problem | The new attribute is needed specify the depth of snow fall during a specified time period. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0244 | EA |  | A | THUNDERSTORM_PRESENT | Flag indicating if a thunderstorm is present. | EA THUNDERSTORM_INTENSITY, EA THUNDERSTORM_PROBABILITY | LOGICAL |  | ATMOSPHERE | UD |  |

Response:	Accepted.

|  | Where | #### Table 6.23 – EDCS Attributes whose label begins with T and Table D.21 – Eas whose label begins with T |
| --- | --- | --- |
|  | Problem | The new attribute is needed to indicate if thunderstorm is present or not. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0245 | EA | HIGH_CLOUD_BASE_ALTITUDE | RD |  | The altitude of the base of a high-level cloud layer, relative to a vertical reference datum; the base of a cloud is defined as the lowest level in which the type of obscuration changes from that corresponding to clear air or haze to that corresponding to water droplets or ice crystals. High-level clouds are generally composed of ice crystals and are of the cirrus, cirrocumulus, and cirrostratus types. High clouds nominally occur within the following altitude ranges: from 3 040 to 7 620 metres (10 000 to 25 000 feet) in the polar regions, between 5 030 to 13 720 metres (16 500 to 45 000 feet) in temperate regions, and between 6 100 to 18 290 metres (20 000 to 60 000 feet) in tropical regions. |  |  |  |  | IR | CATLAS, MINSTR |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| SEDRIS_T0246 | EA | HIGH_CLOUD_COVERAGE | RD |  | The portion of the sky dome covered by all high-level clouds, as observed from the ground. High clouds nominally occur within the following altitude ranges: from 3 040 to 7 620 metres (10 000 to 25 000 feet) in the polar regions, between 5 030 to 13 720 metres (16 500 to 45 000 feet) in temperate regions, and between 6 100 to 18 290 metres (20 000 to 60 000 feet) in tropical regions. |  |  |  |  | IR | CATLAS, MINSTR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T0247 | EA | HIGH_CLOUD_COVERAGE_CLIM | RD |  | The quantity of historical occurrences of high cloud coverage, as observed by a ground observer. High clouds nominally occur within the following altitude ranges: from 3 040 to 7 620 metres (10 000 to 25 000 feet) in the polar regions, between 5 030 to 13 720 metres (16 500 to 45 000 feet) in temperate regions, and between 6 100 to 18 290 metres (20 000 to 60 000 feet) in tropical regions. |  |  |  |  | IR | CATLAS, MINSTR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected (see US comment on this concept).

| SEDRIS_T0248 | EA | HIGH_CLOUD_TOP_ALTITUDE | RD |  | The altitude of the top of a high-level cloud layer, relative to a vertical reference datum. High clouds nominally occur within the following altitude ranges: from 3 040 to 7 620 metres (10 000 to 25 000 feet) in the polar regions, between 5 030 to 13 720 metres (16 500 to 45 000 feet) in temperate regions, and between 6 100 to 18 290 metres (20 000 to 60 000 feet) in tropical regions. |  |  |  |  | IR | CATLAS, GMET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| SEDRIS_T0249 | EA | HIGH_CLOUD_TYPE | RD |  | The type of high cloud. High clouds nominally occur within the following altitude ranges: from 3 040 to 7 620 metres (10 000 to 25 000 feet) in the polar regions, between 5 030 to 13 720 metres (16 500 to 45 000 feet) in temperate regions, and between 6 100 to 18 290 metres (20 000 to 60 000 feet) in tropical regions. |  |  |  |  | IR | CATLAS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | #### Table 6.11 – EDCS Attributes whose label begins with H |
| --- | --- |
|  | Modified definition to include height ranges for high clouds. Reference CATLAS = International Cloud Atlas, World Meteorological Organization Publication # 407 World Meteorological Organization Reference GMET = (see US Comments for full bibliographic information) Glossary of Meteorology, American Meteorological Society Reference MINSTR = Guide to Meteorological Instruments and Methods of Observations, Section 11 WMO Publication No. 8 World Meteorological Organization |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0250 | EA | HIGH_CLOUD_TYPE | RL | HIGH_CLOUD_TYPE_OBS |  |  |  |  |  |  |  |
| ###### SEDRIS_T0251 | EA | HIGH_CLOUD_TYPE | RD |  | The observed high cloud type. |  |  |  |  |  |  |

Response:	Accepted.

|  | #### Table 6.11 – EDCS Attributes whose label begins with H |
| --- | --- |
|  | Need to make it clear that this attribute is applicable to observations of high cloud types and to distinguish this attribute from the one added in SEDRIS T0252 that is more general and applicable to forecast information. Need to make corresponding change in Table 6.39. See related comments SEDRIS T0375. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0252 | EA |  | A | HIGH_CLOUD_TYPE_GENERAL | The classification of clouds, genera, that are associated with high clouds | EA_HIGH_CLOUD_TYPE | ENUMERATION |  | ATMOSPHERE | IR | CATLAS |

Response:	Accepted; revise the definition to “The genera of high clouds.”, and revise the Label to be “_GENERA”.

|  | #### Table 6.11 – EDCS Attributes whose label begins with H |
| --- | --- |
|  | This attribute is needed to define the basic high cloud types. The enumerations given in the SEDRIS T0253 to SEDRIS T0256 below need to be added in Table 6.39. Reference CATLAS = International Cloud Atlas, World Meteorological Organization Publication # 407 World Meteorological Organization |

| Comment Tag | Dic | EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0253 | EE | HIGH_CLOUD_TYPE_GENERAL |  | A | CIRRUS | Detached clouds in the form of white, delicate filaments or white patches or narrow bands. | NR | CATLAS |
| ###### SEDRIS_T0254 | EE | HIGH_CLOUD_TYPE_GENERAL |  | A | CIRROCUMULUS | Thin, white patch, sheet or layer of cloud without shading, composed of very small elements in the form of grains, ripples, etc., merged or separate, and more or less regularly arranged; most of the elements have an apparent width of less than one degree. | NR | CATLAS |
| ###### SEDRIS_T0255 | EE | HIGH_CLOUD_TYPE_GENERAL |  | A | CIRROSTATUS | Transparent, whitish cloud veil of fibrous (hair-like) or smooth appearance, tot | NR | CATLAS |
| - SEDRIS_T0256 | EE | HIGH_CLOUD_TYPE_GENERAL |  | A | NONE_PRESENT | No high clouds present | ND |  |

Response:	Accepted (note any label changes consistent with the response to SEDRIS_T0252).

|  | #### Table 6.39 – EDCS Attribute enumerant codes for attributes whose label begins with H |
| --- | --- |
|  | These enumerants need to be added for the new attribute added SEDRIS T0252. The enumerants need to be added in this order, i.e. code ascending in corresp0onding order, to simplify translation from other standard. Reference CATLAS = International Cloud Atlas, World Meteorological Organization Publication # 407 World Meteorological Organization |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0257 | EA | LOW_CLOUD_BASE_ALTITUDE | RD |  | The altitude of the base of a low-level cloud layer, relative to a vertical reference datum. Low-level clouds are composed of water and are of the stratus, stratocumulus, and fair weather cumulus types. The altitude of the bases of these clouds range from near surface to about 1 980 metres (6 500) feet in all regions. |  |  |  |  | IR | CATLAS, MINSTR |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| ###### SEDRIS_T0258 | EA | LOW_CLOUD_COVERAGE | RD |  | The fraction of the sky dome that is covered by all low level clouds, as observed from the ground. Low-level clouds are composed of water and are of the stratus, stratocumulus, and fair weather cumulus types. The altitude of the bases of these clouds range from near surface to about 1 980 metres (6 500) feet in all regions. |  |  |  |  | IR | CATLAS, MINSTR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| ###### SEDRIS_T0259 | EA | LOW_CLOUD_COVERAGE_CLIM | RD |  | The quantity of historical occurrences of low cloud coverage, as observed by a ground observer. Low-level clouds are composed of water and are of the stratus, stratocumulus, and fair weather cumulus types. The altitude of the bases of these clouds range from near surface to about 1 980 metres (6 500) feet in all regions. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected (see US comment on this concept).

| ###### SEDRIS_T0260 | EA | LOW_CLOUD_TOP_ALTITUDE | RD |  | The altitude of the top of a low-level cloud layer, relative to a vertical reference datum. Low-level clouds are composed of water and are of the stratus, stratocumulus, and fair weather cumulus types. The altitude of the bases of these clouds range from near surface to about 1 980 metres (6 500) feet in all regions. |  |  |  |  | IR | CATLAS, GMET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| SEDRIS_T0261 | EA | LOW_CLOUD_TYPE | RD |  | The type of low cloud. Low-level clouds are composed of water and are of the stratus, stratocumulus, and fair weather cumulus types. The altitude of the bases of these clouds range from near surface to about 1 980 metres (6 500) feet in the mid-latitudes. |  |  |  |  | IR | CATLAS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | #### Table 6.15 – EDCS Attributes whose label begins with L |
| --- | --- |
|  | Corrected definitions. Given height range applies to polar, temperate, and tropical regions. Reference CATLAS = International Cloud Atlas, World Meteorological Organization Publication # 407 World Meteorological Organization Reference GMET = (see US Comments for full bibliographic information) Glossary of Meteorology, American Meteorological Society Reference MINSTR = Guide to Meteorological Instruments and Methods of Observations, Section 11 WMO Publication No. 8 World Meteorological Organization |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0262 | EA | LOW_CLOUD_TYPE | RL | LOW_CLOUD_TYPE_OBS |  |  |  |  |  |  |  |
| ###### SEDRIS_T0263 | EA | LOW_CLOUD_TYPE | RD |  | The observed low cloud type |  |  |  |  |  |  |

Response:	Accepted.

| ###### SEDRIS_T0264 | EA | LOW_CLOUD_TYPE | AC |  |  | EA LOW_CLOUD_TYPE_GENERA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | #### Table 6.15 – EDCS Attributes whose label begins with L |
| --- | --- |
|  | Need to make it clear that this attribute is applicable to observations of low cloud types and to distinguish this attribute from the one added in SEDRIS T0265 that is more general and applicable to forecast information. Need to make corresponding change in Table 6.43. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0265 | EA |  | A | LOW_CLOUD_TYPE_GENERAL | The classification of clouds, genera, that are associated with low clouds | EA LOW_CLOUD_TYPE_OBS | ENUMERATION |  | ATMOSPHERE | IR | CATLAS |

Response:	Accepted; revise the definition to “The genera of low clouds.”, and revise the Label to be “_GENERA”.

|  | #### Table 6.15 – EDCS Attributes whose label begins with L |
| --- | --- |
|  | This attribute is needed to define the basic low cloud types. The enumerations given comments SEDRIS T0266 to SEDRIS T0270 need to be added in Table 6.43. |

| Comment Tag | Dic | EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0266 | EE | LOW_CLOUD_TYPE_GENERAL |  | A | STRATOCUMULUS | Gray or whitish, or both gray and whitish, patch, sheet or layer or cloud which almost always has dark parts, composed of tessellations, rounded masses, rolls, etc., which are non-fibrous (except for virga) and which may or may not be merged; most of the regularly arranged small elements have an apparent width of more than 5 degrees. | NR | CATLAS |
| ###### SEDRIS_T0267 | EE | LOW_CLOUD_TYPE_GENERAL |  | A | STRATUS | Generally gray cloud layer with a fairly uniform base, which may give drizzle, ice prisms, or snow grains. When sun is visible through the cloud, its outline is clearly discernible; does not produce halo phenomena except possibly, at very low temperatures. | NR | CATLAS |
| ###### SEDRIS_T0268 | EE | LOW_CLOUD_TYPE_GENERAL |  | A | CUMULUS | Detached clouds, generally dense and with sharp outlines, developing vertically in the form of rising mounds, domes or towers, of which the bulging upper part often resembles cauliflower; the sunlit parts of these clouds are mostly brilliant white; the base is relatively dark and nearly horizontal. | NR | CATLAS |
| ###### SEDRIS_T0269 | EE | LOW_CLOUD_TYPE_GENERAL |  | A | CUMULONIMBUS | Heavy and dense cloud, with considerable vertical extent, in the form of a mountain or huge towers; at least part of its upper portion is usually smooth, or fibrous or striated, and nearly always flatteded; this part often spreads out in the shape of an anvil or vast plume/. | NR | CATLAS |
| ###### SEDRIS_T0270 | EE | LOW_CLOUD_TYPE_GENERAL |  | A | NONE_PRESENT | No low clouds present | ND |  |

Response:	Accepted (note any label changes consistent with the response to SEDRIS_T0265).

|  | #### Table 6.43 – EDCS Attribute enumerant codes for attributes whose label begins with L |
| --- | --- |
|  | These enumerants need to be added for the new attribute added in SEDRIS T0265. The enumerants need to added in this order, i.e. code ascending in corresp0onding order, to simplify translation from other standard. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0271 | EA | MIDDLE_CLOUD_BASE_ALTITUDE | RD |  | The altitude of the base of a mid-level cloud layer, relative to a vertical reference datum. Middle clouds nominally occur within the following altitude ranges: from 1 980 to 3 960 metres (6 500 to 13 000 feet) in the polar regions, between 1 980 to 7 000 metres (6 500 to 23 000 feet) in temperate regions, and between 1 980 to 18 290 metres (6 500 to 7600 feet) in tropical regions. |  |  |  |  | IR | CATLAS & MINSTR |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| SEDRIS_T0272 | EA | MIDDLE_CLOUD_COVERAGE | RD |  | The fraction of the sky dome that is covered by all middle level clouds, as observed from the ground. Middle clouds nominally occur within the following altitude ranges: from 1 980 to 3 960 metres (6 500 to 13 000 feet) in the polar regions, between 1 980 to 7 000 metres (6 500 to 23 000 feet) in temperate regions, and between 1 980 to 18 290 metres (6 500 to 7600 feet) in tropical regions. |  |  |  |  | IR | CATLAS & MINSTR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T0273 | EA | MIDDLE_CLOUD_COVERAGE_CLIM | RD |  | The quantity of historical occurrences of middle cloud coverage, as observed by a ground observer. Middle clouds nominally occur within the following altitude ranges: from 1 980 to 3 960 metres (6 500 to 13 000 feet) in the polar regions, between 1 980 to 7 000 metres (6 500 to 23 000 feet) in temperate regions, and between 1 980 to 18 290 metres (6 500 to 7600 feet) in tropical regions. |  |  |  |  | IR | CATLAS & MINSTR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected (see US comment on this concept).

| SEDRIS_T0274 | EA | MIDDLE_CLOUD_TOP_ALTITUDE | RD |  | The altitude of the top of a mid-level cloud layer, relative to a vertical reference datum. Middle clouds nominally occur within the following altitude ranges: from 1 980 to 3 960 metres (6 500 to 13 000 feet) in the polar regions, between 1 980 to 7 000 metres (6 500 to 23 000 feet) in temperate regions, and between 1 980 to 18 290 metres (6 500 to 7600 feet) in tropical regions. |  |  |  |  | IR | CATLAS & GMET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted, however only the first sentence will be used from the revised definition.

| SEDRIS_T0275 | EA | MIDDLE_CLOUD_TYPE | RD |  | The type of middle level cloud. Middle clouds nominally occur within the following altitude ranges: from 1 980 to 3 960 metres (6 500 to 13 000 feet) in the polar regions, between 1 980 to 7 000 metres (6 500 to 23 000 feet) in temperate regions, and between 1 980 to 18 290 metres (6 500 to 7600 feet) in tropical regions. |  |  |  |  | IR | CATLAS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | #### Table 6.16 – EDCS Attributes whose label begins with M |
| --- | --- |
|  | Modified definition to include height ranges for middle clouds. The definition is derived from the indicated references. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0276 | EA | MIDDLE_CLOUD_TYPE | RL | MIDDLE_CLOUD_TYPE_OBS |  |  |  |  |  |  |  |
| ###### SEDRIS_T0277 | EA | MIDDLE_CLOUD_TYPE | RD |  | The observed middle cloud type. |  |  |  |  |  |  |

Response:	Accepted.

| ###### SEDRIS_T0278 | EA | MIDDLE_CLOUD_TYPE | AC |  |  | EA MIDDLE_CLOUD_TYPE_GENERA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | #### Table 6.16 – EDCS Attributes whose label begins with L |
| --- | --- |
|  | Need to make it clear that this attribute is applicable to observations of middle cloud types and to distinguish this attribute from the one added in SEDRIS T0256 that is more general and applicable to forecast information. Need to make corresponding change in Table 6.44. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0279 | EA |  | A | MIDDLE_CLOUD_TYPE_GENERAL | The classification of clouds, genera, that are associated with low clouds | EA MIDDLE_CLOUD_TYPE_OBS | ENUMERATION |  | ATMOSPHERE | IR | CATLAS |

Response:	Accepted; revise the definition to “The genera of middle clouds.”, and revise the Label to be “_GENERA”.

|  | #### Table 6.15 – EDCS Attributes whose label begins with L |
| --- | --- |
|  | This attribute is needed to define the basic low cloud types. The enumerants given in comments SEDRIS T0280 to SEDRIS T0283 need to be added in Table 6.44. |

| Comment Tag | Dic | EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0280 | EE | MIDDLE_CLOUD_TYPE_GENERAL |  | A | ALTOCUMULUS | White or gray, or both white and gray, patch, sheet or layer of cloud, generally with shading, composed of laminae, rounded masses, rolls, etc., which are sometimes partly fibrous or diffuse and which may or may not be merged; most of the regularly arranged small elements usually have an apparent width of between one and five degrees. | NR | CATLAS |
| ###### SEDRIS_T0281 | EE | MIDDLE_CLOUD_TYPE_GENERAL |  | A | ALTOSTRATUS | Grayish or bluish cloud sheet or layer or striated, fibrous or uniform appearance, totally or partly covering the sky, and having parts thin enough to reveal the sun at least vaguely, as though ground glass; does not show halo phenomena. | NR | CATLAS |
| ###### SEDRIS_T0282 | EE | MIDDLE_CLOUD_TYPE_GENERAL |  | A | NIMBOSTATUS | Gray cloud layer, often dark, the appearance of which is rendered diffuse by more or less continuously falling rain or snow, which in most cases reaches the ground; thick enough throughout to blot out the sun. | NR | CATLAS |
| ###### SEDRIS_T0283 | EE | MIDDLE_CLOUD_TYPE_GENERAL |  | A | NONE_PRESENT | No low clouds present | ND |  |

Response:	Accepted (note any label changes consistent with the response to SEDRIS_T0279).

|  | #### Table 6.43 – EDCS Attribute enumerant codes for attributes whose label begins with L |
| --- | --- |
|  | These enumerants need to be added for the new attribute added in SEDRIS T0279. The enumerants need to added in this order, i.e. code ascending in corresp0onding order, to simplify translation from other standard. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0284 | EA | OCEAN_SOUND_SPEED | RD |  | The speed of sound in the ocean at a specific time, geographic location, season, and depth. |  |  |  |  | ND |  |

Response:	Accepted.

|  | #### The current definition does not address the specific of the speed of sound. |
| --- | --- |
|  | Replace the definition with the one supplied. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0285 | EA | OBJECT_ID_BASE_NUMBER | RV |  |  |  | INDEX |  |  |  |  |
| SEDRIS_T0286 | EA | OBJECT_VARIANT | RV |  |  |  | INDEX |  |  |  |  |
| SEDRIS_T0287 | EA | SED_GROSS_BOT_LOSS_CLASS | RV |  |  |  | INDEX |  |  |  |  |

Response:	Accepted.

|  | #### Tables 6.18 & 6.22 |
| --- | --- |
|  | By definition, these values are used to assign identification numbers or external indexing, so the value type should be INDEX, not INTEGER |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0288 | EA | TRANSMISSION_LOSS | RD |  | Add: .. For acoustic transmission loss, the starting point is one metre from the acoustic center of the source. … e.g., the electromagnetic band, or frequency range for which the value applies |  |  |  |  |  |  |
| SEDRIS_T0289 | EA | UNDERLYING_MATERIAL_TYPE | RD |  | The type of material composition underlying a terrestrial surface. |  |  |  |  |  |  |
| SEDRIS_T0290 | EA | VOLUME_SCATSTRN_LAYER_FREQ | RD |  | The frequency at which volume layer scattering strength is reported |  |  |  |  |  |  |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0291 | EA | VISIBILITY_REPORT_TYPE | RL | OBS_VISIBILITY_REPORT_TYPE |  |  |  |  |  |  |  |

Response:	Accepted.

|  | #### Table 6.25 – EDCS Attributes whose label begins with V |
| --- | --- |
|  | Change the label as indicated and move to Table 6.18 and move corresponding enumerants from Table 6.53 to Table 6.48. As far as I can determine, this appears to be a US defined parameter, unless it is defined in a ICAO standard. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0292 | EA | YEAR_COMMON_ERA | RD |  | The time period as measured by the Gregorian calendar in whole years since the beginning of the Common Era. |  |  |  |  |  |  |

Response:	Accepted.

|  | #### Table 6.28 |
| --- | --- |
|  | Need change “in years” to “in whole years” to justify value type INTEGER |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0293 | EA | SCND_SRF_THERMAL_CONDITION | RD |  | The physical boundary condition at the secondary surface, generally the top, front, or outer side of a material layer or object, accounting for an object’s interface with the environment at that surface. |  |  |  |  |  |  |

Response:	Accepted.

|  | #### Table 6.22 – EDCS Attributes whose label begins with S |
| --- | --- |
|  | This was the definition for PRI_SRF_THERMAL_CONDITION, change primary surface to secondary surface. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0294 | EA | WIND_SPECCLS_IDENTIFIER | RR |  |  |  |  |  |  | IR | OAML, ???? |

Response:	The editors will investigate the Reference.

|  | #### Table 6.22 – EDCS Attributes whose label begins with W |
| --- | --- |
|  | Type was UD |

| Comment Number | Dct | Current EA label | Current EE label | Type | New Label | Definition | Ref type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0295 | EE | AIRSPACE_ROUTE_TYPE | ALL (Delete all related enumerants. Replace EA with two Eas and distribute Ees accordingly.) | D |  |  |  |  |
| SEDRIS_T0296 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | ADVISORY | A route or area in which ATS provides advice and information for the safe conduct of flight and aircraft movement. | IR | IAN11 |
| SEDRIS_T0297 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | AIR_ROUTE | A corridor pre-established by ATS for the safe conduct of flight and aircraft movement. | IR | IAN11 |
| SEDRIS_T0298 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | AIRWAY | A control area or portion thereof established in the form of a corridor equipped with radio navigation aids. | IR | IAN11 |
| SEDRIS_T0299 | EE | AIRSPACE_ROUTE_TYPE_RNP |  | A | AREA_NAVIGATION | A method of navigation that permits aircraft operation on any desired course within the coverage of station-referenced navigation signals or within the limits of a self-contained system capability. Assigned by ATS upon pilot request. | IR | IAN11 |
| SEDRIS_T0300 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | ATLANTIC | Airspace type over the Atlantic Ocean which requires specific aircraft filing and navigation procedures. | IR | IAN11 |
| SEDRIS_T0301 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | BAHAMAS | Airspace type over the Bahamas which requires specific aircraft filing and navigation procedures. | IR | IAN11 |
| SEDRIS_T0302 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | CANADIAN_CONTROL | Airspace type over Canada (the Canadian Control area) which requires specific aircraft filing and navigation procedures | IR | IAN11 |
| SEDRIS_T0303 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | CORRIDOR | A corridor, or route, between two navigational aids which includes the centerline of the corridor and the airspace up to 7.5 km either side of the centerline and the altitudes between 350 and 7500 metres above mean sea level. | IR | IAN11 |
| SEDRIS_T0304 | EE | AIRSPACE_ROUTE_TYPE_RNP |  | A | DIRECT | Air route on a straight line between two identified NAVAIDs. | IR | IAN11 |
| SEDRIS_T0305 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | HELICOPTER | An air route for helicopter use. | IR | IAN11 |
| SEDRIS_T0306 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | HELICOPTER_NA_ONE_ENGINE | An air route which does not allow traversal by single-engine helicopters. | IR | IAN11 |
| SEDRIS_T0307 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | JET | High-altitude airway marked by longer-range transmitting VHF omnidirectional receivers (VORs) which enable it to be navigated by high-speed jet-powered aircraft. | IR | IAN11 |
| SEDRIS_T0308 | EE | AIRSPACE_ROUTE_TYPE_ATS | LOW-FLYING | A |  | Route assigned by ATS to a low-flying aircraft. | IR | IAN11 |
| SEDRIS_T0309 | EE | AIRSPACE_ROUTE_TYPE_ATS | MILITARY | A |  | Airspace reserved for military operations. | IR | IAN11 |
| SEDRIS_T0310 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | N_AMERICA | Airspace over the North American continent which requires specific aircraft filing and navigation procedures | IR | IAN11 |
| SEDRIS_T0311 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | OCEANIC | Airspace over the Pacific Ocean which requires specific aircraft filing and navigation procedures | IR | Convention on International Civil Aviation, Annex 11, Appendix 1 |
| SEDRIS_T0312 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | PURPLE_AIRWAY | Air route designated as a PURPLE AIRWAY by ATS. | IR | Convention on International Civil Aviation, Annex 11, Appendix 1 |
| SEDRIS_T0313 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | ROYAL_HELICOPTER | Air route reserved for use by a helicopter transporting royalty. | IR | Convention on International Civil Aviation, Annex 11, Appendix 1 |
| SEDRIS_T0314 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | ROYAL_LOW_LEVEL | Air route reserved for low-level flying by an aircraft transporting royalty. | IR | Convention on International Civil Aviation, Annex 11, Appendix 1 |
| SEDRIS_T0315 | EE | AIRSPACE_ROUTE_TYPE_ATS |  | A | REFUEL | A route used when aircraft are being refueled in air. | ND |  |
| SEDRIS_T0316 | EE | AIRSPACE_ROUTE_TYPE_RNP |  | A | TACAN | Air route used by an aircraft, normally military, possessing TACAN with DME but no VOR navigational system capability. | IR | Convention on International Civil Aviation, Annex 11, Appendix 1 |

Response:	Accepted in principle; AI 08-35: the editors will review in coordination with ICAO sources.

|  | Where | Table 6.32 |
| --- | --- | --- |
|  | Problem | Action Item 3: This attribute is being rebuilt: AIRSPACE_ROUTE_TYPE, with enumerants ATS and RNP. Added two attributes, AIRSPACE_ROUTE_TYPE_ATS and AIRSPACE_ROUTE_TYPE_RNP, with their enumerants. This cleans up the list of enumerants and clarifies the two different sets of route types, the set of ATS route types (the services ATS can provide for a given navigation route) and the set of RNP route types (the navigation plans of the aircraft intending to fly a particular route). |
|  | Solution | Add new enumerants for AIRSPACE_ROUTE_TYPE_ATS and AIRSPACE_ROUTE_TYPE_RNP. Reference IAN11= Convention on International Civil Aviation, Annex 11, Appendix 1 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0317 | EE | AIRSPACE_USE_LIMITATION | MINK_FARM | D |  |  |  |  |  |
| SEDRIS_T0318 | EE | ATS_USE_TYPE | ALERT | D |  |  |  |  |  |
| SEDRIS_T0319 | EE | ATS_USE_TYPE | BIRD_HAZARD | D |  |  |  |  |  |
| SEDRIS_T0320 | EE | ATS_USE_TYPE | CONTROL_FIRING | D |  |  |  |  |  |
| SEDRIS_T0321 | EE | ATS_USE_TYPE | DANGER | D |  |  |  |  |  |
| SEDRIS_T0322 | EE | ATS_USE_TYPE | DEFENSE | D |  |  |  |  |  |
| SEDRIS_T0323 | EE | ATS_USE_TYPE | HELICOPTER_PROTECT | D |  |  |  |  |  |
| SEDRIS_T0324 | EE | ATS_USE_TYPE | LOW_FLY | D |  |  |  |  |  |
| SEDRIS_T0325 | EE | ATS_USE_TYPE | MILITARY_FLYING | D |  |  |  |  |  |
| SEDRIS_T0326 | EE | ATS_USE_TYPE | MILITARY_OPERATIONS | D |  |  |  |  |  |
| SEDRIS_T0327 | EE | ATS_USE_TYPE | NON_FREE_FLYING | D |  |  |  |  |  |
| SEDRIS_T0328 | EE | ATS_USE_TYPE | OPERATING | D |  |  |  |  |  |
| SEDRIS_T0329 | EE | ATS_USE_TYPE | PARACHUTE_JUMP | D |  |  |  |  |  |
| SEDRIS_T0340 | EE | ATS_USE_TYPE | PROHIBITED | D |  |  |  |  |  |
| SEDRIS_T0341 | EE | ATS_USE_TYPE | RESTRICTED | D |  |  |  |  |  |
| SEDRIS_T0342 | EE | ATS_USE_TYPE | SPARSELY_SETTLED | D |  |  |  |  |  |
| SEDRIS_T0343 | EE | ATS_USE_TYPE | TEMP_RESERVE | D |  |  |  |  |  |
| SEDRIS_T0344 | EE | ATS_USE_TYPE | WARNING_AREA | D |  |  |  |  |  |

Response:	Accepted in principle; AI 08-35: the editors will review in coordination with ICAO sources.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Several enumerants of ATS_USE_TYPE are redundant with AIRSPACE_USE_LIMITATION’s enumerants. MINK_FARM seems out of place. |
|  | Solution | Either remove the redundant enumerants or connect them as related concepts with their counterparts. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0345 | EE | AEROSOL_TYPE |  | A | SEA_SALT | Salt particles suspended in the air due to the evaporation of sea spray | ND |  |
| SEDRIS_T0346 | EE | AEROSOL_TYPE |  | A | VOLCANIC_DUST | Dust particales produced by a volcanic eruption | ND |  |
| SEDRIS_T0347 | EE | AEROSOL_TYPE |  | A | SULFATES | Sulfates derived from either biogenic gases or SO2 | ND |  |
| ###### SEDRIS_T0348 | EE | AEROSOL_TYPE |  | A | NITRATES | Nitrates derived from NOX | ND |  |
| ###### SEDRIS_T0349 | EE | AEROSOL_TYPE |  | A | SOOT | Soot | ND |  |
| ###### SEDRIS_T0360 | EE | AEROSOL_TYPE |  | A | MARINE | Aerosols that are generated in the marine environment by processes such as breaking waves, and sea spray. | ND |  |
| ###### SEDRIS_T0361 | EE | AEROSOL_TYPE | NO_AEROSOL | RD |  | No aerosols are present. | ND |  |

Response:	Accepted.

|  | #### Table 6.32 – EDCS Attribute enumerant codes for attributes whose label begins with A |
| --- | --- |
|  | These are additional aerosol types that need to be included, and the definition and reference type for NO_AEROSOL needs to be added. |

| Comment Tag | Dic | Current EA Label | EE Label in “natural” order | Old ordering (by EEC) |
| --- | --- | --- | --- | --- |
| ###### SEDRIS_T0362 | EE | ATM_PRESSURE_CHANGE_QUALITY | NO_CHECK | 12 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | GOOD | 11 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | SUSPECT | 13 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_ERROR | 16 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_ERR_REPLACE_GOOD | 14 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_ERR_REPLACE_SUSPECT | 15 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_MISS_PROB_CORRUPT | 17 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_MISS_REPLACE_SUSPECT | 18 |
|  | EE | ATM_PRESSURE_CHANGE_QUALITY | VALUE_MISSING | 19 |
| ###### SEDRIS_T0363 | EE | ATM_PRESSURE_CHANGE_TND | INCREASE_THEN_DECREASE | 16 |
|  | EE | ATM_PRESSURE_CHANGE_TND | INCREASE_THEN_STEADY | 17 |
|  | EE | ATM_PRESSURE_CHANGE_TND | INCREASING | 18 |
|  | EE | ATM_PRESSURE_CHANGE_TND | CHANGE_TO_INCREASE | 12 |
|  | EE | ATM_PRESSURE_CHANGE_TND | STEADY_AND_PRESS_EQUAL | 19 |
|  | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_THEN_INCREASE | 14 |
|  | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_THEN_STEADY | 15 |
|  | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_AND_PRESS_LESS | 13 |
|  | EE | ATM_PRESSURE_CHANGE_TND | CHANGE_TO_DECREASE | 11 |
| ###### SEDRIS_T0364 | EE | ATM_PRESSURE_QUALITY | NO_CHECK | 12 |
|  | EE | ATM_PRESSURE_QUALITY | GOOD | 11 |
|  | EE | ATM_PRESSURE_QUALITY | SUSPECT | 13 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_ERROR | 16 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_ERR_REPLACE_GOOD | 14 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_ERR_REPLACE_SUSPECT | 15 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_MISS_PROB_CORRUPT | 17 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_MISS_REPLACE_SUSPECT | 18 |
|  | EE | ATM_PRESSURE_QUALITY | VALUE_MISSING | 19 |
| ###### SEDRIS_T0365 | EE | BELOW_STATION_CLD_CVRG | NONE_PRESENT | 19 |
|  | EE | BELOW_STATION_CLD_CVRG | ‘lt1_OKTA | 11 |
|  | EE | BELOW_STATION_CLD_CVRG | 2_OKTA | 12 |
|  | EE | BELOW_STATION_CLD_CVRG | 3_OKTA | 13 |
|  | EE | BELOW_STATION_CLD_CVRG | 4_OKTA | 14 |
|  | EE | BELOW_STATION_CLD_CVRG | 5_OKTA | 15 |
|  | EE | BELOW_STATION_CLD_CVRG | 6_OKTA | 16 |
|  | EE | BELOW_STATION_CLD_CVRG | 7_OKTA | 17 |
|  | EE | BELOW_STATION_CLD_CVRG | 8_OKTA | 18 |
|  | EE | BELOW_STATION_CLD_CVRG | SKY_OBSCURED | 11 |
|  | EE | BELOW_STATION_CLD_CVRG | PARTIAL_OBSCURATION | 12 |
| ###### SEDRIS_T0366 | EE | BELOW_STATION_CLD_TOP_TYP | FRAGMENTED | 17 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | CONTINUOUS_FLAT_TOPS | 14 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | SMALL_BREAKS_FLAT_TOPS | 21 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | LARGE_BREAKS_FLAT_TOPS | 18 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | CONTINUOUS_UNDULATING_TOPS | 16 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | SMALL_BREAKS_UNDULATE_TOPS | 22 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | LARGE_BREAKS_UNDULATE_TOPS | 19 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | CONTINUOUS_TOWERING_TOPS | 15 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | WAVE_GROUPS_WITH_TOWERING | 23 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | MULTIPLE_LAYERS_AND_LEVELS | 20 |
|  | EE | BELOW_STATION_CLD_TOP_TYP | CLOUD_NOT_VISIBLE | 13 |
| ###### SEDRIS_T0367 | EE | BELOW_STATION_CLD_TYPE | CIRRUS | 15 |
|  | EE | BELOW_STATION_CLD_TYPE | CIRROCUMULUS | 13 |
|  | EE | BELOW_STATION_CLD_TYPE | CIRROSTRATUS | 14 |
|  | EE | BELOW_STATION_CLD_TYPE | ALTOCUMULUS | 11 |
|  | EE | BELOW_STATION_CLD_TYPE | ALTOSTRATUS | 12 |
|  | EE | BELOW_STATION_CLD_TYPE | NIMBOSTRATUS | 18 |
|  | EE | BELOW_STATION_CLD_TYPE | STRATOCUMULUS | 22 |
|  | EE | BELOW_STATION_CLD_TYPE | STRATUS | 23 |
|  | EE | BELOW_STATION_CLD_TYPE | CUMULUS | 17 |
|  | EE | BELOW_STATION_CLD_TYPE | CUMULONIMBUS | 16 |
|  | EE | BELOW_STATION_CLD_TYPE | NO_CLOUDS_OR_NOT_VISIBLE | 20 |
|  | EE | BELOW_STATION_CLD_TYPE | NOT_VISIBLE | 21 |
|  | EE | BELOW_STATION_CLD_TYPE | NO_CLOUDS | 15 |
| ###### SEDRIS_T0368 | EE | CLOUD_TYPE | CIRRUS | 15 |
|  | EE | CLOUD_TYPE | CIRROCUMULUS | 13 |
|  | EE | CLOUD_TYPE | CIRROSTRATUS | 14 |
|  | EE | CLOUD_TYPE | ALTOCUMULUS | 11 |
|  | EE | CLOUD_TYPE | ALTOSTRATUS | 12 |
|  | EE | CLOUD_TYPE | NIMBOSTRATUS | 19 |
|  | EE | CLOUD_TYPE | STRATOCUMULUS | 20 |
|  | EE | CLOUD_TYPE | STRATUS | 21 |
|  | EE | CLOUD_TYPE | CUMULUS | 18 |
|  | EE | CLOUD_TYPE | CUMULONIMBUS | 17 |
|  | EE | CLOUD_TYPE | NO_CLOUDS_OR_NOT_VISIBLE | 16 |
| ###### SEDRIS_T0369 | EE | DUEPOINT_QUALITY | NO_CHECK | 12 |
|  | EE | DUEPOINT_QUALITY | GOOD | 11 |
|  | EE | DUEPOINT_QUALITY | SUSPECT | 13 |
|  | EE | DUEPOINT_QUALITY | VALUE_ERROR | 16 |
|  | EE | DUEPOINT_QUALITY | VALUE_ERR_REPLACE_GOOD | 14 |
|  | EE | DUEPOINT_QUALITY | VALUE_ERR_REPLACE_SUSPECT | 15 |
|  | EE | DUEPOINT_QUALITY | VALUE_MISS_PROB_CORRUPT | 17 |
|  | EE | DUEPOINT_QUALITY | VALUE_MISS_REPLACE_SUSPECT | 18 |
|  | EE | DUEPOINT_QUALITY | VALUE_MISSING | 19 |
| ###### SEDRIS_T0370 | EE | HIGH_CLOUD_TYPE | NO_HIGH_CLOUDS | 20 |
|  | EE | HIGH_CLOUD_TYPE | CIRRUS_FIBRATUS | 14 |
|  | EE | HIGH_CLOUD_TYPE | PATCHY_SPISSATUS | 21 |
|  | EE | HIGH_CLOUD_TYPE | CIRRUS_SPISSATUS | 17 |
|  | EE | HIGH_CLOUD_TYPE | CIRRUS_UNCINUS | 18 |
|  | EE | HIGH_CLOUD_TYPE | CIRRUS_le45_DEG | 16 |
|  | EE | HIGH_CLOUD_TYPE | CIRRUS_gt45_DEG | 15 |
|  | EE | HIGH_CLOUD_TYPE | CIRROSTRATUS_COVERING | 12 |
|  | EE | HIGH_CLOUD_TYPE | CIRROSTRATUS_PARTIAL | 13 |
|  | EE | HIGH_CLOUD_TYPE | CIRROCUMULUS | 11 |
|  | EE | HIGH_CLOUD_TYPE | HIGH_CLOUDS_INVISIBLE | 19 |
| ###### SEDRIS_T0371 | EE | LOW_CLOUD_TYPE | NO_LOW_CLOUDS | 17 |
|  | EE | LOW_CLOUD_TYPE | CUMULUS_HUMULIS | 14 |
|  | EE | LOW_CLOUD_TYPE | CUMULUS_MEDIOCRIS | 15 |
|  | EE | LOW_CLOUD_TYPE | CUMULONIMBUS_CALVUS | 11 |
|  | EE | LOW_CLOUD_TYPE | STRATOCUMULUS_CUMULOGEN | 18 |
|  | EE | LOW_CLOUD_TYPE | STRATOCUMULUS_OTHER | 19 |
|  | EE | LOW_CLOUD_TYPE | STRATUS_NEBULOSUS | 21 |
|  | EE | LOW_CLOUD_TYPE | STRATUS_FRACTUS | 20 |
|  | EE | LOW_CLOUD_TYPE | CUMULUS_AND_STRATO | 13 |
|  | EE | LOW_CLOUD_TYPE | CUMULONIMBUS_CAPILLATUS | 12 |
|  | EE | LOW_CLOUD_TYPE | LOW_CLOUDS_INVISIBLE | 16 |
| ###### SEDRIS_T0372 | EE | LOWEST_CLOUD_COVERAGE | NONE_PRESENT | 19 |
|  | EE | LOWEST_CLOUD_COVERAGE | lt1_OKTA | 11 |
|  | EE | LOWEST_CLOUD_COVERAGE | 2_OKTA | 12 |
|  | EE | LOWEST_CLOUD_COVERAGE | 3_OKTA | 13 |
|  | EE | LOWEST_CLOUD_COVERAGE | 4_OKTA | 14 |
|  | EE | LOWEST_CLOUD_COVERAGE | 5_OKTA | 15 |
|  | EE | LOWEST_CLOUD_COVERAGE | 6_OKTA | 16 |
|  | EE | LOWEST_CLOUD_COVERAGE | 7_OKTA | 17 |
|  | EE | LOWEST_CLOUD_COVERAGE | 8_OKTA | 18 |
|  | EE | LOWEST_CLOUD_COVERAGE | PARTIAL_OBSCURATION | 20 |
|  | EE | LOWEST_CLOUD_COVERAGE | SKY_OBSCURED | 21 |
| ###### SEDRIS_T0373 | EE | MIDDLE_CLOUD_TYPE | NO_MIDDLE_CLOUDS | 20 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOSTRATUS_TRANSLUCIDUS | 16 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOSTRATUS_OPACUS | 15 |
|  | EE | MIDDLE_CLOUD_TYPE | SINGLE_LEVEL_ALTOCUMULUS | 21 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOCUMULUS_PATCHES | 13 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOCUMULUS_TRANSLUCIDUS | 14 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOCUMULUS_CUMULOGEN | 12 |
|  | EE | MIDDLE_CLOUD_TYPE | MULTILAYER_ALTOCUMULUS | 19 |
|  | EE | MIDDLE_CLOUD_TYPE | ALTOCUMULUS_CASTELLANUS | 11 |
|  | EE | MIDDLE_CLOUD_TYPE | CHAOTIC_ALTOCUMULUS | 17 |
|  | EE | MIDDLE_CLOUD_TYPE | MIDDLE_CLOUDS_INVISIBLE | 18 |
| ###### SEDRIS_T0374 | EE | TOTAL_CLOUD_COVER_CODED | NONE_PRESENT | 19 |
|  | EE | TOTAL_CLOUD_COVER_CODED | ‘lt1_OKTA | 11 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 2_OKTA | 12 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 3_OKTA | 13 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 4_OKTA | 14 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 5_OKTA | 15 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 6_OKTA | 16 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 7_OKTA | 17 |
|  | EE | TOTAL_CLOUD_COVER_CODED | 8_OKTA | 18 |
|  | EE | TOTAL_CLOUD_COVER_CODED | SKY_OBSCURED | 21 |
|  | EE | TOTAL_CLOUD_COVER_CODED | PARTIAL_OBSCURATION | 20 |

Response:	Accepted.

|  | #### Table 6.xx – EDCS Attribute enumerant codes for attributes whose label begins with A, B, C, with H, L, M, T |
| --- | --- |
|  | This enumeration is based on enumerations from other standards. Reorder and recode the enumerations. It simplifies the conversion if the order of the enumerants if maintained. |

| Comment Tag | Dictionary | Current EC Label | Change Type | Label | # Definition | Related Concepts | Groups | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0375 | EC |  | A | ATM_OBS_TRSRF_CHARSET | An evaluation of the state of the atmosphere as observed from a point at the surface of the earth | EC ATMOSPHERE, EC ATM_OBS_CHARSET | ATMOSPHERE | UD |  |

|  | #### Where Table C.2 – Table 5.4 – EDCS Classifications whose label begins with A and ECs whose label begins with A |
| --- | --- |
|  | Problem: Need to add this classification to make the set of atmospheric observations classifications more complete. |

Response:	Accepted, pending resolution of the general issue identified by comment SEDRIS_G005: AI 08-02.

| Comment Tag | Dictionary | Current EA Label | Change Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Groups | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0376 | EA |  | A | FOG_PRESENT | Flag indicating if fog is present. | EA FOG_COVERAGE, EA FOG_PROBABILITY | LOGICAL |  | ATMOSPHERE | UD |  |

Response:	Duplicate of SEDRIS_T0242.

|  | Table 6.9 – EDCS Attributes whose label begins with F and Table D.7 – Eas whose label begins with F |
| --- | --- |
|  | The new attribute is needed to indicate if fog is present or not. |

| Comment Tag | Dictionary | Current EA Label | Change Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Groups | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0378 | EA |  | A | SNOW_ACCUMULATION | The depth of snow that has accumulated over a specified time interval. | EA SNOW_DEPTH, EA SNOW_DEPTH_EQUIVALENT | REAL | LENGTH | ATMOSPHERE | UD |  |

Response:	Duplicate of SEDRIS_T0243.

|  | #### Table 6.22 – EDCS Attributes whose label begins with S and Table D.20 – Eas whose label begins with S |
| --- | --- |
|  | The new attribute is needed specify the depth of snow fall during a specified time period. |

| Comment Tag | Dictionary | Current EA Label | Change Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Groups | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0379 | EA |  | A | THUNDERSTORM_PRESENT | Flag indicating if a thunderstorm is present. | EA THUNDERSTORM_INTENSITY, EA THUNDERSTORM_PROBABILITY | LOGICAL |  | ATMOSPHERE | UD |  |

Response:	Duplicate of SEDRIS_T0244.

|  | #### Table 6.23 – EDCS Attributes whose label begins with T and Table D.21 – Eas whose label begins with T |
| --- | --- |
|  | The new attribute is needed to indicate if thunderstorm is present or not. |

SEDRIS T0380 through SEDRIS T0498 Withdrawn.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0499 | EE | ATM_PRESSURE_CHANGE_TND | INCREASE_THEN_STEADY | RD |  | Increasing, then steady; or Increasing, then Increasing more slowly. Atmospheric pressure now higher than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0500 | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_THEN_STEADY | RD |  | Decreasing or steady, then increasing; or increasing, then increasing more rapidly Atmospheric pressure now higher than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0501 | EE | ATM_PRESSURE_CHANGE_TND | INCREASE_THEN_DECREASE | RD |  | Increasing, then decreasing; atmospheric pressure the same or higher than 3 hours ago | NR | WMOCODE210 |
| SEDRIS_T0502 | EE | ATM_PRESSURE_CHANGE_TND | CHANGE_TO_DECREASE | RD |  | Steady or increasing, then decreasing; or decreasing, then decreasing more rapidly. Atmospheric pressure now lower than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0503 | EE | ATM_PRESSURE_CHANGE_TND | CHANGE_TO_INCREASE | RD |  | Decreasing or steady, then increasing; or increasing, then increasing more rapidly Atmospheric pressure now higher than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0504 | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_AND_PRESS_LESS | RD |  | Decreasing (steadily or unsteadily). Atmospheric pressure now lower than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0505 | EE | ATM_PRESSURE_CHANGE_TND | DECREASE_THEN_INCREASE | RD |  | Decreasing, then increasing; atmospheric pressure the same or lower than 3 hours ago | NR | WMOCODE210 |
| SEDRIS_T0506 | EE | ATM_PRESSURE_CHANGE_TND | INCREASING | RD |  | Increasing (steadily or unsteadily) Atmospheric pressure now higher than 3 hours ago. | NR | WMOCODE210 |
| SEDRIS_T0507 | EE | ATM_PRESSURE_CHANGE_TND | STEADY_AND_PRESS_EQUAL | RD |  | Steady; atmospheric pressure the same as 3 hours ago | NR | WMOCODE210 |

Response:	Accepted.

|  | #### Table 6.32 – EDCS Attribute enumerant codes for attributes whose label begins with A |
| --- | --- |
|  | The current definitions are either incorrect or did not match the definitions from the specified reference. WMOCODE210 = World Meteorological Organization Manual on Codes, Volume 2, International Codes (WMO-No. 306) Code Table 0-10-63 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0508 | EE | BELOW_STATION_CLD_CVRG | All enumerants | RR |  |  | QR | WMOCODE1 |
| SEDRIS_T0509 | EE | BELOW_STATION_CLD_TOP_TYP | All enumerants | RR |  |  | QR | WMOCODE1 |

Response:	Accepted.

|  | #### TABLE 6.33 EDCS Attribute enumerant codes for attributes whose label begins with B |
| --- | --- |
|  | The above enumerants are taken from the given reference. WMOCODE1 = World Meteorological Organization Manual on Codes, Volume 1, International Codes (WMO-No. 306) Part B, descriptor 0-20-011 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0510 | EE | BELOW_STATION_CLD_CVRG |  | A | SCATTERED | Between 3/8 and 4/8 coverage inclusive | IR | FMH1_952 & WMOCODE1 |
| SEDRIS_T0511 | EE | BELOW_STATION_CLD_CVRG |  | A | BROKEN | Between 5/8 and 7/8 coverage inclusive | IR | FMH1_952 & WMOCODE1 |
| SEDRIS_T0512 | EE | BELOW_STATION_CLD_CVRG |  | A | FEW | Between 1/8 and 2/8 coverage inclusive | IR | FMH1_952 & WMOCODE1 |
| SEDRIS_T0513 | EE | BELOW_STATION_CLD_CVRG |  | A | INDISCERNABLE | Cloud cover is indiscernible for reasons other than fog or other meteorological phenomena, or observations not made | IR | WMOCODE1 |

Response:	Accepted.

|  | #### Table 6.33 – EDCS Attribute enumerant codes for attributes whose label begins with B |
| --- | --- |
|  | These additional enumerants are required to complete the enumerations list and make it compatible with the WMO enumeration. These enumerants should be added to end of the BELOW_STATION_CLD_CVRG enumerant list as reordered by comments SEDRIS T0365 in the order given. Reference WMOCODE1 = World Meteorological Organization Manual on Codes, Volume 1, International Codes (WMO-No. 306) Part B, descriptor 0-20-011 Reference FMH1_952= Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 9.5.2 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | # Definition | Related Concepts | Value Type | # EQ Label | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0514 | EA | CLOUD_TYPE | RL | CLOUD_TYPE_SKY_COVER |  |  |  |  |  |  |  |

Response:	Accepted.

|  | #### Table 6.6 – EDCS Attributes whose label begins with C |
| --- | --- |
|  | The label needs to be changed to clarify that the enumerations apply to observed sky cover, to distinguish it from similar attributes, and the new label is more consistent with the definition. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0515 | EE | LIGHTNING_POLARITY | NEGATIVE | RD |  | The lightning stroke has a negative polarity. |  |  |
| ###### SEDRIS_T0516 | EE | LIGHTNING_POLARITY | POSITIVE | RD |  | The lightning stroke has a positive polarity. |  |  |

Response:	Rejected; Rob Cox will research: AI 08-32.

|  | #### Table 6.43 – EDCS Attribute enumerant codes for attributes whose label begins with L |
| --- | --- |
|  | No definitions are currently given. Add the above definitions. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0517 | EE | PRECIPITATION_INTENSITY | LIGHT | RD |  | For rain and ice pellets: up to 2,54 millimetres (0,10 inches) per hour; Or up to 0,254 millimetres (0,01 inches) in 6 minutes. For snow: Visibility > ½ mile. | QR | FMH1_841 |
| ###### SEDRIS_T0518 | EE | PRECIPITATION_INTENSITY | MODERATE | RD |  | For rain and ice pellets: more than 2,54 millimetres (0,10 inches) up to 7,62 millimetres (0,30 inches) per hour; Or more than 0,254 millimetres (0,01 inches) up to 0,762 millimetres (0,03 inches) in 6 minutes. For snow: Visibility > ¼ mile but  ½ mile. | QR | FMH1_841 |
| ###### SEDRIS_T0519 | EE | PRECIPITATION_INTENSITY | HEAVY | RD |  | For rain and ice pellets: more than 7,62 millimetres (0,30 inches) per hour; more than 0,762 millimetres (0,03 inches) in 6 minutes. For snow: Visibility  ¼ mile. | QR | FMH1_841 |

Response:	Accepted; the editors will revise as necessary.

|  | #### Table 6.47 EDCS Attribute enumerant codes for attributes whose label begins with P |
| --- | --- |
|  | The current definitions apply only to rain and ice pellets. Expand the definitions to include snow. The definitions are combinations of the rain & ice pellets and snow definitions from the indicated reference. Reference FMH1_841= Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 8.4.1 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0520 | EE | SNOW_MEASUREMENT_CONDITION | ALL ENUMERANTS | RR |  |  | NR | WMOCODE |
| SEDRIS_T0521 | EE | SNOW_MEASUREMENT_CONDITION |  | A | NONE | NO SNOW PRESENT | NR | WMOCODE |

Response:	Accepted.

|  | #### Table 6.50 EDCS Attribute enumerant codes for attributes whose label begins with S |
| --- | --- |
|  | These enumerations are defined in the referenced publication. The NONE enumerant is missing and needs to be added. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0522 | EE | TOTAL_CLOUD_COVER_CODED | ALL ENUMERANTS | RR |  |  | QR | WMOCODE1 |
| ###### SEDRIS_T0523 | EE | TOTAL_CLOUD_COVER_CODED |  | A | SCATTERED | Between 3/8 and 4/8 coverage inclusive | QR | FMH1_952 |
| ###### SEDRIS_T0524 | EE | TOTAL_CLOUD_COVER_CODED |  | A | BROKEN | Between 5/8 and 7/8 coverage inclusive | QR | FMH1_952 |
| ###### SEDRIS_T0525 | EE | TOTAL_CLOUD_COVER_CODED |  | A | FEW | Between 1/8 and 2/8 coverage inclusive | QR | FMH1_952 |
| ###### SEDRIS_T0526 | EE | TOTAL_CLOUD_COVER_CODED |  | A | INDISCERNABLE | Cloud cover is indiscernible for reasons other than fog or other meteorological phenomena, or observations not made | IR | WMOCODE1 |

Response:	Accepted.

|  | #### Table 6.51 – EDCS Attribute enumerant codes for attributes whose label begins with T |
| --- | --- |
|  | These additional enumerants are required to complete the enumerations list and make it compatible with the WMO enumeration. These enumerants should be added to end of the TOTAL_CLOUD_COVER_CODED enumerant list as reordered by comments SEDRIS T0374 in the order given. Reference FMH1_952= Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 9.5.2 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce Reference WMOCODE1 = World Meteorological Organization Manual on Codes, Volume 1, International Codes (WMO-No. 306) Part B, descriptor 0-20-011 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0527 | EE | VISIBILITY_DIST_QUALITY | ERRONEOUS | RR |  |  | IR | NCDCM |
| ###### SEDRIS_T0528 | EE | VISIBILITY_DIST_QUALITY | GOOD | RR |  |  | IR | NCDCM |
| ###### SEDRIS_T0529 | EE | VISIBILITY_DIST_QUALITY | SUSPECT | RR |  |  | IR | NCDCM |

Response:	Accepted.

|  | #### Table E.23 – Ees for Eas whose label begins with V |
| --- | --- |
|  | These enumerations are defined in the referenced publication. Reference NCDCM = NATIONAL CLIMATIC DATA CENTER, DOCUMENTATIONMANUAL, HOURLY SURFACE AIRWAYS OBSERVATIONS (TD-3280) |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0530 | EE | VISIBILITY_DIST_QUALITY | NO_CHECK | RD |  | No check preformed | ND |  |

Response:	Accepted.

|  | #### Table 6.53 – EDCS Attribute enumerant codes for attributes whose label begins with V |
| --- | --- |
|  | No definition given, add this definition. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0531 | EE | VISIBILITY_REPORT_TYPE | MINIMUM | RD |  | Minimum observed visibility during reporting period. | QR | FMH1_6 |
| ###### SEDRIS_T0532 | EE | VISIBILITY_REPORT_TYPE | PREVAILING | RD |  | The visibility that is considered representative of visibility conditions at the station; the greatest distance that can be seen throughout at least half the horizon circle, not necessarly continuous. | IR | FMH1_6 |
| ###### SEDRIS_T0533 | EE | VISIBILITY_REPORT_TYPE | PREVAILING_VARIABLE_HIGH | RD |  | The maximum visibility reported if the prevailing visibility rapidly increases and decreases by ½ statute miles or more, during the time of observation, and the visibility is less than 3 miles. | IR | FMH1_6 |
| ###### SEDRIS_T0534 | EE | VISIBILITY_REPORT_TYPE | PREVAILING_VARIABLE_LOW | RD |  | The minimum visibility reported if the prevailing visibility rapidly increases and decreases by ½ statute miles or more, during the time of observation, and the visibility is less than 3 miles. | IR | FMH1_6 |
| ###### SEDRIS_T0535 | EE | VISIBILITY_REPORT_TYPE | SECTOR | RD |  | The visibility in a specific direction that represents at lease a 45 degree arc of the horizon circle. | IR | FMH1_6 |
| ###### SEDRIS_T0536 | EE | VISIBILITY_REPORT_TYPE | TOWER | RD |  | The prevailing visibility determined from airport traffic control tower (ATCT) at stations that also report surface visibility. | IR | FMH1_6 |
| ###### SEDRIS_T0537 | EE | VISIBILITY_REPORT_TYPE | TOWER_VARIABLE_HIGH | RD |  | The maximum visibility reported if the tower visibility rapidly increases and decreases by ½ statute miles or more, during the time of observation, and the visibility is less than 3 miles. | IR | FMH1_6 |
| ###### SEDRIS_T0538 | EE | VISIBILITY_REPORT_TYPE | TOWER_VARIABLE_LOW | RD |  | The minimum visibility reported if the tower visibility rapidly increases and decreases by ½ statute miles or more, during the time of observation, and the visibility is less than 3 miles. | IR | FMH1_6 |

Response:	Accepted.

|  | #### Table 6.53 – EDCS Attribute enumerant codes for attributes whose label begins with V |
| --- | --- |
|  | These enumerations are defined in the referenced publication. These enumerations need to be moved to Table 6.48 Reference FMH1_6= Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 6.0 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T0539 | EE | VISIBILITY_REPORT_TYPE |  | A | UNKNOWN | The visibility type is unknown. | ND |  |

Response:	Accepted.

|  | #### Table 6.53 – EDCS Attribute enumerant codes for attributes whose label begins with V |
| --- | --- |
|  | This is a valid enumerant as determined for the FMH1 and JMCDM. Should be moved to Table 6.48 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0540 | EE | WEATHER_MSRMNT_TECHNIQUE | AUTOMATED | RD |  | Measured using automated methods, e.g., processed from a sensor array. |  |  |
| SEDRIS_T0541 | EE | WEATHER_MSRMNT_TECHNIQUE | MANUAL | RD |  | Measured using manual methods, e.g., visual observation |  |  |

Response:	Accepted.

|  | #### Table 6.54 – EDCS Attribute enumerant codes for attributes whose label begins with W |
| --- | --- |
|  | The definitions in the current document were reversed. The above changes correct this. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0542 | EE | WIND_DIRECTION_QUALITY | ALL ENUMERANTS | RR |  |  | QR | NCDCM |
| SEDRIS_T0543 | EE | WIND_SPEED_QUALITY | ALL ENUMERANTS | RR |  |  | QR | NCDCM |

Response:	Accepted.

|  | #### Table 6.54 – EDCS Attribute enumerant codes for attributes whose label begins with W |
| --- | --- |
|  | The definition is defined in the specified reference. Reference NCDCM = NATIONAL CLIMATIC DATA CENTER, DOCUMENTATIONMANUAL, HOURLY SURFACE AIRWAYS OBSERVATIONS (TD-3280) |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0544 | EE | BEACH_PORTION | BACKSHORE | AC |  |  |  |  | EC_BACKSHORE_PRECISE |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0545 | EE | BEACH_PORTION | BACKSHORE | RD |  | The area of shore lying between the average hightide mark and the vegetation, affected by waves only during severe storms. |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle; the editors will consider the related concepts from IHO S-57 and revise as necessary.

| SEDRIS_T0546 | EE | BEACH_PORTION | NEARSHORE | AC |  |  |  |  | EC_NEARSHORE_PRECISE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0547 | EE | BEACH_PORTION | NEARSHORE | RD |  | The region of land extending from the backshore to the beginning of the offshore zone. |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle; the editors will consider the related concepts from IHO S-57 and revise as necessary.

| SEDRIS_T0548 | EE | BERTH_TYPE | WHARF | AC |  |  |  |  | EC_WHARF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0549 | EE | BERTH_TYPE | WHARF | RD |  | A structure built alongside or out into the water as a landing place for boats and ships, sometimes with a protective covering or enclosure. |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0550 | EE | BERTH_TYPE | QUAY | RD |  | A platform that runs along the edge of a port or harbour, where boats are loaded and unloaded. |  | ND |  |
| SEDRIS_T0551 | EE | BERTH_TYPE | PIER | RD |  | A platform built on stilts jutting out into a body of water, used as a boat dock, a place from which to fish, or as an entertainment center. |  | ND |  |

Response:	Accepted in principle; the editors will consider the related concepts from IHO S-57 and revise as necessary.

| SEDRIS_T0552 | EE | BERTH_TYPE | PIER | AC |  |  |  |  | EC_RECREATIONAL_PIER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Add missing definitions, related concepts. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0553 | EE | BOUNDARY_TYPE | ADMINISTRATIVE | AC |  |  |  |  | EC_ADMINISTRATIVE_BOUNDARY |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0554 | EE | BOUNDARY_TYPE | ADMINISTRATIVE | RD |  | A line of demarcation (boundary) between administratively controlled regions. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0555 | EE | BOUNDARY_TYPE | ARMISTICE_LINE | RD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0556 | EE | BOUNDARY_TYPE | CEASE_FIRE_LINE | D |  |  |  |  |  |

Response:	Accepted in principle; the editors will investigate and provide an appropriate definition.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | CEASE FIRE LINE is currently redundant with ARMISTICE LINE in this context. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0557 | EA | CLOUD_WATER_MIXING_RATIO | AC |  |  |  |  |  |  |  |  | EA_MIXING_RATIO |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | CLOUD_WATER_MIXING_RATIO is just a special case of MIXING_RATIO, but is not marked as a related concept. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0558 | EE | CHANGE_POINT_TYPE | BOUNDARIES | RL | BOUNDARY |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0559 | EE | CHANGE_POINT_TYPE | BOUNDARIES | AC |  |  |  |  | EC_BOUNDARY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0560 | EE | CHANGE_POINT_TYPE | BOUNDARIES | RD |  | A line of demarcation between two regions. |  | UD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0561 | EE | CHANGE_POINT_TYPE | HYDROGRAPHY | AC |  |  |  |  | EC_HYDROGRAPHY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0562 | EE | CHANGE_POINT_TYPE | HYDROGRAPHY | RD |  | A body of water or its adjoining shores, e.g. an aqueduct, canal, lake, or river. |  | UD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0563 | EE | CHANGE_POINT_TYPE | OBSTACLES | RL | OBSTACLE |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0564 | EE | CHANGE_POINT_TYPE | OBSTACLES | AC |  |  |  |  | EC_MISC_OBSTACLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0565 | EE | CHANGE_POINT_TYPE | OBSTACLES | RD |  | An obstacle. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0566 | EE | CHANGE_POINT_TYPE | ROAD_WIDTH_CHANGE | D |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will investigate and revise as necessary.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Undefined enumerants, missing related concepts. ROAD_WIDTH_CHANGE is not a category of object, so it doesn’t fit the EA’s definition. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0567 | EE | COLLECTION_TYPE | DATA_NOT_REQUESTED | D |  |  |  |  |
| SEDRIS_T0568 | EE | MISSING_COLLECTION_REASON | DATA_NOT_REQUESTED | RD |  | The described data was not requested by the user. | ND |  |
| SEDRIS_T0569 | EE | COLLECTION_TYPE | DATA_NOT_REQUIRED | D |  |  |  |  |
| SEDRIS_T0570 | EE | MISSING_COLLECTION_REASON | DATA_NOT_REQUIRED | RD |  | The described data was not required by the user. | ND |  |
| SEDRIS_T0571 | EE | COLLECTION_TYPE | DIFFERENT_HEIGHT_THRESHOLD | D |  |  |  |  |
| SEDRIS_T0572 | EE | MISSING_COLLECTION_REASON | DIFFERENT_HEIGHT_THRESHOLD | RD |  | The described data was collected using a different height threshold than some threshold specified elsewhere. | ND |  |
| SEDRIS_T0573 | EE | COLLECTION_TYPE | LOW_DATA_COLLECT_CRITERIA | D |  |  |  |  |
| SEDRIS_T0574 | EE | MISSING_COLLECTION_REASON | LOW_DATA_COLLECT_CRITERIA | RD |  | The described data was collected with low data collection criteria. | ND |  |
| SEDRIS_T0575 | EE | COLLECTION_TYPE | NO_AVAILABLE_IMAGERY | D |  |  |  |  |
| SEDRIS_T0576 | EE | COLLECTION_TYPE | NO_AVAILABLE_MAP_SOURCE | D |  |  |  |  |
| SEDRIS_T0577 | EE | COLLECTION_TYPE | NO_SUITABLE_IMAGERY | D |  |  |  |  |
| SEDRIS_T0578 | EE | COLLECTION_TYPE | TOO_ROUGH_TO_COLLECT | D |  |  |  |  |
| SEDRIS_T0579 | EE | MISSING_COLLECTION_REASON | TOO_ROUGH_TO_COLLECT | RD |  | The region was too poorly defined to permit the collection of data. | ND |  |
| SEDRIS_T0580 | EE | COLLECTION_TYPE | NOT_COLLECTED | A |  | Data was not collected. | ND |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Most of COLLECTION_TYPE’s enumerants are redundant with those of MISSING_COLLECTION_REASON. |
|  | Solution | Delete COLLECTION_TYPE’s duplicate enumerants, provide proper definitions for those of MISSING_COLLECTION_REASON, and map instances of the former to the latter. |

| SEDRIS_T0581 | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | CONTOUR_INTERVAL_RELATION uses the undefined term ‘contour interval’. |
|  | Solution | Define what a ‘contour interval is, and possibly add an EC for it. |

Response:	AI 08-14: Paul Foley will investigate with other “mapping issues”.

| SEDRIS_T0582 | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | CONTROL_POINT_TYPE uses the undefined term ‘control point’. |
|  | Solution | Define what a ‘control point’ is, and possibly add an EC for it, if it is not synonymous with CADASTRAL_CONTROL_POINT. If it is synonymous, change the definition and label of CONTROL_POINT_TYPE to specify ‘cadastral’. |

Response:	Rejected; revise Label to SURVEY_CONTROL_POINT_TYPE and refine definition to use “surveying”.

| SEDRIS_T0583 | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | MAP_OBJECT_IDENTIFIER’s definition implies some structure that could be used to link to an external reference, but the definition seems to be missing some context. |
|  | Solution | Supply the missing context |

Response:	AI 08-14: Paul Foley will investigate.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0584 | EA | OBJECT_RANKING | D |  |  |  |  |  |  |  |  |  |

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | OBJECT_RANKING seems redundant with ORDINAL_RANK. |
|  | Solution | Delete OBJECT_RANKING. |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0585 | EA | POPULATION | AC |  |  | EA_POPULATION_RANGE |  |  |  |  |  |
| SEDRIS_T0586 | EA | POPULATION_RANGE | AC |  |  | EA_POPULATION |  |  |  |  |  |
| SEDRIS_T0589 | EA | WATER_DEPTH | AC |  |  | EA_WATER_DEPTH_RANGE |  |  |  |  |  |
| SEDRIS_T0590 | EA | WATER_DEPTH_RANGE | AC |  |  | EA_WATER_DEPTH |  |  |  |  |  |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing related concepts where pairs of concepts are similar. |
|  | Solution | Add the missing 2 way relationships. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0591 | EE | CHARGED_PARTICLE_SPECIES | ALPHA | RD |  | A form of ionizing radiation ejected from the nucleus of a radioactive material. The particle consists of 2 protons and 2 neutrons: the equivalent of a helium ion. Also known as alpha radiation. | IR | FFENVSCI |
| SEDRIS_T0592 | EE | CHARGED_PARTICLE_SPECIES | ELECTRON | RD |  | One of the elementary particles of an atom. Negatively charged electrons surround the positively charged nucleus of an atom and occur in numbers equal to the number of protons in the nucleus. An electron has a mass 1/1837 that of a proton. | IR | FFENVSCI |
| SEDRIS_T0593 | EE | CHARGED_PARTICLE_SPECIES | HYDROGEN_ION | RD |  | Hydrogen atom without an electron; chemical symbol H+. | IR | FFENVSCI |
| SEDRIS_T0594 | EE | CHARGED_PARTICLE_SPECIES | PROTON | RD |  | One of the elementary nuclear particles. The particle is located in the nucleus of atoms, has a mass number of 1, and carries a positive charge. The number of protons in the nucleus of an atom is referred to as the atomic number. | IR | FFENVSCI |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0595 | EE | COMMUNICATION_TYPE | BEACON | AC |  |  |  |  | EC_BEACON |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0596 | EE | COMMUNICATION_TYPE | BEACON | RD |  | A fixed, usually elevated, device that serves as an aid to marine or aeronautical navigation, usually by emitting a signal warning of its location, such as a flashing light or radar pulse. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0597 | EE | COMMUNICATION_TYPE | NAVIGATION_LIGHT | AC |  |  |  |  | EC_NAVIGATION_LIGHT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0598 | EE | COMMUNICATION_TYPE | NAVIGATION_LIGHT | RD |  | A specially constructed device that displays a luminous or lighted aid to navigation. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0599 | EE | COMMUNICATION_TYPE | RADAR_STATION | AC |  |  |  |  | EC_RADAR_STATION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0600 | EE | COMMUNICATION_TYPE | RADAR_STATION | RD |  | The location of a radar device. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0601 | EE | COMMUNICATION_TYPE | SATELLITE_GROUND_STATION | AC |  |  |  |  | EC_SATELLITE_GROUND_STATION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0602 | EC | SATELLITE_GROUND_STATION | AC |  |  | EE_COMMUNICATION_TYPE_SATELLITE_GROUND_STATION |  |  |  |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0603 | EC | SATELLITE_GROUND_STATION | RD |  | The ground support structure that interfaces with a celestial body (e.g. satellite, space station, orbiter, or moon base) that orbits a planet, where this ground station supports communications with the celestial body, e.g. a groundbased satellite dish. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0605 | EE | EXISTENCE_STATUS | CONTINUOUS_OPERATION | D |  |  |  |  |  |
| SEDRIS_T0606 | EE | EXISTENCE_STATUS | INACTIVE | D |  |  |  |  |  |
| SEDRIS_T0607 | EE | EXISTENCE_STATUS | MAINTAINED | D |  |  |  |  |  |
| SEDRIS_T0608 | EE | EXISTENCE_STATUS | NOT_MAINTAINED | D |  |  |  |  |  |
| SEDRIS_T0609 | EE | EXISTENCE_STATUS | NON_TIDAL | D |  |  |  |  |  |
| SEDRIS_T0610 | EE | EXISTENCE_STATUS | TIDAL | D |  |  |  |  |  |
| SEDRIS_T0611 | EE | EXISTENCE_STATUS | NOT_USABLE | D |  |  |  |  |  |
| SEDRIS_T0612 | EE | EXISTENCE_STATUS | OPERATIONAL | D |  |  |  |  |  |
| SEDRIS_T0613 | EE | EXISTENCE_STATUS | RUINED | D |  |  |  |  |  |
| SEDRIS_T0614 | EE | EXISTENCE_STATUS | WATCHED | RL | MONITORED |  |  |  |  |

Response:	Accepted in principle; the editors will review and incorporate as appropriate.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | EXISTENCE_STATUS has several enumerants that are redundant with other Eas, or are redundant with one another. |
|  | Solution | - Map EE_EXISTENCE_STATUS_MAINTAINED to EA_MAINTAINED with a value of TRUE, EE_EXISTENCE_STATUS_NOT_MAINTAINED to EA_MAINTAINED with a value of FALSE - Map NON_TIDAL to TIDE_INFLUENCED with a value of FALSE, and map TIDAL to the same EA with TRUE. - Map NOT_USABLE and INACTIVE to EA_OPERATIONAL with a value of FALSE, and map OPERATIONAL to EA_OPERATIONAL with a value of TRUE. - CONTINUOUS_OPERATION is redundant with EE_OPERATING_RESTRICTIONS_TYPE_CONTINUOUS. - RUINED is covered by DESTROYED. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0615 | EE | EXISTENCE_STATUS | PERMANENT | D |  |  |  |  |  |
| SEDRIS_T0616 | EE | EXISTENCE_STATUS | TEMPORARY | D |  |  |  |  |  |

Response:	Accepted in principle; the editors will review and incorporate as appropriate.

| SEDRIS_T0617 | EE | VEHICULAR_SURFACE_TYPE | PERMANENT | D |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0618 | EE | VEHICULAR_SURFACE_TYPE | TEMPORARY | D |  |  |  |  |  |

Response:	Rejected.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Several enumerated Eas have entries that are redundant with the two logical values of EA_PERMANENT. |
|  | Solution | Map EE_EXISTENCE_STATUS_PERMANENT, EE_EXISTENCE_STATUS_TEMPORARY to appropriate values of EA_PERMANENT; similarly map the corresponding enumerants in VEHICULAR_SURFACE_TYPE. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0619 | EE | INTEREST_TYPE | FAMOUS_PLACE_OF_RESIDENCE | RL | FAMOUS_RESIDENCE |  |  |  |  |
| SEDRIS_T0620 | EE | INTEREST_TYPE | FAMOUS_PLACE_OF_RESIDENCE | RD | The object is a famous place of residence. |  | ND |  |  |
| SEDRIS_T0621 | EE | INTEREST_TYPE | HISTORIC_BATTLEFIELD | RD | The object is an historic battlefield. |  | ND |  |  |
| SEDRIS_T0622 | EE | INTEREST_TYPE | HISTORIC_MONUMENT | RD | The object is an historic battlefield. |  | ND |  |  |
| SEDRIS_T0623 | EE | IRRIGATION_TYPE | SPRINKLER | RD | Add “e.g. a sprinkler”. |  |  |  |  |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Undefined, sometimes poorly labeled enumerants. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0624 | EA | IRRIGATION_TYPE | RL | IRRIGATION_METHOD |  |  |  |  |  |  |  |  |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Undefined, sometimes poorly labeled Eas. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0625 | EE | BOTTOM_MATERIAL_TYPE | BEDROCK | AC |  |  |  |  | EE_SURFACE_MATERIAL_TYPE_BEDROCK |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0626 | EE | BOTTOM_MATERIAL_TYPE | SEAMOUNT | D |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will investigate.

| SEDRIS_T0627 | EE | BOTTOM_MATERIAL_TYPE | CORAL | AC |  |  |  |  | EE_SURFACE_MATERIAL_TYPE_CORAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0628 | EE | BOTTOM_MATERIAL_TYPE | CORAL | RD |  | The calcium carbonate skeleton of certain colonial anthozoan coelenterates known as coral. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0629 | EE | BOTTOM_MATERIAL_TYPE | PEAT | AC |  |  |  |  | EE_SURFACE_MATERIAL_TYPE_PEAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Undefined enumerants, missing related concepts; SEAMOUNT doesn’t belong here. |
|  | Solution | Map current instances of EE_BOTTOM_MATERIAL_TYPE_SEAMOUNT to EE_WATER_BOTTOM_MORPHOLOGY_SEAMOUNT. Synchronize remaining definitions with those of EA_SURFACE_MATERIAL_TYPE, and add related concepts information. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0630 | EA | VEHICULAR_SURFACE_TYPE | D |  |  |  |  |  |  |  |  |

Response:	Reject; revise Label to TRAFFICWAY_SURFACE_TYPE; ensure that all of the enumerants also appear in SURFACE_MATERIAL_TYPE.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | VEHICULAR_SURFACE_TYPE is redundant with SURFACE_MATERIAL_TYPE. |
|  | Solution | See also separate comments about redundancies with EA_PERMANENT. - Map the following VEHICULAR_SURFACE_TYPE Ees to their counterparts in SURFACE_MATERIAL_TYPE: BITUMINOUS, BRICK, CLAY, CONCRETE, CORAL, GRAVEL, ICE, LATERITE, MEMBRANE, SAND, SNOW. - COMPOSITE_NON_PERMANENT maps to COMPOSITE_ge50_PCT_PERM - COMPOSITE_PERMANENT maps to COMPOSITE_lt50_PCT_PERM - GRADED_SOIL maps to GRADED - MACADAM maps to MACADAM_CRUSHED_ROCK - MIX_IN_PLACE maps to MIX Left over: ASPHALT, CORDUROY, HARD, LOOSE, MIXED_CONCRETE_AND_ASPHALT, NATURAL, SOD, STEEL_PLANKING, UNGRADED, UNPAVED |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0631 | EA | ACCURACY | RL | ACCURACY_SURVEYED_POSITION |  |  |  |  |  |  |  |

Response:	Rejected; revise Label to POSITION_ACCURACY_STATEMENT and definition to “Statement of the accuracy of the position.”.

| SEDRIS_T0632 | EA | ELEVATION_ACCURACY | D |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected; revise Label to ELEVATION_ACCURACY_STATEMENT and revise the definition similarly.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0633 | EE | VERTICAL_MSRMNT_QUALITY | ESTIMATED | RD |  | The measurement is a rough calculation based on an estimated value, or an approximate measurement of a value. |  | ND |  |

Response:	Accepted.

| SEDRIS_T0634 | EE | VERTICAL_MSRMNT_QUALITY | ESTIMATED | AC |  |  |  |  | EE_POSITION_QUALITY_ESTIMATED |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0635 | EA | HYDROGRAPHIC_ORIGIN | D |  |  |  |  |  |  |  |  |

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | All the enumerants of HYDROGRAPHIC_ORIGIN are redundant with some of those of EXISTENCE_STATUS. |
|  | Solution | Map all current instances of HYDROGRAPHIC_ORIGIN enumerants to their counterparts in EXISTENCE_STATUS. - HYDROGRAPHIC_ORIGIN_CONTROLLED maps to EXISTENCE_STATUS_CONTROLLED. - HYDROGRAPHIC_ORIGIN_MAN_MADE maps to EXISTENCE_STATUS_MAN_MADE. - HYDROGRAPHIC_ORIGIN_NATURAL maps to EXISTENCE_STATUS_NATURAL. [Alternate solution: EXISTENCE_STATUS should be split up between EXISTENCE_STATUS and HYDROGRAPHIC_ORIGIN. EXISTENCE_STATUS should renamed SITE_EXISTENCE_STATUS. ] |

Response:	Rejected.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0636 | EE | FISHING_FACILITY_TYPE | STAKE | AC |  |  |  |  | EC_FISH_STAKES |
| SEDRIS_T0637 | EE | FISHING_FACILITY_TYPE | TRAP | AC |  |  |  |  | EC_FISH_TRAP |
| SEDRIS_T0638 | EE | FISHING_FACILITY_TYPE | TUNA_NET | AC |  |  |  |  | EC_TUNA_NETS |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing related concepts. |

| Comment | Dic | Current EA Label | Current EE Label | Type | New Label | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0639 | EE | MTRL_COMPOSITION_UNDERLYING |  | A | AIR | The mixture of invisible odorless tasteless gases (as nitrogen and oxygen) that surrounds the earth, i.e. air. | ND |  |
| SEDRIS_T0640 | EE | SURFACE_MTRL_CATEGORY |  | A | BARK | The tough exterior covering of a woody root or stem, i.e. bark. | ND |  |
| SEDRIS_T0641 | EE | MTRL_COMPOSITION_CATEGORY |  | A | CARBON | A nonmetallic chemical element, atomic number 6, that exists in graphite and diamond. | ND |  |
| SEDRIS_T0642 | EE | MTRL_COMPOSITION_SECONDARY |  | A | CARBON | A nonmetallic chemical element, atomic number 6, that exists in graphite and diamond. | ND |  |
| SEDRIS_T0643 | EE | SURFACE_MTRL_CATEGORY |  | A | CARBON_COMPOSITE | A nonmetallic chemical element (atomic number 6) composite that exists in graphite and diamond. | ND |  |
| SEDRIS_T0644 | EE | MTRL_COMPOSITION_CATEGORY |  | A | CARDBOARD | A stiff light material made from wastepaper pulp, i.e. cardboard. | ND |  |
| SEDRIS_T0645 | EE | MTRL_COMPOSITION_CATEGORY |  | A | CEILING_TILE | Ceiling tile. | ND |  |
| SEDRIS_T0646 | EE | SURFACE_MTRL_CATEGORY |  | A | CEILING_TILE | Ceiling tile. | ND |  |
| SEDRIS_T0647 | EE | MTRL_COMPOSITION_CATEGORY |  | A | CERAMIC | Of or relating to the manufacture of any product (as earthenware, porcelain, or brick) made essentially from a nonmetallic mineral (as clay) by firing at a high temperature, i.e. ceramic. | ND |  |
| SEDRIS_T0648 | EE | SURFACE_MTRL_CATEGORY |  | A | CERAMIC | Of or relating to the manufacture of any product (as earthenware, porcelain, or brick) made essentially from a nonmetallic mineral (as clay) by firing at a high temperature, i.e. ceramic. | ND |  |
| SEDRIS_T0649 | EE | SOIL_TYPE |  | A | CLAY | An earthy material that is plastic when moist but hard when fired, that is composed mainly of fine particles of hydrous aluminum silicates and other minerals, and that is used for brick, tile, and pottery, i.e. clay. | ND |  |
| SEDRIS_T0650 | EE | SFC_COMPACTION_DESCRIPTION |  | A | CORDUROY | A road made of logs across muddy or swampy ground, i.e. corduroy. | ND |  |
| SEDRIS_T0651 | EE | BULK_WETNESS_CONDITION |  | A | DRY | Dry. | ND |  |
| SEDRIS_T0652 | EE | MTRL_COMPOSITION_CATEGORY |  | A | EPOXY | A flexible usually thermosetting resin made by copolymerization of an epoxide with another compound having two hydroxyl groups and used chiefly in coatings and adhesives, i.e. an epoxy. | ND |  |
| SEDRIS_T0653 | EE | MTRL_COMPOSITION_SECONDARY |  | A | EPOXY | A flexible usually thermosetting resin made by copolymerization of an epoxide with another compound having two hydroxyl groups and used chiefly in coatings and adhesives, i.e. an epoxy. | ND |  |
| SEDRIS_T0654 | EE | SURFACE_MTRL_CATEGORY |  | A | EPOXY | A flexible usually thermosetting resin made by copolymerization of an epoxide with another compound having two hydroxyl groups and used chiefly in coatings and adhesives, i.e. an epoxy. | ND |  |
| SEDRIS_T0655 | EE | SURFACE_MTRL_CATEGORY |  | A | FABRIC | A material that resembles cloth, i.e. fabric. | ND |  |
| SEDRIS_T0656 | EE | MTRL_COMPOSITION_CATEGORY |  | A | FIBER | A very strong lightweight synthetic fiber made especially by carbonizing acrylic fiber at high temperatures | ND |  |
| SEDRIS_T0657 | EE | MTRL_COMPOSITION_SECONDARY |  | A | FIBERGLASS | Glass in fibrous form used in making various products (as glass wool for insulation) or a composite structural material of plastic and fiberglass. | ND |  |
| SEDRIS_T0658 | EE | SURFACE_MTRL_CATEGORY |  | A | FIBERGLASS | Glass in fibrous form used in making various products (as glass wool for insulation) or a composite structural material of plastic and fiberglass. | ND |  |
| SEDRIS_T0659 | EE | SFC_ROUGHNESS_DESCRIPTION |  | A | FLAT_COARSE | Flat and loose or rough in texture. | ND |  |
| SEDRIS_T0660 | EE | SFC_ROUGHNESS_DESCRIPTION |  | A | FLAT_MIRRORED | Flat, polished, and smooth surface (as of glass). | ND |  |
| SEDRIS_T0661 | EE | SFC_ROUGHNESS_DESCRIPTION |  | A | FLAT_SMOOTH | Flat and not sharp or harsh, i.e. smooth. | ND |  |
| SEDRIS_T0662 | EE | SFC_ROUGHNESS_DESCRIPTION |  | A | FLAT_SMOOTH_POLISHED | Flat, smooth, and glossy | ND |  |
| SEDRIS_T0663 | EE | SFC_COMPACTION_DESCRIPTION |  | A | FLEXIBLE | Able to bend or be bent repeatedly without damage, i.e. flexible. | ND |  |
| SEDRIS_T0664 | EE | BULK_WETNESS_CONDITION |  | A | FLOODED | Covered with water as a result of a river overflowing or a pipe bursting, i.e. flooded. | ND |  |
| SEDRIS_T0665 | EE | MTRL_COMPOSITION_CATEGORY |  | A | FOAM | Spongy rubber of fine texture made from latex by foaming (as by whipping) before vulcanization | ND |  |
| SEDRIS_T0666 | EE | BULK_WETNESS_CONDITION |  | A | FROZEN_OR_PERMAFROST | Covered by or made into ice, i.e. frozen or permafrost. | ND |  |
| SEDRIS_T0667 | EE | MTRL_COMPOSITION_CATEGORY |  | A | GRAPHNOL | High-strength graphite developed for missile nose tips for the U. S. Navy, i.e. graphnol. | ND |  |
| SEDRIS_T0668 | EE | SURFACE_MTRL_CATEGORY |  | A | GRAPHNOL_COMPOSITE | A high-strength graphite composite developed for missile nose tips for the U.S. Navy, i.e. graphnol composite. | ND |  |
| SEDRIS_T0669 | EE | VEGETATION_TYPE |  | A | GRASS | Any of a large family (Gramineae) of monocotyledonous mostly herbaceous plants with jointed stems, slender sheathing leaves, and flowers borne in spikelets of bracts | ND |  |
| SEDRIS_T0670 | EE | SOIL_TYPE |  | A | GRAVELY | A surface covered with gravel. | ND |  |
| SEDRIS_T0671 | EE | SOIL_TYPE |  | A | GRAVELY_LOAM | A soil consisting of a friable mixture of varying proportions of clay, silt, sand, and gravel, i.e. gravelly loam. | ND |  |
| SEDRIS_T0672 | EE | SFC_COMPACTION_DESCRIPTION |  | A | HARD | Firm, stiff, or rigid and not easily cut, pierced, or bent, i.e. hard. | ND |  |
| SEDRIS_T0673 | EE | MTRL_COMPOSITION_CATEGORY |  | A | INFRAGOLD | A specially formulated white reflectance coating for use over the UV-VIS-NIR wavelength region, i.e. infragold. | ND |  |
| SEDRIS_T0674 | EE | SURFACE_MTRL_CATEGORY |  | A | INFRAGOLD | A specially formulated white reflectance coating for use over the UV-VIS-NIR wavelength region, i.e. infragold. | ND |  |
| SEDRIS_T0675 | EE | SFC_COMPACTION_DESCRIPTION |  | A | LIGHT | Weighing comparatively little, i.e. light. | ND |  |
| SEDRIS_T0676 | EE | MTRL_COMPOSITION_SECONDARY |  | A | LIME | A caustic highly infusible solid that consists of calcium oxide often together with magnesium oxide, that is obtained by calcining forms of calcium carbonate (as shells or limestone), and that is used in building (as in mortar and plaster) and in agriculture, i.e. lime. | ND |  |
| SEDRIS_T0677 | EE | SOIL_TYPE |  | A | LIME | A caustic highly infusible solid that consists of calcium oxide often together with magnesium oxide, that is obtained by calcining forms of calcium carbonate (as shells or limestone), i.e. lime. | ND |  |
| SEDRIS_T0678 | EE | SOIL_TYPE |  | A | LOAM | A soil consisting of a friable mixture of varying proportions of clay, silt, and sand, i.e. loam. | ND |  |
| SEDRIS_T0679 | EE | SFC_COMPACTION_DESCRIPTION |  | A | LOOSE | Not compact or dense in texture or arrangement, i.e. loose. | ND |  |
| SEDRIS_T0680 | EE | BULK_WETNESS_CONDITION |  | A | MOIST | Slightly wet or damp, i.e. moist. | ND |  |
| SEDRIS_T0681 | EE | SFC_COMPACTION_DESCRIPTION |  | RD | NATURAL | Having a particular characteristic by nature. | ND |  |
| SEDRIS_T0682 | EE | MTRL_COMPOSITION_CATEGORY |  | A | PAINT_CHIP | A small usually thin and flat piece of paint, i.e. a paint chip. | ND |  |
| SEDRIS_T0683 | EE | BULK_WETNESS_CONDITION |  | A | PATCHY_ICE | Ice occurring only in patches, rather than throughout an area, or consisting only of Ice patches. | ND |  |
| SEDRIS_T0684 | EE | BULK_WETNESS_CONDITION |  | A | PATCHY_WET | Wetness occurring only in patches, rather than throughout an area, or consisting only of wet patches. | ND |  |
| SEDRIS_T0685 | EE | MTRL_COMPOSITION_CATEGORY |  | A | PERMAGOLD | A specially formulated coating, i.e. permagold. | ND |  |
| SEDRIS_T0686 | EE | MTRL_COMPOSITION_SECONDARY |  | A | PHENOLIC | Containing or derived from a corrosive poisonous crystalline acidic compound C6H5OH present in coal tar and wood tar that in dilute solution is used as a disinfectant, i.e. phenolic. | ND |  |
| SEDRIS_T0687 | EE | SURFACE_MTRL_CATEGORY |  | A | PLASMAGOLD | A specially formulated coating, i.e. plasmagold. | ND |  |
| SEDRIS_T0688 | EE | MTRL_COMPOSITION_SECONDARY |  | A | PLASTIC | Any of numerous organic synthetic or processed materials that are mostly thermoplastic or thermosetting polymers of high molecular weight and that can be made into objects, films, or filaments, i.e. plastic. | ND |  |
| SEDRIS_T0689 | EE | MTRL_COMPOSITION_UNDERLYING |  | A | RUBBER | An elastic substance, i.e. rubber. | ND |  |
| SEDRIS_T0690 | EE | SOIL_TYPE |  | A | SAND | Soil containing 85 percent or more of sand and a maximum of 10 percent of clay, i.e. sand. | ND |  |
| SEDRIS_T0691 | EE | SOIL_TYPE |  | A | SANDY_LOAM | A sand soil consisting of a friable mixture of varying proportions of clay, silt, and sand, i.e. sandy loam. | ND |  |
| SEDRIS_T0692 | EE | MTRL_COMPOSITION_UNDERLYINGK |  | A | SEMI_INFINITE | A solid body that extends to inifinity in all but one direction and is characterized by a single identified surface. | ND |  |
| SEDRIS_T0693 | EE | MTRL_COMPOSITION_CATEGORY |  | A | SILICA | Silicon dioxide SiO2 occurring in crystalline, amorphous, and impure forms, i.e. silica. | ND |  |
| SEDRIS_T0694 | EE | SURFACE_MTRL_CATEGORY |  | A | SILICA | Silicon dioxide SiO2 occurring in crystalline, amorphous, and impure forms, i.e. silica. | ND |  |
| SEDRIS_T0695 | EE | MTRL_COMPOSITION_CATEGORY |  | A | SILICONE | Any of various polymeric organic silicon compounds obtained as oils, greases, or plastics and used especially for water-resistant and heat-resistant lubricants, varnishes, binders, and electric insulators, i.e. silicone. | ND |  |
| SEDRIS_T0696 | EE | MTRL_COMPOSITION_SECONDARY |  | A | SILICONE | Any of various polymeric organic silicon compounds obtained as oils, greases, or plastics and used especially for water-resistant and heat-resistant lubricants, varnishes, binders, and electric insulators, i.e. silicone. | ND |  |
| SEDRIS_T0697 | EE | SURFACE_MTRL_CATEGORY |  | A | SILICONE | Any of various polymeric organic silicon compounds obtained as oils, greases, or plastics and used especially for water-resistant and heat-resistant lubricants, varnishes, binders, and electric insulators, i.e. silicone. | ND |  |
| SEDRIS_T0698 | EE | SOIL_TYPE |  | A | SILTY_CLAY | An earthy material deposit of sediment that is plastic when moist but hard when fired, that is composed mainly of fine particles of hydrous aluminum silicates and other minerals, and that is used for brick, tile, and pottery, i.e. silty clay. | ND |  |
| SEDRIS_T0699 | EE | SOIL_TYPE |  | A | SILTY_LOAM | A soil deposit of sediment consisting of a friable mixture of varying proportions of clay, silt, and sand that is plastic when moist but hard when fired, that is composed mainly of fine particles of hydrous aluminum silicates and other minerals, and that is used for brick, tile, and pottery, i.e. silty loam. | ND |  |
| SEDRIS_T0700 | EE | BULK_WETNESS_CONDITION |  | A | SNOW | Water vapor in the atmosphere that has frozen into ice crystals and then falls to the ground in the form of flakes, i.e. snow. | ND |  |
| SEDRIS_T0701 | EE | SFC_COMPACTION_DESCRIPTION |  | A | SOFT | Easily shaped, bent, or cut, i.e. soft. | ND |  |
| SEDRIS_T0702 | EE | MTRL_COMPOSITION_CATEGORY |  | A | SPECTRALON | A proprietary thermoplastic formulation of polytetrafluoroethylene (PTFE), i.e. spectralon. | ND |  |
| SEDRIS_T0703 | EE | SURFACE_MTRL_CATEGORY |  | A | SPECTRALON | A proprietary thermoplastic formulation of polytetrafluoroethylene (PTFE), i.e. spectralon. | ND |  |
| SEDRIS_T0704 | EE | MTRL_COMPOSITION_CATEGORY |  | A | SUBSTRATE | The material of which something is made and from which it derives its special qualities, i.e. substrate. | ND |  |
| SEDRIS_T0705 | EE | MTRL_COMPOSITION_CATEGORY |  | A | TAR | A dark brown or black bituminous usually odorous viscous liquid obtained by destructive distillation of organic material (as wood, coal, or peat), i.e. tar. | ND |  |
| SEDRIS_T0706 | EE | MTRL_COMPOSITION_UNDERLYING |  | A | TAR | A dark brown or black bituminous usually odorous viscous liquid obtained by destructive distillation of organic material (as wood, coal, or peat), i.e. tar. | ND |  |
| SEDRIS_T0707 | EE | SURFACE_MTRL_CATEGORY |  | A | TAR | A dark brown or black bituminous usually odorous viscous liquid obtained by destructive distillation of organic material (as wood, coal, or peat), i.e. tar. | ND |  |
| SEDRIS_T0708 | EE | MTRL_COMPOSITION_CATEGORY |  | A | TEFLON | Used for synthetic fluorine-containing resins used especially for molding articles and for coatings to prevent sticking, i.e. 101eflon. | ND |  |
| SEDRIS_T0709 | EE | SURFACE_MTRL_CATEGORY |  | A | TEFLON | Used for synthetic fluorine-containing resins used especially for molding articles and for coatings to prevent sticking, i.e. 101eflon. | ND |  |
| SEDRIS_T0711 | EE | MTRL_COMPOSITION_CATEGORY |  | A | TITANIUM | A silvery gray light strong metallic element, atomic number 22, found combined in ilmenite and rutile and used especially in alloys (as steel) and combined in refractory materials and in coatings. | ND |  |
| SEDRIS_T0712 | EE | SURFACE_MTRL_CATEGORY |  | A | TITANIUM | A silvery gray light strong metallic element, atomic number 22, found combined in ilmenite and rutile and used especially in alloys (as steel) and combined in refractory materials and in coatings. | ND |  |
| SEDRIS_T0713 | EE | BULK_WETNESS_CONDITION |  | A | TOTAL_ICE | Water that has frozen into solid form, not patchy. | ND |  |
| SEDRIS_T0714 | EE | MTRL_COMPOSITION_UNDERLYING |  | A | VARIABLE | A quantity that may assume any one of a set of values | ND |  |
| SEDRIS_T0715 | EE | MTRL_COMPOSITION_UNDERLYING |  | A | VINYL | A polymer of a vinyl compound or a product. | ND |  |
| SEDRIS_T0716 | EE | BULK_WETNESS_CONDITION |  | A | WET | Covered, soaked, or dampened with water or some other liquid. | ND |  |
| SEDRIS_T0717 | EE | BULK_WETNESS_CONDITION |  | A | WET_RUBBER | Rubber that is covered, soaked, or dampened with water or some other liquid | ND |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Material enumerations |
| --- | --- | --- |
|  | Problem | Supplying missing definitions, enumerants requested to support users of EDCS (see especially SCR-jrm-102, SCR-jrm-103, and SCR-jrm-104). |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0718 | EE | MINE_TYPE | STRIP | RD |  | The process of removing mineral deposits that are found close enough to the surface so that the construction of tunnels is not necessary. The soil and strata that cover the deposit are removed to gain access to the mineral deposit. Also called surface mining. | IR | FFENVSCI |
| SEDRIS_T0719 | EE | MINE_TYPE | BELOW_SURFACE | RD |  | A technique of removing coal and mineral ores from the Earth that involves cutting tunnels below the surface to gain access to the deposit(s). | IR | FFENVSCI |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0720 | EE | NAVIGATION_LINE_TYPE | TRANSIT_LINE | RD |  | A line along which two features are observed to lie directly in line by an observer on a vessel which also lies on that transit line. When the observer can obtain a twotransit fix by observing yet another two features on a transit line, the observer can fix the position of the vessel at that time. | IR | http://education.qld.gov.au/tal/kla/compass/html/cncha.htm |

Response:	Accepted in principle; the editors will review IHO S-57 and revise as appropriate.

| Com Tag | Dic | Current EA Label | Current EE Label | Ch. Typ | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0721 | EE | PREDOMINANT_COLOUR | BLACK_AND_GREEN | RD | Black and green appear to be equally predominant. | ND |  |
| SEDRIS_T0722 | EE | PREDOMINANT_COLOUR | BLACK_AND_RED | RD | Black and red appear to be equally predominant. | ND |  |
| SEDRIS_T0723 | EE | PREDOMINANT_COLOUR | BLACK_AND_WHITE | RD | Black and white appear to be equally predominant. | ND |  |
| SEDRIS_T0724 | EE | PREDOMINANT_COLOUR | BLACK_AND_YELLOW | RD | Black and yellow appear to be equally predominant. | ND |  |
| SEDRIS_T0725 | EE | PREDOMINANT_COLOUR | BLUE_AND_GREEN | RD | Blue and green appear to be equally predominant. | ND |  |
| SEDRIS_T0726 | EE | PREDOMINANT_COLOUR | GREEN_AND_BLACK | RD | Green and black appear to be equally predominant. | ND |  |
| SEDRIS_T0727 | EE | PREDOMINANT_COLOUR | GREEN_AND_BROWN | RD | Green and brown appear to be equally predominant. | ND |  |
| SEDRIS_T0728 | EE | PREDOMINANT_COLOUR | GREEN_AND_RED | RD | Green and red appear to be equally predominant. | ND |  |
| SEDRIS_T0729 | EE | PREDOMINANT_COLOUR | GREEN_AND_WHITE | RD | Green and white appear to be equally predominant. | ND |  |
| SEDRIS_T0730 | EE | PREDOMINANT_COLOUR | GREY_AND_BLACK | RD | Grey and black appear to be equally predominant. | ND |  |
| SEDRIS_T0731 | EE | PREDOMINANT_COLOUR | GREY_AND_GOLD | RD | Grey and gold appear to be equally predominant. | ND |  |
| SEDRIS_T0732 | EE | PREDOMINANT_COLOUR | GREY_AND_GREEN | RD | Grey and green appear to be equally predominant. | ND |  |
| SEDRIS_T0733 | EE | PREDOMINANT_COLOUR | GREY_AND_WHITE | RD | Grey and white appear to be equally predominant. | ND |  |
| SEDRIS_T0734 | EE | PREDOMINANT_COLOUR | LAVENDER_AND_RED_BRICK | RD | Lavender and red brick appear to be equally predominant. | ND |  |
| SEDRIS_T0735 | EE | PREDOMINANT_COLOUR | RED_AND_BLACK | RD | Red and black appear to be equally predominant. | ND |  |
| SEDRIS_T0736 | EE | PREDOMINANT_COLOUR | RED_AND_GREEN | RD | Red and green appear to be equally predominant. | ND |  |
| SEDRIS_T0737 | EE | PREDOMINANT_COLOUR | RED_AND_GREY | RD | Red and grey appear to be equally predominant. | ND |  |
| SEDRIS_T0738 | EE | PREDOMINANT_COLOUR | RED_AND_WHITE | RD | Red and white appear to be equally predominant. | ND |  |
| SEDRIS_T0739 | EE | PREDOMINANT_COLOUR | TAN_AND_BLACK | RD | Tan and black appear to be equally predominant. | ND |  |
| SEDRIS_T0740 | EE | PREDOMINANT_COLOUR | TAN_AND_BLUE | RD | Tan and blue appear to be equally predominant. | ND |  |
| SEDRIS_T0741 | EE | PREDOMINANT_COLOUR | TAN_AND_BROWN | RD | Tan and brown appear to be equally predominant. | ND |  |
| SEDRIS_T0742 | EE | PREDOMINANT_COLOUR | TAN_AND_DARK_YELLOW | RD | Tan and dark yellow appear to be equally predominant. | ND |  |
| SEDRIS_T0743 | EE | PREDOMINANT_COLOUR | TAN_AND_GREEN | RD | Tan and green appear to be equally predominant. | ND |  |
| SEDRIS_T0744 | EE | PREDOMINANT_COLOUR | TAN_AND_GREY | RD | Tan and grey appear to be equally predominant. | ND |  |
| SEDRIS_T0745 | EE | PREDOMINANT_COLOUR | WHITE_AND_BLACK | RD | White and black appear to be equally predominant. | ND |  |
| SEDRIS_T0746 | EE | PREDOMINANT_COLOUR | WHITE_AND_BLUE | RD | White and blue appear to be equally predominant. | ND |  |
| SEDRIS_T0747 | EE | PREDOMINANT_COLOUR | WHITE_AND_GREEN | RD | White and green appear to be equally predominant. | ND |  |
| SEDRIS_T0748 | EE | PREDOMINANT_COLOUR | WHITE_AND_ORANGE | RD | White and orange appear to be equally predominant. | ND |  |
| SEDRIS_T0749 | EE | PREDOMINANT_COLOUR | WHITE_AND_RED | RD | White and red appear to be equally predominant. | ND |  |
| SEDRIS_T0750 | EE | PREDOMINANT_COLOUR | WHITE_AND_YELLOW | RD | White and yellow appear to be equally predominant. | ND |  |
| SEDRIS_T0751 | EE | PREDOMINANT_COLOUR | YELLOW_AND_BLACK | RD | Yellow and black appear to be equally predominant. | ND |  |
| SEDRIS_T0752 | EE | PREDOMINANT_COLOUR | YELLOW_AND_BLUE | RD | Yellow and blue appear to be equally predominant. | ND |  |
| SEDRIS_T0753 | EE | PREDOMINANT_COLOUR | YELLOW_AND_GREEN | RD | Yellow and green appear to be equally predominant. | ND |  |
| SEDRIS_T0754 | EE | PREDOMINANT_COLOUR | YELLOW_AND_RED | RD | Yellow and red appear to be equally predominant. | ND |  |

Response:	AI 08-22: Paul Foley will investigate (see response to comments SEDRIS_T453 and SEDRIS_T454).

|  | Where | PREDOMINANT_COLOUR |
| --- | --- | --- |
|  | Problem | The ‘and’ in the existing definition was intended to indicate that “These colours all appear to be predominant, or they may be present in equal amounts.” |
|  | Solution | Clarify the definitions. |

| Com Tag | Dic | Current EA Label | Current EE Label | Ch. Typ | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0755 | EE | PREDOMINANT_COLOUR | BLACK_BROWN_GREY | RD | Predominance cannot be distinguished between black, brown, and grey. | ND |  |
| SEDRIS_T0756 | EE | PREDOMINANT_COLOUR | BLACK_WHITE_GREY | RD | Predominance cannot be distinguished between black, white, and grey. | ND |  |
| SEDRIS_T0757 | EE | PREDOMINANT_COLOUR | GREY_BLUE_BLACK | RD | Predominance cannot be distinguished between grey, blue, and black. | ND |  |
| SEDRIS_T0758 | EE | PREDOMINANT_COLOUR | LAVENDER_YELLOW_BROWN | RD | Predominance cannot be distinguished between lavender, yellow, and brown. | ND |  |
| SEDRIS_T0759 | EE | PREDOMINANT_COLOUR | OATMEAL_BLACK_GREY | RD | Predominance cannot be distinguished between oatmeal, black, and grey. | ND |  |
| SEDRIS_T0760 | EE | PREDOMINANT_COLOUR | OATMEAL_TAN_GREY | RD | Predominance cannot be distinguished between oatmeal, tan, and grey. | ND |  |
| SEDRIS_T0761 | EE | PREDOMINANT_COLOUR | OATMEAL_WHITE_BLACK | RD | Predominance cannot be distinguished between oatmeal, white, and black. | ND |  |
| SEDRIS_T0762 | EE | PREDOMINANT_COLOUR | OATMEAL_WHITE_BLUE | RD | Predominance cannot be distinguished between oatmeal, white, and blue. | ND |  |
| SEDRIS_T0763 | EE | PREDOMINANT_COLOUR | RED_WHITE_BLACK | RD | Predominance cannot be distinguished between red, white, and black. | ND |  |
| SEDRIS_T0764 | EE | PREDOMINANT_COLOUR | RED_WHITE_TAN | RD | Predominance cannot be distinguished between red, white, and tan. | ND |  |
| SEDRIS_T0765 | EE | PREDOMINANT_COLOUR | TAN_BLACK_SEAGREEN | RD | Predominance cannot be distinguished between tan, black, and sea green. | ND |  |
| SEDRIS_T0766 | EE | PREDOMINANT_COLOUR | TAN_BLACK_WHITE | RD | Predominance cannot be distinguished between tan, black, and white. | ND |  |
| SEDRIS_T0767 | EE | PREDOMINANT_COLOUR | TAN_GREY_BLACK | RD | Predominance cannot be distinguished between tan, grey, and black. | ND |  |
| SEDRIS_T0768 | EE | PREDOMINANT_COLOUR | WHITE_PURPLE_RED_BRICK | RD | Predominance cannot be distinguished between white, purple, and red brick. | ND |  |

Response:	AI 08-22: Paul Foley will investigate (see response to comments SEDRIS_T453 and SEDRIS_T454).

|  | Where | PREDOMINANT_COLOUR |
| --- | --- | --- |
|  | Problem | Existing definition is comma delimited, which was intended to indicate that the data provider cannot determine predominance between colours indicated or the colour may be texture or mixture. |
|  | Solution | Clarify the definitions. |

| Com Tag | Dic | Current EA Label | Current EE Label | Ch. Typ | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1767 | EE | PREDOMINANT_COLOUR | AMBER | RD | A brownish yellow, i.e., amber | ND |  |
| SEDRIS_T1768 | EE | PREDOMINANT_COLOUR | BLUE | RD | Blue. | ND |  |
| SEDRIS_T0769 | EE | PREDOMINANT_COLOUR | BUFF | RD | A pale, light, or moderate yellowish pink to yellow, including moderate orange-yellow to light yellowish brown, i.e,. buff | ND |  |
| SEDRIS_T0770 | EE | PREDOMINANT_COLOUR | CHOCOLATE | RD | A grayish to deep reddish brown to deep grayish brown, i.e,. chocolate. | ND |  |
| SEDRIS_T0771 | EE | PREDOMINANT_COLOUR | DARK_COLOURED | RD | An indefinable dark colour. | ND |  |
| SEDRIS_T0772 | EE | PREDOMINANT_COLOUR | LIGHT_COLOURED | RD | An indefinable light colour. | ND |  |
| SEDRIS_T0773 | EE | PREDOMINANT_COLOUR | ORANGE | RD | Orange. | ND |  |
| SEDRIS_T0774 | EE | PREDOMINANT_COLOUR | PINK | RD | Pink. | ND |  |
| SEDRIS_T0775 | EE | PREDOMINANT_COLOUR | VIOLET | RD | Reddish-blue, i.e., violet. | ND |  |
| SEDRIS_T0776 | EE | PREDOMINANT_COLOUR | CHARCOAL | A |  | Charcoal | ND |
| SEDRIS_T0777 | EE | PREDOMINANT_COLOUR | DARK_BROWN | A |  | Dark Brown | ND |
| SEDRIS_T0778 | EE | PREDOMINANT_COLOUR | DARK_GRAY | A |  | Dark Gray | ND |
| SEDRIS_T0779 | EE | PREDOMINANT_COLOUR | DARK_GREEN | A |  | Dark Green | ND |
| SEDRIS_T0780 | EE | PREDOMINANT_COLOUR | DARK_RED | A |  | Dark Red | ND |
| SEDRIS_T0781 | EE | PREDOMINANT_COLOUR | LIGHT_GRAY | A |  | Light Gray | ND |
| SEDRIS_T0782 | EE | PREDOMINANT_COLOUR | LIGHT_RED | A |  | Light Red | ND |
| SEDRIS_T0783 | EE | PREDOMINANT_COLOUR | LIGHT_YELLOW | A |  | Light Yellow | ND |
| SEDRIS_T0784 | EE | PREDOMINANT_COLOUR | MAROON | A |  | Maroon | ND |
| SEDRIS_T0785 | EE | PREDOMINANT_COLOUR | MUSTARD_YELLOW | A |  | Mustard Yellow | ND |
| SEDRIS_T0786 | EE | PREDOMINANT_COLOUR | SILVER | A |  | Silver | ND |
| SEDRIS_T0787 | EE | PREDOMINANT_COLOUR | SMOKE_GRAY | A |  | Smoke Gray | ND |

Response:	AI 08-22: Paul Foley will investigate (see response to comments SEDRIS_T453 and SEDRIS_T454).

|  | Where | PREDOMINANT_COLOUR |
| --- | --- | --- |
|  | Problem | Several ‘atomic’ colours are currently undefined. DMSO has requested several atomic colours that are not currently provided. |
|  | Solution | Supply the missing definitions, add the requested new colours. |

| Com Tag | Dic | Current EA Label | Current EE Label | Ch. Typ | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0788 | EE | PREDOMINANT_COLOUR | BLACK_RED_BLACK | RD | Black, red, and black appear in succession. | ND |  |
| SEDRIS_T0789 | EE | PREDOMINANT_COLOUR | BLACK_YELLOW_BLACK | RD | Black, yellow, and black appear in succession. | ND |  |
| SEDRIS_T0799 | EE | PREDOMINANT_COLOUR | GREEN_RED_GREEN | RD | Green, red, and green appear in succession. |  |  |
| SEDRIS_T0800 | EE | PREDOMINANT_COLOUR | GREEN_YELLOW_BLACK | RD | Green, yellow, and black appear in succession. | ND |  |
| SEDRIS_T0801 | EE | PREDOMINANT_COLOUR | RED_GREEN_RED | RD | Red, green, and red appear in succession. | ND |  |
| SEDRIS_T0802 | EE | PREDOMINANT_COLOUR | WHITE_GREEN_WHITE | RD | White, green, and white appear in succession. | ND |  |
| SEDRIS_T0803 | EE | PREDOMINANT_COLOUR | WHITE_RED_GREEN | RD | White, red, and green appear in succession. | ND |  |
| SEDRIS_T0804 | EE | PREDOMINANT_COLOUR | YELLOW_BLACK_YELLOW | RD | Yellow, black, and yellow appear in succession. | ND |  |
| SEDRIS_T0805 | EE | PREDOMINANT_COLOUR | YELLOW_RED_WHITE | RD | Yellow, red, and white appear in succession. | ND |  |

Response:	AI 08-22: Paul Foley will investigate (see response to comments SEDRIS_T453 and SEDRIS_T454).

|  | Where | Table 6.47 – EDCS Attribute enumerant codes for attributes whose label begins with P |
| --- | --- | --- |
|  | Problem | Given the ‘-‘ in the existing definitions and the colours listed: These colours are typically applied to signaling devices (buoys) where the sequence of colours is the important factor. |
|  | Solution |  |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0806 | EE | WARNING_SIGNAL_STATION_TYPE | TIDE_GAUGE | RD |  | A gauge for measuring the height of the tide. | IR | FACC, Annex A |  |

Response:	Rejected.

| SEDRIS_T0807 | EE | WARNING_SIGNAL_STATION_TYPE | TIDE_GAUGE | AC |  |  |  |  | EC_TIDE_GAUGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0808 | EE | WATER_TYPE | ALKALINE | RD |  | Alkaline water. Alkalinity is generally a measure of the ability of water to neutralize acids, and is measured by determining the amount of acid required to lower the pH of water to 4.5. In natural waters, the alkalinity is effectively the bicarbonate ion concentration plus twice the carbonate ion concentration, expressed in milligrams per liter calcium carbonate. | IR | FFENVSCI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0809 | EE | WATER_TYPE | BRACKISH | RD |  | Brackish water with a salt content between 1000 and 4000 parts per million. | IR | FFENVSCI |  |
| SEDRIS_T0810 | EE | WATER_TYPE | FRESH | RD |  | Fresh water without significant amounts of dissolved sodium chloride (salt). The opposite of seawater. Water characteristic of rain, rivers, ponds, and most lakes. | IR | FFENVSCI |  |
| SEDRIS_T0811 | EE | WATER_TYPE | MINERAL | RD |  | Mineral water containing at least 250 parts per million total dissolved solids (consisting primarily of dissolved inorganic salts). The dissolved inorganic salts are present in the water when drawn from the aquifer and carried to the surface. | IR | FFENVSCI |  |
| SEDRIS_T0812 | EE | WATER_TYPE | POTABLE | RD |  | Potable water suitable for human consumption |  |  |  |

Response:	Accepted (there are no definitions at present).

| SEDRIS_T0813 | EE | WATER_TYPE | PRECIPITATION | AC |  |  |  |  | EC_PRECIPITATION_REGION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0814 | EE | WEATHER_TYPE | RAIN | AC |  |  |  |  | EC_RAIN_REGION |
| SEDRIS_T0815 | EE | WEATHER_TYPE | FOG | AC |  |  |  |  | EC_FOG_REGION |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Various Eas are marked as related concepts, but are not similar concepts. On the other hand, some similar concepts aren’t indicated, and some enumerants aren’t defined. |
|  | Solution | Remove the incorrect relationships. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0816 | EE | NAUTICAL_GATE_TYPE | CAISSON | RD |  | A gate at the end of a dry dock that excludes the water after pumping out the dock; it often contains the pumping engines. | IR | FACC, Annex A |  |

| SEDRIS_T0817 | EE | NAUTICAL_GATE_TYPE | CAISSON |  |  |  |  |  | EC_CAISSON |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing definition and related concept. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0818 | EE | RELIGIOUS_DENOMINATION | BUDDHIST | RL | BUDDHISM_UNSPECIFIED |  |  |  |
| SEDRIS_T0819 | EE | RELIGIOUS_DENOMINATION | BUDDHIST | RD |  | A religion based on the life and teachings of Siddhartha Gautama, later known as Buddha. | IR | LEVIN |
| SEDRIS_T0820 | EE | RELIGIOUS_DENOMINATION | CHRISTIAN_UNDEFINED | RL | CHRISTIAN_UNSPECIFIED |  |  |  |
| SEDRIS_T0821 | EE | RELIGIOUS_DENOMINATION | CHRISTIAN_UNDEFINED | RD |  | A religion focusing on the life of Jesus Christ and his teachings, whose denominations fall into 3 broad categories: Eastern Orthodoxy, Protestantism, and Roman Catholicism. | IR | LEVIN |
| SEDRIS_T0822 | EE | RELIGIOUS_DENOMINATION | CONFUCIANISM | A |  | A system of thought developed in China beginning in about 500 B.C.E., founded by Confucius. | IR | LEVIN |
| SEDRIS_T0823 | EE | RELIGIOUS_DENOMINATION | GREEK_ORTHODOX | RL | EASTERN_ORTHODOX |  |  |  |
| SEDRIS_T0824 | EE | RELIGIOUS_DENOMINATION | HINDUISM_UNSPECIFIED | A |  | A collection of many related religious beliefs and practices that are accepted within the framework of Hindu society, where ‘Hindu’ is a catchall term that includes most of the thousands of different religious groups that have evolved in India since 1500 B.C.E. | IR | LEVIN |
| SEDRIS_T0825 | EE | RELIGIOUS_DENOMINATION | MORMONISM | A |  | A Christian sect founded by Joseph Smith, jr. in 1830 during the Second Great Awakening. The term ‘Mormonism’ is applied to the Church of Jesus Christ of LatterDay Saints and such splinter groups as the Reorganized Church of Jesus Christ of LatterDay Saints. | IR | LEVIN |
| SEDRIS_T0826 | EE | RELIGIOUS_DENOMINATION | SIHKISM | A |  | A religion founded by Guru Nanak (1469 1539), primarily practiced in the Punjab region of northwestern India. | IR | LEVIN |
| SEDRIS_T0827 | EE | RELIGIOUS_DENOMINATION | SHINTO | RD |  | The indigenous religion of Japan, also called kaminomichi, “the way of the gods/spirits”. | IR | LEVIN |
| SEDRIS_T0828 | EE | RELIGIOUS_DENOMINATION | MOSLEM | RL | ISLAM_UNSPECIFIED |  |  |  |
| SEDRIS_T0829 | EE | RELIGIOUS_DENOMINATION | MOSLEM | RD |  | A religion based on the tenets expressed in the Quran, i.e. Islam. | IR | LEVIN |
| SEDRIS_T0830 | EE | RELIGIOUS_DENOMINATION | PROTESTANT | RL | PROTESTANTISM_UNSPECIFIED |  |  |  |
| SEDRIS_T0831 | EE | RELIGIOUS_DENOMINATION | PROTESTANT | RD |  | One of the many sects of Western Christianity whose origins can be traced to the Reformation triggered by Martin Luther’s publication of his NinetyFive Theses. | IR | LEVIN |
| SEDRIS_T0832 | EE | RELIGIOUS_DENOMINATION | ROMAN_CATHOLIC | RL | ROMAN_CATHOLICISM |  |  |  |
| SEDRIS_T0833 | EE | RELIGIOUS_DENOMINATION | ROMAN_CATHOLIC | RD |  | The largest sect of Western Christianity, characterized by a highly structured, hierarchical organization headed by the pope. | IR | LEVIN |
| SEDRIS_T0834 | EE | RELIGIOUS_DENOMINATION | SHIITE | A |  | One of the major sects of Islam, found mainly in Iran, distinguished by the belief that divine leadership can only pass to direct relatives of Muhammad and who thus consider Ali as the Prophet’s rightful successor. | IR | LEVIN |
| SEDRIS_T0835 | EE | RELIGIOUS_DENOMINATION | SUFISM | A |  | A mystical movement within Islam which was the main group responsible for spreading Islam beyond its political boundaries in Africa and Asia, and which consists of about seventy Sufi orders. | IR | LEVIN |
| SEDRIS_T0836 | EE | RELIGIOUS_DENOMINATION | SUNNI | A |  | One of major sects of Islam, constituting nearly 90% of the Muslims in the world, distinguished by the belief that the Muslim leader can only come from the Qurayish tribe and who trace their line of succession to Abu Bakr. | IR | LEVIN |
| SEDRIS_T0837 | EE | RELIGIOUS_DENOMINATION | ZEN_BUDDHISM | A |  | A sect of Buddhism believing that one can achieve enlightenment through strict selfdiscipline and meditation; also known as Ch’an. | IR | LEVIN |

Response:	Accepted in principle; see the response to SC_T020 through SC_T048.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Undefined enumerants. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0838 | EA | SPECIFIC_HUMIDITY | RL | ABSOLUTE_HUMIDITY |  |  |  |  |  |  |  |  |
| SEDRIS_T0839 | EA | SPECIFIC_HUMIDITY | RD |  | The amount of water vapour in a unit mass of air, usually expressed in grams of water per kilogram of air. |  |  |  |  | IR | FFENVSCI |  |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Definition does not agree with |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0840 | EE | TREE_TYPE | DECIDUOUS | D |  |  |  |  |  |
| SEDRIS_T0841 | EE | TREE_TYPE | EVERGREEN | D |  |  |  |  |  |

Response:	Rejected.

| SEDRIS_T0842 | EE | TREE_TYPE | EVERGREEN | RD |  | Copy/paste error from DECIDUOUS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Where |  |
| --- | --- | --- |
|  | Problem | VEGETATION_TYPE_DECIDUOUS is redundant with TREE_TYPE_DECIDUOUS, VEGETATION_TYPE_EVERGREEN is redundant with TREE_TYPE_EVERGREEN. If they’re kept, the definition of EVERGREEN needs to be fixed. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0843 | EE | VOLCANIC_FORMATION_TYPE | CALDERA | RD |  | A roughly circular topographic and structural depression, varying in diameter from about 1 to 100 km (but up to 70 km in some martian examples), and formed by the foundering and collapse of a magma chamber roof into its underlying magma body. Slumping and erosion of the caldera walls may enlarge the topographic rim of the depression well beyond the structural rim. Later injection of magma into the chamber can cause doming of the caldera floor to create a resurgent caldera. | IR | DESCI |  |
| SEDRIS_T0844 | EE | VOLCANIC_FORMATION_TYPE | CINDER_CONE | RD |  | Volcanic cone built of pyroclastic material (cinders and scoriaceous ejecta), usually of basaltic or andesitic composition, dominated by fragments 2 64 millimetres in diameter. These airfall deposits build a straightsided cone with slopes of about 30 arc degrees (the angle of repose). Complex varieties may occur, including breached or gaping, and conewithincone types. Also known as scoria cone, ash cone. | IR | DESCI |  |
| SEDRIS_T0845 | EE | VOLCANIC_FORMATION_TYPE | COMPOSITE | RL | STRATO_VOLCANO |  |  |  |  |
| SEDRIS_T0846 | EE | VOLCANIC_FORMATION_TYPE | COMPOSITE | RD |  | A volcano built up of layers of lava alternating with beds of ash and other pyroclastics and with material eroded from higher slopes of the cone. | IR | DESCI |  |
| SEDRIS_T0847 | EE | VOLCANIC_FORMATION_TYPE | SHIELD | RL | SHIELD_VOLCANO |  |  |  |  |
| SEDRIS_T0848 | EE | VOLCANIC_FORMATION_TYPE | SHIELD | RD |  | A volcano with a broad, lowangle cone, produced by a Hawaiiantype eruption. Such an eruption produces very fluid, mobile, basaltic lava, characterized by spectacular fire fountains, with little explosive activity. The lava flows that result are often formed by the accumulation of firefountained lava and are termed ‘spatterfed flows’. | IR | DESCI |  |
| SEDRIS_T0849 | EE | VOLCANIC_FORMATION_TYPE | VOLCANO | RD |  | A naturally occurring vent or fissure at the Earth’s surface through which erupt molten, solid, and gaseous materials. The viscosity, gas content, and rate of extrusion of the magma probably determine the shape of the mountain built by the eruptions. The magma may reach the surface either through a single channel (see central vent volcano) or through a series of vertical fractures (see fissure volcano). | IR | DESCI |  |
| SEDRIS_T0860 | EE | VOLCANIC_FORMATION_TYPE | VOLCANO | RL | VOLCANO_UNSPECIFIED |  |  |  |  |

Response:	Accepted (the editors will integrate will similar ED comments).

|  | Where | VOLCANIC_FORMATION_TYPE |
| --- | --- | --- |
|  | Problem | Undefined enumerants. Reference DESCI = Allaby, A. and M. Allaby. A Dictionary of Earth Sciences. 2nd edition. Oxford: Oxford University Press, 1999. ISBN 0-19-280079-5. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0861 | EA | WEATHER_TYPE | RL | PRESENT_WEATHER |  |  |  |  |  |  |  |
| SEDRIS_T0862 | EA | WEATHER_TYPE | RD |  | A description of the present weather at a location;. Present weather includes precipitation, obscurations, well-developed dust/sand whirls, squalls, tornadic activity, sandstorms, and dust storms. Present weather may be evaluated instrumentally, manually, or through a combination of instrumental and manual methods. |  |  |  |  | IR | FMH1_8 |

Response:	Accepted in principle; revise “present” to “observed” and remove the second sentence from the definition.

|  | #### Where | #### Table 6.26 – EDCS Attributes whose label begins with W & Table D.24 – Eas whose label begins with W |
| --- | --- | --- |
|  | Problem | The enumeration is from the WMO Manual on Codes, International Codes, WMO-Mo. 306 and the code for present weather. By renaming the EAS, it will be clearer that they correspond. Reference FMH1_8= Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 8 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0863 | EE | WETLAND_SURFACE_FORM | ATLANTIC_PLATEAU_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0864 | EE | WETLAND_SURFACE_FORM | BASIN_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0865 | EE | WETLAND_SURFACE_FORM | BASIN_SWAMP | AC |  |  |  |  | EC_SWAMP |
| SEDRIS_T0866 | EE | WETLAND_SURFACE_FORM | BLANKET_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0867 | EE | WETLAND_SURFACE_FORM | DOMED_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0868 | EE | WETLAND_SURFACE_FORM | FLAT_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0869 | EE | WETLAND_SURFACE_FORM | LOWLAND_POLYGON_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0870 | EE | WETLAND_SURFACE_FORM | PALSA_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0871 | EE | WETLAND_SURFACE_FORM | PEAT_PLATEAU_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0872 | EE | WETLAND_SURFACE_FORM | POLYGONAL_PEAT_PLATEAU_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0873 | EE | WETLAND_SURFACE_FORM | SHALLOW_BASIN | AC |  |  |  |  | EC_MARSH |
| SEDRIS_T0874 | EE | WETLAND_SURFACE_FORM | SHORE_MARSH | AC |  |  |  |  | EC_MARSH |
| SEDRIS_T0875 | EE | WETLAND_SURFACE_FORM | SLOPE_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0876 | EE | WETLAND_SURFACE_FORM | STREAM_MARSH | AC |  |  |  |  | EC_MARSH |
| SEDRIS_T0877 | EE | WETLAND_SURFACE_FORM | STREAM_SWAMP | AC |  |  |  |  | EC_SWAMP |
| SEDRIS_T0878 | EE | WETLAND_SURFACE_FORM | STRING_BOG | AC |  |  |  |  | EC_BOG |
| SEDRIS_T0879 | EE | WETLAND_SURFACE_FORM | VENEER_BOG | AC |  |  |  |  | EC_BOG |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing related concepts for WETLAND_SURFACE_FORM. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0888 | EE | WELL_TYPE | ARTESIAN | RD |  | A well drilled into a confined geological stratum holding water that is draining from an area higher than the wellhead. The resulting pressure forces the water to the surface, eliminating the need for a pump. A freeflowing well. | IR | FFENVSCI |  |
| SEDRIS_T0889 | EE | WELL_TYPE | WATER_HOLE | RD |  | A small natural depression in which water collects, especially a pool where animals come to drink. | IR | AHD |  |
| SEDRIS_T0890 | EE | WELL_TYPE | WATER_HOLE | RL | WATERING_HOLE |  |  |  |  |

Response:	Accepted.

| SEDRIS_T0891 | EE | WELL_TYPE | WATER_HOLE | AC |  |  |  |  | EC_WATERING_HOLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0892 | EE | WELL_TYPE | FOUNTAIN | RD |  | An artificial spring with water. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T0893 | EE | WELL_TYPE | FOUNTAIN | AC |  |  |  |  | EC_FOUNTAIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing definitions, related concepts for WELL TYPE. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0894 | EE | OBSERVATION_PLATFORM_TYPE | AIRCRAFT | AC |  |  |  |  | EC_AIRCRAFT |
| SEDRIS_T0895 | EE | OBSERVATION_PLATFORM_TYPE | MOBILE_GROUND_VEHICLES | AC |  |  |  |  | EC_GROUND_VEHICLE |
| SEDRIS_T0896 | EE | OBSERVATION_PLATFORM_TYPE | SHIP | AC |  |  |  |  | EC_WATER_SURFACE_VEHICLE |
| SEDRIS_T0897 | EE | TRANSPORTATION_USE | AIRCRAFT | AC |  |  |  |  | EC_AIRCRAFT |
| SEDRIS_T0898 | EE | TRANSPORTATION_USE | AQUEDUCT | AC |  |  |  |  | EC_AQUEDUCT |
| SEDRIS_T0899 | EE | TRANSPORTATION_USE | BOAT | AC |  |  |  |  | EC_WATER_SURFACE_VEHICLE |
| SEDRIS_T0900 | EE | TRANSPORTATION_USE | BUS | AC |  |  |  |  | EC_GROUND_VEHICLE |
| SEDRIS_T0901 | EE | TRANSPORTATION_USE | CANAL | AC |  |  |  |  | EC_CANAL |
| SEDRIS_T0902 | EE | TRANSPORTATION_USE | RAILROAD | AC |  |  |  |  | EC_RAILROAD |
| SEDRIS_T0903 | EE | TRANSPORTATION_USE | SHIP | AC |  |  |  |  | EC_WATER_SURFACE_VEHICLE |
| SEDRIS_T0904 | EE | TRANSPORTATION_USE | SUBWAY | AC |  |  |  |  | EC_SUBWAY |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Missing related concepts. |

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def |  | Value Type |  | Related Concepts |  | Ref Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0905 | EA | OCEAN_SURFACE_TEMPERATURE | RD |  | Temperature of the water measured, estimated, or predicted at or near the ocean surface. Ocean surface temperature may qualified by other attributes. (e.g.: OCEAN_SURFACE_OBS_POINT) |  |  |  |  |  |  |  |
| SEDRIS_T0906 | EA | OCEAN_SURFACE_TEMPERATURE | RT |  |  |  |  |  |  |  | ND |  |
| SEDRIS_T0907 | EA |  | A | OCEAN_SURFACE_OBS_POINT | The point relative to ocean surface at which water characteristics are observed, measured or collected. |  | ENUMERATED |  | OCEAN_SURFACE |  | ND |  |

Response:	Accepted.

| SEDRIS_T0908 | EA | OCEAN_BASIN | RD |  | The basin (depression of the sea floor more or less equidimensional in plan view and of variable extent) in which an ocean lies. |  |  |  |  |  | IR | BOWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0909 | EA |  | A | SEA_BASIN | The basin (depression of the sea floor more or less equidimensional in plan view and of variable extent) in which a sea lies. |  | ENUMERATED |  | OCEAN, BATHYMETRY |  | IR | BOWD |

Response:	AI 08-31: Louis Hembree will investigate (include EC OCEAN, EC SEA, these two EAs and the proposed enumerants for correctness and completeness).

|  | Table 6.18 |
| --- | --- |
|  | Action Item 9: Ocean_surface_temperature is confusing. Determine how to handle it in EDCS. Scrub EA_OCEAN_BASIN enumerant list. Current list is a combination of OCEAN_BASIN and SEA_BASIN. |
|  | Clarify the definition and add an observation point attribute. Add SEA_BASIN. Clean up OCEAN_BASIN enumerants to correspond to the appropriate EA, either OCEAN_BASIN or SEA_BASIN. Reference BOWD = Bowditch, American Practical Navigator, DMA/HTC, 1981 |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Def | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0910 | EE | OCEAN_SURFACE_OBS_POINT |  | A | OCEAN_ACTUAL_SURFACE | The actual ocean surface (e.g., at the crest, midpoint, or trough of any wave present) | ND |  |
| SEDRIS_T0911 | EE | OCEAN_SURFACE_OBS_POINT |  | A | OCEAN_BELOW_ACTUAL_SURFACE | A point within ten meters below the actual ocean surface. | IR | U.S.Navy Aerographer’s Mate Rate Training Manual, Module 2, Miscellaneous Observations and Codes |
| SEDRIS_T0912 | EE | OCEAN_SURFACE_OBS_POINT |  | A | OCEAN_SFC_SHIP_INJECTION | The depth of the ship’s seawater intake injection port. | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 1 |
| SEDRIS_T0913 | EE | OCEAN_SURFACE_OBS_POINT |  | A | OCEAN_SURFACE_GENERIC | A generic (undefined) surface at the top of the ocean. | ND |  |

Response:	Accepted; the editors will revise the Labels to remove “OCEAN”.

| SEDRIS_T0914 | EE | OCEAN_BASIN | ARCTIC | RT |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0915 | EE | OCEAN_BASIN | ATLANTIC | RT |  |  | ND |  |
| SEDRIS_T0916 | EE | OCEAN_BASIN | BALTIC | D |  |  |  |  |
| SEDRIS_T0917 | EE | OCEAN_BASIN | INDIAN | RT |  |  | ND |  |
| SEDRIS_T0918 | EE | OCEAN_BASIN | MEDITERRANEAN | D |  |  |  |  |
| SEDRIS_T0919 | EE | OCEAN_BASIN | N_ATLANTIC | RT |  |  | ND |  |
| SEDRIS_T0920 | EE | OCEAN_BASIN | N_PACIFIC | RT |  |  | ND |  |
| SEDRIS_T0921 | EE | OCEAN_BASIN | NORWEGIAN_SEA | D |  |  |  |  |
| SEDRIS_T0922 | EE | OCEAN_BASIN | PACIFIC | RT |  |  | ND |  |
| SEDRIS_T0923 | EE | OCEAN_BASIN | S_ATLANTIC | RT |  |  | ND |  |
| SEDRIS_T0924 | EE | OCEAN_BASIN | S_PACIFIC | RT |  |  | ND |  |
| SEDRIS_T0925 | EE | SEA_BASIN | NORWEGIAN | A |  | ? | ND |  |
| SEDRIS_T0926 | EE | SEA_BASIN | BALTIC | A |  | Baltic Sea | ND |  |
| SEDRIS_T0927 | EE | SEA_BASIN | BLACK | A |  | Black Sea | ND |  |
| SEDRIS_T0928 | EE | SEA_BASIN | CARIBBEAN | A |  | Caribbean Sea | ND |  |
| SEDRIS_T0929 | EE | SEA_BASIN | CASPIAN | A |  | Caspian Sea | ND |  |
| SEDRIS_T0930 | EE | SEA_BASIN | MEDITERRANEAN | A |  | Mediterranean Sea | ND |  |
| SEDRIS_T0931 | EE | SEA_BASIN | NORWEGIAN | A |  | Norwegian Sea | ND |  |
| SEDRIS_T0932 | EE | SEA_BASIN | ADRIATIC | A |  | Adriatic Sea | ND |  |

Response:	Accepted.

|  | Table 6.18 |
| --- | --- |
|  | Action Item 9: Scrub EA_OCEAN_BASIN enumerant list. Current list is a combination of OCEAN_BASIN and SEA_BASIN. |
|  | Add SEA_BASIN. Clean up OCEAN_BASIN enumerants to correspond to the appropriate EA, either OCEAN_BASIN or SEA_BASIN. |

| SEDRIS_T0933 | EA | PICTORIAL_REPRESENTATION | RD |  | Indicator of availability of pictorial representation. |  |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0934 | EA | PICTORIAL_REPRESENTATION | RV |  |  |  | STRING (can accommodate an 80-character file reference, a URL, or simple “existence of” statement) |  |  |  |  |  |

Response:	AI 08-23: Paul Foley will investigate (see response to US_T453).

|  | Table 6.19 |
| --- | --- |
|  | Action Item 11. Check on PICTORIAL_REPRESENTATION from FACC. |
|  | Changed definition and value type to correspond to FACC usage. |

| SEDRIS_T0935 | EA |  | A | OBSTRUCTION | Anything that stops or blocks a way or passage; an obstructing obstacle. This allows for the attribution of objects as obstructions, rather than the identification of an object simply as an obstruction. |  | LOGICAL |  | NAUTICAL, AEROUNAUTICS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted with: deletion of the second sentence, revise definition to better fit this attribute concept, and add “PHYSICAL_” to the Label.

| SEDRIS_T0936 | Withdrawn |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0937 | EA |  | A | SFC_RFLCT_DIR_HEMI | The ratio of the total hemispherically integrated reflected flux from a sample to the directional flux (directional irradiance integrated over the directional solid angle). Is generally a function of incidence angle. Also known as the hemispherical albedo, plane albedo and hemispherical reflectance. This quantity may be obtained by integrating the bi-directional reflectance over all angles of reflection. This attribute can further be clarified by other attributes, e.g., ELECTROMAGNETIC_BAND. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0938 | EA |  | A | INCIDENCE_ANGLE | The angle subtended by a source vector of radiation and the surface normal |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | WATT |
| SEDRIS_T0939 | EA |  | A | GLINT_ANGLE | The angle subtended by the glint vector and the surface normal, where the glint vector is the vector that bisects the source vector and the viewing vector at a surface. Used by the Phong and Sandford-Robertson BRDF models. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | WATT |
| SEDRIS_T0940 | EA |  | A | VOL_SINGLE_SCAT_ALBEDO | The ratio of the total amount of power scattered by a particle volume to the total power removed (total extinction) from the incident wave |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0941 | EA |  | A | OPP_EFFECT_ANG_WIDTH | A factor governing the angular half-width of the opposition-effect component of the Hapke BRDF model. It is equal to half product of the particle extinction radious and the volume extinction coefficient. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0942 | EA |  | A | VOL_PHASE_FUNCTION | A factor, generally a function of g, the phase angle between the source vector and viewing vector, that describes the angular pattern into which an incident radiance is scattered by a volume of particles. Sometimes assumed incidence angle independent and used as a scalar. Used by the Hapke and Beard-Maxwell BRDF Models. |  | REAL | RECIPROCAL STERADIAN | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0943 | EA |  | A | PHASE_ANGLE_HAPKE | The angle between the source vector of illumination and the viewing vector. Used by the Hapke and Beard-Maxwell models. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0944 | EA |  | A | DIR_HEMI_REFL_PERP_POLARIZED | The directional-hemispherical (diffuse) reflectance with a perpendicularly polarized source. Generally a function of incidence angle, but without angular dependence specified, assumed incidence angle independent. |  | REAL | INV_STERADIAN | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0945 | EA |  | A | DIR_HEMI_REFL_PARAL_POLARIZ | The directional-hemispherical (diffuse) reflectance with a parallel polarized source. Generally a function of incidence angle, but without angular dependence specified, assumed incidence angle independent. |  | REAL | INV_STERADIAN | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0946 | EA |  | A | FRESNEL_REFL_NORMAL_INC | Fresnel reflectance at normal incidence. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0947 | EA |  | A | BEARD_MAXWELL_SHOBS_TAU_PARM | One of two input parameters to the Shadowing and Obscuration term in the Beard-Maxwell BRDF model. Used to curve-fit to actual BRDF measurements It is the denominator of the exponential argument in this term. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0948 | EA |  | A | BEARD_MAXWELL_SHOBS_OMEGA | One of two input parameters to the Shadowing and Obscuration term in the Beard-Maxwell BRDF model. It is used to curve-fit to actual BRDF measurements It is the denominator of the of the fraction involving incidence angle in this term. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0949 | EA |  | A | ZERO_BISTATIC_BRDF | Zero-bistatic BRDF measurement as a function of incidence angle from the surface normal. It is the BRDF measured in the back-direction of the source. |  | REAL | INV_ STERADIAN | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0950 | EA |  | A | COS_BETA_ANGLE | The cosine of the angle over which a zero bistatic BRDF measurement is made. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0951 | EA |  | A | SPECULAR_LOBE_SIZE | The size of the of specular lobe in the Beard Maxwell BRDF model. |  | REAL | STERADIAN | EM_PHENOMENON |  | IR | Nonconventional Exploitation Factors (NEF) Modeling ORD 257-96 (National Institute of Standards and Technology (NIST), 1996) |
| SEDRIS_T0952 | EA |  | A | OPPOSITION_EFFECT_AMPL | The peak amplitude of the opposition-effect component of the Hapke BRDF Model. It is the ratio of the light scattered from near the illuminated surface of a particle volume to the total amount of light scattered at zero phase angle, (g=0), where the phase angle g is the angle between the source vector and viewing vector. This term and the Opposition Effect Angular Width Parameter are sufficient to define the Opposition Effect Factor as a function of phase angle. |  | REAL | UNITLESS | EM_PHENOMENON |  | IR | HAPKE |
| SEDRIS_T0953 | EA | BRDF | RT |  |  |  | REAL | UNITLESS |  |  | IR | HAPKE |

Response:	Accepted in principle, the editors will review and incorporate.

| SEDRIS_T0954 |  | CRITICAL_LAYER_ALTITUDE_E | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0955 |  | CRITICAL_LAYER_ALTITUDE_F1 | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| SEDRIS_T0956 |  | CRITICAL_LAYER_ALTITUDE_F2 | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| SEDRIS_T0957 |  | CRITICAL_LAYER_FREQUENCY_E | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| SEDRIS_T0958 |  | CRITICAL_LAYER_FREQUENCY_F1 | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| SEDRIS_T0959 |  | CRITICAL_LAYER_FREQUENCY_F2 | RR |  |  |  |  |  |  | IR | Space Physics Text Book of Oulu, http://www.oulu.fi/~spaceweb/textbook/ionosphere.html |
| SEDRIS T0960 to T0961 | Withdrawn |  |  |  |  |  |  |  |  |  |  |

Response:	Accepted.

|  | Action Item 16: Examine SURFACE_THERMAL_INDEX and SURFACE_THERMAL_MODEL. |
| --- | --- |
|  | Eliminate “model-specific” language, identifying the particular data being specified by the model-dependent “thermal index.” Reference HAPKE = Theory of Reflectance and Emittance Spectroscopy (Hapke, B., 1993, Cambridge Press: York) Reference WATT = Advanced Animation and Rendering Techniques (Watt, A. and Watt, M., 1992, ACM Press: Essex, England) |

| SEDRIS_T0962 | EA | BREAKER_ANGLE_DIRECTION | RL | BREAKER_PRIMARY_ANGLE_DIR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	AI 08-36: Louis Hembree will research.

| SEDRIS_T0963 | EA | BREAKER_ANGLE_DIRECTION | AC |  |  |  |  |  | OCEAN, OCEAN_HYDRODYNAMICS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	The editors will determine whether to accept the proposed changes based on the results of AI 08-12 assigned to the editors (see response to SEDRIS_T0024) to clarify the intent of “related concept” and its utility.

| SEDRIS_T0964 | EA | BREAKER_PRIMARY_ANGLE_DIR | RR |  |  |  |  |  |  |  | IR | NAMRTM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	AI 08-36: Louis Hembree will research.

| SEDRIS_T0965 | EA |  | A | BREAKER_HEIGHT | The average height of the highest one-third of 100 observed breaking waves. |  | REAL | LENGTH | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0966 | EA |  | A | SWELL_PRIMARY_WAVE_DIR | The horizontal angle, measured in a clockwise direction from above, from planetary geodetic north to the direction from which the primary swell waves are arriving |  | REAL | PLANE_ANGLE | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 2 |
| SEDRIS_T0967 | EA |  | A | SWELL_SECONDARY_WAVE_DIR | The horizontal angle, measured in a clockwise direction from above, from planetary geodetic north to the direction from which the secondary swell waves are arriving. |  | REAL | PLANE_ANGLE | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 2 |
| SEDRIS_T0968 | EA |  | A | SWELL_PRIMARY_WAVE_PERIOD | The time it takes, usually measured in seconds, for a complete cycle (usually crest to crest or trough to trough) of the primary long-range swell wave to pass a given fixed point. |  | REAL | TIME | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |
| SEDRIS_T0969 | EA |  | A | SWELL_SECONDARY_WAVE_PERIOD | The time it takes, usually measured in seconds, for a complete cycle (usually crest to crest or trough to trough) of the secondary long-range swell wave to pass a given fixed point. |  | REAL | TIME | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |
| SEDRIS_T0970 | EA |  | A | SWELL_PRIMARY_WAVE_HEIGHT | The average height of the highest 1/3 of observed primary long-range swell waves. |  | REAL | LENGTH | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |
| SEDRIS_T0971 | EA |  | A | SWELL_SECONDARY_WAVE_HEIGHT | The average height of the highest 1/3 of observed secondary long-range swell waves. |  | REAL | LENGTH | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |
| SEDRIS_T0972 | EA |  | A | WIND_INDUCED_WAVE_PERIOD | The time it takes, usually measured in seconds, for a complete cycle (usually crest to crest or trough to trough) of a wind-induced wave to pass a given fixed point. |  | REAL | TIME | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |
| SEDRIS_T0973 | EA |  | A | WIND_INDUCED_WAVE_HEIGHT | The average height of the highest 1/3 observed wind-induced waves |  | REAL | LENGTH | OCEAN, OCEAN_HYDRODYNAMICS |  | IR | U.S. Navy Aerographer’s Mate Rate Training Manual |

Response:	AI 08-36: Louis Hembree will research.

|  | Where | Table 6.5 (B) and Table 6.26 (W) |
| --- | --- | --- |
|  | Problem | Action Item 17. Examine WAVE_HEIGHT, BREAKER_HEIGHT, WAVE_PERIOD, WAVE_DIRECTION, and BREAKER_DIRECTION to determine appropriate codes for primary and secondary waves. |
|  | Solution | Separate waves into wind-waves and swell-waves. Note primary and secondary swell. Include wave period in wind and swell waves. Note that breakers can have primary and secondary directions, but only primary directions are reported. Reference NAMRTM = U.S. Navy Aerographer’s Mate 1&C Rate Training Manual, NAVEDTRA, revised 1985. |

| Comment Tag | Dic | Current Label | Chg Type | Label | Defn |  |  |  |  |  | Ref Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0974 | EA | WATER_BOTTOM_MORPHOLOGY | RD | . | The form and structure of the bottom (floor) of water basins |  |  |  |  |  |  |  |
| SEDRIS_T0975 | EA | WATER_BOTTOM_MORPHOLOGY | RT |  |  |  |  |  |  |  | ND |  |

Response:	Accepted in principle; the editors will review in concert with IHO S-57 comments from ED.

| SEDRIS_T0976 | EA | WATER_BOTTOM_OBJECT_TYPE | RL | WATER_BOTTOM_TRANS_OBJECT (water_bottom_transient_object) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0977 | EA | WATER_BOTTOM_OBJECT_TYPE | RD |  | Object located on the bottom of a water body which is transient, i.e., impermanent. |  |  |  |  |  |  |  |

Response:	Rejected.

|  | Table 6.26 |
| --- | --- |
|  | Action Item 19. Review WATER_BOTTOM_MORPHOLOGY, WATER_BOTTOM_OBJECT_TYPE. Determine whether PINNACLE is a valid enumerant. |
|  | Clarify definitions. Note that WATER_BOTTOM_OBJECT_TYPE indicates transient, rather than permanent, objects. Delete PINNACLE, since it is a permanent bottom feature. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0978 | EE | WATER_BOTTOM_OBJECT_TYPE | FISH_HAVEN | RD |  | A haven for fish. | ND |  |
| SEDRIS_T0979 | EE | WATER_BOTTOM_OBJECT_TYPE | LESS_WATER_REPORTED | RD |  | Reports of less water (indicating presence of an unknown bottom object) received. | ND |  |

Response:	Accepted (unclear what “less water reported” means).

| SEDRIS_T0980 | EE | WATER_BOTTOM_OBJECT_TYPE | PINNACLE | D |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T0981 | EE | WATER_BOTTOM_OBJECT_TYPE | SHOALING | RD |  | Bottom is shoaling, indicating possible presence of a bottom object. | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0982 | EE | WATER_BOTTOM_OBJECT_TYPE | UNEXPLODED_ORDNANCE | RD |  | Unexploded ordnance. | ND |  |
| SEDRIS_T0983 | EE | WATER_BOTTOM_OBJECT_TYPE | UNKNOWN_OBSTRUCTION | RD |  | Unknown obstruction. | ND |  |
| SEDRIS_T0984 | EE | WATER_BOTTOM_OBJECT_TYPE | WELL | RD |  | Remains of a well located on the bottom. | ND |  |
| SEDRIS_T0985 | EE | WATER_BOTTOM_OBJECT_TYPE | WRECKAGE | RD |  | Wreckage. | ND |  |

Response:	Accepted.

|  | #### Table 6.54 – EDCS Attribute enumerant codes for attributes whose label begins with W |
| --- | --- |
|  | Action Item 19. Review WATER_BOTTOM_OBJECT_TYPE. May need to remove PINNACLE. |
|  | Cleaned up definitions. Removed PINNACLE, as it is not a transient feature. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0986 | EE | WATER_CURRENT_TYPE | GENERAL_FLOW | D |  |  |  |  |
| SEDRIS_T0987 | EE | WATER_CURRENT_TYPE | RIFLOW | D |  |  |  |  |

Response:	The editors will investigate in concert with integration of comments on IHO S-57 from ED.

| SEDRIS_T0989 | EE | WATER_CURRENT_TYPE | EBB | RL | TIDAL_EBB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T0990 | EE | WATER_CURRENT_TYPE | EBB | RD |  | The outflow (ebb) period of a tidal current. | ND |  |
| SEDRIS_T0991 | EE | WATER_CURRENT_TYPE | FLOOD | RL | TIDAL_FLOOD |  |  |  |
| SEDRIS_T0992 | EE | WATER_CURRENT_TYPE | FLOOD | RD |  | The inflow (flood) period of a tidal current. | ND |  |
| SEDRIS_T0993 | EE | WATER_CURRENT_TYPE | LONGSHORE | RD |  | A current which flows horizontal to a coastline. | ND |  |
| SEDRIS_T0994 | EE | WATER_CURRENT_TYPE | OCEAN_FLOW | RL | OCEAN |  |  |  |
| SEDRIS_T0995 | EE | WATER_CURRENT_TYPE | OCEAN_FLOW | RD |  | A permanent current which exists in ocean waters. | ND |  |
| SEDRIS_T0996 | EE | WATER_CURRENT_TYPE | RIP | RD |  | Current produced by water piling up along a shore which is deflected by an opposing current or bottom irregularities, causing a current flow in excess of 0.5 kt. | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 2. |
| SEDRIS_T0997 | EE | WATER_CURRENT_TYPE | TIDAL | A | TIDAL | Current caused by the attraction of the moon and, to a lesser degree, of the sun. | ND |  |
| SEDRIS_T0999 | EE | WATER_CURRENT_TYPE | WIND_DRIVEN | A | WIND_DRIVEN | Current generated by the drag of a semi-permanent atmospheric wind on the surface of a water body. | ND |  |
| SEDRIS_T1000 | EE | WATER_CURRENT_TYPE | LITTORAL | A |  | Another name for LONGSHORE current. | IR | U.S. Navy Aerographer’s Mate Rate Training Manual, Module 2 |

Response:	Accepted in principle; the editors will review in concert with US and ED comments on these concepts.

|  | Table 6.54 |
| --- | --- |
|  | Action Item 20. Check out GENERAL_FLOW, RIFLOW before deletion. Validate the enumerant domain… |
|  | Since neither is a valid current type (per U.S. Navy Aerographer’s Mate Rate Training Manual), delete them both. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T2000 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 1_HZ | 1 Hertz | UD | none |
| ###### SEDRIS_T1001 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 1r25_HZ | 1,25 Hertz | UD | none |
| ###### SEDRIS_T1002 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 1r5_HZ | 1,5 Hertz | UD | none |
| ###### SEDRIS_T1003 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 2_HZ | 2 Hertz | UD | none |
| ###### SEDRIS_T1004 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 2r5_HZ | 2,5 Hertz | UD | none |
| ###### SEDRIS_T1005 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 3r15_HZ | 3,15 Hertz | UD | none |
| ###### SEDRIS_T1006 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 4_HZ | 4 Hertz | UD | none |
| ###### SEDRIS_T1007 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 5_HZ | 5 Hertz | UD | none |
| ###### SEDRIS_T1008 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 6r3_HZ | 6,3 Hertz | UD | none |
| ###### SEDRIS_T1009 | EE | ACOUSTIC_FREQUENCY_CODED |  | A | 8_HZ | 8 Hertz | UD | none |

Response:	Accepted.

|  | #### The One Third octave data should start at 1 hertz to cover the modeling of the ocean. |
| --- | --- |
|  | Add this enumerant. |

| ###### SEDRIS_T1010 | EE | SEASON | SPRING | RD |  | The season of the year in which plants begin to vegetate. In the Northern Hemisphere, spring begins astronomically at the vernal equinox and ends at the summer solstice. In the Southern Hemisphere, the limits are the autumnal equinox and the winter solstice. | IR | BOWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T1011 | EE | SEASON | SUMMER | RD |  | The warmest season of the year. In the Northern Hemisphere, summer begins astronomically at the summer solstice and ends at the autumnal equinox. In the Southern Hemisphere, the limits are the winter solstice and the vernal equinox. | IR | BOWD |
| ###### SEDRIS_T1012 | EE | SEASON | FALL | RD |  | The season marking the end of the growing season. In the Northern Hemisphere, autumn begins astronomically at the autumnal equinox and ends at the winter solstice. In the Southern Hemisphere, the limits are the vernal equinox and the summer solstice. | IR | BOWD |
| ###### SEDRIS_T1013 | EE | SEASON | WINTER | RD |  | The coldest season of the year. In the Northern Hemisphere, winter begins astronomically at the winter solstice and ends at the vernal equinox. In the Southern Hemisphere, the limits are the summer solstice and the autumnal equinox. |  |  |

Response:	Rejected; use the NSOED “default” definitions (need to specify the astronomic versions).

| ###### SEDRIS_T1014 | EE | SEASON | FALL | RL | AUTUMN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Table 6.50 |
| --- | --- |
|  | Action Item 18. Check in BOWDITCH for definition and reference for SEASON enumerants. |
|  | Provide sound scientific definitions. Change “FALL” to official term “AUTUMN.” Reference BOWD = Bowditch, American Practical Navigator, DMA/HTC, 1981 |

| SEDRIS_T1015 | EE | NSC_SOURCE | COMNAVFE | RD |  | Commander, Naval Forces Far East (a post-War operational command) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1016 | EE | NSC_SOURCE | COMNAVPAC | RD |  | Commander, Naval Forces Pacific (a post-War operational command) |  |  |

Response:	Accepted in principle; the editors will integrate with similar comments from US on these concepts.

|  | Table 6.45 |
| --- | --- |
|  | Action Item 24. Check out enumerants for NSC_SOURCE. |
|  | Verify expansion of acronyms. |

| ###### SEDRIS_T1017 | EE | MINE_OTHER_POSITION_TYPE | ACTIVE | D |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle; the editors will investigate.

| ###### SEDRIS_T1018 | EE | MINE_INFO_ANTI_HUNTING | ACOUSTIC_IMPEDANCE | RD |  | The capability of a mine to avoid detection using acoustic impedance. | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T1019 | EE | MINE_INFO_ANTI_HUNTING | ACOUSTIC_TRANSPARENCY | RD |  | The capability of a mine to avoid detection by appearing acoustically transparent. | ND |  |
| ###### SEDRIS_T1020 | EE | MINE_INFO_ANTI_HUNTING | ANECHOIC_COATING | RD |  | The capability of a mine to avoid detection by reason of its anechoic coating. | ND |  |
| ###### SEDRIS_T1021 | EE | MINE_INFO_ANTI_HUNTING | AUTOMATIC_MINE_BURIAL | RD |  | The capability of a mine to avoid detection through automatic case burial | ND |  |
| ###### SEDRIS_T1022 | EE | MINE_INFO_ANTI_HUNTING | IRREGULAR_SHAPING | RD |  | The capability of a mine to avoid detection by possessing an irregularly-shaped case. | ND |  |
| ###### SEDRIS_T1023 | EE | MINE_INFO_ANTI_HUNTING | NON_METALLIC_CASE | RD |  | The capability of a mine to avoid detection by possessing a non-metallic case. | ND |  |
| ###### SEDRIS_T1024 | EE | MINE_INFO_ANTI_HUNTING | SONAR_DECOY | RD |  | The capability of a mine to avoid detection by possessing sonar decoys | ND |  |
| ###### SEDRIS_T1025 | EE | MINE_INFO_ANTI_RECOVERY | MOORING_LEVEL_SWITCH | RD |  | The capability of a mine to prevent recovery through a mooring-level switch. | ND |  |
| ###### SEDRIS_T1026 | EE | MINE_INFO_ANTI_RECOVERY | STRIPPING_EQUIPMENT | RD |  | The capability of a mine to prevent recovery using stripping equipment. | ND |  |
| ###### SEDRIS_T1027 | EE | MINE_INFO_ANTI_SWEEP_WIRE | CHAIN_MOORING | RD |  | The capability of a mine to resist being swept by possessing a chain mooring. | ND |  |
| ###### SEDRIS_T1028 | EE | MINE_INFO_ANTI_SWEEP_WIRE | CUTTERS | RD |  | The capability of a mine to resist being swept by possessing cutters. | ND |  |
| ###### SEDRIS_T1029 | EE | MINE_INFO_ANTI_SWEEP_WIRE | GRAPNEL | RD |  | The capability of a mine to resist being swept by having grapnel hooks. | ND |  |
| SEDRIS_T1030 | EE | MINE_INFO_ANTI_SWEEP_WIRE | SENSITIVE_TUBING | RD |  | The capability of a mine to resist being swept by having sensitive-tubing. | ND |  |
| SEDRIS_T1031 | EE | MINE_INFO_ANTI_SWEEP_WIRE | SPROCKET | RD |  | The capability of a mine to resist being swept by having sprockets. | ND |  |
| SEDRIS_T1032 | EE | MINE_STATUS | FOULED | RD |  | Mine anchor is fouled, i.e., it has picked up a cable, debris, rock, or coral, or is wrapped in its own chain. | IR | U.S. Navy MINEMAN Rate Training Manual, Vol. 7 |

Response:	Accepted.

|  | Table 6.44 |
| --- | --- |
|  | Action Item 36-39. Research MINE_ACOUSTIC_ACTUATION frequency ranges. Fill in definitions of MINE_INFO_ANTI_SWEEP_WIRE, ANTI_HUNTING, and ANTI_RECOVERY. Check on ACTIVE as a mine position type. Define FOULED mines. |
|  | MINE_ACOUSTIC_ACTUATION frequency ranges are classified; therefore, they are not available for insertion into this document. Definitions and references added for MINE_INFO_ANTI_SWEEP_WIRE, ANTI_HUNTING, and ANTI_RECOVERY. ACTIVE was deleted as a mine position type. FOULED mine defined. |

| SEDRIS_T1033 | Action Item 26. Reynolds number is dimensionless, but on the scale of 1 X 10**5. See: http://www.efunda.com/formulae/fluids/calc_reynolds.cfm |
| --- | --- |

Response:	The editors will investigate in conjunction with other NB comments on this concept.

| SEDRIS_T1034 | Action Item 34. No reference to OCEAN_PROUD explosive mine was found. It may be that the mine is prow-mounted, but I cannot confirm that. |
| --- | --- |

Response:	The editors will investigate.

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T1035 | EE | BOT_RETURN_OBSTACLE_STATUS | CLASSIFIED | AD |  | The classification of the return of the side scan sonar as an OBSTACLE. | UD | none |
| ###### SEDRIS_T1036 | EE | BOT_RETURN_ROCK_STATUS | CLASSIFIED | AD |  | The classification of the return of the side scan sonar as a rock. | UD | none |
| ###### SEDRIS_T1037 | EE | BOT_RETURN_SEABED_STATUS | CLASSIFIED | AD |  | The classification of the return of the side scan sonar as SEABED. | UD | none |
| ###### SEDRIS_T1038 | EE | BOT_RETURN_STATUS | CLASSIFIED | AD |  | The classification of the return of the side scan sonar as bottom. | UD | none |
| ###### SEDRIS_T1039 | EE | BOT_RETURN_WRECK_STATUS | CLASSIFIED | AD |  | The classification of the return of the side scan sonar as a wreck. | UD | None |
| SEDRIS_T1040 | EE | BOT_RETURN_OBSTACLE_STATUS | DETECTED | AD |  | The initial observation that the OBSTACLE exists. | UD | none |
| SEDRIS_T1041 | EE | BOT_RETURN_ROCK_STATUS | DETECTED | AD |  | The initial observation that the rock exists. | UD | none |
| SEDRIS_T1042 | EE | BOT_RETURN_SEABED_STATUS | DETECTED | AD |  | The initial observation that the SEABED exists. | UD | none |
| SEDRIS_T1043 | EE | BOT_RETURN_STATUS | DETECTED | AD |  | The initial observation that the bottom return. | UD | None |
| SEDRIS_T1044 | EE | BOT_RETURN_WRECK_STATUS | DETECTED | AD |  | The initial observation that the wreck exists. | UD | None |
| ###### SEDRIS_T1045 | EE | BOT_RETURN_OBSTACLE_STATUS | IDENTIFIED | AD |  | The final identification of the OBSTACLE on a navigation chart or map. | UD | none |
| ###### SEDRIS_T1046 | EE | BOT_RETURN_ROCK_STATUS | IDENTIFIED | AD |  | The final identification of the rock on a navigation chart or map. | UD | none |
| ###### SEDRIS_T1047 | EE | BOT_RETURN_SEABED_STATUS | IDENTIFIED | AD |  | The final identification of the SEABED on a navigation chart or map. | UD | none |
| ###### SEDRIS_T1048 | EE | BOT_RETURN_STATUS | IDENTIFIED | AD |  | The final identification of the bottom. | UD | None |
| ###### SEDRIS_T1049 | EE | BOT_RETURN_WRECK_STATUS | IDENTIFIED | AD |  | The final identification of the wreck on a navigation chart or map. | UD | None |

Response:	Accepted (currently these concepts have no definition).

|  | #### The definitions are missing. |
| --- | --- |
|  | Add these definitions. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### SEDRIS_T1050 | EE | SNOISE_SEASON_TYPE | LAND | D |  |  |  |  |

Response:	AI 08-33: Paul Berner will investigate and propose refinement of the EA definition and/or enumerants as appropriate.

|  | #### This enumerant is incorrect for this EA. |
| --- | --- |
|  | Remove the EE |

| Comment Tag | Dic | Current EA Label | Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1189 | EA | BARGE_LOAD_RSTRCTS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1190 | EA | BEARING_FROM_SEAWARD | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1191 | EA | BERTH_IDENT | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1192 | EA | CALM_SEA_STATE_FRACTION | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1193 | EA | CENTER_FREQUENCY_BAND | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1194 | EA | COLOUR_INFORMATION | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1195 | EA | COMMUNICATION_CHANNEL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1196 | EA | COVERED_DRAIN | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1197 | EA | CURRENT_DIR_10HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1198 | EA | CURRENT_DIR_11HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1199 | EA | CURRENT_DIR_1HR | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1200 | EA | CURRENT_DIR_1HR_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1201 | EA | CURRENT_DIR_1HR_BTREFLVL. | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1202 | EA | CURRENT_DIR_2HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1203 | EA | CURRENT_DIR_2HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1204 | EA | CURRENT_DIR_2HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1205 | EA | CURRENT_DIR_3HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1206 | EA | CURRENT_DIR_3HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1207 | EA | CURRENT_DIR_3HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1208 | EA | CURRENT_DIR_4HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1209 | EA | CURRENT_DIR_4HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1210 | EA | CURRENT_DIR_4HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1211 | EA | CURRENT_DIR_5HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1212 | EA | CURRENT_DIR_5HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1213 | EA | CURRENT_DIR_5HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1214 | EA | CURRENT_DIR_6HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1215 | EA | CURRENT_DIR_6HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1216 | EA | CURRENT_DIR_7HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1217 | EA | CURRENT_DIR_8HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1218 | EA | CURRENT_DIR_9HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1219 | EA | CURRENT_RATE_10HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1220 | EA | CURRENT_RATE_11HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1221 | EA | CURRENT_RATE_1HR | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1222 | EA | CURRENT_RATE_1HR_ATREFLVL. | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1223 | EA | CURRENT_RATE_1HR_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1224 | EA | CURRENT_RATE_2HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1225 | EA | CURRENT_RATE_2HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1226 | EA | CURRENT_RATE_2HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1227 | EA | CURRENT_RATE_3HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1228 | EA | CURRENT_RATE_3HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1229 | EA | CURRENT_RATE_3HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1230 | EA | CURRENT_RATE_4HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1231 | EA | CURRENT_RATE_4HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1232 | EA | CURRENT_RATE_4HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1233 | EA | CURRENT_RATE_5HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1234 | EA | CURRENT_RATE_5HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1235 | EA | CURRENT_RATE_5HRS_BTREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1236 | EA | CURRENT_RATE_6HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1237 | EA | CURRENT_RATE_6HRS_ATREFLVL | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1238 | EA | CURRENT_RATE_7HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1239 | EA | CURRENT_RATE_8HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1240 | EA | CURRENT_RATE_9HRS | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1241 | EA | DEPTH_BELOW_OCEAN_FLOOR | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1242 | EA | ELECTRONIC_DEPTH | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1243 | EA | END_DATE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1244 | EA | EXTERNAL_PAINT_SCHEME | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1245 | EA | FERRY_CROSSING_TIME | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1246 | EA | FIRST_BOUNDARY_SIDE_NAME | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1247 | EA | FISHING_FACILITY_TYPE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1248 | EA | FISHING_VESSEL_COUNT | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1249 | EA | FLOW_DIRECTION | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1250 | EA | HARBOUR_FACILITY_TYPE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1251 | EA | HEIGHT_ABOVE_SEA_BOTTOM | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1252 | EA | HIGH_WATER_CURRENT_DIR | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1253 | EA | HIGH_WATER_CURRENT_RATE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1254 | EA | HYDROGRAPHIC_BARRIER_TYPE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1255 | EA | HYDROGRAPHIC_LIGHT_TYPE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1256 | EA | ICE_COVERED | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1257 | EA | ICE_KEEL_COUNT | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1258 | EA | ICE_TYPE | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1259 | EA | IMO_ADOPTED | RT |  |  |  |  |  |  | ND |  |
| SEDRIS_T1260 | EA | INITIAL_LIGHT_SECTOR_ANGLE | RT |  |  |  |  |  |  | ND |  |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Several of the Eas currently marked as having reference type UD can be changed to ND. |

| Comment Tag | Dic | Current EA Label | Change Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1261 | EA | BY_PRODUCT | A |  | By-product or effect of production, mining, or agricultural activity. |  | ENUM |  | LAND_INDUSTRY | ND |  |
| SEDRIS_T1262 | EA | PRIMARY_PRODUCT | RD |  | Primary product resulting from a production, mining, or agricultural activity. |  |  |  |  | ND |  |
| SEDRIS_T1263 | EA | RAW_MATERIAL | A |  | Primary raw material involved in a production activity. |  | ENUM |  | LAND_INDUSTRY | ND |  |

Response:	Accepted.

|  | #### Where | #### Table 6.19 |
| --- | --- | --- |
|  | Problem | Action Item 40 from H3 ad hoc meeting, Ft. Knox. Investigate category of PRIMARY_PRODUCT; need proper definition. Examine enumerants and supply correct definitions. |
|  | Solution | Split EA_PRIMARY_PRODUCT into three Eas, two of which are new: RAW_MATERIAL, PRIMARY_PRODUCT, and BY_PRODUCT. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Change Type | Label | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1264 | EE | PRIMARY_PRODUCT | EARTHEN | D |  |  |  |  |
| SEDRIS_T1265 | EE | PRIMARY_PRODUCT | GREEN_ROCKS | D |  |  |  |  |
| SEDRIS_T1266 | EE | PRIMARY_PRODUCT | GROUND | D |  |  |  |  |
| SEDRIS_T1267 | EE | PRIMARY_PRODUCT | OOZE | D |  |  |  |  |
| SEDRIS_T1268 | EE | PRIMARY_PRODUCT | PART_METAL | D |  |  |  |  |
| SEDRIS_T1269 | EE | PRIMARY_PRODUCT | VOLCANIC | D |  |  |  |  |

Response:	Accepted, however the editors will review to determine if any concepts were inadvertently deleted.

|  | #### Where | #### Table 6.19 |
| --- | --- | --- |
|  | Problem | Action Item 40 from H3 ad hoc meeting, Ft. Knox. Investigate category of PRIMARY_PRODUCT; need proper definition. Examine enumerants and supply correct definitions. EA_PRIMARY_PRODUCT currently covers 3 concepts: raw materials, primary products, and by-products. Some enumerants do not fall into any of those 3 categories, however. |
|  | Solution | Delete the completely inappropriate enumerants from PRIMARY_PRODUCT, without transferring their concepts into other enumerants elsewhere. (A different block of comments addresses enumerants that are being transferred to other Eas, as PRIMARY_PRODUCT is being split up into 3 Eas: RAW_MATERIAL, BY_PRODUCT, and PRIMARY_PRODUCT.) See PRI_MATERIAL_TYPE and its cohorts for possible alternate enumerants to use. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Change Type | Label | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1270 | EE | PRIMARY_PRODUCT | ALUMINUM | D |  |  |  |  |
| SEDRIS_T1271 | EE | BY_PRODUCT | ALUMINUM | A |  | A highly ductile, malleable and conductive element, atomic number 13, that is resistant to corrosion and wear, and is widely used in alloys for beverage cans, household utensils, aircraft and automobile parts, electrical equipment, and many other products. | ND |  |
| SEDRIS_T1272 | EE | PRIMARY_PRODUCT | AMMUNITION | RD |  | Any object or material designed to be fired, exploded, or otherwise released by a weapon, especially bullets, shot, shells, and other such projectiles, e.g. ammunition. | ND |  |
| SEDRIS_T1273 | EE | PRIMARY_PRODUCT | ASH | D |  |  |  |  |
| SEDRIS_T1274 | EE | BY_PRODUCT | ASH | A |  | The solid residue left when combustible material is thoroughly burned or is oxidized by chemical means, e.g., ash. | ND |  |
| SEDRIS_T1275 | EE | PRIMARY_PRODUCT | ASPHALT | D |  |  |  |  |
| SEDRIS_T1276 | EE | RAW_MATERIAL | ASPHALT | A |  | A dark tarry, bituminous material found naturally or distilled from petroleum, mixed with crushed stone, gravel, or sand, e.g. asphalt. | ND |  |
| SEDRIS_T1277 | EE | PRIMARY_PRODUCT | AUTOMOBILES_AND_TRUCKS | RL | AUTOMOTIVE_VEHICLE |  |  |  |
| SEDRIS_T1278 | EE | BY_PRODUCT | CINDER | A |  | A partly burned and extinguished piece of combustible material or particles of such a material, e.g. a cinder. | ND |  |
| SEDRIS_T1279 | EE | PRIMARY_PRODUCT | CINDERS | D |  |  |  |  |
| SEDRIS_T1280 | EE | RAW_MATERIAL | CORAL_HEAD | A |  | The head, or top, of a growth of coral. | ND |  |
| SEDRIS_T1281 | EE | PRIMARY_PRODUCT | CORAL_HEAD | D |  |  |  |  |
| SEDRIS_T1282 | EE | PRIMARY_PRODUCT | DIATOM | D |  |  |  |  |
| SEDRIS_T1283 | EE | RAW_MATERIAL | DIATOMACEOUS EARTH | A | A soft, fine, porous siliceous sedimentary deposit, coloured yellow, light grey, or white, and composed chiefly of the microscopic skeletons of diatoms, e.g. diatomaceous earth. |  | ND |  |
| SEDRIS_T1284 | EE | PRIMARY_PRODUCT | LIQUEFIED_NATURAL_GAS | RD |  | Liquefied inflammable gas occurring underground, consisting chiefly of methane and other simple paraffins and often found associated with petroleum. | ND |  |
| SEDRIS_T1285 | EE | PRIMARY_PRODUCT | LIQUEFIED_PETROLEUM_GAS | RD |  | Liquefied gas produced from petroleum. | ND |  |
| SEDRIS_T1286 | EE | PRIMARY_PRODUCT | MATTES | D |  |  |  |  |
| SEDRIS_T1287 | EE | BY_PRODUCT | MATTE | A |  | An impure and unfinished metallic product of the smelting of various ores, especially those of copper; e.g., matte. | ND |  |
| SEDRIS_T1288 | EE | PRIMARY_PRODUCT | MILK | RD |  | The milk of a domesticated animal, such as a cow or goat. | ND |  |
| SEDRIS_T1289 | EE | PRIMARY_PRODUCT | MINERAL_OIL | RD |  | A general type of oil refined from petroleum, e.g. mineral oil. | ND |  |
| SEDRIS_T1290 | EE | PRIMARY_PRODUCT | MUD | D |  |  |  |  |
| SEDRIS_T1291 | EE | BY_PRODUCT | MUD | A |  | Any very wet, soft soil, e.g. mud. | ND |  |
| SEDRIS_T1292 | EE | PRIMARY_PRODUCT | NO_PRODUCT | RD |  | None. | ND |  |
| SEDRIS_T1293 | EE | PRIMARY_PRODUCT | NO_PRODUCT | RL | NONE |  |  |  |
| SEDRIS_T1294 | EE | PRIMARY_PRODUCT | PETROLEUM | RD |  | Unrefined mineral oil, petroleum. | ND |  |
| SEDRIS_T1295 | EE | PRIMARY_PRODUCT | PALM | RD |  | Palm trees and related products | ND |  |
| SEDRIS_T1296 | EE | PRIMARY_PRODUCT | PALMETTO | RD |  | Palmetto plants | ND |  |
| SEDRIS_T1297 | EE | PRIMARY_PRODUCT | PRESTRESSED_CONCRETE | RD |  | A composition of stone chippings, sand, gravel, pebbles, etc., formed into a mass with cement, reinforced by steel rods by steel rods or wires which have been tensioned while the concrete is setting to compress the concrete, thereby strengthening it. | ND |  |
| SEDRIS_T1298 | EE | PRIMARY_PRODUCT | RADIOACTIVE_MATERIAL | D |  |  |  |  |
| SEDRIS_T1299 | EE | BY_PRODUCT | RADIOACTIVE_MATERIAL | A |  | Any substance that spontaneously emits ionizing radiation, e.g. radioactive material. | ND |  |
| SEDRIS_T1300 | EE | RAW_MATERIAL | RADIOACTIVE_MATERIAL | A |  | Any substance that spontaneously emits ionizing radiation, e.g. radioactive material. | ND |  |
| SEDRIS_T1301 | EE | PRIMARY_PRODUCT | REFUSE | D |  |  |  |  |
| SEDRIS_T1302 | EE | BY_PRODUCT | REFUSE | A |  | Discarded material of any kind, e.g., Rubbish or refuse. | ND |  |
| SEDRIS_T1303 | EE | PRIMARY_PRODUCT | RUBBLE | D |  |  |  |  |
| SEDRIS_T1304 | EE | BY_PRODUCT | RUBBLE | A |  | Waste fragments of stone, e.g., rubble. | ND |  |
| SEDRIS_T1305 | EE | PRIMARY_PRODUCT | SAWDUST | D |  |  |  |  |
| SEDRIS_T1306 | EE | BY_PRODUCT | SAWDUST | A |  | Small particles of wood detached from a tree, plank, or other wood during the process of sawing, e.g. sawdust. | ND |  |
| SEDRIS_T1307 | EE | PRIMARY_PRODUCT | SAWDUST_WOOD_CHIPS | D |  |  |  |  |
| SEDRIS_T1308 | EE | BY_PRODUCT | SAWDUST_WOOD_CHIP | A |  | Small or moderate-sized chunks of wood detached from a tree, plank, or other wood during the process of sawing. May be further delineated specifically as EE SAWDUST and EE WOOD_CHIP. |  |  |
| SEDRIS_T1309 | EE | PRIMARY_PRODUCT | SCHIST | D |  |  |  |  |
| SEDRIS_T1310 | EE | RAW_MATERIAL | SCHIST | A |  | A strongly foliated, crystalline, coarse-grained metamorphic rock with lamellar mineral constituents that allow it to split easily into thin flakes or slabs, e.g. schist. | ND |  |
| SEDRIS_T1311 | EE | PRIMARY_PRODUCT | SCORIA | D |  |  |  |  |
| SEDRIS_T1312 | EE | RAW_MATERIAL | SCORIA | A |  | A cindery, vesicular crust formed on the surface of basaltic or andesitic lava as a result of the escape and expansion of gases before solidification, e.g. scoria. | ND |  |
| SEDRIS_T1313 | EE | PRIMARY_PRODUCT | SCRAP_METAL | D |  |  |  |  |
| SEDRIS_T1314 | EE | BY_PRODUCT | SCRAP_METAL | A |  | Waste fragments of metal, e.g. scrap metal. | ND |  |
| SEDRIS_T1315 | EE | PRIMARY_PRODUCT | SEWAGE | D |  |  |  |  |
| SEDRIS_T1316 | EE | BY_PRODUCT | SEWAGE | A |  | Any liquid-borne waste that contains animal or plant matter in suspension or solution, chemicals in solution, or soils and storm water, e.g. sewage. | ND |  |
| SEDRIS_T1317 | EE | PRIMARY_PRODUCT | SLAG | D |  |  |  |  |
| SEDRIS_T1318 | EE | BY_PRODUCT | SLAG | A |  | The vitreous mass left as a residue by the smelting of metallic ore, e.g. slag. | ND |  |
| SEDRIS_T1319 | EE | PRIMARY_PRODUCT | SLUDGE | D |  |  |  |  |
| SEDRIS_T1320 | EE | BY_PRODUCT | SLUDGE | A |  | A soft, soupy or muddy stream-bottom deposit, especially a black ooze formed on the bottom of a lake, e.g. sludge. | ND |  |
| SEDRIS_T1321 | EE | PRIMARY_PRODUCT | SNOW | D |  |  |  |  |
| SEDRIS_T1322 | EE | RAW_MATERIAL | SNOW | A |  | The partially frozen vapour of the atmosphere falling in flakes characterized by their whiteness and lightness. | ND |  |
| SEDRIS_T1323 | EE | PRIMARY_PRODUCT | SPICULES | D |  |  |  |  |
| SEDRIS_T1324 | EE | RAW_MATERIAL | SPICULE | A |  | One or other of the points of the basidia or sporophores in fungals. | ND |  |
| SEDRIS_T1325 | EE | PRIMARY_PRODUCT | SPONGE | D |  |  |  |  |
| SEDRIS_T1326 | EE | RAW_MATERIAL | SPONGE | A |  | The soft, porous, fibrous framework of certain animals or colonies of the group Porifera, especially of the genera Spongia and Hippospongia, from which the living matter has been removed, and which readily absorbs liquid and remains soft when wet, e.g. sponge. | ND |  |
| SEDRIS_T1327 | EE | RAW_MATERIAL | STEEL | A |  | Any of various alloys of the elements iron and carbon containing less than 2,5% carbon, usually also with lesser amounts of other elements, having substantial qualities of strength, hardness, and malleability. |  |  |
| SEDRIS_T1328 | EE | PRIMARY_PRODUCT | SUGAR | RD |  | A sweet, crystalline substance, white when pure, obtained from a great variety of plant juices, but chiefly from those of the sugar-cane and sugar-beet, and forming an important article of human food. | ND |  |
| SEDRIS_T1329 | EE | PRIMARY_PRODUCT | TAILINGS | D |  |  |  |  |
| SEDRIS_T1330 | EE | BY_PRODUCT | TAILINGS | A |  | Refuse material from processed ore, e.g. tailings. | ND |  |
| SEDRIS_T1331 | EE | PRIMARY_PRODUCT | TELECOMMUNICATIONS | RL | TELECOMMUNICATION |  |  |  |
| SEDRIS_T1332 | EE | PRIMARY_PRODUCT | TELECOMMUNICATIONS | RD |  | Support for communication over long distances, esp. by electrical means such as by telegraphy, telephony, or broadcasting, including the branch of technology concerned with this. | ND |  |
| SEDRIS_T1333 | EE | PRIMARY_PRODUCT | THATCH | RD |  | Material used in thatching; straw or similar material with which roofs are covered. | ND |  |
| SEDRIS_T1334 | EE | PRIMARY_PRODUCT | TOBACCO | RD |  | The leaves of the tobacco plant dried and variously prepared, forming a narcotic and sedative substance widely used for smoking, chewing, in the form of snuff, and to a slight extent in medicine. | ND |  |
| SEDRIS_T1335 | EE | PRIMARY_PRODUCT | WASTE | D |  |  |  |  |
| SEDRIS_T1336 | EE | BY_PRODUCT | WASTE | A |  | Anything not used; e.g. waste. | ND |  |
| SEDRIS_T1337 | EE | PRIMARY_PRODUCT | WINE | RD |  | Wine. | ND |  |
| SEDRIS_T1338 | EE | PRIMARY_PRODUCT | WOOD_CHIPS | D |  |  |  |  |
| SEDRIS_T1339 | EE | BY_PRODUCT | WOOD_CHIP | A |  | Moderate-sized chunk of wood detached from a tree, plank, or other wood during the process of sawing, e.g. wood chip. | ND |  |

Response:	Accepted, however the editors will review.

|  | #### Where | #### Table 6.19 |
| --- | --- | --- |
|  | Problem | Action Item 40 from H3 ad hoc meeting, Ft. Knox. Investigate category of PRIMARY_PRODUCT; need proper definition. Examine enumerants and supply correct definitions. EA_PRIMARY_PRODUCT currently covers 3 concepts: raw materials, primary products, and by-products. |
|  | Solution | Split EA_PRIMARY_PRODUCT into three Eas, two of which are new: RAW_MATERIAL, PRIMARY_PRODUCT, and BY_PRODUCT. This block of comments deals with enumerants that are being deleted from PRIMARY_PRODUCT and added to a new EA, BY_PRODUCT. The new enumerants’ definitions are being supplied/updated where noted, and sometimes they are being relabeled. NOTE: Corrected typo in definition of SEWAGE. Elsewhere, updated to provide ‘e.g.’ fragments in definitions where needed to obtain the desired label. |

| Comment Tag | Dic | Current EA Label | Current EE Label | Change Type | Label | Definition | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1340 | EE | RAW_MATERIAL | BAMBOO | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1341 | EE | RAW_MATERIAL | BASALT | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1342 | EE | RAW_MATERIAL | BAUXITE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1343 | EE | RAW_MATERIAL | BEDROCK | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1344 | EE | RAW_MATERIAL | BOULDERS | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1345 | EE | RAW_MATERIAL | CALCAREOUS | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1346 | EE | RAW_MATERIAL | CHALK | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1347 | EE | RAW_MATERIAL | CIRRIPEDIA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1348 | EE | RAW_MATERIAL | CLAY | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1349 | EE | RAW_MATERIAL | COAL | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1350 | EE | RAW_MATERIAL | COBBLE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1351 | EE | BY_PRODUCT | COKE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1352 | EE | RAW_MATERIAL | CONGLOMERATE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1353 | EE | RAW_MATERIAL | COPPER | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1354 | EE | RAW_MATERIAL | CORAL | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1355 | EE | RAW_MATERIAL | DOLOMITE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1356 | EE | BY_PRODUCT | ERODED_LANDS | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1357 | EE | RAW_MATERIAL | FLYSCH | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1358 | EE | RAW_MATERIAL | FORAMINIFERA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1359 | EE | RAW_MATERIAL | GLOBIGERINA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1360 | EE | RAW_MATERIAL | GRANITE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1361 | EE | RAW_MATERIAL | ICE | move from PRIMARY_PRODUCT |  | Frozen water. | ND |  |
| SEDRIS_T1362 | EE | RAW_MATERIAL | IRON | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1363 | EE | RAW_MATERIAL | LAVA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1364 | EE | RAW_MATERIAL | LEAD | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1365 | EE | RAW_MATERIAL | LIME | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1366 | EE | RAW_MATERIAL | LOESS | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1367 | EE | RAW_MATERIAL | MADREPORES | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1368 | EE | RAW_MATERIAL | MANGANESE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1369 | EE | RAW_MATERIAL | MARBLE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1370 | EE | RAW_MATERIAL | MARL | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1371 | EE | BY_PRODUCT | OIL_BLISTER | RD; move from PRIMARY_PRODUCT |  | An underground swelling containing oil. | ND |  |
| SEDRIS_T1372 | EE | RAW_MATERIAL | POLYZOA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1373 | EE | RAW_MATERIAL | PORPHYRY | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1374 | EE | RAW_MATERIAL | PTEROPODS | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1375 | EE | RAW_MATERIAL | PUMICE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1376 | EE | RAW_MATERIAL | QUARTZ | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1377 | EE | RAW_MATERIAL | RADIOLARIA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1378 | EE | RAW_MATERIAL | SEA_TANGLE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1379 | EE | RAW_MATERIAL | SEAWEED | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1380 | EE | RAW_MATERIAL | SHELL | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1381 | EE | RAW_MATERIAL | SILT | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1382 | EE | RAW_MATERIAL | SNOW | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1383 | EE | RAW_MATERIAL | SNOW_OR_ICE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1384 | EE | RAW_MATERIAL | STONE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1385 | EE | RAW_MATERIAL | TALC | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1386 | EE | RAW_MATERIAL | TRAVERTINE | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1387 | EE | RAW_MATERIAL | TUFA | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1388 | EE | RAW_MATERIAL | URANIUM | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1389 | EE | RAW_MATERIAL | VOLCANIC_ASH | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1390 | EE | RAW_MATERIAL | WOOD | move from PRIMARY_PRODUCT |  |  |  |  |
| SEDRIS_T1391 | EE | RAW_MATERIAL | ZINC | move from PRIMARY_PRODUCT |  |  |  |  |

Response:	Accepted, however the editors will review.

|  | #### Table 6.19 |
| --- | --- |
|  | Action Item 40. Investigate category of PRIMARY_PRODUCT; need proper definition. Examine enumerants and supply correct definitions. |
|  | Divided EA_PRIMARY_PRODUCT into three pieces: RAW_MATERIAL, PRIMARY_PRODUCT, and BY_PRODUCT. |

| Comment Tag | Dic | Current EA Label | Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1393 |  | ALONGSHORE_CURRENT_SPEED | RD |  | Speed of a current paralleling the shore largely within the surf zone. The current is caused by the excess water brought to the zone by the small net mass transport of wind waves. Alongshore currents feed into rip currents. |  |  |  |  | IR | BOWD |
| SEDRIS_T1394 |  | AVERAGE_WATER_DEPTH | RD |  | The mean vertical distance from a given water level to the bottom. The charted depth is the vertical distance from the tidal datum to the bottom. |  |  |  |  | IR | BOWD |
| SEDRIS_T1395 |  | AVERAGE_WATER_SPEED | RD |  | Mean rate of motion of a water parcel. |  |  |  |  | IR | BOWD |
| SEDRIS_T1396 |  | BEARING_AND_RECIPROCAL | RD |  | The horizontal direction of one terrestrial point from another, expressed as the angular distance from a reference direction. It is usually measured from 0000 to 3600. The reciprocal is one differing by 1800, or one measured in the opposite direction. |  |  |  |  | IR | BOWD |

Response:	Accepted in principle; the editors will review.

| SEDRIS_T1397 |  | HYDROGRAPHIC_DEPTH | RR |  |  |  |  |  |  | IR | BOWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1398 |  | HYDROGRAPHIC_DEPTH_INFO | RD |  | The accuracy of a hydrographic depth measurement. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1399 |  | HYDROGRAPHIC_DEPTH_INFO | RL | HYDROGRAPHIC_DEPTH_ACCURACY |  |  |  |  |  |  |  |

Response:	Rejected.

| SEDRIS_T1400 |  | HYDROGRAPHIC_DRYING_HEIGHT | RD |  | The height above the referenced vertical datum of those features which are periodically covered and exposed by the rise and fall of the tide. |  |  |  |  | IR | BOWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Definitions that are incorrect or have no references, but which can be supplied from Bowditch. |
|  | Solution | Supply the definitions from Bowditch (see BOWD comment against bibliography). Reference BOWD = Bowditch, American Practical Navigator, DMA/HTC, 1981 |

| Comment Tag | Dic | Current EA Label | Type | Label | Definition | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1401 | EA | BULK_WETNESS_CONDITION | A |  | Wetness condition of the primary material or object. |  | ENUMERATED |  |  | ND |  |

Response:	Accepted in principle; the editors will review the list of proposed enumerants for consistency.

| SEDRIS_T1402 | EA | DEPTH_OF_OCEAN_FLOOR | A |  | Depth of the ocean floor |  | REAL |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1403 | EA | ELECTRON_DENSITY | RD |  | The number of electrons per unit volume. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1404 | EA | EM_BAND | RD |  | The scientifically-accepted designation for a range of frequencies within the electromagnetic spectrum. |  |  |  |  | ND |  |
| SEDRIS_T1405 | EA | EM_PROPAGATION_CONDITION | RD |  | Variation in the level of propagated electromagnetic energy caused by interaction of the emitted energy with the vertical density gradient of the atmosphere. |  |  |  |  | ND |  |
| SEDRIS_T1406 | EA | EXTINCTION_COEFF_ALTITUDE | RD |  | The altitude above the planetary surface at which a given extinction coefficient value occurs. |  |  |  |  | ND |  |

Response:	Accepted in principle; the editors will review.

| SEDRIS_T1407 | EA | FERRY_TYPE | RD |  | The type of route, either guided or unguided, which a ferry follows. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1408 | EA | FERRY_TYPE | RL | FERRY_ROUTE_TYPE |  |  |  |  |  |  |  |

Response:	Accepted in principle; the editors will review IHO S-57 and revise as appropriate.

| SEDRIS_T1409 | EA | HARBOUR_IDENTIFIER | RD |  | The textual identifier of a harbour. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1410 | EA | HIGHEST_GROUND_WATER_LEVEL | RD |  | Highest level of ground water for a given period. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected, however the term ANNUAL should be added to the Label.

| SEDRIS_T1411 | EA | HYDROGRAPHIC_LIGHT_TYPE | RL | NAUTICAL_LIGHT_TYPE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1412 | EA | HYDROGRAPHIC_ORIGIN | RL | HYDROGRAPHIC_OBJECT_ORIGIN |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1413 | EA | HYDROGRAPHIC_ORIGIN | RT |  |  |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T1414 | EA | HYDROGRAPHIC_SURFACE | RD |  | The hydrographic depth of an object relative to a specified vertical datum. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1415 | EA | HYDROGRAPHIC_SURFACE | RL | HYDROGRAPHIC_RELATIVE_DEPTH |  |  |  |  |  |  |  |

Response:	Accepted.

| SEDRIS_T1416 | EA | HYDROGRAPHIC_TYPE | RD |  | The persistence, over time, of an inland hydrographic feature. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1417 | EA | HYDROGRAPHIC_TYPE | RL | HYDROGRAPHIC_PERMANENCE |  |  |  |  |  |  |  |

Response:	Accepted.

| SEDRIS_T1418 | EA | HYDROLOGIC_TYPE | RD |  | The persistence, over time, of a hydrologic feature. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1419 | EA | HYDROLOGIC_TYPE | RL | HYDROLOGIC_PERMANENCE |  |  |  |  |  |  |  |

Response:	Accepted.

| SEDRIS_T1420 | EA | HYPSOGRAPHIC_ELEVATION_DEPICTION | RD |  | Depiction of an elevation on a hypsographic map. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Rejected.

| SEDRIS_T1421 | EA | ICE_DRAFT_RMS | RD |  | The average depth of the surface ice cover within a nominal 111,120 kilometres (60 nautical miles) square cell of the ocean, calculated using the root-mean square method. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

| SEDRIS_T1422 | EA | ICE_FACTOR | RL | ICE_ACCRETION_VERTICAL_EFFECT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted in principle; the editors will review and revise as necessary.

| SEDRIS_T1423 | EA | SFC_COMPACTION_DESCRIPTION | A |  | Surface compaction of the surface of the primary material or object. |  | ENUMERATED |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1424 | EA | SFC_WETNESS_CONDITION | A |  | Wetness condition on the surface of the primary material or object. |  | ENUMERATED |  |  | ND |  |

Response:	Accepted in principle; the editors will review the list of proposed enumerates for consistency.

| SEDRIS_T1425 | EA | INLAND_WATER_OBSTRUCTION | RD |  | Indicates that the object located in an inland water body is an obstruction to vessel movement. |  |  |  |  | ND |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Response:	Accepted.

|  | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | Hydrographic/hydrologic Eas with labels that are less than optimal, and definitions that are confusing or incorrect, and which can be fixed with ND definitions. Also, JRM (one of the SEDRIS associates) has submitted Eas for describing surfaces and material properties. |

## Clause 7

| SEDRIS_T1051 | ##### Clause 7, Table 7.9, Tables 7.11-13 |
| --- | --- |
|  | Equivalence Class Membership should distinguish between the SI Standard Unit, compatible units and Deprecated units – for clarity |
|  | In the Equivalence Class Membership column bold the standard SI Unit label for the EQ, and italicize (alternatively, strike through) the Deprecated Unit labels. For example, for TIME: DAY, HOUR, MINUTE, SECOND, YEAR (alternatively: DAY, HOUR, MINUTE, SECOND, YEAR) Add explanation to the text in table 7.9 – Field: “Equivalence class membership” Contents: The list of EULs that are members of the EQ, in which the EUL of the SI standard unit for the EQ is bolded, and the EULs of deprecated units (if any) are italicized. |

Response:	Accepted, with added explanatory text to be added.

| Comment Tag | Dic | Current EU Label | Chg Type | Label | Symbol | EQ Label | # Definition | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1052 | ###### EU | AREIC_POWER_LEVEL | ###### RL | ###### AREIC_POWER_RATIO |  |  |  |  |
| SEDRIS_T1053 | ###### EU | AREIC_POWER_LEVEL | ###### RD |  |  |  | ###### areic power ratio (level); Power ratio divided by area. |  |

Response:	AI 08-34: Paul Berner will investigate.

|  | #### Table 7.4, Table 7.11 |
| --- | --- |
|  | ##### A reference value is not given, so it is a ratio. |

## Clause 8

| SEDRIS_T1054 | ##### 8.3 EDCS Group Dictionary |
| --- | --- |
|  | Relation between Tables 8.5 and annex F needs clarification. |
|  | Add to end of first paragraph: The first four fields are found in Table 8.5. The Label fields together with last tow fields are found in Annex F. |

Response:	Accepted.

| SEDRIS_T1055 | ##### Table 8.9 ECs in the ACOUSTIC group |
| --- | --- |
|  | Wrong group name |
|  | Change to: Table 8.9 ECs in the ACOUSTIC PHENOMENON group |

Response:	Accepted.

| SEDRIS_T1056 | ##### Table 8.9 ECs in the ACOUSTIC group |
| --- | --- |
|  | Missing ACOUSTIC_DEVICE |
|  | Add ACOUSTIC_DEVICE to Table 8.9 and Add ACOUSTIC_PHENOMENON to label ACOUSTIC_DEVICE in table C.2 |

Response:	Accepted.

| Comment Tag | Dic | Current EG Label | Chg Type | Label | # Definition | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1057 | ## EG | HYDROGRAPHIC_TRNSP | RD |  | Of, or related to, the actions or processes of transporting something; conveyance of people and goods, from one place to another and involving ocean, seas, lakes, rivers, or other inland or marine bodies of water |  |  |

Response:	Rejected.

|  | #### Table 8.5 – EDCS Group Dictionary Entries |
| --- | --- |
|  | Need to explicitly add “ocean” & “inland or marine” to definition as “Hydrographic” has an inland connotation. Note that, for example, BEACON is in this group. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1058 | EC | WATERING_HOLE | AG |  |  |  | ANIMAL |  |  |

Response:	Accepted.

|  | Where | Table 5.20 and clause C, table C.20 |
| --- | --- | --- |
|  | Problem | ECs that ought to belong to the ANIMAL group but don’t. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1059 | EC | BRUSH_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1060 | EC | CROP_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1061 | EC | GRASS_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1062 | EC | LAND_REGION | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1063 | EC | MIXED_VEGETATION_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1064 | EC | SCRUB_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1065 | EC | TREED_TRACT | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1066 | EC | UNDERMINED_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1067 | EC | UNVEGETATED_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |
| SEDRIS_T1068 | EC | WOODY_GRASS_LAND | AG |  |  |  | PHYSIOGRAPHY |  |  |

Response:	Accepted.

|  | Where | Table 5.26 and clause C, table C.26 |
| --- | --- | --- |
|  | Problem | ECs that ought to belong to the PHYSIOGRAPHY group but don’t., and missing related concepts with ECs. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1069 | EC | AUTOMOTIVE_VEHICLE_STATION | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1070 | EC | DRAGON_TEETH | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1071 | EC | ENGINEER_BERM | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1072 | EC | ENGINEER_DITCH_OR_BERM | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1073 | EC | FENDER_CUSHION | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1074 | EC | VEHICLE_BARRIER | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1075 | EC | VEHICLE_LOT | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1076 | EC | VEHICLE_WEIGH_SCALE | AG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1077 | EC | WIRE_OBSTACLE | AG |  |  |  | VEHICLE |  |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Clause 8 |
| --- | --- | --- |
|  | Problem | ECs that have something to do with vehicles (e.g. exist solely to obstruct vehicles, places where vehicles are stored) but aren’t in the VEHICLE group. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1078 | EC | PONTOON | DG |  |  |  | VEHICLE |  |  |
| SEDRIS_T1079 | EC | PONTOON | AG |  |  |  | LAND_TRANSPORTATION |  |  |

Response:	Accepted.

|  | Where | Clause 8 |
| --- | --- | --- |
|  | Problem | PONTOON’s definition does not make clear why it should be in the VEHICLE group when BRIDGE is not, for example. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1080 | EC | BREAKLINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1081 | EC | CHANGE_LINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1082 | EC | CHANNEL_EDGE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1083 | EC | CLIFF_BOTTOMLINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1084 | EC | CLIFF_TOPLINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1085 | EC | COMPLEX_OUTLINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1086 | EC | DATA_QUALITY_BOUNDARY | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1087 | EC | LIGHT_LINE | AG |  |  |  | DEMARCATION |  |  |
| SEDRIS_T1088 | EC | SAND_LINE | AG |  |  |  | DEMARCATION |  |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Clause 5 |
| --- | --- | --- |
|  | Problem | ECs that should be in the DEMARCATION group, but aren’t. |
|  | Solution | Add 2 way ‘related concepts’ relationships between EC_BOUNDARY and these ECs, if SEDRIS G000 was accepted. These are all instances of |

Response:	“Related concepts’ portion withdrawn.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1089 | EA | AIRSPACE_ALTITUDE_LIMITS | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1090 | EA | AIRSPACE_USE_BOUNDARY | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1091 | EA | ATM_BOUNDARY_LAYER_ALTITUDE | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1092 | EA | TROPOPAUSE_ALTITUDE | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1093 | EA | DELINEATION_CERTAIN | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1094 | EA | DELINEATION_CERTAIN | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1095 | EA | LOWEST_AIRSPACE_ALT_LIMIT | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1096 | EA | HIGHEST_AIRSPACE_ALT_LIMIT | AG |  |  |  |  |  | DEMARCATION |  |  |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Table 8.69 Eas in the DEMARCATION group |
| --- | --- | --- |
|  | Problem | Eas that ought to be in the DEMARCATION group but aren’t. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1097 | EC | CADASTRAL_CONSTRUCTION | AG |  |  |  | SURVEY |  |  |
| SEDRIS_T1098 | EC | CHANGE_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1099 | EC | FIDUCIAL_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1100 | EC | GEODETIC_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1101 | EC | INLAND_WATER_ELEVATION | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1102 | EC | NAMED_LOCATION | AG |  |  |  | POSITION |  |  |

Response:	Accepted.

| SEDRIS_T1103 | EC | TEXT_DESCRIPTION | AG |  |  |  | POSITION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1104 | EC | TEXT_DESCRIPTION | RL | DESCRIBED_POINT |  |  |  |  |  |

Response:	AI 08-14: To be included in the resolution of the “map analysis” AI.

| SEDRIS_T1105 | EC | TIDE_DATA_POINT | AG |  |  |  | POSITION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1106 | EC | TIDE_DATA_POINT | RG |  |  |  | SURVEY |  |  |
| SEDRIS_T1107 | EC | TERRAIN | RG |  |  |  | ABSTRACTION |  |  |

Response:	Accepted.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1108 | EA | ASSOCIATED_TEXT | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1109 | EA | ASSOCIATED_TEXT | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1110 | EA | CLASSIFICATION_NAME | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1111 | EA | CLASSIFICATION_NAME | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1112 | EA | MAINTAINED | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1113 | EA | MAINTAINED | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1114 | EA | NAME | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1115 | EA | NAME | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1116 | EA | OBJECT_LABEL | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1117 | EA | OBJECT_LABEL | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1118 | EA | PICTORIAL_REPRESENTATION | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1119 | EA | PICTORIAL_REPRESENTATION | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1120 | EA | PRI_DISPLAY_MODE | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1121 | EA | PRI_DISPLAY_MODE | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1122 | EA | QUALCHAR_RELIABILILITY | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1123 | EA | QUALCHAR_RELIABILILITY | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1124 | EA | RELEASABILITY_RSTRCTS | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1125 | EA | RELEASABILITY_RSTRCTS | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |

Response:	Accepted in principle; the editors will review.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1126 | EA | AIRSPACE_ALTITUDE_LIMITS | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1127 | EA | AIRSPACE_USE_BOUNDARY | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1128 | EA | ATM_BOUNDARY_LAYER_ALTITUDE | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1129 | EA | TROPOPAUSE_ALTITUDE | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1130 | EA | DELINEATION_CERTAIN | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1131 | EA | DELINEATION_CERTAIN | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1132 | EA | LOWEST_AIRSPACE_ALT_LIMIT | AG |  |  |  |  |  | DEMARCATION |  |  |  |
| SEDRIS_T1133 | EA | HIGHEST_AIRSPACE_ALT_LIMIT | AG |  |  |  |  |  | DEMARCATION |  |  |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Table 8.69 Eas in the DEMARCATION group |
| --- | --- | --- |
|  | Problem | Eas that ought to be in the DEMARCATION group but aren’t. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Def | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1134 | EC | CADASTRAL_CONSTRUCTION | AG |  |  |  | SURVEY |  |  |
| SEDRIS_T1135 | EC | CHANGE_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1136 | EC | FIDUCIAL_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1137 | EC | GEODETIC_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1138 | EC | INLAND_WATER_ELEVATION | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1139 | EC | NAMED_LOCATION | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1140 | EC | TEXT_DESCRIPTION | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1141 | EC | TEXT_DESCRIPTION | RL | DESCRIBED_POINT |  |  |  |  |  |
| SEDRIS_T1142 | EC | TIDE_DATA_POINT | AG |  |  |  | POSITION |  |  |
| SEDRIS_T1143 | EC | TIDE_DATA_POINT | RG |  |  |  | SURVEY |  |  |
| SEDRIS_T1144 | EC | TERRAIN | RG |  |  |  | ABSTRACTION |  |  |

Response:	Accepted in principle; the editors will review.

| Comment Tag | Dic | Current EA Label | Chg Type | Label | Def | Related Concepts | Value Type | EQ Label | Grps | Ref Type | Ref | Related Concepts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1145 | EA | ASSOCIATED_TEXT | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1146 | EA | ASSOCIATED_TEXT | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1147 | EA | CLASSIFICATION_NAME | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1148 | EA | CLASSIFICATION_NAME | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1149 | EA | MAINTAINED | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1150 | EA | MAINTAINED | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1151 | EA | NAME | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1152 | EA | NAME | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1153 | EA | OBJECT_LABEL | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1154 | EA | OBJECT_LABEL | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1155 | EA | PICTORIAL_REPRESENTATION | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1156 | EA | PICTORIAL_REPRESENTATION | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1157 | EA | PRI_DISPLAY_MODE | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1158 | EA | PRI_DISPLAY_MODE | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1159 | EA | QUALCHAR_RELIABILILITY | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1160 | EA | QUALCHAR_RELIABILILITY | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |
| SEDRIS_T1161 | EA | RELEASABILITY_RSTRCTS | DG |  |  |  |  |  | SURVEY |  |  |  |
| SEDRIS_T1162 | EA | RELEASABILITY_RSTRCTS | AG |  |  |  |  |  | ABSTRACT_CONCEPT |  |  |  |

Response:	Accepted in principle; the editors will review.

|  | Where | Table 8.69 Eas in the SURVEY group |
| --- | --- | --- |
|  | Problem | Eas that ought to be in the SURVEY group but aren’t, or are there but belong elsewhere. |

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Defn | Related Concepts | Grps | Ref Type | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_T1163 | EC | BOATING_CLUB | AG |  |  |  | RECREATION |  |  |
| SEDRIS_T1164 | EC | CAMP | AG |  |  |  | RECREATION |  |  |
| SEDRIS_T1165 | EC | MOBILE_HOME | AG |  |  |  | RECREATION |  |  |
| SEDRIS_T1166 | EC | PUBLIC_INN_OR_RESTAURANT | AG |  |  |  | RECREATION |  |  |
| SEDRIS_T1167 | EC | PARTICLE_ACCELERATOR | AG |  |  |  | ANCILLARY_ARTEFACT |  |  |
| SEDRIS_T1168 | EC | BEACON | RG |  |  |  | ACOUSTIC_PHENOMENON |  |  |

Response:	Accepted in principle; the editors will review.

## Annex H

| SEDRIS_T1169 | ##### Table 5.x |
| --- | --- |
|  | Undefined acronyms |
|  | Undefined Acronym; Add acronym to Table H.2; Definition. CANAL_CN; CN; Centreline or Nexus. IFR_REGION; IFR; Instrument Flight Rules. ILS_TERMINAL_SGMT; ILS; Instrument Landing System. INS_ALIGNMENT_PAD; INS; Inertial Navigation System. MAIN_TELECOM_EXCHANGE; TELECOM; TeleCommunications. MARITIME_NAVAID; NAVAID; Navigation Aid. R_AND_D_BUILDING; R_AND_D; Research and Development. |

Response:	Accepted in principle; the editors will investigate.

| SEDRIS_T1170 | ##### Table 6.x |
| --- | --- |
|  | Undefined acronyms |
|  | Undefined Acronym; Add acronym to Table H.3; Definition. DIR_SNOISE_SPECPL_DANES; DANES; Directional Ambient noise Estimation System. OMNIDIR_SNOISE_SPECPL_TAPPS; TAPPS; Towed Array Performance Prediction System. OMNIDIR_SNOISE_SPECPL_FANM; FANM; Fast Ambient Noise Model. RGB_COLOUR_BLUE; RGB; Red/green/blue. RUNWAY_REIL_INTENSITY; REIL; Runway End Identifier Lights. SOLAR_WIND_SPEED_GSE_X; GSE; Geocentric Solar Ecliptic. TSS_IMO_ADOPTED; TSS; Traffic Separation Scheme. UTM_GRID_EASTING; UTM; Universal Trans-Mercator. |

Response:	Accepted.

| Air Traffic Services | ATS |
| --- | --- |
| Amplitude | AMPL |
| Angle | ANG |
| Component | COMP |
| Cosine | COS |
| General | GEN |
| Hemisphere | HEMI |
| Incident(ce) | INC |
| Opposite | OPP |
| Parallel | PARAL |
| Parameter | PARM |
| Perpendicular | PERP |
| Reflectance | REFL |
| Route Navigation Plan | RNP |
| Scattering | SCAT |
| Shadowing and obscuration | SHOBS |
| Surface | SFC |
| Transient | TRANS |
| Volume | VOL |

| SEDRIS_T1171 | ##### Table H.3 |
| --- | --- |
|  | Needed acronyms for Eas are missing. |
|  | Add the missing acronyms. |

Response:	Accepted.

## Bibliography

| SEDRIS_T1172 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | We currently have a number of undefined environmental science terms. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = FFENVSCI Reference = Wyman, B. and L. H. Stevenson. The Facts on File Dictionary of Environmental Science. 2001. New edition. ISBN 0-8160-4234-9. |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1173 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | We currently have a number of undefined environmental science terms. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = DESCI Reference = Allaby, A. and M. Allaby. A Dictionary of Earth Sciences. 2nd edition. Oxford: Oxford University Press, 1999. ISBN 0-19-280079-5. |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1174 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | We do not currently use a reference for RELIGIOUS DENOMINATION. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = LEVIN Reference = Levinson, D. Religion: A CrossCultural Encyclopedia. New York: Oxford University Press, 1996. ISBN 0-19-512311-5. |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1175 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The reference used to supply new definitions for the enumerants of SNOW_MEASUREMENT_CONDITION is not in the bibliography. |
|  | Solution | Add the following entry to the bibliography (check with Louis Hembree about more specific sections of the reference for this one) IDENTIFIER = WMOCODE Reference = World Meteorological Organization Manual on Codes, Volume 1, International Codes (WMO-No. 306) |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1176 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The reference used to supply new definitions for the enumerants of ATM_PRESSURE_CHANGE_TND, BELOW_STATION_CLD_CVRG, BELOW_STATION_CLD_TOP_TYP is not in the bibliography. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = WMOCODE1 Reference = World Meteorological Organization Manual on Codes, Volume 1, International Codes (WMO-No. 306) Part B, descriptor 0-20-011 |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1177 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The reference used to supply new definitions for the enumerants of ATM_PRESSURE_CHANGE_TND is not in the bibliography. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = WMOCODE210 Reference = World Meteorological Organization Manual on Codes, Volume 2, International Codes (WMO-No. 306) Code Table 0-10-63 |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1178 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The reference used to supply new definitions for the enumerants of AIRSPACE_ROUTE_TYPE is not in the bibliography. |
|  | Solution | Add the following entry to the bibliography. IDENTIFIER = IAN11 Reference = Convention on International Civil Aviation, Annex 11, Appendix 1 |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1179 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The following Ees come from references not currently in the bibliography, if the changes proposed in these comments are accepted: VISIBILITY_DIST_QUALITY’s Ees, WIND_DIRECTION_QUALITY’s Ees, WIND_SPEED_QUALITY’s Ees |
|  | Solution | Add the following reference to the bibliography Identifier = [NCDCM] Reference = NATIONAL CLIMATIC DATA CENTER, DOCUMENTATIONMANUAL, HOURLY SURFACE AIRWAYS OBSERVATIONS (TD-3280) |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1180 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The proposed the following proposed Eas come from references not currently in the bibliography: BRDF, OPPOSITION_EFFECT_AMPL, OPP_EFFECT_ANG_WIDTH, PHASE_ANGLE_HAPKE , SFC_RFLCT_DIR_HEMI, SFC_REFL_AMBIENT_COMP, VOL_PHASE_FUNCTION, VOL_SINGLE_SCAT_ALBEDO, |
|  | Solution | Add the following reference to the bibliography Identifier = [HAPKE] Reference = Theory of Reflectance and Emittance Spectroscopy (Hapke, B., 1993, Cambridge Press: York) |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1181 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The following Eas come from references not currently in the bibliography, if the changes proposed in these comments are accepted: BREAKER_PRIMARY_ANGLE_DIR |
|  | Solution | Add the following reference to the bibliography Identifier = [NAMRTM] Reference = U.S. Navy Aerographer’s Mate 1&C Rate Training Manual, NAVEDTRA, revised 1985. |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1182 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The following proposed Eas come from references not currently in the bibliography: GLINT_ANGLE, INCIDENCE_ANGLE, SFC_REFLECTIVITY_DIFFUSE, SFC_REFLECTIVITY_GEN |
|  | Solution | Add the following reference to the bibliography Identifier = [WATT] Reference = Advanced Animation and Rendering Techniques (Watt, A. and Watt, M., 1992, ACM Press: Essex, England) |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1183 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The proposed new definitions for the following enumerated Eas come from references not currently in the bibliography: VISIBILITY_REPORT_TYPE’s enumerants |
|  | Solution | Add the following reference to the bibliography Identifier = [FMH1_6] Reference = Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 6.0 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1184 | Where | Bibliography. |
| --- | --- | --- |
|  | Problem | The EA_WEATHER_TYPE enumeration’s new definition is being supplied from a WMO reference not currently in the bibliography. |
|  | Solution | Add the following entry to the bibliography. Identifier = FMH1_8 Reference = Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 8 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1185 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The proposed new definitions for the following enumerated Eas come from references not currently in the bibliography: PRECIPITATION_INTENSITY’s enumerants |
|  | Solution | Add the following reference to the bibliography Identifier = [FMH1_841] Reference = Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 8.4.1 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1186 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The proposed new definitions for the following enumerated Eas come from references not currently in the bibliography: BELOW_STATION_CLOUD_COVERAGE, TOTAL_CLOUD_COVER_CODED’s enumerants |
|  | Solution | Add the following reference to the bibliography Identifier = [FMH1_952] Reference = Federal Meteorological Handbook No. 1 Surface Weather Observations and Reports, FCM-H1-1995, Section 9.5.2 Office of the Federal Coordinator for Meteorological Services and Supporting Research US Department of Commerce |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1187 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | The proposed new definitions for the following enumerated Eas come from references not currently in the bibliography: HIGH_CLOUD_BASE_ALTITUDE |
|  | Solution | Add the following reference to the bibliography Identifier = [CATLAS] Reference = International Cloud Atlas, World Meteorological Organization Publication # 407 World Meteorological Organization Identifier = [GMET] (see US Comments for full bibliographic information) Glossary of Meteorology, American Meteorological Society Identifier = [MINSTR] Reference = Guide to Meteorological Instruments and Methods of Observations, Section 11 WMO Publication No. 8 World Meteorological Organization |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1188 | Where | Bibliography |
| --- | --- | --- |
|  | Problem | We do not currently use standard references for mineralogy. |
|  | Solution | Add the following entries to the Bibliography. The reference has been in print in various incarnations since it was written in 1848. Identifier = DANAMM Reference = Dana, J. D., C. Klein and C. S. Hurlbut, jr.Manual of Mineralogy. 20th edition. New York: John Wiley & Sons. 1985. ISBN 0-471-80580-7. Identifier = DANASM This was the first comprehensive book on mineralogy in English, written in 1837; through subsequent revisions, it has remained a standard reference work. Reference = Dana, J.D. Dana’s System of Mineralogy. 7th edition. Volume 1, 1944; volume 2, 1951; volume 3, 1962. New York: John Wiley & Sons, Rewritten by C. Palache, H. Berman, and C. Frondel. |

Response:	Accepted in principle; the editors will revise and use as appropriate.

| SEDRIS_T1392 | Where | Clause 6 |
| --- | --- | --- |
|  | Problem | No bibliography entry exists for the reference used for the new definitions, enumerants of the following: OCEAN_BASIN, SEA_BASIN |
|  | Solution | Add the following to the bibliography: Identifier = BOWD Reference = Bowditch, American Practical Navigator, DMA/HTC, 1981 |

Response:	Accepted in principle; the editors will revise and use as appropriate.

# Editorial

## Foreword

## Introduction

| SEDRIS_E001 | ##### Introduction, First paragraph |
| --- | --- |
|  | Needs better wording |
|  | Replace with: The EDCS supports the encoding and communication of qualitative and quantitative information associated with physical environments, both real world and artificial. The EDCS is a set of dictionaries of encodings of environmental objects, phenomena, concepts, and associated data constructs, and also includes: - rules for expanding dictionaries through registration, - conventions for applying the encodings in information processing applications, and - organizational schemes to aid the access of dictionary content. EDCS Dictionaries for the following categories of information are defines in this International Standard: |

Response:		Accepted in principle (e.g., merge the first two sentences into a single paragraph, change the use of EDCS to “this IS” in the first two uses); the editors will merge the proposal with the original wording and provide a proposed revision to the SEDRIS Organization.

| SEDRIS_E002 | ##### Introduction, j. |
| --- | --- |
|  | The single data type enumerant is not representative of the many data types. |
|  | Change (enumerants, units) to (data types, units) |

Response:	Accepted.

| SEDRIS_E003 | ##### 4.2.1 Definitions |
| --- | --- |
|  | The concept of an EDCS definition may be confused with the concept of a lexicon definition |
|  | Change first sentence: The definition field of an EDCS Dictionary Entry is a precise statement of the nature, properties, scope or essential qualities of the concept embodied in the entry, and may include the name or names for the concept as part of the definition. |

Response:	Accepted in principle; the editors will determine how best to address this issue.

| SEDRIS_E004 | ##### 4.7 EDCS unit scales, first paragraph, last sentence |
| --- | --- |
|  | Confusing |
|  | Change to: When the multiplicative constant (scale factor) in that linear equation is one from a list of a certain powers of ten or powers of 210 (and the additive constant is zero), this International Standard specifically treats them as EDCS Unit Scales (ESs). |

Response:	Accepted.

| #### SEDRIS_E005 | ##### Annex G Change and deprecation |
| --- | --- |
|  | The use of the word magnitude could be confusing in this context |
|  | Change all occurrences of “magnitude” with “extent”. |

Response:		Rather than rename, the editors will remove the use of such language throughout the clause, as it is unnecessary.

| Comment Tag | Dic | Current EC Label | Chg Type | Label | Definition | Related Concepts | Grps | Ref Type | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEDRIS_E006 | EC | HUMMOCK |  |  | A parcel of land of higher elevation than that surrounding it, within a swamp, bog, or marsh. |  |  |  |  |

|  | #### The definition has a double that. |
| --- | --- |
|  | Replace with “than that” |

Response:	Accepted.

| SEDRIS_E007 | ##### Annex C, D, E, G, H, I |
| --- | --- |
|  | HTML Title blocks are incorrect. |
|  | Change to match titles in index |

Response:	Accepted.

| SEDRIS_E008 | ##### Annex H.2 Overview |
| --- | --- |
|  | Sentence is incomplete. |
|  | Change to: For each EDCS Dictionary (EC through EG), the corresponding Table H.2 through H.9 lists words and their abbreviations, as well as phrases and their acronyms, to be used for constructing labels. |

Response:	Accepted.

| SEDRIS_E009 | ##### Annex I, Title block |
| --- | --- |
|  | Title says Annex H |
|  | Change title from Annex H to Annex I |

Response:	Accepted.